20/07/2022

Scott Copperthwaite

2026635

“Water – The main culprit behind many OME problems”

Course: OME level 4 – Principles of Design Engineering

2198

Word Count (Excluding title page, abstract, contents, references, Figures, and any words within graphs):

# Abstract

A report focused on the detrimental effects that water (H2O) could have on Ordnance, Munitions and Explosives. A broad focus on not just explosives but on also the munitions encased within explosives, such as bullets and the mechanism that fire such materials. Background information is integrated and investigated into the physical properties of water. These include the atomic structure of water, atmospheric conditions and relative humidity, and the potential for electro-static discharge of charged particles within an atmosphere, notably where water is present around explosive charges. Furthered by a chemical view of water involving its polarity due to its bonding structure and its amphoteric nature. Amphoteric nature is inclusive of understanding the  acid/base reactions  of water as a solvent as well as subsequent redox reactions that can create corrosion. Mitigation techniques, tools and protection have been prescribed in order to help lessen the adverse effects for OME materials. Leading to conclusions drawn from the entire report.

Contents

Abstract	1

1.	Introduction	2

2.	Physical Properties of Water and effects on OME	2

2.1.	Structural States	2

2.2.	Relative Humidity	3

2.3.	Electro- Static Discharge (ESD)	4

3.	Chemical properties of Water and effects on OME	5

3.1.	Polarity	5

3.2.	Amphoteric Nature	5

3.3.	Redox Reactions	6

4.	Mitigation Techniques	7

5.	Conclusion	7

6.	References	8

7.	Bibliography	10

# Introduction

When viewed from Space, water in both liquid and frozen form can be seen. In fact, water covers around 75% on the Earth Surface, the abundance of which formed the basis for life. For all its benefits, water is anathema to the ordnance, munitions, and explosives community. The chemical and physical properties of water in all its states, including that within the atmosphere have created many issues in the OME field. The ramifications can be experienced at the top level in ordnance, which is the mechanism used to fire the munition projectile, all the way through from sub-system munition that cases the explosive charge and down to the component level that is the explosive charge within the munition itself.

An explosive is  a substance or device that can be manufactured to produce a volume of rapidly expanding gas. They are highly energetic compounds that generate high pressure gases when a physical or chemical stimuli is added. Such release of gases includes NO, H2O, CO, and CO2 [1] . Explosives can be defined to several major groups such as low, or deflagrating explosives, primary and secondary high explosives as well as blasting agents [2]. Low or deflagrating explosives tend to be composed of an oxidant that decomposes rapidly but at a slow burn rate and their major usage is in propellants. High explosives detonate faster than the speed of sound and don’t deflagrate to create a rapid decomposition of high pressure and tend to be used in the creation of Military munitions.

# Physical Properties of Water and effects on OME

## Structural States

Water is the only known substance to exist in all three known states of matter: liquid, solid and gas. In liquid state water has Hydrogen bonded by two Oxygen atoms in an almost perfect tetrahedron structure. In a solid state, water has a crystalline hexagon structure with a lower density than water due to the voids left within the hexagon structure, Figure 2.1.1. When there is enough energy to break the hydrogen bonds, water becomes its third state, water vapour, or gaseous. Water vapour is directly attributed relative humidity. The boiling point and freezing points of water are 100 degrees and 0 degrees Celsius respectively. Water is also known as the universal solvent due to its polar nature.

Figure 2.1.1 – Structure of water in liquid and solid state [3]

A dielectric constant is a measure of how easy/difficult a material is polarised with an electric field. The dialectic constant of water is around 80.4 at 20 degrees [4]. The high nature of this enables an important role in water acting as solvent [5]. Water, also, in a dense and sufficiently hot enough environment can play the part of a catalyst in which, it speeds a chemical reaction without being consumed within explosive reactions [6]

## Relative Humidity

Humidity is the direct measurement of water vapour, within the air. For explosives, relative humidity measurement (RH) is integral to preservation of explosive charges. RH measures the amount of water in the air in relation to the maximum amount of water vapour that could be present. There is a direct correlation between the higher the temperature and increased water vapour present in the air. RH influences the rates of chemical reactions as well as the physical changing within an object, such as size or structural strength. Both ends of the spectrum can dramatically alter, detrimentally, the object exposed to the changes [7].

RH is considered a “Saturation” issue, as higher levels can infiltrate and become absorbed into an explosive mixing process. A fully completed High explosive charge, once packed, and sealed that is saturated with moisture can be extremely dangerous. The locked moisture can create microscopic cracks within the explosive material and create structural weaknesses [8]. The moisture can react with the carbon within explosives to create bubbles of carbon monoxide, known as Outgassing. This creates an undesirable effect in the completed explosive munition. High Humidity of water vapour also creates an oxidation reaction on the structural surface on the ordnance. Which will eventually lead to more weaknesses within the overall firing structure of the munition known as corrosion [9].

Figure 2.2.1 – Corrosion rate increase on Steel [10]

## Electro- Static Discharge (ESD)

If materials are placed in contact with one another and then separated, friction can lead to negatively charged electrons on one material being transferred to the other. The material that has lost an electron becomes positively charged whereas the other becomes negatively charged. This is known as the triboelectric effect and is the creation of a discharge of static electricity. Upon a material having created a charge, it is now defined as an electrostatic charged material and the charge on it can be transferred to another material creating an ESD event. Materials with no static charge have an equal number of neutrons, protons, and electron, which is the case for water. Due to water having all its electrons bound within H2O or OH- within the air, it enables the air itself to be a good conductor, enabling it to dissipate any built-up charge into smaller unnoticeable burst. The event still happens but is spread out and not focused into one large discharge. Should an event occur, the discharge can break the carbon-carbon and oxygen-oxygen bonds to form new bonds between carbon-oxygen. The new bonds contain less energy than the previous and thus the left-over energy is released in form of heat, in turn, heat of combustion/initiation of the charge.

The triboelectric effect can depend upon a variety of factors including separation speed, area of the contact between the two materials, temperature of the environment, chemistry of the materials, RH and importantly particle size of explosive. Powdered processes are more susceptible to ESD events than that of completed, compacted, shaped charges due to surface area. Generally, it is a safe approach to treat all explosives, in all conditions as vulnerable to ESD occurrences [11].

# Chemical properties of Water and effects on OME

## Polarity

Water has unusual chemical properties due to the polarity of its molecules, specifically the hydrogen bonds . Water consists of two Hydrogen atoms covalently bonded to an Oxygen atom. The shared pair of electrons are attracted towards the oxygen atoms greater than the hydrogen atoms. This attraction creates paired opposite charges (Dipoles) [5]. The partial positive charge is attributed to the hydrogen atom whereas the partial negative charge to the Oxygen atom. Each molecule of water has the potential to bond itself further to a maximum of 4 Hydrogen bods with other water molecules. With all bonds, energy is a necessity to break and form bonds and subsequently energy is released upon these interactions [12]

H2O has a structure that is considered “bent” due to two pairs of unshared Oxygen electrons that are of a repelling nature set at 104.5 degrees to create a tetrahedron shape. H2O is the chemical formula and colloquially known as Dihydrogen Oxide in the field of chemistry. This is shown in Figure 3.1.1

Figure 3.1.1- Molecular structure of water [13]

## Amphoteric Nature

Water has a very important property known as the Amphoteric Nature of water. Due to its highly polar structure, water, in its liquid state has the ability to act as both an acid by donating a proton to a base, or as a base by using its lone pair of electrons to accept a proton. Water at 25 degrees has equal concentration of H+ and OH- molecules, both are concentrated at 1.0 x 10-7 Moles [12]

When acting as base, water decreases the H+ in a solution. For instance, if HCL dissolves in water it will separate into protons of H+ and ions of CL-. The proton is then accepted by the water to form a hydronium ion (H3O+).

HCl (aq) + H2O(l) → H3O+(aq) + Cl–(aq)

When acting as an acid, water increases the H+ in a solution. For instance, it will donate a proton to NH3.

H2O(aq) + NH3(aq) ⇆ (NH4)+ + OH–

[14]

Consequently, it is this ability of accepting and donating protons that allows water to attract with other molecular compounds. The positively and negatively charged hydrogen and oxygen within water creates a stripping effect to molecules. Although a relatively weak base and acid, it is its abilities as a strong solvent to induce this stripping effect that can disrupt formed bonds within explosive compounds, disintegrating the bonds and creating chemical instability [12].

## Redox Reactions

Oxidation is the loss electrons whereas reduction is the gain of electrons, these processes combine into the umbrella term of Redox. In terms of waters redox ability, it behaves like its Amphoteric nature. When present to a strong electron donor water will act as an oxidising agent, whereas in the presence on a strong electron receiver, water acts a reducing agent. Due to it being a weak oxidising or reducing agent, it used again as solvent medium to expediate the process [15].

Due to the oxygen present within the water molecule, water has strong corrosive abilities. Corrosion is an electrochemical process of oxidation in metals. Iron (III) oxide, commonly known as rust, is produced when as a result of the oxygen within water, be it atmospheric or liquid form. Rust is the general term to describe red oxides in Iron, stell or its alloys. The only metals that rust is Iron and Stell and these are commonly used within ordnance and munitions. However, oxidation also occurs on other materials but under different names, e.g., tarnish on silver and Verdigris on copper [16]. There a rea series of chemical reactions that are undergone to result in the final chemical formula of rust below:

4Fe + 3O2 + 6H2O → 4Fe(OH)3.

Once rusting starts, it is a process that is very difficult to stop. Free oxygen and dissolved iron form the oxide that releases electrons that then traverse and flow to another un-rusted part to continue the process anew. Without proper coating or protection on ordnance to protect from rusting, the consequences could be malfunction of the system, misfiring or unexpected chemical incident with the munition and explosive charge.

# Mitigation Techniques

Having identified the numerous detrimental effects of water within the OME ecosystem, developments have had to be made to mitigate the physical and chemical reactions resulting from water exposure. A vast area of mitigation comes in the form of environmental control. Controlling the environment enables the producing OME factory to control the ingress/exposure of water within the atmosphere that can potentially infiltrate into the production of explosives from powder production to shaped compacted charges. It also aides in mitigating ESD and reducing the corrosive effects of water. For potential ESD events, adjusting RH within an ordnance factory is one key aspect and one layer against the potential initiation of explosives via ESD. Management of ESD states that the dissipation of charge and the decay of such should occur within a 2 to 4 seconds [17]. This can be achieved by minimising friction on surface areas and by introducing conductive flooring and footwear to provide a low resistance path to the earth. Personal protective equipment on the operator such as conductive footing and cotton-based overalls can add an extra layer of protection [8]

The corrosive nature of water on ordnance can create hydrogen embrittlement, that is the absorption of hydrogen within the metal crystalline structure causing it to become brittle. This embrittlement can be intergranular or trans granular. Generally, corrosion penetrating within a metal can structurally weaken the whole system with devastating consequences when firing live, explosive munitions [18]. The UK and US governments spend billions annually to prevent and mitigate such effects, the US in fact spent $20.8 billion in 2013. The application of coatings to firearms and general ordnance, such as paint based, oils or waxes can provide a protective layer where the corrosive reactions from water cannot penetrate to the main frame of the ordnance. The British government use a varying combination of such methods to mitigate this, albeit these coatings are classified. [19]

# Conclusion

It can be concluded that the physical and chemical properties of water have multiple adverse effects in the OME field. These are inclusive of, but not limited to, corrosion of the ordnance, the cracking/outgassing of the munitions and as well chemical decomposition of the explosive charge itself. All of which can lead to misfiring, incomplete explosion, shorting of the detonation effect and desensitisation [20]

However, alongside the adverse consequences of water exposure, it has been seen that relative humidity of an environment can create an environment that acts as a mitigator against accidental initiation. A study by the Lawrence National laboratory also discovered another unexplored positive consequence of water interaction within an explosive. They found that H and OH atoms in water can transport oxygen that was part of nitrogen to carbon fuel in PETN explosives (hen under extreme temperatures). This reaction served as a catalyst within the explosive that speeds up the chemical reactions without being consumed during initiation. In essence, they concluded that when detonating PETN, water acting as both an oxidiser and reducer, worked in tandem to help transport oxygen as a fuel to further the yield of the explosion and super critical temperatures [6].

It is a fine balance to mitigate the adverse effects of exposure within OME but utilising the molecules within water as extra fuel and oxidiser to improve the yields of explosives.

# References

| [1] | C. V. &. A. Covaci, “Hyphenated and non-hyphenated chromatographic techniques for trace level explosives in water bodies – a review,” International Journal of Environmental Analytical Chemistry, vol. 98, no. 5, pp. 387-412, 2018. |
| --- | --- |
| [2] | E. S. S. Sydney Meyers, “Industrial explosives - a brief history of their development and use,” Journal of Hazardous Materials, vol. 23, no. 2, pp. 183-201, 1990. |
| [3] | D. Mubiru, “Blogspot,” Blogspot, April 2017. [Online]. Available: http://mubirupepu.blogspot.com/2017/04/structure-of-water-and-ice.html. [Accessed July 2022]. |
| [4] | L. G. M. a. C. V. J. L. Aragones, “Dielectric Constant of Ices and Water: A Lesson about Water Interactions,” Journal of Physical Chemistry, vol. 115, no. 23, pp. 5745-5758, 2011. |
| [5] | Vedantu, “Properties of Water,” Vedantu. |
| [6] | Lawrence Livermore National Laboratory, “Water acts as catalyst in explosives,” Lawrence Livermore National Laboratory, 2009. |
| [7] | M. G. Lawrence, “The Relationship between Relative Humidity and the Dewpoint Temperature in Moist Air: A Simple Conversion and Applications,” Bulletin of the American Meteorological Society, vol. 86, no. 2, pp. 225-234, 2005. |
| [8] | Explosives Research and Development Establishment, “The Effect of Temperature and Relative Humidity on the Accumulation of Electrostatic Charges on Fabrics and Primary Explosives,” Defense Technical Information Center, 1971. |
| [9] | C. J. M. a. T. S. Yoder, “"Effects of Temperature and Humidity on the Characterization of C-4 Explosive Threats,” Sensing and Imaging, vol. 13, no. 2, pp. 89-100, 2012. |
| [10] | Calorex, “Humidity Pro,” 2022. |
| [11] | L. Turker, “Electrostatic Discharge and Energetic Materials,” Chemsitry Journal, vol. 2, 2019. |
| [12] | B. Y. University, “Chemical Characteristics and Functions of Water in Physiology,” Brigham Young University, 2022. |
| [13] | Universtiy of Tokyo, “Tetrahedrality is key to the uniqueness of water,” Tetrahedrality is key to the uniqueness of water, 2018. |
| [14] | D. E. a. W. Kauzmann, The Structure and Properties of Water, Oxford University Press, 2005. |
| [15] | J. W. M. Justin M. Shorb, ChemPrime, 2020. |
| [16] | Y. H. a. N. M. I. Suzuki, “Nature of Atmospheric Rust on Iron,” Journal of Electrochemical Society, vol. 127, no. 10, 1980. |
| [17] | S. N. R. S. P. J.Paasi, “Performance of ESD protective materials at low relative humidity,” Journal of Electrostatics, Vols. 51-52, pp. 429-434, 2001. |
| [18] | OFFICE OF THE UNDER SECRETARY OF DEFENSE (ACQUISITION TECHNOLOGY AND LOGISTICS) WASHINGTON DC OFC OF CORROSION POLICY AND OVERSIGHT, “Corrosion Prevention and Control Planning Guidebook for Military Systems and Equipment,” 2014. |
| [19] | S. J. Dalewicz-Kitto, “Stopping corrosion around the world: an investigation into protective surface coatings on arms and armour,,” Journal of the Institute of Conservation, vol. 39, no. 1, pp. 44-56, 2016. |
| [20] | D. H. Y. K. Hyongdoo Jang, “Effects of Water Deck on Rock Blasting Performance,” Creative Commons, 2018. |
| [21] | N. Gardner, “Explosive,” Encylopedia Britannica, 19 February 2019. |
| [22] | P. Streatfield, “Explosions: Inventions and Uses,” The Oxford Scientist, 2020. |
| [23] | S. N. R. V. S. S. P. M. J. Paasi, “Performance of ESD protective materials at low relative humidity,” Journal of Electrostatics,, Vols. 51-52, pp. 429-434, 2001. |
| [24] | J.-X. G. D. P. F. Wan, “Absolute humidity, relative humidity: which is more important in representing severity of electrostatic discharge,” Electronic Letters, vol. 49, no. 23, pp. 1451-1452, 2013. |
| [25] | J. M. B. Judith C. Pennington, “Environmental fate of explosives,” Thermochimica Acta, vol. 384, no. 1-2, pp. 163-172, 2002. |
| [26] | J. M. B. Cynthia B. Price, “Technical report: Transformation of 2,4,6-Trinitrotoluene Under Controlled EH/pH Conditions,” US Army Corps of Engineers, Vicksburg, MS, 1995. |
| [27] | iRing INC -Knowledge, “Watergel Explosives - Background,” iRing INC, 20181. |
| [28] | FRTR , “Properties and Behavior of Explosives”. |
| [29] | Khan Academy, “Structure of water and hydrogen bonding,” Ap College Biology, 2020. |
| [30] | C. B. e. al, ““Defense Infrastructure: DOD’s 2013 Facilities Corrosion Study Addressed Reporting Elements,” 2014. |
| [31] | H. Schwarcz, “How Dynamite Spawned the Nobel Prizes,” 8 October 2021. [Online]. Available: https://www.mcgill.ca/oss/article/history/how-dynamite-spawned-nobel-prizes. [Accessed July 2022]. |
| [32] | A. Z. Ł. M. M. K. Wojciech Pawłowski, “The air humidity effect on the detection of TNT, PETN and NG by the FAIMS technique,,” Sensors and Actuators B: Chemical, vol. 247, pp. 343-348, 2017. |

# Bibliography

S. J. Allardi, A. Brunet, and A. Lewicke, Chemistry. Unit 8, When chemicals meet water : the properties of solutions. District of Columbia: Annenberg Learner, 2014.

D. Kalderis, A. L. Juhasz, R. Boopathy, and S. Comfort, “Soils contaminated with explosives: Environmental fate and evaluation of state-of-the-art remediation processes (IUPAC Technical Report),” Pure and applied chemistry., vol. 83, no. 7, pp. 1407–1484, 2011.

J. M. B. Cynthia B. Price, “Technical report: Transformation of 2,4,6-Trinitrotoluene Under Controlled EH/pH Conditions,” US Army Corps of Engineers, Vicksburg, MS, 1995.

J. M. B. Judith C. Pennington, “Environmental fate of explosives,” Thermochimica Acta, vol. 384, no. 1-2, pp. 163-172, 2002.

Talebzadeh et al, "Dependence of ESD Charge Voltage on Humidity in Data Centers: Part I-Test Methods," ASHRAE Trans, vol. 121, pp. 58-70, 2015. Available: https://www.proquest.com/scholarly-journals/dependence-esd-charge-voltage-on-humidity-data/docview/1725205886/se-2

P.A. Buxton and K. Mellanby, “The Measurement and Control of Humidity”, Bulletin of Entomological Research, vol. 25, no. 2, pp. 171-175, 1934

H. Schwarcz, "How Dynamite Spawned the Nobel Prizes," 8 October 2021. [Online]. Available: https://www.mcgill.ca/oss/article/history/how-dynamite-spawned-nobel-prizes. [Accessed July 2022].

A. Z. Ł. M. M. K. Wojciech Pawłowski, "The air humidity effect on the detection of TNT, PETN and NG by the FAIMS technique,," Sensors and Actuators B: Chemical, vol. 247, pp. 343-348, 2017

| [1] | C. V. &. A. Covaci, "Hyphenated and non-hyphenated chromatographic techniques for trace level explosives in water bodies – a review," International Journal of Environmental Analytical Chemistry, vol. 98, no. 5, pp. 387-412, 2018. |
| --- | --- |
| [2] | E. S. S. Sydney Meyers, "Industrial explosives - a brief history of their development and use," Journal of Hazardous Materials, vol. 23, no. 2, pp. 183-201, 1990. |
| [3] | D. Mubiru, "Blogspot," Blogspot, April 2017. [Online]. Available: http://mubirupepu.blogspot.com/2017/04/structure-of-water-and-ice.html. [Accessed July 2022]. |
| [4] | L. G. M. a. C. V. J. L. Aragones, "Dielectric Constant of Ices and Water: A Lesson about Water Interactions," Journal of Physical Chemistry, vol. 115, no. 23, pp. 5745-5758, 2011. |
| [5] | Vedantu, "Properties of Water," Vedantu. |
| [6] | Lawrence Livermore National Laboratory, "Water acts as catalyst in explosives," Lawrence Livermore National Laboratory, 2009. |
| [7] | M. G. Lawrence, "The Relationship between Relative Humidity and the Dewpoint Temperature in Moist Air: A Simple Conversion and Applications," Bulletin of the American Meteorological Society, vol. 86, no. 2, pp. 225-234, 2005. |
| [8] | Explosives Research and Development Establishment, "The Effect of Temperature and Relative Humidity on the Accumulation of Electrostatic Charges on Fabrics and Primary Explosives," Defense Technical Information Center, 1971. |
| [9] | C. J. M. a. T. S. Yoder, ""Effects of Temperature and Humidity on the Characterization of C-4 Explosive Threats," Sensing and Imaging, vol. 13, no. 2, pp. 89-100, 2012. |
| [10] | Calorex, "Humidity Pro," 2022. |
| [11] | L. Turker, "Electrostatic Discharge and Energetic Materials," Chemsitry Journal, vol. 2, 2019. |
| [12] | B. Y. University, "Chemical Characteristics and Functions of Water in Physiology," Brigham Young University, 2022. |
| [13] | Universtiy of Tokyo, "Tetrahedrality is key to the uniqueness of water," Tetrahedrality is key to the uniqueness of water, 2018. |
| [14] | D. E. a. W. Kauzmann, The Structure and Properties of Water, Oxford University Press, 2005. |
| [15] | J. W. M. Justin M. Shorb, ChemPrime, 2020. |
| [16] | Y. H. a. N. M. I. Suzuki, "Nature of Atmospheric Rust on Iron," Journal of Electrochemical Society, vol. 127, no. 10, 1980. |
| [17] | S. N. R. S. P. J.Paasi, "Performance of ESD protective materials at low relative humidity," Journal of Electrostatics, Vols. 51-52, pp. 429-434, 2001. |
| [18] | OFFICE OF THE UNDER SECRETARY OF DEFENSE (ACQUISITION TECHNOLOGY AND LOGISTICS) WASHINGTON DC OFC OF CORROSION POLICY AND OVERSIGHT, "Corrosion Prevention and Control Planning Guidebook for Military Systems and Equipment," 2014. |
| [19] | S. J. Dalewicz-Kitto, "Stopping corrosion around the world: an investigation into protective surface coatings on arms and armour,," Journal of the Institute of Conservation, vol. 39, no. 1, pp. 44-56, 2016. |
| [20] | D. H. Y. K. Hyongdoo Jang, "Effects of Water Deck on Rock Blasting Performance," Creative Commons, 2018. |
| [21] | N. Gardner, "Explosive," Encylopedia Britannica, 19 February 2019. |
| [22] | P. Streatfield, "Explosions: Inventions and Uses," The Oxford Scientist, 2020. |
| [23] | S. N. R. V. S. S. P. M. J. Paasi, "Performance of ESD protective materials at low relative humidity," Journal of Electrostatics,, Vols. 51-52, pp. 429-434, 2001. |
| [24] | J.-X. G. D. P. F. Wan, "Absolute humidity, relative humidity: which is more important in representing severity of electrostatic discharge," Electronic Letters, vol. 49, no. 23, pp. 1451-1452, 2013. |
| [25] | J. M. B. Judith C. Pennington, "Environmental fate of explosives," Thermochimica Acta, vol. 384, no. 1-2, pp. 163-172, 2002. |
| [26] | J. M. B. Cynthia B. Price, "Technical report: Transformation of 2,4,6-Trinitrotoluene Under Controlled EH/pH Conditions," US Army Corps of Engineers, Vicksburg, MS, 1995. |
| [27] | iRing INC -Knowledge, "Watergel Explosives - Background," iRing INC, 20181. |
| [28] | FRTR , "Properties and Behavior of Explosives". |
| [29] | Khan Academy, "Structure of water and hydrogen bonding," Ap College Biology, 2020. |
| [30] | C. B. e. al, "“Defense Infrastructure: DOD’s 2013 Facilities Corrosion Study Addressed Reporting Elements," 2014. |
| [31] | H. Schwarcz, "How Dynamite Spawned the Nobel Prizes," 8 October 2021. [Online]. Available: https://www.mcgill.ca/oss/article/history/how-dynamite-spawned-nobel-prizes. [Accessed July 2022]. |
| [32] | A. Z. Ł. M. M. K. Wojciech Pawłowski, "The air humidity effect on the detection of TNT, PETN and NG by the FAIMS technique,," Sensors and Actuators B: Chemical, vol. 247, pp. 343-348, 2017. |