EXAMINATIONS 2018 / 2019

ARHOLIADAU  2018  / 2019

FACULTY OF: Architecture, Computing & Engineering

| NAME OF COURSE/PROGRAMME: | HND/BSc Engineering Programmes |
| --- | --- |
| YEAR OR LEVEL: | Level 4 |
| NAME OF MODULE/UNIT/EXAM PAPER: | Engineering Science 1 |
| MODULE NO: | M4X02192 |
| DATE OF EXAMINATION: |  |
| EXAMINATION TIMES: |  |
| START TIME: |  |
| FINISH TIME: |  |
| TIME ALLOWED: | 2 hours |

INSTRUCTIONS TO CANDIDATES:

Answer all questions.

g = 9.81m/s2

Give answers to 2.d.p.

You are allowed a scientific calculator and a dictionary.

A formula e car performs a straight-line acceleration from 0 to 100km/h in 3 seconds.  It then accelerates up to its top velocity of 225km/h in 10 seconds, when the brakes are immediately applied and it comes back to rest in 65m.  Construct a velocity time graph [5 marks] and calculate:

- The rate of acceleration from 0-100km/h. [6 marks]

- The rate of acceleration from 100km/h to 225km/h. [5 marks]

- The braking time. [4 marks]

- And total displacement of the car.                         [8 marks]

Figure 1: FIA Formula E car.

2.

At one circuit the Formula E car has to travel up a incline of 20ᵒ at a velocity of 140km/h.  Given that the car has a mass of 888kg and that it is using 150kW of power, construct a forces diagram [5 marks] and calculate the coefficient of friction between the tyres and the incline.  	 	   [12 marks]

3.

The rotor of the electric motor of a Formula E car, accelerates up to 20000rpm in 0.5 seconds, where it reaches its peak torque of 145Nm.   Calculate:

- The mass moment of inertia of the rotor.

[9 marks]

- The mass of the rotating parts.	Given the radius of the rotor has a diameter of 260mm           [5 marks]

- The power developed at this rpm in kW.        [3 marks]

- The maximum kinetic Energy of the rotating parts in kJ.			    [3 marks]

Figure 2: Electric motor

Children use a playground swing similar to the one show in in figure 3.

Given that the maximum displacement of the swing from its rest point is 1750mm and that is has a frequency of 0.65Hz.  Calculate:

a. the angular velocity of the swing.     [4 marks]

b. the periodic time of the motion of the swing.                                                                                                                                       [2 marks]

Given that the force exerted on the swing cannot exceed 10kN calculate the maximum mass that the swing can take.                [4 marks]

4.

Figure 3: Nest swing