Old Paxton Street Car Park Development

Site Waste Management Plan

| Project cost (estimated) | £40,000,000 |
| --- | --- |
| Floor area (m2) | 18,000m2 (30m x 30m x 20 floors) |
| Project start date | 30/03/2020 |
| Project end date | 03/01/2022 |
| Site location description | The site is located at the Old Paxton Street Car Park, near Swansea City Centre |
| Client | UWTSD Developments |
| Principal Contractor | EJ Construction |

Description of Project, Materials Used and Construction Method

- The scheme comprises of the construction of a £40,000,000, twenty-storey r.c concrete frame building with three floors of underground basement parking. The building consists of a 900m2 (30m x 30m) footprint and is situated in the Marina area of Swansea. - The building foundations consist of reinforced concrete slabs on load bearing piles. Reinforced diaphragm walls form act as a retaining element for the three levels of basement, spanning 14 metres. - The building superstructure is made up of a Kingspan “Benchmark Dri-Design” façade system, pre-fabricated aluminium cassettes which are fixed to the concrete frame. - The aim of this Site Waste Management Plan is to reduce the environmental impact of the project by following the waste hierarchy - reduce, reuse and recycle materials.

| Roles and Responsibilities to Distribute Regarding Waste Management |  |
| --- | --- |
| Responsible Party for Drafting SWMP | Project Manager (Client) |
| Responsible Party for Implementing/Distributing SWMP Distribute to – - Client - Relevant Sub Contractors - CDM Co-Ordinator) | Project Manager (Main Contractor) |
| Waste Coordinator | Project Manager (Main Contractor) |
| Analysis of Material Quantities | Buyer (Main Contractor) |
| Determine Delivery Dates | Programmer & Project Manager (Main Contractor) |
| Obtain Quotations & Place Orders | Buyer (Main Contractor) |
| Quantity & Quality Check on Delivery | Site Team (Main Contractor) |
| Material Site Storage & Damage Prevention | Site Team (Main Contractor) |
| Obtain Relevant Information from Sub-Contractors | Project Manager (Main Contractor) |
| Sites that Waste is being delivered to |  |

Upon investigating the predicted waste that will be generated during this project, the following targets have been set in order to maximise savings environmentally and financially –

| Waste Category | Material Types | Target of Disposal/Treatment |
| --- | --- | --- |
| Inert | Non-contaminated soil, brick, block, concrete. | 20-30% re-use on site. |
| Non-Hazardous | Metals, plastics, plasterboards. | 90% recycled off site |
| Hazardous | Oil, asbestos, diesel, fluorescent tubes. | 100% disposal from site. |

| Identification of types of waste on site during the construction stages specified |  |  |
| --- | --- | --- |
| Building Element | Activity | Waste Material |
| Sub-Structure | Soil Excavation | Soil (Assumed Non-Contaminated) |
|  | Installation of Diaphragm Walls – Cast in situ | Concrete Steel Reinforcement |
|  | Installation of Load Bearing Piles – Cast in situ | Concrete Steel Reinforcement |
|  | Installation of Floor Slabs (Foundations) and Raft Foundation – All cast in situ | Concrete Steel Reinforcement |
| Superstructure | Installation of R.C Concrete Frame, Floors & Columns | Concrete Steel Reinforcement |
| Façade | “Kingspan Benchmark Dri-Design” Cassettes Pre-Fabricated, Pre-Insulated Panels with Aluminium Cladding & Glazed Units | Aluminium Glass Packaging – Plastic, Cardboard, Pallets Insulation |

| Approximate calculation of Waste Volumes and Masses |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Building Element | Material & Code | Waste Classification | Quantity | Wastage (%) | Total Waste |
| Soil Excavation | Soil (17.05.04) | - Inert | 10,890m3 (- the 30% to be utilised for Landscaping) | N/A | 7,623m3 11,434.5 tonnes |
| GF Raft Foundations | Concrete (17.01.01) | - Inert | 900m3 1,116 tonnes | 7% | 63m3 78.12 tonnes |
|  | Steel (17.04.05) | - Non-Hazardous | 1,800m2 180 tonnes | 3% | 54m2 5.4 tonnes |
| Concrete Sub-Slabs (GF, Floor -1, Floor –2) | Concrete (17.01.01) | - Inert | 2,160m3 2,678.4 tonnes | 7% | 151.2m3 187.48 tonnes |
|  | Steel (17.04.05) | - Non-Hazardous | 2,700m2 270 tonnes | 3% | 81m2 8.1 tonnes |
| Diaphragm Walls | Concrete (17.01.01) | - Inert | 826m3 892.8 tonnes | 7% | 57.82m3 71.69 tonnes |
|  | Steel (17.04.05) | - Non-Hazardous | 1,652m2 165.2 tonnes | 3% | 49.56m2 4.95 tonnes |
| Load Bearing Piles | Concrete (17.01.01) | - Inert | 40.68m3 50.44 tonnes | 7% | 2.84m3 3.53 tonnes |
|  | Steel (17.04.05) | - Non-Hazardous | 4.06 tonnes | 3% | 0.12 tonnes |
| Superstructure Concrete Frame Columns | Concrete (17.01.01) | - Inert | 550.8m3 682.9 tonnes | 3% | 16.52m3 20.48 tonnes |
|  | Steel (17.04.05) | - Non-Hazardous | 110.16 tonnes | 3% | 3.30 tonnes |
| Superstructure Concrete Frame Floors (Pre-Fabricated) | Concrete (17.01.01) | - Inert | 3,600m3 4,464 tonnes | 0% | Zero waste (Pre-fabricated) |
|  | Steel (17.04.05) | - Non-Hazardous | 18,000m2 1,800 tonnes | 0% | Zero waste (Pre-fabricated) |
| Kingspan Dri-Design Cassette Façade System | Aluminium (17.04.02) | Non-Hazardous | 4,320m2 | 0% | Zero waste (Pre-fabricated) |
|  | Insulation (17.06.04) | Non-Hazardous | 4,320m2 | 0% | Zero waste (Pre-fabricated) |
|  | Glass (17.02.02) | Non-Hazardous | 2,880m2 | 0% | Zero Waste (Pre-Fabricated) |
| Estimated Totals of Waste | Concrete 361.3 tonnes (291.38m3) | Steel 21.87 tonnes | Insulation Zero | Glass Zero | Aluminium Zero |

| Actions taken to minimise waste on site |  |  |
| --- | --- | --- |
| Action Taken | Responsible Person/Party | Description of Action/Potential Volume Reduced |
| Accurate Ordering of Materials | Contractor - Buyer/Estimator | By taking accurate measurements to work out the exact quantities required, the Contractor will reduce the amount of surplus waste on site. |
| Good Housekeeping | Site Manager/Sub-Contractors | Good housekeeping will result in less materials getting lost or damaged. This is exceptionally important with items that need to be stored inside and away from the elements such as plasterboard and insulation. All sub-contractors will be asked to be responsible for the removal of any waste they bring on site. |
| Specify Standard Material Dimensions | Design Team | By specifying standard material dimensions, the Design Team are reducing the amount of “cut offs” made by the site team (for example plasterboard sizes, wall heights or ceiling lengths specified to a standard dimension will mean the drylining contractor will cut less boards. |
| Offsite Fabrication | Design Team | Offsite fabrication will greatly reduce the potential for mistakes, reducing the likelihood of reusing materials on site as everything is produced in the quality controlled environment of a factory. It also promotes just in time delivery, meaning less opportunity for material to be damaged on site in storage. |
| Design for Flexible Usage | Design Team | By specifying flexible design elements, the Design Team will increase the opportunity to reduce wastage on site. This could mean specifying structures that are easy to reconstruct/adapt such as steel frames, mitigating the need to fully demolish the building in future refurbishment works. |
| Good Workmanship | Sub-Contractors/ Tradesmen | By utilising Quality Plans and frequently inspecting work on site, the Site Manager will maintain high standards of workmanship on site, leading to less material being wasted. |
| On Site Training/Toolbox Talks | Site Manager | In order to communicate the SWMP to the site team, the Site Manager should hold toolbox talks explaining the ways in which waste should be managed on site. This will be specific to different trades and will be specific to the elements of construction that are taking place (e.g. a toolbox talk could be held with the Carpenters to convey the message of reusing timber cut offs where possible, rather than just disposing of them). |
| Reduce Packaging Waste | Buyers/Design Team | Contractors & Sub-Contractors should look to use manufacturers who aim to use less packaging, leading to less wastage such as plastic wrapping. For specialist materials, the Design Team should specify these products. “More than 1/3 of construction waste is packaging. You can cut it by 40% by using suppliers actively trying to reduce waste.” (http://www.wrap.org.uk/sites/files/wrap/Onsite%20Plasterers.pdf) |

“If you are arranging for your waste to be collected by someone else, to comply with the Waste Duty of Care, you must check that you are using a registered waste carrier”

(https://naturalresources.wales/permits-and-permissions/)

Carrier to provide the following information -

- Duty of Care transfer paperwork

- Classification of waste

- Waste Analysis

- Interpretation of Site Investigation reports

| Destination of Waste (Re-Use, Recycle, Dispose) Specifying Volumes, as Well as Dealers and Cost. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type of Waste | Total Volume/Mass | On Site Re-Use/Recycling/Recovery | Off Site Re-Use/Recycling/Recovery | Dealer |
| Soil | 10,890m3 | Cut & Fill exercise – 30% of excavated soil to be reused for “fill” and landscaping. | Soil can be re-distributed to other sites by the carrier, provided it is not contaminated. | Derwen Group, Neath 70% to be re-distributed to other construction sites. |
| Concrete | 291.38m3 361.3 tonnes | 20% to be used for secondary aggregate on site. | Supplier to take 20% of concrete wastage back to their yard. | Returned to Cemex Swansea |
| Steel | 21.87 tonnes | N/A | Segregate from other waste and send to be recycled. | Derwen Group, Neath |

(https://www.derwengroup.co.uk/muckaway-haulage/ Date Accessed 28/3/2020)

| Budget for Waste Management |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type of Waste | Total Volume & Mass | Number of Skips/Trucks | Cost Per Skip/Trip | Total Cost |
| Soil (Inert) | 7,623m3 11,434 tonnes | 24 tonne Articulated Dump Truck 458 trips | £170 | £77,860 |
| Concrete | 291.38m3 361.3 tonnes | Supplier to take 80% of concrete wastage back to their yard. | £100 per m3 | £23,310 |
| Metals | 52.07m3 21.87 tonnes | 20 yard skips (15m3) x 4 skips | £300 | £1,200 |
| Mixed | Miscellaneous waste | 1 x 10 yard skip | £170 | £170 |
| Total |  |  |  | £102,540 |

Note – The above figures are not inclusive of waste from internal works. Projected waste from internal works to include items such as tiles, plasterboard, insulation and timber.

Actions for Implementation, Monitoring and Review

During the project, the SWMP should be reviewed regularly to ensure all items are being accounted for and that all responsible parties are familiar with their various roles in reducing waste on site. This would ideally be checked during Client/Contractor meetings which would typically be held monthly. It would be encouraged that the Contractor would internally review the SWMP weekly/fortnightly. Upon completion of the project, the Contractor and Client should have a final review of the SWMP, checking that the following items have been captured and accounted for: - Confirmation that the plan has been checked, monitored and updated by the Project Manager on a regular basis. - Any improvements than can be made in the future and any issues/complications that arose during the project in regard to Waste Management. - Compare the estimated quantities of waste against the actual quantities. For this reason, it is vital that the Contractor keeps records of waste transfer notes. - Estimates of any cost savings gained from implementing a SWMP and any clear environmental benefits. Finally, it is essential that a copy of the SWMP is readily accessible for a minimum of three years after practical completion of the project.

Appendix

Calculations

- Soil Excavation - Vo = 30m x 30m x 11m= 9,900m3

Vb = 30m x 30m x 11m x 1.10 = 10,890m3

(-30% to be utilised for Landscaping)

30% of 10,890m3 = 3,267m3

10,890m3 3,267m3 = 7,623m3 waste

7,623m3 x 1.5 = 11,434 tonnes waste

- GF Raft

Concrete – 30m x 30m x 1m = 900m3

7% of 900m3 = 63m3

963m3 – 900m3 = 63m3 waste

63m3 x 1.24 = 78.12 tonnes waste

Steel – 30m x 30m = 900m2

3% of 900m2 = 27m2

100kg/m2 = 2.7 tonnes of waste

(Multiply by two as it is assumed there is two layers of steel reinforcement – both at the top and bottom of the raft)

Volume of Waste = 54m2

Mass of waste = 5.4 tonnes

(Note – Figures are indicative only. Structural Engineer to specify correct quantities of steel within specifications and bar bending schedules. This will greatly reduce steel wastage)

- Sub Slabs

Concrete -  30m x 30m x 0.8m = 720m3

720m3/floor x 3 floors = 2,160m3

7% of 2,160m3 = 151.2m3

151.2m3 x 1.24 = 187.48 tonnes

Steel – 30m x 30m = 900m2 per floor

100kg/m2 of concrete = 270 tonnes

3% of 900m2 = 27m2

Mass of waste = 8.1 tonnes

- Diaphragm Walls

Concrete – 30m x 30m x 14m = 12,600m3 (Outer Sections)

29m x 29m x 14m = 11,774m3 (Inner Sections)

= Volume of 826m3

7% of 826m3 = 57.82m3 waste

57.82m3 x 1.24 = Mass of 71.69 tonnes of waste

Steel – 29.5m x 14m = 413m2

413m2 x 4 = 1,652m2

3% of 1,652m2 = 49.56m2 of waste

100kg/m2 of steel; 100kg x 1,652m2 = 165.2 tonnes

3% of 165.2 tonnes = 4.95 tonnes of waste

- Load Bearing Piles

Depth of 4m, diameter of 600mm

Concrete – π x 0.3m2 x 4m = 1.13m3

6 x 6 rows = 36 piles

36 x 1.13m3 = 40.68m3

Steel – 100kg/m3

100kg x 40.68m3 = 4.06 tonnes

3% waste of 4.06 tonnes = 0.12 tonnes

- Concrete Frame – Columns

Concrete –  0.850m (length) x 0.300m (width) x 60m (height) = 15.3m3

15.3m3 x 36 columns per floor = 550.8m3

3% of 550.8m3 = 16.52m3

16.52m3 x 1.24 = 20.48 tonnes

Steel -   200kg/m3 – 200kg x 550.8m3 = 110.16 tonnes

3% of 110.16 tonnes = 3.30 tonnes

7.External Cladding System

Aluminium Cladding Panels – 30m x 60m = 1,800m2

1,800m2 x 4 = 7,200m2

60% Glazed Panels = 4,320m2

Glazed Panels – 30m x 60m = 1,800m2

1,800m2 x 4 (for each side of the build) = 7,200m2

40% Aluminium Cladding = 2,880m2

References

(https://www.derwengroup.co.uk/muckaway-haulage/ Date Accessed 28/3/2020)

(https://naturalresources.wales/permits-and-permissions/)

(http://www.wrap.org.uk/sites/files/wrap/Onsite%20Plasterers.pdf)