| # Creating and Deploying A Virtualized Network |  |
| --- | --- |
| 02/03/26 Software Engineering Network Operating Systems and Services Camden Henderson (2404086) |  |

Introduction

NOSS2026 is a multinational organization that depends on virtualized network infrastructure for managing IT services. This company has two primary networks across two different locations, Swansea and London. NOSS2026 has employed my skills for the purpose of designing and implementing a suitable virtualized network solution. The network requires a Windows Server and Debian server, whilst the domain is managed through SWANSEA.ACCA. This is essential for enabling communication and exchange between the two locations. I will do prior research before the implementation of Hypervisors, IP addressing, routing, and network services. This will influence the design decisions made for the network. A reflection will be produced to assess and justify any design decisions made and decide what could be done differently.

Task 1

Hypervisor

Hypervisors are software that allows for the creation and use of virtual machines [1]. Multiple virtual machines can be run on a single physical computer host by sharing resources. Multiple different operating systems (such as Windows and Linux) can be run on a single machine. Hypervisors are an essential element used in cloud computing. Users can access a virtualized environment whilst IT technicians can control the infrastructure and sensitive data. Hypervisors are categorized into Type 1 (bare metal hypervisor) and Type 2. Type 1 hypervisors operate on a bare metal server with direct access to its hardware resources [2]. It is a suitable hypervisor for business environments in which they need to virtualize company services [3]. An advantage of a type-1 hypervisor is high security, due to only one application running on it directly, and better performance due to its bare-metal nature. This makes it suitable for less powerful hosts. They are also more complex to implement compared to type 2, and only one can be deployed on a single machine.

Type 2 hypervisor is a virtual machine manager that is installed as software on an existing operating system [4]. This makes it easier to install compared to Type 1. However, the performance depends on the host computer, which can increase security vulnerabilities. Examples of type 2 hypervisors include VMware workstation and Oracle VirtualBox.

A Type 1 hypervisor would be suitable for NOSS2026/ This is due to needing scalability and security whilst supporting windows server 2022 and Debian domain services. High security is provided by strict isolation, and performance is bolstered with resource efficiency. Because it minimizes latency and can utilize multiple servers, it is the preferred hypervisor. However, I chose to use a Type 2 hypervisor as it was the more practical solution within the constraints.

IP Addressing

IP addressing is a logical method of identifying where data is sent as packets and where it needs to be delivered. This allows for communication between devices on networks. When data is sent across the internet, it is divided into smaller packets which contain a source and destination header inside of it. This allows the router to send it to the correct locations. IPv4 addresses follow a 32 bit integer format represented in dotted decimal notation such as 192.168.0.10. IPv4 routes packets across networks and is responsible for logical addressing [5]. The first three portions of the address identify the network, and the last portion identifies the host. The main issue with IPv4 is that it has a limited number of addresses. It holds exactly 4,294,967,286 unique values [6]. This is not enough to cover all of humanity which is roughly 8 billion people. This problem was solved through the adoption of IPv6, which is a 128-bit hexadecimal format[7]. This supports many more devices and is scalable for the long term. However, many companies still utilize IPv4 due to easier configuration, and existing infrastructure already using that format. Subnetting is a technique which divides a larger network into smaller manageable networks. This improves security management and reduces unnecessary traffic. Routers allow subnets to communicate with each other.

Figure 1: How a packet travels between computers[8]

Routing

Routing is the process of selecting a network path across one of more networks [9]. It determines the most efficient path for packets to travel from the source to the destination. Routers follow a set of rules called router protocols. These rules dictate how routers communicate and exchange information through efficient data transmission paths. Common protocols include Routing Information Protocol (RIP) and Enhanced Interior Gateway Routing Protocol (EIGRP). Traversal paths are determined based on metrics such as bandwidth. Routing can be classified as either Static of Dynamic. Static Ip routing includes manually configuring routes, so paths will remain unless updated. This improves security due to less traffic being introduced. Dynamic routing updates routing tables to allow routers to adapts to any network changes without manual assistance[10]. This improves tolerance but is more complex to configure for organizations but is more adaptable because it's automatic.

A static outing approach is suitable for NOSS2026 because the Debian server is a DHCP relay. This means it forwards client requests between subnets to the windows host server. This means it has limited routing paths. Dynamic routing would be more beneficial but use to reduce complexity and need a reliable connection; static Ip will be used.

Network Services

VPN (Virtual Protection Network) let organizations add a layer of security to devices in separate locations. They are encrypted network connections that let users access an organizations service with protection. VPNs can also be used for employees in a different location who need to access their companies' email and services and work remotely.

DHCP (Dynamic Host Configuration Protocol) is a network management protocol which assigns IP addresses to each host on a network. Automatically assigning P’s removes any user error which may cause incorrect addresses to be entered. The DHCP relay used in debian ensures users on different networks obtain IP addresses from a central server without needing multiple DHCP servers.

AD DS (Active Directory Domain Service) is a method for storing directory information and making it available for administrators and users[13]. It allows for control over users within a domain, and technicians can use group policies to manage access across both London and Swansea servers.

Task 2 - Documentation

Screenshot 1 – 4 virtual machines

Screenshot 2 – Downloading Windows Server 2022 ISO.

Screenshot 3 – Downloading Debian Server

Screenshots 4 – Windows Server 2022 Settings & Network Adapter Names.

Screenshot 5 – Windows Client Settings

Screenshot 6 – Debian Server Settings

Screenshot 7 – Debian Client Settings

Screenshot 8 – Connecting Windows Client to Internet

Screenshot 9 – Renaming Windows Server 2022 to SD-C

Screenshot 10 – Windows Client pinging SD-C

Screenshot 11 – IP config of windows server

Screenshot 12 – IP config of client machine

Screenshot 13 – Windows Server 2022 IPv4 Settings

Screenshot 14 – Windows Client IPv4 Settings

Screenshot 15 – changing name of windows client machine to “SDC-C-CLIENT"

Screenshot 16 – Debian Host Network Adapters

Screenshot 17 – Debian Host IP Configuration

Screenshot 18 – Debian Client Network Adapters

Screenshot 19 – Debian Client IP Configuration

Screenshot 20 – Debian Host unsuccessfully pinging client machine.

Screenshot 21 – Debian Host successfully pinging Client, fixed incorrect Ip configuration.

Screenshot 22 – Debian Client successfully pinging Host

Task 3 – Reflection

I was met with several challenges when implementing the virtual machine solution. And they were successfully troubleshooted. A notable challenge occurred when attempting to connect the Windows Host and Client together, and I needed to test the connection with a ping. The Ip addresses attached and they were on the same host. After confirming that the network settings were connected, I concluded that the Firewall was blocking ICMP traffic. After disabling the firewall, the problem was resolved and the two machines could successfully ping each other.

Another challenge I encountered was setting up the Debian host and client to ping each other. When attempting a ping from either machine, it would resort to a “destination host unreachable error”. I concluded that the internal networks might have been misspelled, but this was incorrect. I tried many different methods, but I eventually solved this error by swapping around the network adapters. The adapters were assigned to the wrong network, which is why the machines couldn't find each other. After swapping the adapters to the correct location, they could successfully ping.

This project improved my skills regarding implementing both Linux and windows machines. I learned various new Linux commands as well as the ability to effectively troubleshoot any problems I may enter again. I gained a better overall understanding of how to deploy networks and the resources that needed to be allocated, as well as whether to choose a type 1 or type to hypervisor depending on the organizational requirements.

References

[1]vmware, “What is a Hypervisor? | VMware Glossary,” www.vmware.com, 2024. https://www.vmware.com/topics/hypervisor

‌[2]Amazon Web Services, “What’s the Difference Between Type 1 and Type 2 Hypervisors?,” Amazon Web Services, Inc., Feb. 12, 2026. https://aws.amazon.com/compare/the-difference-between-type-1-and-type-2-hypervisors/#whats-the-difference-between-type-1-and-type-2-hypervisors--axi6re (accessed Mar. 02, 2026).

[3]O. Pankevych, “Type 1 vs. Type 2 Hypervisor: What Is The Difference?,” StarWind Blog, Jul. 20, 2023. https://www.starwindsoftware.com/blog/type-1-vs-type-2-hypervisor-what-is-the-difference/

‌

[4]“What Is A Hosted Hypervisor (Type 2 Hypervisor)?,” SearchITOperations. https://www.techtarget.com/searchitoperations/definition/Type-2-hypervisor-hosted-hypervisor

‌

[5]GeeksforGeeks, “IPv4 Datagram Header,” GeeksforGeeks, Jul. 03, 2015. https://www.geeksforgeeks.org/computer-networks/introduction-and-ipv4-datagram-header/

[6]“IPv4 Address Report,” ipv4.potaroo.net. https://ipv4.potaroo.net/

[7]GeeksforGeeks, “Internet Protocol version 6 (IPv6),” GeeksforGeeks, Aug. 22, 2017. https://www.geeksforgeeks.org/computer-networks/internet-protocol-version-6-ipv6/

‌[8]Ace Networker, “A Packet’s Journey - How Data Travels Across a Computer Network,” YouTube, Sep. 25, 2021. https://www.youtube.com/watch?v=aFR1kKKJoic (accessed Mar. 03, 2026).

[9]Cloudflare, “What Is routing? | IP Routing | Cloudflare,” Cloudflare, 2024. Available: https://www.cloudflare.com/learning/network-layer/what-is-routing/

‌

[10]“Static vs. Dynamic IP Address: Similarities and Differences,” Fortinet. https://www.fortinet.com/uk/resources/cyberglossary/static-vs-dynamic-ip

‌

[11]NCSC, “Virtual Private Networks (VPNs),” www.ncsc.gov.uk, Jun. 29, 2021. https://www.ncsc.gov.uk/collection/device-security-guidance/infrastructure/virtual-private-networks

‌

[12]Fortinet, “What Is DHCP? How Does DHCP Work? Why Is It Important?,” Fortinet, 2024. https://www.fortinet.com/resources/cyberglossary/dynamic-host-configuration-protocol-dhcp

‌

[13]Microsoft, “Active directory domain services overview,” Microsoft, Mar. 11, 2025. https://learn.microsoft.com/en-us/windows-server/identity/ad-ds/get-started/virtual-dc/active-directory-domain-services-overview

‌

‌

‌

‌