Machine Workshop Tom Schultze 1802180

19th November 2019

2nd Machining Session with Dave Cooper

Machine Workshop

Milling session

Health and Safety

-ALWAYS before entering workshop wear appropriate safety equipment.

Equipment worn this session:

-Eye protection: safety goggles

-Protective clothing: dust coat provided by uni and steel toe caps

- Remove burs from workpiece (WP)

- Square up WP on level surface

- Start measuring WP

- Measured and marked WP after technical drawing (Fig. 1)

Figure 1: Technical drawing of WP

Dave explains M6= 6mm outer diameter and thread pitch usually 1mm

6-1=5

Use 5mm tap drill

- Use centre punch

- Centre drill to WP

- Drill relief hole with pillar drill

- Repeat for every hole

Die= to thread bolts

Tap= to thread hole

- To tap get tap wrench perpendicular to workpiece

- Use lubricant as to not destroy threads

- 360 turn, 180 turn back as to remove shavings and to preserve thread

- Use all the thread of tap wrench

- Use hacksaw for roughing process (Fig. 2)

Figure 2: Roughed out WP

- For milling process

- Zero y and z

- Take off 0.5mm at once

- Set the shoulder

- Zero z and x

- And mill the required material amount

End of session