Embedded C Assignment

Embedded C Code

#include <main.h>					// include main header file

#define LCD_ENABLE_PIN PIN_D2				// define pins

#define LCD_RS_PIN PIN_D0				//

#define LCD_RW_PIN PIN_D1				//

#define LCD_DATA4 PIN_D4				//

#define LCD_DATA5 PIN_D5				//

#define LCD_DATA6 PIN_D6				//

#define LCD_DATA7 PIN_D7				//

#define SW1_A PIN_A2 					//

#define SW1_B PIN_A3					//

#define SW1_C PIN_A4					//

#define SW1_D PIN_A5					//

#define motA PIN_C0					//

#define motb PIN_C1					//

#define but  PIN_A1					//

#include <lcd.c>					// inlcude lcd header file

void analog(void);				 	//define fucntions

void switch1_A(void);					//

void switch1_B(void);					//

void switch1_C(void);					//

void switch1_D(void);					//

unsigned int adc;								//define global variables

unsigned int kitt[15] = {1, 2, 4, 8, 16, 32, 64, 128, 64, 32, 16, 8, 4, 2, 1}; //define kitt array

void main()							//main function

{

setup_adc_ports(sAN0);					// initialise adc channel

setup_adc(ADC_CLOCK_INTERNAL|ADC_TAD_MUL_0);		// setup adc operation

lcd_init();							// initialise the LCD

while(TRUE)						//start main while loop

{

if(input(SW1_A))			//When switch A is selected carry out the function

switch1_A();					//call function

if(input(SW1_B))			//When switch B is selected carry out the function

switch1_B();					//call function

if(input(SW1_C))			//When switch C is selected carry out the function

switch1_C();					//call function

if(input(SW1_D))			//When switch D is selected carry out the function

switch1_D();					//call function

}

}

void analog(void)

{

set_adc_channel(0);					//read adc channel 0

delay_us(30);					// wait 30uS

adc = read_adc();					// store value in variable ADC

}

void switch1_A(void)						//function for when switch is in A position

{

analog();						//call analog function

printf(lcd_putc,"\f");				//clear lcd screen

lcd_gotoxy(5,1);					// read the analog input

printf(lcd_putc,"value %u ",adc);			//print lcd message

printf("\fvalue %u \n\r",adc); 			//print v terminal message

output_b(adc);					//output vale to port b

delay_ms(500);					//delay 1 second

}

void switch1_B(void)						//function for when switch is in B postion

{

analog();					   	//call analog function

printf("\f");					//clear lcd screen

if(adc<75)						//if variable reistior value is less than 75

{

output_high(motA);					//turn motor clockwise

output_low(motB);					//

printf(lcd_putc,"\f");				//clear lcd screen

lcd_gotoxy(1,2);					//assign location of lcd message, line 2

printf(lcd_putc,"Clockwise %u",adc);		//print lcd message

delay_ms(500);  					//delay 0.5 seconds

}

else if(adc>175)					//if variable resistor value is greater than 175

{

output_low(motA);					//turn motor clockwise

output_high(motB);					//

printf(lcd_putc,"\f");				//clear lcd screen

lcd_gotoxy(1,2);					//assign location of lcd message, line 2

printf(lcd_putc,"Anti-Clockwise %u",adc);		//print lcd message

delay_ms(500);					//delay 0.5 seconds

}

else if(adc>=76 && adc<=174)			//if variable resistor value is between 76 & 174

{

output_low(motA);					//stop motor

output_low(motB);					//

printf(lcd_putc,"\f");				//clear lcd screen

lcd_gotoxy(1,2);					//assign location of lcd message, line 2

printf(lcd_putc,"Motor Stop");			//print lcd message

delay_ms(500);					//delay 0.5seconds

}

}

void switch1_C(void)						//function for when switch is in C position

{

if(input(but))					//if button is pressed

{

output_high(motA);				//turn motor clockwise

output_low(motB);				//

printf(lcd_putc,"\f");				//clear lcd screen

lcd_gotoxy(21,1);				//assign location of lcd message, line 3

printf(lcd_putc,"Button On");			//print lcd message

printf("\fButton On");				//print v terminal message

delay_ms(500);					//delat 0.5 seoconds

}

else						//when button is not pressed

{

output_low(motA);					//stop motor

output_low(motB);					//

printf(lcd_putc,"\f");				//clear lcd screen

lcd_gotoxy(21,1);					//assign location to print message, line 3

printf(lcd_putc,"Button Off");			//print lcd message

printf("\fButton Off");				//print v terminal message

delay_ms(500);

}

}

void switch1_D(void)						//function for when switch is in D position

{

analog();						//call analog function

for(int i=0;i<=15;i++)				//for loop which increments through array 'kitt'

{

output_b(kitt[i]);				//output value on 'b' pins

delay_ms(150);					//delay 0.15 seconds

printf(lcd_putc,"\f");				//clear lcd screen

lcd_gotoxy(25,2);				//assign location of lcd message, line 4

printf(lcd_putc,"Kitt Mode");			//print lcd message

printf("\fKitt Mode");				//print v terminal message

}

}

Screenshots