| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 3 | LEFEL 3 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| January 2022 | IONAWR 2022 |

MODULE CODE: ACHA3000

INTRODUCTION TO MATHS & SCIENCE

TIME:  1 ½ HOURS

answer all questions

Where needed, take the value of gravity to be 9.81m/s2

ANSWER ALL QUESTIONS

- Write down the units for the following:

- Area square meter(meter2) 								                [1 mark]

- Volume cube meter(meter3)								    [1 mark]

- Mass kg									    [1 mark]

- Force N Newton									    [1 mark]

- Weigh N Mewton								    [1 mark]

- Given the equations for density can be written as:

Density=MassVolume

D=m/v

Write down the units of density.						    [1 mark]

Kg/meter cube(kg/m3)

- Calculate the magnitude, direction and specify a bearing of the resultant for the following vectors:

a.

5N

10N						 [13 marks]

b.

15N							9N

12N						[15 marks]

- A ship is towed as a constant speed by two tug boats, each pulling the ship with a force of 10kN.  The angle between the tug boat cables is 50 degrees. Draw a vector diagram [2 marks] and calculate:

- The resultant force on the ship,						[6 marks]

- The drag force on the ship.						[2 marks]

- A uniform diving board has a length of 5m, and a weight of 250N.  It is bolted to the ground at one end, and projects a distance of 4m beyond the edge of the swimming pool.  A person of weight 500N stands on the free end of the diving board, draw a free body diagram of the diving board [4 marks] and calculate:

- The force on the bolts.							[3 marks]

- The force on the edge of the swimming pool.				[3 marks]

- A Circuit has a 12V battery as a DC source and 4 resistors. Resistors R2 and R3 are connected in parallel and in series with R1 and R4

a. Which direction is conventional current flow in ‘A’ positive to negative or ‘B’ negative to positive 										[1 mark]

b. What is the circuits total resistance 						[2 marks]

c. what is the current draw on the battery						[2 marks]

d. what is the voltage across the resistor (R4)				[2 marks]

e. what is the current through the  resistor (R2)				[3 marks]

f. what is the current through the  resistor (R3)				[2 marks]

g. Calculate which resistor transfers the highest amount of energy 		[3 marks]

Diwedd/End