#include <main.h>

#define LCD_ENABLE_PIN PIN_D2

#define LCD_RS_PIN PIN_D0

#define LCD_RW_PIN PIN_D1

#define LCD_DATA4 PIN_D4

#define LCD_DATA5 PIN_D5

#define LCD_DATA6 PIN_D6

#define LCD_DATA7 PIN_D7

#include <lcd.c>

//Variables 					// define functions

void analog(void);

void print2lcd(int);

void motor(void);

unsigned int value;

unsigned int adc;

void read_switch(void);

unsigned int switsh=0;

unsigned int switsh_way;

void terminal(void);

void night_rider(void);

unsigned int lightsB[8]={8,4,2,1};		// Array defining the states for the first set of traffic lights (lightsB), These values correspond to specific output configurations for controlling the lights

unsigned int lightsA[8]={128,64,32,16}; 	// Array defining the states for the second set of traffic lights (lightsA, These values correspond to specific output configurations for controlling the lights, The numbers are chosen because the system operates on an 8-bit board

unsigned int lightsB1[8]={1,2,4,8};		// Array defining the states for the first set of traffic lights (lightsB), These values correspond to specific output configurations for controlling the lights

unsigned int lightsA1[8]={16,32,64,128}; 	// Array defining the states for the second set of traffic lights (lightsA, These values correspond to specific output configurations for controlling the lights, The numbers are chosen because the system operates on an 8-bit board

void Button(void);

int last_button_state =0;

int kittModeDisplayed = 0;

void main()

{

setup_adc_ports(sAN0);							// initialise adc channel

setup_adc(ADC_CLOCK_INTERNAL|ADC_TAD_MUL_0);				// setup adc operation

lcd_init();							// initialise the LCD

while(TRUE)					// Start an infinite loop

{

{

analog();

if(input(pin_a2)) Motor();		// Check if the switch connected to pin a2

if(input(pin_a3)) terminal();		// Check if the switch connected to pin a3

if(input(pin_a4)) Button();		// Check if the switch connected to pin a4

if(input(pin_a5)) night_rider();	// Check if the switch connected to pin a5

}

}

}

void Motor()

{

{

lcd_gotoxy(1,1);			// Print to LCD display

printf(lcd_putc, " %3u",value);		// Display potenciometer value

}

if(value > 175) 				// If ADC value1 is greater than 175

{

output_high(Pin_C0);  			// Turn on motor (Pin C0)

output_low(Pin_C1);   			// Turn off motor (Pin C1)

lcd_gotoxy(21,1);

printf(lcd_putc, "Clockwise"); 		//display "Clockwise on LCD"

break;

}

else if(value < 75) 			 // If ADC value1 is less than 75

{

output_low(Pin_C0);   			// Turn off motor (Pin C0)

output_high(Pin_C1);  			// Turn on motor (Pin C1)

lcd_gotoxy(21,1);

printf(lcd_putc, "Anticlockwise");	 //display "Anti clockwise on LCD"

break;

}

else 					// If ADC value1 is between 76 - 174

{

output_low(Pin_C0);   			// Turn off motor (Pin C0)

output_low(Pin_C1);   			// Turn off motor (Pin C1)

lcd_gotoxy(21,1);

printf(lcd_putc, " Stopped      "); 		//display "Stopped on LCD"

}

}

void analog(void)

{

set_adc_channel(0);		//read adc channel 0

delay_us(30);			// wait

value = read_adc();		// store value in variable ADC

}

void terminal(void)

{

output_b(value & 0xff);		// This sends the ADC value to port B, only using the last 8 bits

lcd_gotoxy(7,1);			// This sets where the next text will appear on the LCD

printf(lcd_putc, " %3u",value);	// Display the ADC value on the LCD

delay_ms(200);			// wait

printf(lcd_putc, "\f");		// This command clears everything on the LCD

output_b(0x00);			// Set all bits of port B to low

}

void night_rider(void) 				// Function to control night rider style

{

unsigned int q;   				// Declare a variable 'q' for loop iteration

for (q=0;q<4;q++)			// Loop through the second set of lights (lightsA)

{

output_b(lightsA[q]);  			//output to port b

delay_ms(200);				// Delay 200ms

}

for (q=0;q<4;q++) 				// Declare a variable 'q' for loop iteration

{

output_b(lightsB[q]);  			//output to port b

delay_ms(200);				// Delay 200ms

}

for (q=0;q<4;q++) 			// Declare a variable 'q' for loop iteration

{

output_b(lightsB1[q]);  			//output to port b

delay_ms(200);				// Delay 200ms

}

for (q=0;q<4;q++)			// Loop through the second set of lights (lightsA)

{

output_b(lightsA1[q]);  			//output to port b

delay_ms(200);				// Delay 200ms

}

lcd_gotoxy(26,2);

printf(lcd_putc, "Kitt mode    ");	// display "Kitt mode"

// Check if the message has been displayed before

if (kittModeDisplayed == 0) {

printf("Kitt mode \r\n"); 					// Display "Kitt mode" only once

kittModeDisplayed = 1; 					// Set the flag to indicate the message has been displayed

}

if (input(pin_a1)) { 						// Assuming pin_a1 is the button

kittModeDisplayed = 0; 					// Reset the flag when the button is pressed

}

delay_ms(200);							//wait

}

void Button(void)

{

int current_button_state = input(pin_a1);			//read the curret button state

if(current_button_state != last_button_state)			//cheks if the current button state is not the same as its previous state

{

if (current_button_state)

{

printf(lcd_putc, "\f");

printf("Button on \r\n");			//print to the virutal terminal

output_low(PIN_C1);				//set pin C1 low for clockwise motor movement

output_high(PIN_C0);				//set pin C0 high for clockwise motor movement

lcd_gotoxy(24,1);				//go to center of line 3 on the LCD display

printf(lcd_putc,"Button pressed ");		//print to the LCD display

}

else

{

printf(lcd_putc, "\f");

printf("Button off \r\n");			//print to the virtual terminal

output_low(PIN_C1);				//set pin C1 low to turn the motor off

output_low(PIN_C0);				//set pin C1 low to turn the motor off

lcd_gotoxy(24,1);				//go to center of line 3 on the LCD display

printf(lcd_putc,"Button released ");		//print to the LCD display

}

last_button_state = current_button_state;		//update last button state

}

}

Comments

2.. 20   3.. 30    4.. 40   5.. 10

****************************************

2.. Your code has a number of structures and all work except you don't put the value of RV1 on the LEDS in section 1 and you don't output the values to the virtual terminal in section 2. Your statements indicate what each line does 65%

3.. Your program flows nicely 72%

4.. Your program mostly achieves the desired results. 68%

5.. Your program is nicely laid out and indented.72%