Conrod Dynamic Forces Report

Richard Osborne

1704921

Abstract

This report will show the forces acting upon a conrod during motion of an internal combustion engine. The specifications of components will be changed slightly to observe the difference in force generated. The report shows that upon making these changes, there is a considerable change in the maximum tensile force applied to the conrod.

Contents

Title									p1

Abstract & Contents						p2

List of figures							p3

Introduction & Methodology					p4

Analysis								p5

Conclusion								p13

References	 & Research/Literature				p14

List of Figures

Figure 1: Forces observed with standard component specifications (max rpm)

Fig 2: Forces observed with standard component specifications (cruising speed)

Fig 3: Forces observed with standard component specifications (low rpm)

Fig 4: Graph showing conrod oscillations at 6800rpm

Fig 5: Forces observed with smaller crank radius

Fig 6: Graph showing conrod oscillations with smaller crank radius

Fig 7: Forces observed with larger Crank Radius

Fig 8: Graph showing conrod oscillations with larger crank radius

Fig 9: Forces observed with a longer conrod

Fig 10: Graph showing conrod oscillations with longer conrod

Fig 11: Forces observed with shorter conrod

Fig 12: Graph showing conrod oscillations with shorter conrod

Fig 13: Forces observed with greater Piston mass

Fig 14: Graph showing conrod oscillations with greater Piston mass

Fig 15: Forces observed with less Piston mass

Fig 16: Graph showing conrod oscillations with less Piston mass

Fig 17: Forces observed with lighter piston and longer conrod

Fig 18: Forces observed with smaller crank radius and longer conrod

Fig 19: Forces observed with smaller crank radius, longer conrod and lighter piston

Introduction

This report will focus on observing the change in force acting upon a conrod inside of an internal combustion engine. A number of component specifications will be altered slightly from manufacturer standard.

In addition to component changes, the forces will be monitored at the maximum speed of 6800rpm, a cruising speed of 3350rpm and a low engine speed of 1200rpm. The maximum engine speed is the speed that will mainly be used, as it will give a better understanding of the force the conrod can handle.

The engine being used for this experiment is a Toyota 2JZ-GTE engine. This engine is commonly known for its application in the 1990’s MK4 Toyota Supra. This engine has also been used in the Toyota aristo V.

The purpose of this report is to see if there is any improvement that can be made in the form of a reduction in maximum tensile force that is applied to a conrod housed inside this engine.

Methodology

The way in which the forces will be monitored in this report is through spreadsheet and graph analysis using Microsoft Excel, the component data will be entered and altered to reveal the changes in maximum tensile force on a conrod during the four stroke cycle. Numerous spread sheets will be created and analysed to understand the changes that occur while adjusting the specifications.

Analysis

The engine chosen for this experiment was a Toyota 2JZ-GTE from a Mk4 Toyota Supra. The standard manufacturer specifications for the engine are as follows, Crank shaft radius (half stroke) - 43mm, conrod length - 142mm and Piston Mass - 0.37kg. These specifications were the base for this experiment. As shown in figures 1, 2 and 3, the specifications of the Crank shaft radius, conrod length and Mass of Piston remain as the standard manufacturer specification but the engine speed is altered.

Fig 1

Fig 2

Fig 3

Each of these graphs represents the force applied to the conrod at 6800rpm (maximum rpm), 3350rpm and 1200rpm. It was found that as the rpm was periodically decreased the maximum tensile force also decreased, going from 10497Newtons at 6800rpm to 2547N at 3350rpm and down as low as 327N at 1200rpm.

After discovering this, small alterations began to take place to observe what would happen to the maximum tensile force with the engine at the maximum rpm of 6800.

Fig 5

Firstly, as shown in Fig 5, by reducing the Crank shaft radius, the maximum tensile force reduced to 8161N.

Fig 4

The distance between the top of the conrod and the crankshaft with standard components can be seen in fig 4, where it was shown that there was a distance of 85mm between them.

Fig 6

However, as shown in fig 6, when the crank shaft radius was reduced from 43mm to 35mm (reduction of 8mm) the distance reduced to 69mm (a difference of 16mm).

Fig 7

Fig 7 shows the effects on force when the Crank shaft radius was increased to 50mm (7mm increase from standard), the maximum tensile force increased to 12645N.

Fig 8

The distance between the top of the conrod and the Crank shaft also increased to 99mm as shown here in fig 8.

After altering the Crank shaft radius, alterations to the length of the conrod took place. The first alteration was an increase in length by 12mm giving an overall conrod length of 154mm.

Fig 9

Fig 9 shows us that the maximum tensile force reduced slightly from 10479N to 10290N.

Fig 10

Fig 12

Figures 10 and 12 show us that the distance between the top of the conrod and the Crank shaft remains at 85mm, as found with standard spec, but at different peaks of height and depth. This was unexpected as it was believed that the overall distance would simply increase or decrease by the same amount that the conrod length was increased or decreased.

Fig 11

Figure 11 displays the maximum tensile force when the conrod length is reduced by 12mm to 130mm, the outcome was an increase to 10703N.

Fig 13

Figure 13 shows the effects that occurred when the Piston mass was increased by 0.08kg to 0.45kg. The result of this was an increase of the maximum tensile force to take it up to 12745N.

Fig 14

Fig 16

Figures 14 and 16 show the distance between the top of the conrod and the Crank shaft does not change at all when the Piston mass is altered.

Fig 15

In figure 15 the maximum tensile force on the conrod is dramatically reduced with the Piston mass much lighter at 0.29kg (0.08kg reduction) resulting in a force of 8213N.

After differing singular components, evidence would suggest that by altering multiple components would be beneficial to better the immense forces acting on the conrod.

Fig 17

Figure 17 represents the decrease in force when the Piston mass was reduced and the conrod length was increased, this is a great improvement over the force applied with standard component specifications, taking the force from 10479N, down to 8065N. Although this is a vast improvement, there was still better results to be achieved.

Fig 18

As shown in figure 18, by reducing the Crank radius to 35mm and keeping the conrod length extended to 154mm, the force reduced again to 8036N. But the largest reduction was found in Fig 19, when the Piston mass was also included at the lesser weight of 0.29kg. The maximum tensile force was then down as low as 6298N.

Fig 19

Conclusion

In this experiment it was found that while increasing the mass of the Piston, increasing the Crank shaft radius and reducing the length of the conrod the maximum tensile force acting on the conrod increased to levels that were unsettling for the strength of the conrod.

While doing the opposite, the maximum tensile force applied to the conrod was massively reduced.

If the components used could be made from lighter materials while still maintaining maximum strength, the longevity of the engine would improve, and higher power output could easily be achieved.

References

- http://my.is/forums/f114/piston-rod-weight-comparisons-oem-vs-aftermarket-433190/ JeffTsai, August 8, 2011

- http://www.maxspeedingrods.co.uk/high-performance-toyota-supra-2jzgte-2jz-2jzge-4340-en24-chrome-moly-forged-h-beam-connecting-rods-conrod-with-arp-bolts-x4pcs.html

- http://engineswork.com/engines/toyota-2jz-gte-2jz-ge-2jz-fse.html

Research/Literature

- http://www.superstreetonline.com/how-to/engine/1505-guide-to-understanding-the-toyota-2jz-gte-engine/ Aaron Bonk, Jun 29, 2015

- http://my.is/forums/f114/piston-rod-weight-comparisons-oem-vs-aftermarket-433190/ JeffTsai, August 8, 2011

- http://emweb.unl.edu/Mechanics-Pages/Luke-schreier/unzip/Tension%20and%20Compression%20in%20Connecting%20Rods%20VI.htm, Luke Schreier, April 26, 1999