| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 5 | LEFEL 5 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| January 2025 | Ionawr 2025 |

MODULE CODE: ACDA5001

Electrical Engineering

TIME:  2 HOURS

answer ALL THE questions

Equipment or materials required – Calculator

Adnoddau neu ddefnyddiau sydd eu hangen - Cyfrifiannell

ANSWER ALL THE QUESTIONS

Question 1 [25 marks]

- An Alternating voltage, used to power an AC motor is described by the equation V=100Sin (150πt-0.25). Determine the following parameters to evaluate the motor’s power supply characteristics.

- Peak to peak voltage							[2 Marks]

- RMS voltage								[2 Marks]

- Frequency									[2 Marks]

- Phase Angle								[2 Marks]

Answer:

- peak to peak voltage

Peak to Peak Voltage  = 2×Amplitude =2×100		[1 Mark]

Peak to Peak Voltage=200V						[1 Mark]

- RMS value

VRMS=Vm2=1002 								[1 Mark]

VRMS=70.7V								[1 Mark]

- Frequency

We know that ω=2πf=150π

2πf=150π 								[1 Mark]

2f=150

f=75Hz									[1 Mark]

- Phase angle

ϕ=0.25 radians lagging

0.25 radians=0.25×180πo						[1 Mark]

ϕ=14.32o								[1 Mark]

- A coil with an inductance of 0.1H and a resistance of 5Ω is used in a relay circuit. When this circuit is switched on to a 220V supply. Calculate the steady state current in the circuit and the rate of rise of current at t=0 and at t=2τ and analyse the results. [where τ=LR]	[13 Marks]

Answer:

The Steady state current  I0=VR=2205=44A					[2 Marks]

To Find rate of Rise of current at t=0

The rise in Current is given by

i=I01-e-tτ								[1 Mark]

i=VR1-e-RtL

i=22051-e-5t0.1								[1 Mark]

i=441-e-50t

i=44-44e-50t

The rate of change of Current with respect to time can be determined by differentiating the above equation with respect to t.

didt=0-44(-50)e-50t								[2 Marks]

didt=2200e-50t

At t=0

didt=2200e-50×0 								[1 Mark]

didt=2200e0

didt=2200 A/Sec

At t=0, the rate of rise of Current is 2200A/Sec					[2 Marks]

At t=2τ

didt=2200e-50×2τ 								[1 Mark]

didt=2200e-50×2LR

didt=2200e-50×20.15

didt=2200e-2

didt=2200×0.135

didt=297A/Sec

At t=2τ, the rate of rise of Current is 297A/Sec					[2 Marks]

Rate of rise of current has reduced when t=2τ					[1 Mark]

- Describe eddy current loss in a transformer. List one technique to minimise eddy current loss.		[4 Marks]

Answers:

Eddy current loss in a transformer occurs when circulating currents, known as eddy currents, are induced in the core due to the alternating magnetic field. These currents generate heat as they flow through the core's resistance, leading to energy loss.		[3 Marks]

Eddy current loss can be minimised by laminating the core.		[1 Mark]

Question 2 [25 marks]

- A supply voltage of V1=25V is given to the circuit shown in fig:1. Determine the following

- Calculate the load current.						[9 Marks]

- Determine the value of load resistor for maximum power transfer across the load.										[3 Marks]

- Draw a simplified circuit diagram for the circuit shown in fig.1	[3 Marks]

- Calculate the Maximum power transferred across the load (PL).	[4 Marks]

Fig: 1

Answer:

- The Load Current,

Voltage across 25Ω resistor,

To find current in the loop, I=2535=0.71A

V25Ω=0.71×25=17.75V=Vth 	 						[3 Marks]

To Find Rth

R1 parallel to R3=7.143Ω

7.143Ω series with R2=22.143 Ω

Rth=22.143Ω 								[3 Marks]

IL=VthRth+RL=17.7522.14+22.14=0.401A 						[3 Marks]

- Value of load resistor, when maximum power is transferred across the load,

According to Maximum power transfer theorem, maximum power will be transferred across the load, when the Load resistor is equal to internal resistance. Hence Internal resistance is 22.143 Ω										[3 Marks]

- Draw a simplified circuit diagram for the circuit shown in fig.1

[3 Marks]

- The maximum power transferred across the load (PL).

PL=IL2RL=0.2422×22.143							[3 Marks]

=1.2968W									[1 Mark]

This is the maximum power transfered across the load.

- Describe the functions of the following components used in DC machines

- Field System								[3 Marks]

- Armature									[3 Marks]

Answers:

  - Field System

It consists of number of poles bolted inside the circular frame .It provide uniform magnetic field.									[1 Mark]

The mmf (magnetomotive force) produced by the field windings provides a magnetic flux that passes through the pole pieces, the air gap, the armature and the frame.												[2 Marks]

  - Armature

The armature consists of coils of wire wound around a metal core. When the armature rotates within the magnetic field created by the stationary field magnets, an electromagnetic force (EMF) is induced in the armature coils due to electromagnetic induction. 									[3 Marks]

Question 3 [25 marks]

- A 400V, three phase star connected alternator supplies a delta – connected load as shown in fig: 2. Each phase of which has a resistance of 30Ω and inductive reactance 40Ω. Analyse the system and:

- Calculate the current supplied by the alternator,			[7 Marks]

- Determine the output power of the alternator,				[4 Marks]

- Calculate the Apparent power (kVA) of the alternator.		[3 Marks]

Neglect the losses in the line between the alternator and the load.

Fig: 2

Answer:

- the current supplied by the alternator,

Consider the Load: Phase Current =Ip=VpZp

Vp=VL for delta connection. Hence Vp=400V					[1 Mark]

Phase impedance, Zp=(Rp2+XL2)=302+402=50Ω			[2 Marks]

Hence, Ip=VpZp=40050=8A 								[2 Marks]

For delta connection line current, IL=3Ip=3×8=13.86A			[2 Marks]

13.86A is the current supplied by the alternator.

- the output power of the alternator,

Alternator output power is equal to the power dissipated by the load .

P=3VLILCosϕ									[1 Mark]

Where cosϕ=RPZp=3050

cosϕ=0.6										[1 Mark]

Hence, P=3×400×13.86×0.6						[1 Mark]

P=5.76KW										[1 Mark]

- the kVA of the alternator.

Alternator Output kVA, S=3VLIL							[1 Mark]

S=340013.86									[1 Mark]

S=9.60kVA										[1 Mark]

- A 415V, three phase AC motor has a power output of 12.75kW and operates at a power factor of 0.77 lagging and with an efficiency of 85%. If the motor is delta-connected, Apply you knowledge to determine the following,

- Input Power 		[3 Marks]

- Line current 		[3 Marks]

Answer:

- The Power input

Efficiency=Power OutputPower Input

85100=12750Power input		[1 Mark]

Power Input=12750 ×10085=15000W		[2 Marks]

- The Line current

IL=P34150.77=1500034150.77		[2 Marks]

IL=27.10A		[1 Mark]

- A generator is connected to a 60Ω load and supplied with a current of 8A. If the armature resistance is 5Ω. Calculate the terminal voltage and the generated e.m.f.											[5 Marks]

Answer:

Terminal Voltage: V=IaRL=8×60						[1 Mark]

V=480V										[1 Mark]

Generated EMF, E=V+IaRa=480+(8×5)					[2 Marks]

E=520Volts										[1 Mark]

Question 4 [25 marks]

- A RLC series circuit consists of R=75Ω, L=125mH and C=200μF. The circuit is excited by a sinusoidal source of voltage 115V, 60Hz. Using Complex functions analyse and determine the following.

- Impedance in the circuit		[7 Marks]

- Current across the circuit						[2 Marks]

- Voltage across each element		[6 Marks]

- Apparent power								[2 Marks]

- Active Power								[2 Marks]

- Reactive power								[2 Marks]

- Construct a Phasor diagram to represent the circuit			[4 Marks]

Answers:

- Impedance in the circuit

Inductive Reactance= jXL=j2πfL

=j2π×60×125×10-3=j47.124Ω		[1 Mark] Capacitive  Reactance= -jXC=-j12πfc

-jXC=-j12π×60×200×10-6=-j13.263Ω 		[1 Mark]

Total Reactance=jX=jXL-jXC=j47.124-j13.263=j33.861Ω 		[1 Mark]

Impedance, Z=R+jX=75+j33.861		[1 Mark]

To find the polar form

Z=R2+X2=752+33.8612=82.29Ω		[1 Mark]

ϕ=tan-1XR=tan-133.86175=24.3o		[1 Mark]

Impedance=82.29∠24.3oΩ		[1 Mark]

- Current across the circuit

Voltage= 115∠0oV

By Ohm’s Law,

I=VZ=115∠0o82.29∠24.3o=1.397∠0o-24.3o=1.397∠-24.3oA 		[2 Marks]

- Voltage across each element

VR=I×R=1.397∠-24.3o×75=104.78∠-24.3oV 		[2 Marks]

VL=I×jXL=1.397∠-24.3o×j47.124

VL=1.397∠-24.3o×47.124 ∠90o=65.832 ∠65.7oV		[2 Marks]

VC=I×(-jXC)=1.397∠-24.3o×(-j13.263)

VC=1.397∠-24.3o×13.263 ∠-90o=18.528 ∠-114.3oV		[2 Marks]

- Apparent power

S=VI=115×1.397=160.66VA		[2 Marks]

- Active Power

P=VICosϕ=115×1.397Cos24.3o=146.47W		[2 Marks]

- Reactive power

Q=VISinϕ=115×1.397sin24.3o=66.1VAR		[2 Marks]

- Construct a Phasor diagram to represent the circuit

[4 Marks]

Diwedd/End