Applied Computing

Wales Institute of Science and Art (WISA)

# Advanced Networking (ACCA5024)

Lab 8: Static Routing

n.kaliyamurthy@uwtsd.ac.uk                                                                                                                            1

Design the topology and execute basic configuration as stated below

Refer to the above topology for addressing information.

Tasks:

- Set Up the Topology and Initialize Devices

- Configure Devices and Verify Connectivity

Assign static IPv4 information to the PC interfaces.

Configure basic router settings. (Refer to our previous labs. Basic Configurations includes hostname, passwords, line configurations, device security, domain lookups, banners)

Configuring your network with complete information is highly recommended.

Verify network connectivity.

- Configure Static, Default and Loopback addresses for all the routers Configure IPv4 Address for all the router interfaces.

R1 interface GigabitEthernet0/0 description S1 subnet ip address 192.168.10.1 255.255.255.0 duplex auto speed auto ! interface GigabitEthernet0/1 no ip address duplex auto speed auto shutdown ! interface GigabitEthernet0/2 no ip address duplex auto speed auto shutdown ! interface Serial0/3/0 description int to R2 ip address 10.0.0.1 255.255.255.252 clock rate 2000000 ! interface Serial0/3/1 no ip address clock rate 2000000 shutdown ! R2 interface GigabitEthernet0/0 description S2 subnet ip address 192.168.20.1 255.255.255.0 duplex auto speed auto ! interface GigabitEthernet0/1 no ip address duplex auto speed auto shutdown ! interface GigabitEthernet0/2 no ip address duplex auto speed auto shutdown ! interface Serial0/3/0 description to R1 ip address 10.0.0.2 255.255.255.252 ! interface Serial0/3/1 description to R3 ip address 20.0.0.1 255.255.255.252 ! R3 interface GigabitEthernet0/0 description S3 subnet ip address 192.168.30.1 255.255.255.0 duplex auto speed auto ! interface GigabitEthernet0/1 no ip address duplex auto speed auto shutdown ! interface GigabitEthernet0/2 no ip address duplex auto speed auto shutdown ! interface Serial0/3/0 no ip address clock rate 2000000 shutdown ! interface Serial0/3/1 description to R2 ip address 20.0.0.2 255.255.255.252 clock rate 2000000 !

Configure loopback address (Lo1) as 50.50.50.50/32 at Router R2.

interface Loopback0

description Loopback

ip address 55.55.55.55 255.255.255.255

!

On R1, R2 and R3 configure IPv4 static route to connect other networks. Do not include Lo1 network.

R1 Routing Table Gateway of last resort is 10.0.0.2 to network 0.0.0.0 10.0.0.0/8 is variably subnetted, 2 subnets, 2 masks C 10.0.0.0/30 is directly connected, Serial0/3/0 L 10.0.0.1/32 is directly connected, Serial0/3/0 20.0.0.0/30 is subnetted, 1 subnets S 20.0.0.0/30 [1/0] via 10.0.0.2 192.168.10.0/24 is variably subnetted, 2 subnets, 2 masks C 192.168.10.0/24 is directly connected, GigabitEthernet0/0 L 192.168.10.1/32 is directly connected, GigabitEthernet0/0 S 192.168.20.0/24 [1/0] via 10.0.0.2 S 192.168.30.0/24 [1/0] via 10.0.0.2 S* 0.0.0.0/0 [1/0] via 10.0.0.2 R2 Routing Table Gateway of last resort is not set 10.0.0.0/8 is variably subnetted, 2 subnets, 2 masks C 10.0.0.0/30 is directly connected, Serial0/3/0 L 10.0.0.2/32 is directly connected, Serial0/3/0 20.0.0.0/8 is variably subnetted, 2 subnets, 2 masks C 20.0.0.0/30 is directly connected, Serial0/3/1 L 20.0.0.1/32 is directly connected, Serial0/3/1 55.0.0.0/32 is subnetted, 1 subnets C 55.55.55.55/32 is directly connected, Loopback0 S 192.168.10.0/24 [1/0] via 10.0.0.1 192.168.20.0/24 is variably subnetted, 2 subnets, 2 masks C 192.168.20.0/24 is directly connected, GigabitEthernet0/0 L 192.168.20.1/32 is directly connected, GigabitEthernet0/0 S 192.168.30.0/24 [1/0] via 20.0.0.2 R3 Routing Table Gateway of last resort is 20.0.0.1 to network 0.0.0.0 10.0.0.0/30 is subnetted, 1 subnets S 10.0.0.0/30 [1/0] via 20.0.0.1 20.0.0.0/8 is variably subnetted, 2 subnets, 2 masks C 20.0.0.0/30 is directly connected, Serial0/3/1 L 20.0.0.2/32 is directly connected, Serial0/3/1 S 192.168.10.0/24 [1/0] via 20.0.0.1 S 192.168.20.0/24 [1/0] via 20.0.0.1 192.168.30.0/24 is variably subnetted, 2 subnets, 2 masks C 192.168.30.0/24 is directly connected, GigabitEthernet0/0 L 192.168.30.1/32 is directly connected, GigabitEthernet0/0 S* 0.0.0.0/0 [1/0] via 20.0.0.1

Configure default static route to reach Lo1 network using next-hop ip address at R1

R1 Routing Table

Gateway of last resort is 10.0.0.2 to network 0.0.0.0

S* 0.0.0.0/0 [1/0] via 10.0.0.2

Configure default static route to reach Lo1 network using exit interface at R3

R3 Routing Table

Gateway of last resort is 20.0.0.1 to network 0.0.0.0

S* 0.0.0.0/0 [1/0] via 20.0.0.1

Interpret the output from the routing table.

L - local C - connected S - static * - candidate default This means that C shows adjacent subnets, L generally shows the interface for that adjacent subnet, and S shows manually entered static routes, and S* shows the manually entered default static route.

Verify the status of the interfaces.

All non-used interfaces are set to down, and all in use are set to up as seen above

Verify the status of the network connectivity.

From Lap1: C:\>ping 10.0.0.2 Pinging 10.0.0.2 with 32 bytes of data: Reply from 10.0.0.2: bytes=32 time=1ms TTL=254 Reply from 10.0.0.2: bytes=32 time=1ms TTL=254 Reply from 10.0.0.2: bytes=32 time=2ms TTL=254 Reply from 10.0.0.2: bytes=32 time=2ms TTL=254 Ping statistics for 10.0.0.2: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 1ms, Maximum = 2ms, Average = 1ms C:\>ping 20.0.0.2 Pinging 20.0.0.2 with 32 bytes of data: Reply from 20.0.0.2: bytes=32 time=15ms TTL=253 Reply from 20.0.0.2: bytes=32 time=4ms TTL=253 Reply from 20.0.0.2: bytes=32 time=3ms TTL=253 Reply from 20.0.0.2: bytes=32 time=2ms TTL=253 Ping statistics for 20.0.0.2: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 2ms, Maximum = 15ms, Average = 6ms C:\>ping 192.168.20.11 Pinging 192.168.20.11 with 32 bytes of data: Request timed out. Reply from 192.168.20.11: bytes=32 time=10ms TTL=126 Reply from 192.168.20.11: bytes=32 time=1ms TTL=126 Reply from 192.168.20.11: bytes=32 time=1ms TTL=126 Ping statistics for 192.168.20.11: Packets: Sent = 4, Received = 3, Lost = 1 (25% loss), Approximate round trip times in milli-seconds: Minimum = 1ms, Maximum = 10ms, Average = 4ms C:\>ping 192.168.30.11 Pinging 192.168.30.11 with 32 bytes of data: Request timed out. Reply from 192.168.30.11: bytes=32 time=2ms TTL=125 Reply from 192.168.30.11: bytes=32 time=12ms TTL=125 Reply from 192.168.30.11: bytes=32 time=2ms TTL=125 Ping statistics for 192.168.30.11: Packets: Sent = 4, Received = 3, Lost = 1 (25% loss), Approximate round trip times in milli-seconds: Minimum = 2ms, Maximum = 12ms, Average = 5ms This shows that all subnets can communicate with each other

Record your routers running configurations here!!!  Do it separately for R1, R2 and R3

R1 running configuration:

Building configuration...

Current configuration : 1711 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

security passwords min-length 7

!

hostname R1

!

login block-for 120 attempts 3 within 60

!

!

enable secret 5 $1$mERr$EJnmB234UvJf9yoQMWYJK/

!

!

!

!

!

!

no ip cef

no ipv6 cef

!

!

!

username admin password 7 0822455D0A1613030B1B0D1739

!

!

license udi pid CISCO2911/K9 sn FTX15246LYN-

!

!

!

!

!

!

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name networking.com

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0

description S1 subnet

ip address 192.168.10.1 255.255.255.0

duplex auto

speed auto

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/3/0

description int to R2

ip address 10.0.0.1 255.255.255.252

clock rate 2000000

!

interface Serial0/3/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

ip classless

ip route 0.0.0.0 0.0.0.0 10.0.0.2

ip route 192.168.20.0 255.255.255.0 10.0.0.2

ip route 192.168.30.0 255.255.255.0 10.0.0.2

ip route 20.0.0.0 255.255.255.252 10.0.0.2

ip route 0.0.0.0 0.0.0.0 55.55.55.55

!

ip flow-export version 9

!

!

ip access-list extended sl_def_acl

deny tcp any any eq telnet

deny tcp any any eq www

deny tcp any any eq 22

permit tcp any any eq 22

!

no cdp run

!

banner motd ^CThis is a secure system. Authorized Access Only^C

!

!

!

!

!

line con 0

exec-timeout 5 0

password 7 0822455D0A1606181C1B0D1739

login

!

line aux 0

!

line vty 0 4

exec-timeout 5 0

password 7 0822455D0A1613030B1B0D1739

login local

transport input ssh

!

!

!

end

R2 running configuration:

Building configuration...

Current configuration : 1644 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

security passwords min-length 7

!

hostname R2

!

login block-for 120 attempts 3 within 60

!

!

enable secret 5 $1$mERr$EJnmB234UvJf9yoQMWYJK/

!

!

!

!

!

!

ip cef

no ipv6 cef

!

!

!

username admin password 7 0822455D0A1613030B1B0D1739

!

!

license udi pid CISCO2911/K9 sn FTX152487S3-

!

!

!

!

!

!

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name networking.com

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface Loopback0

description Loopback

ip address 55.55.55.55 255.255.255.255

!

interface GigabitEthernet0/0

ip address 192.168.20.1 255.255.255.0

duplex auto

speed auto

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/3/0

ip address 10.0.0.2 255.255.255.252

!

interface Serial0/3/1

ip address 20.0.0.1 255.255.255.252

!

interface Vlan1

no ip address

shutdown

!

ip classless

ip route 192.168.10.0 255.255.255.0 10.0.0.1

ip route 192.168.30.0 255.255.255.0 20.0.0.2

ip route 192.168.20.0 255.255.255.0 GigabitEthernet0/0

!

ip flow-export version 9

!

!

ip access-list extended sl_def_acl

deny tcp any any eq telnet

deny tcp any any eq www

deny tcp any any eq 22

permit tcp any any eq 22

!

banner motd ^CThis is a secure system. Authorised access only^C

!

!

!

!

!

line con 0

exec-timeout 5 0

password 7 0822455D0A1606181C1B0D1739

login

!

line aux 0

!

line vty 0 4

exec-timeout 5 0

password 7 0822455D0A1613030B1B0D1739

login local

transport input ssh

!

!

!

end

R3 running configuration:

Building configuration...

Current configuration : 1627 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

security passwords min-length 7

!

hostname R3

!

login block-for 120 attempts 3 within 60

!

!

enable secret 5 $1$mERr$EJnmB234UvJf9yoQMWYJK/

!

!

!

!

!

!

no ip cef

no ipv6 cef

!

!

!

username admin password 7 0822455D0A1613030B1B0D1739

!

!

license udi pid CISCO2911/K9 sn FTX1524CV8A-

!

!

!

!

!

!

!

!

!

ip ssh version 2

no ip domain-lookup

ip domain-name networking.com

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0

ip address 192.168.30.1 255.255.255.0

duplex auto

speed auto

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/3/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/3/1

ip address 20.0.0.2 255.255.255.252

clock rate 2000000

!

interface Vlan1

no ip address

shutdown

!

ip classless

ip route 0.0.0.0 0.0.0.0 20.0.0.1

ip route 192.168.10.0 255.255.255.0 20.0.0.1

ip route 192.168.20.0 255.255.255.0 20.0.0.1

ip route 10.0.0.0 255.255.255.252 20.0.0.1

!

ip flow-export version 9

!

!

ip access-list extended sl_def_acl

deny tcp any any eq telnet

deny tcp any any eq www

deny tcp any any eq 22

permit tcp any any eq 22

!

no cdp run

!

banner motd ^CThis is a secure system. Authorized Access Only^C

!

!

!

!

!

line con 0

exec-timeout 5 0

password 7 0822455D0A1606181C1B0D1739

login

!

line aux 0

!

line vty 0 4

exec-timeout 5 0

password 7 0822455D0A1613030B1B0D1739

login local

transport input ssh

!

!

!

end