Recap IPV4 Addressing

- For the following IP addresses deduce the network ID and Broadacast address

- 192.168.10.253 /24

- 192.168.10.253 /25

- 192.168.10.253 /26

- 192.168.10.253 /27

- 192.168.10.253 /30

NETWORK  TOPOLOGY

Recap -OSPF

Work in groups of 3

- Use Cisco Real Devices to implement the given Topology

- Design and assign IPV4 Addresses to topology devices and links

- Configure  Single Area OSPF

RID for R1 is 1.1.1.1

RID for R2 is 2.2.2.2

RID for R3  is 3.3.3.3

RID for  R4 is 4.4.4.4.

- Make  sure PCs can ping each other

- Identify DR, BDR and Drothers