University of Wales Trinity Saint David

School of Engineering

Heat Exchanger Laboratory Analysis

Jeffrey Elliss

1802644

Motorsport Engineering Beng

24/04/20

Thermofluid Mechanics, M5X00406

# Contents

Table of Contents

Contents	1

Introduction	3

Method	3

Heat Exchanger Basic Principles	4

Heat Exchanger Equations	4

Data	6

6

8

Conclusion	9

Evaluation	10

# Introduction

Heat exchanges have a multitude of purposes and have been used for a long period of time. From steam trains to the most modern hybrid supercars different forms of heat exchangers have been used to move energy in different ways. In a steam train a parallel flow heat exchanger is used to boil the water using the heat from a fire. The water becomes steam expanding and drives the pistons providing a driving force through a crank to the trains wheels. In modern vehicles radiators and oil coolers are used these are cross flow heat exchangers where the air flow is perpendicular to the water flow.

# Method

Experiments 1 and 2

- Setup the TD1007 Heat exchanger as specified by the TD1007_HX laboratory guide.

- Note the ambient air pressure, this will be used to determine the air flow rate.

- Check and note the orifice diameter of the air inlet. It is stamped on the top of the orifice plate.

- Turn on the fan and move the outlet plate until the pressure gauge reads 250Pa with a tolerance of +-5 Pa.

- Turn on the pump and use the dial to adjust it to read 0.25l/minute with a tolerance of +- 0.01L/min.

- Once the reading is taken move the outlet plate to adjust the air pressure. Between readings ensure that the temperature readings have settled and are no longer fluctuating.

- Repeat the process gradually reducing the air pressure until all the required readings have been taken.

Experiment 3

1) Setup the TD1007 Heat exchanger as specified by the TD1007_HX laboratory guide.

2) Note the ambient air pressure, this will be used to determine the air flow rate.

3) Check and note the orifice diameter of the air inlet. It is stamped on the top of the orifice plate.

4) Turn on the fan and move the outlet plate until the pressure gauge reads 250Pa with a tolerance of +-5 Pa.

5) Turn on the pump and use the dial to adjust it to read 2.25l/minute with a tolerance of +- 0.01L/min. Note here that the pump would not go over 2l/min and as such an extra reading was taken at a lower value to ensure sufficient data was collected.

6) Between readings ensure that the temperature readings have settled and are no longer fluctuating.

7)Once the reading is taken adjust the flow rate to the next measurement and check that the other variables are steady.

8) Repeat steps 6 to 7 ensuring the air pressure is correct at the start of each test and adjusting the water flow according to your chosen values.

# Heat Exchanger Basic Principles

A heat exchanger is used to move energy between one flow and another either to heat or cool the fluids. For example, in a refrigerator a heat exchanger moves the energy from inside the fridge to outside cooling the interior. Or in a steam train where the heat from a coal fire is used to heat water flowing through tubes in a boiler turning it to steam creating pressure differentials which are used to drive the wheels.

The amount of energy that can be transferred by a heat exchanger is determined by a few factors, the temperature gradient between the two flows, the length of the tubes and the direction of the flows relative to each other. There are three main types of heat exchanger parallel flow, counter flow and cross flow. In a parallel flow heat exchanger both flows run in the same direction, in a counter flow heat exchanger the flows run opposite to each other and in a cross flow heat exchanger such as in a car radiator the flows are perpendicular to each other.

# Heat Exchanger Equations

In order to evaluate a heat exchanger, we must calculate the amount of energy transferred and the rate at which it is transferred.

Q=U*A*∆TEquation 1-Heat Transfer rate

Equation 1 where Q is heat transfer rate in Watts, U is the overall heat transfer coefficient, A is the heat transfer area and delta T is the change in temperature. This equation comes from basic thermodynamics principles and is common to all heat exchangers.

∆TLM=∆To-∆TilnΔTo∆TiEquation 2-Log mean temperature difference

Equation 2 shows how to calculate the log mean temperature difference, this is useful for accounting for the curve of the temperature graphs between the hot and cold flow streams.

From the first law of thermodynamics the energy contained in a flowing fluid is proportional to the specific heat capacity of the fluid, the amount of fluid flowing and the temperature gradient. As such equation 3 shows the rate of heat transfer.

Q=mcp∆TEquation 3-Heat transfer rate

Equation 3 is our fundamental equation for showing the amount of energy transferred between flows and therefore showing how effective a heat exchanger is.

ρw=(999.839+16.952T-7.990*10-3T2-46.25*10-6T3+105.846*10-9T4-281.030*10-12T5/(1+16.887*10-3T))	Equation 4-Water Density

Water density varies with temperature to correct for this equation 4 Is used.

The specific heat capacity of the two fluids is essential to accurately determining the heat exchangers operation characteristics. The specific heat capacity of air remains relatively constant and is assumed to be 1005j/kgK in the calculations. However. the specific heat capacity of water varies with temperature more significantly. Equation 5 uses a known point at 15 Celsius to calculate the specific heat capacity of water at different temperatures. At this temperature the specific heat capacity of water is 4185.5 J/kgK.

(CpwAt T)CpwAt 15C=0.996185+0.0002874T+1001005.26+0.01160*10-0.036TEquation 5-Specific heat capacity of water at varying temperature and constant pressure

Efficiencies (temp efficiencies and energy efficiencies)

ηw=Tw1-Tw2Tw1-Ta1*100	Equation 6-Temperature efficiency water

Equation 6 calculates the temperature efficiency of the water stream, a ratio between the initial difference between the water and air temperatures and the difference between the start and final water temperatures.

ηa=Ta2-Ta1Tw1-Ta1	Equation 7-Temperature efficiency air

Equation 7 calculates the temperature efficiency of the air stream, a ratio between the initial difference between the water and air temperatures and the difference between the start and final air temperatures.

η=ηw+ηa2	Equation 8-Mean temperature efficiency

Equation 8 shows the mean temperature efficiency of the two streams, this gives a more accurate picture of the effectiveness of the heat exchanger at different temperatures. Averaging across the two streams means that if one stream is saturated or the efficiency of one stream is significantly reduced when the other is increased the efficiency of the whole system is reflected.

U=Qw/(A*LMTD)		Equation 9-Overall heat transfer coefficient

Equation 9 calculates the overall heat transfer coefficient in W/M^2K allowing for better analysis of the capabilities and efficiency of a heat exchanger by showing the amount of heat transfer relative to the mean area over which the heat transfer is occurring, and log mean temperature difference. This allows different types of heat exchanger to be compared more easily.

# Data

Figure 1-Log mean temperature difference for experiments 1 and 2

Figure 1 shows the differences in log mean temperature difference caused by the differing initial water temperatures.

Figure 2-Heat transfer water against air.

Figure 2 plots the heat transfer from the air and the water against the water flow, the air values follow a smooth curve as expected however the water values have two anomalies in them. These are most likely due to reading errors or changes in ambient conditions. In spite of the errors it is clear that the energy moves from one stream to the other as both data sets follow the same trend.

Figure 3-Experiment 1 temperature efficiencies

Figure 3 shows an increase in mean and water temperature efficiency with the increased air flow rate, however the temperature efficiency of the air notably reduces. As the airflow increases the efficiency of the water stream increases as the amount of heat leaving the water stream increases.

Figure 4-Experiment 2 temperature efficiencies

Figure 4 shows a small increase in efficiency as the air flow rate increases. Increased air flow increases the total amount of heat energy that can be transferred and as air flow speed increases it becomes more likely that the flow will be turbulent allowing for greater heat transfer again. The efficiency of the air stream reduces in the same way as in experiment 1.

Figure 5-Experiment 3 temperature efficiencies

Figure 5 shows that increasing the water flow rate reduces the overall temperature efficiency of this heat exchanger. The trend of the water and air temperature efficiencies is inverted compared to experiment 1 and 2.

Figure 6-Heat transfer coefficient for experiments 1 and 2

Figure 6 shows the heat transfer coefficients from experiments 1 and 2 against the outlet pressure which is directly linked to the flow rate through the system. Experiment 1 shows a slightly lower peak U of 433 W/m^2K compared to experiment two’s 469 W/m^2K. This is due to the higher temperature gradient in experiment 2 with a Tw1 of 60.3 degrees compared to a Tw1 of 49.4 degrees the starting temperature difference leads to a greater LMTD in experiment 2.

Figure 7-Experiment 3 heat transfer coefficient

Figure 7 again shows some odd points in the experiment 3 data set. The heat transfer coefficient shows a general trend of increasing with increased water flow. However, at the peak flow the heat transfer coefficient reduces. As well as another dip at 1.25 l/min, it is likely that these are data collection or experimental errors. Potential errors may come from fluctuations in ambient temperature or pressure, human error (misreading the equipment or inaccurate inputs such as setting pressure or flow rate out of the recommended tolerances). This data set should be taken again to confirm that the trend shown is accurate as there are obvious anomalous results.

Varying the water flow appears to have a much smaller effect on the heat transfer coefficient than varying the air flow rate. Experiment 3 where the water flow is varied shows a change of 7 W/m^2K whereas experiments 1 and 2 have a much wider range of 175 W/m^2K.

# Conclusion

In experiments 1 and 2 there is a large disparity between the energy lost by the hot stream and the energy absorbed by the cold stream. However, in experiment 3 the energy lost by the hot steam is approximately the same as the energy absorbed by the cold stream. As the two setups are the same and only one variable is changed in each experiment it is unusual that such a disparity of results is seen and is likely due to an error.

The performance of any give heat exchanger is dependent on a few variables, the mass flow rate of each of the streams, the layout of the heat exchanger, the effective area over which heat transfer takes place and the temperature gradient between the streams. Higher flow rates in either stream increase the amount of heat transfer in the heat exchanger. Varying the air flow rate has a greater effect on the amount of heat transfer than varying the water flow rate.

# Evaluation

The results of the first two experiments appear to be consistent and show obvious and strong trends. However, the results of experiment 3 while showing trends similar to those in the first two experiments have obvious errors in the heat transfer of the water flow. Ideally each experiment would be repeated three times to confirm the measurements.

When conducting experiment, the original plan suggests taking 5 readings ranging from 0.25 l/min to 2.25l/min. This was not possible as the equipment would not pump above 2l/min. With this in mind it was necessary to take an extra reading at 0.75l/min to ensure that there were sufficient data points to make any trend obvious and to give a greater degree of certainty in the results.