- Passwords of the devices should be changed and should all be different and stronger.

- The switches should not use the default SVI and management VLAN

- The switchports should use different VLAN’s instead of VLAN 1

- Switchports that are not used should be shut down and moved to an unused VLAN.

- Port security should be used

- DTP negotiation being active on trunk ports is a security risk.

- OSPF is active on the LAN interfaces. Passive interfaces reduce unnecessary traffic on the network.

- SSH is not being used on the network.

- 802.1x should be implemented.

- IP source guard should be implemented

- CDP should be turned off or restricted

- DTP is active except on ports connected to PC4 and PC5.

- Legacy STP that is being used on switches should be changed to RSTP

- Trunk links are using Native VLAN 1 which should be changed

- VLAN 99 is only created on SWB2 and its not being used nor does it have a name so it should be removed.