Tomos Hall

2219188@student.uwtsd.ac.uk

Assignment 1 – November 2023

Electronic systems design

# Abstract

A Data acquisition sensor was designed to measure different variables in a two-hour test drive. Sensors were selected to measure engine rpm, engine temperature, suspension travel, pitch roll and yaw, and road speed.

A circuit was designed to acquire data from the selected sensors and store it to an SD card via a PIC microcontroller.

Table of Contents

0

Abstract	1

Introduction	3

Task 1	4

Designing a data acquisition system	0

Engine RPM	0

Vehicle road speed (MPH)	1

Engine temperature	1

Suspension travel	2

Pitch, roll and yaw	2

Microcontroller selection	3

Connector selection	3

Storage requirements	4

Power supply selection	0

Signal conditioning and overvoltage protection	1

Circuit design	3

Block diagram	8

Operation flow chart	9

PCB design	11

Enclosure selection	15

Build costs	16

Recommendations	18

Appendix 1 – Voltage regulator data sheet	20

Appendix 2 – Microcontroller data sheet	21

Appendix 3 – Gyro IC data sheet	22

Appendix 4 – SNG-SPSC-001 data sheet	23

Appendix 5 – Enclosure data sheet	24

Appendix 6 – Suspension sensor data sheet	25

Appendix 7 – temperature sensor data sheet	27

# Introduction

In the automotive industry, when a prototype vehicle is constructed, testing is required to analyse performance. Sensors are installed throughout the vehicle, collecting a range of data which is then processed and stored for analysis. This report details the research carried out into these data collection systems, determining their electronic requirements. The report continues to explain the design process of a new data collection system, analysing data from several sensors throughout the car. Data is collected and processed using a PIC microcontroller, the program for this PIC is detailed in a flowchart along with a block diagram detailing the circuit's construction. A full circuit diagram and PCB layout is included for the circuit as well as build costs. A conclusion reflects on possible improvements.

# Task 1

There are many data acquisition (DAQ) systems on the market, each designed to collect and process data in similar ways. For vehicular testing, external sensors are positioned in key areas throughout the vehicle. Data from these sensors then undergoes signal conditioning before being processed by a computer system, often a microcontroller. Once processed the data is stored to an external storage device for analysing [1]. A popular DAQ system is the SIRIUS R1DB made by DEWEsoft [2]. Another popular system is the Granite 10 produced by Cambell scientific [3] . An innovative system is produced by DEWETRON, where each aspect of the system is modular and so interchangeable [4] . This allows the user to select the digital and analogue inputs required, the power supply utilised and even the signal conditioning module. These systems are compared in table 1.

A benefit of the DEWESoft DAQ is that it is an all-in-one instrument, this refers to its built in LED screen. This allows the user to process and analyse the data without the need of an additional device. Another benefit is the signal conditioning offered. This produces a reliable input from the sensors as only the desired signal is analysed [4]. The granite 10 offers onboard storage in the form of DDR3 SDRAM which is beneficial for streamed data. It also offers built in GPS used for network synchronization [3]. The DEWETRON is perhaps the most useful DAQ as it can be completely customised. The modular construction allows the user to configure a purpose built DAQ system making the product extremely versatile.

Table 1- Comparison of portable data acquisition systems for automotive testing [2] [4, 3]

| DAQ | INPUTS | POWER SUPPLY | FEATURES | Signal conditioning | Benefits |
| --- | --- | --- | --- | --- | --- |
| DEWEsoft SIRIUS® R1DB | Up to 16 analogue inputs | 2 x 72Wh Li-Ion cells | SSD data logger | High dynamic range | High-end signal conditioning |
|  | Up to 8 counter/encoder inputs |  | Multi-touch LED screen | High signal to noise ratio |  |
|  | Up to 24 digital inputs |  |  | high speed synchronized angle and speed measurement | All in one instrument |
|  | Up to 9 CAN bus ports |  |  | Channel to channel and channel to ground isolation |  |
| Cambell scientific - Granite 10 | Up to 8 counters | 9.6 to 32 Vdc input | MicroSD storage | Not specified | Built in GPS |
|  |  |  | DDR3 SDRAM 2GB |  |  |
|  | Up to 8 digital inputs |  | GPS |  | Onboard storage |
| DEWETRON | User exchangeable input modules | User exchangeable input modules | Modular structure | User exchangeable input modules | Completely customisable |  |

# Designing a data acquisition system

To design a microcontroller based DAQ system the component selection is extremely important to ensure accurate data is produced. The first step in designing is to select the sensors required as this will determine the types of inputs necessary when choosing the microcontroller.

## Engine RPM

Engine RPM is a measurement of rotational speed, this can be measured in various ways. For this purpose, the measurement must be non-contact. For this type of measurement an optical encoder is commonly used which detects changes in light [5]. A reflective sticker can be used to trigger the sensor. The encoder counts every time the sticker is detected and therefore calculates the revolutions per minute. This solution however may not be practical in an automotive environment. Over a two-hour test drive the sticker could be corrupted by oil or dirt from the engine. This would result in inaccurate data.

| Sensor | Supply voltage | Output type | IP rating | Operating temperature | Cost |
| --- | --- | --- | --- | --- | --- |
| Honeywell LCZ260 Hall effect sensor | 4.5-26 Vdc | Digital (square wave) | IP68 | -40 to 125°C | £120.38 |
| Honeywell General Purpose Industrial VRS Magnetic Speed Sensor | Passive | Analogue | N/A | -40 to 107°C | £163.25 |
| Honeywell SNG-S series | 8-16v | Digital | IP69K | -40 to 140°C | £68.04 |
| Honeywell ZH10 series | 4-24v | Digital | IP67 | -40 to 100°C | £148.04 |

A magnetic speed sensor would overcome this problem. This type of sensor utilises the hall effect to detect rotation [6]. As the teeth of the fly wheel pass by the sensor, the internal magnetic field is altered, developing a voltage. This voltage can be processed to determine rotational speed [7]. Table 2 details a comparison of suitable magnetic speed sensors with data taken from datasheets.

Table 2 - Magnetic speed sensor comparison [8] [9, 10, 11, 12]

From the comparison table the Honeywell SNG-s series sensor is the most suitable option as it has the highest IP rating making it the most durable against water and dust. It also operates at a suitable supply voltage for the project as most vehicular electronics use a 12v supply [13]. The price point of the SNG-s series is also attractive as it is more affordable than similar sensors. The SNG-s sensor produces a digital output which makes it more compatible with a PIC microcontroller as an analogue to digital convertor is not required.

The sensor functions by detecting the movement of ferrous material due to alterations in the internal magnet’s flux density. This type of sensor can be positioned at a revolving gear and will detect the difference in depth between the teeth of the gear. This produces a digital pulse, the frequency of which can be used to determine the RPM [14].  This can be mounted using jubilee clips or cable ties to detect the movement of the engine fly wheel, from this reading the engine RPM can be calculated using the MCU.

## Vehicle road speed (MPH)

Vehicle road speed is another measurement of rotational speed and so can be measured using the same type of sensor. Using the same sensor as engine RPM simplifies the inputs of the design and lowers production cost. The Honeywell SNP-S sensor can be mounted to the brake calliper of the car targeting the spokes of the wheel. For non-ferrous wheels, a magnet can be mounted to the spoke to engage the sensor.

## Engine temperature

Engine temperature must be measured using the engine coolant. The sensor must be reusable for different vehicles and so cannot contact the coolant directly. This means standard temperature probe cannot be utilised, temperature must be measured by the outside temperature of the coolant container. This can be achieved using a stick on RTD sensor or an infra-red temperature sensor. Table 3 compares several options.

| Sensor | Supply voltage | Output type | IP Rating | Operating temperature | Measuring range | Cost |
| --- | --- | --- | --- | --- | --- | --- |
| RTD patch sensor | Passive | Analogue | N/A | -50 to 150°C | -50 to 150°C | £61.62 |
| Calex PC21MT-0 | 6-28Vdc | Analogue | IP65 | 0 to 70°C | 0 to 250°C | £207 |
| RS PRO V IR | 5-28Vdc | Analogue | IP65 | 0 to 70°C | 0-1000°C | £177.06 |
| Optris CATCSLT15SFCB3K | 5-30Vdc | Analogue/Digital | IP63 | -20 to 80°C | -50 to 1030°C | £203.38 |

Table 3 - Temperature sensor comparison [15] [16] [17] [18]

From table 3, the Calex PC21MT-0 sensor is the most suited temperature sensor. Although the RTD sensor would be a more affordable option, the sticky patch would not be reusable as required. The Calex and the RS Pro sensors offer similar parameters, however the Calex sensor is housed in stainless steel which proves far more durable than the plastic RS Pro. The Calex sensor operates at a suitable voltage, has a good IP rating, and measures the required temperature range.

## Suspension travel

Suspension travel can be measured using a type of variable resistor. The output resistance is proportional to the extending and compression of the suspension arm. Table 4 compares several options.

| Sensor | Supply voltage | Output type | IP Rating | Operating temperature | Cost |
| --- | --- | --- | --- | --- | --- |
| RS PRO linear transducer | Not specified | Analogue | Not listed | -40 to 105°C | £134.165 |
| Gefran linear transducer | Max 60 V | Analogue | IP65 | -30 to 100°C | £239.23 |
| SP2 compact string pot | Max 30V | Analogue | IP50 | -18° to 71°C | £433.62 |

Table 4 - Comparison of suspension travel sensor [19] [20] [21]

From table 4, the Gerfran linear transducer is the most suited option as it has the best IP rating. This is advantageous as the sensor is installed to the underside of the car where water contamination could be problematic. Its piston style shape is beneficial for installation to the suspension in comparison to the string pot which may be harder to mount.

## Pitch, roll and yaw.

Pitch, roll and yaw can be measured using a digital gyroscope. The gyroscope can be installed directly to the PCB providing the casing is secured to a flat surface. Table 5 compares several digital gyroscopes.

| Sensor | Supply voltage | Number of pins | Output type | Scale range | Communication | Cost |
| --- | --- | --- | --- | --- | --- | --- |
| ANALOG DEVICES ADXRS450BRGZ | 3.1-5.25V | 16 | Digital | ± 400°/s | SPI | £64.95 |
| TRONICS GYPRO3300 | 5V | N/A | Digital | ±300°/s | SPI | £579,11 |
| LSM6DSV16BXTR | 1.7 - 3.6V | 14 | Digital | Programable | SPI / I²C & MIPI I3C® | £5.48 |

Table 5 - Comparison of gyroscopes for Pitch, roll and yaw [22] [23] [24]

From the table the most suited sensor is the LSM6DSV16BXTR, a digital gyro IC, as it has a programmable scale range, can communicate using I2C and is a more affordable price.

# Microcontroller selection

Having selected the sensors, the MCU can be selected based on the number of inputs utilised. Using the Microchip PIC selection tool, several suitable MCUs are presented [25]. Figure 1 compares the MCUs.

From figure 1, the most suitable MCU is the PIC18F26J53 as it offers 13 ADC inputs, 64KB flash memory with a low pin count of 28 [26]. From the sensor selection only five analogue, two digital and one I2c inputs are required. The PIC18F26J53 provides more than enough input pins for the project, allowing for further requirements. PIC18F26J53 is also recommended for automotive design [26].

Figure 1 - PIC comparison [25]

Connector selection

As the system is to be used outdoors and in a high vibration environment, the connectors selected are important. Each sensor requires a connector with a locking mechanism. The Honeywell SNG-S sensor used for RPM and road speed has a built-in female AMP Superseal connector [9]. As this connector meets the requirements, having an IP rating of 67 and a locking mechanism, it makes sense to continue its use where applicable. A full connector kit including the pin terminals and socket terminals can be purchased [27]. In addition to this, a data cable is required, the datasheet specifies 0.30-0.50mm2 wire size [27]. RS produces a suitable 3 core data cable which includes screening and shielding, making it durable enough for automotive testing [28]. The sensor selected for suspension travel also has a three-core output so the same connector can be utilised again. For the temperature sensor, a two-pin connector is required, which is available from the same manufacturer [29].  For connection to the PCB, Molex kk 254 connectors are a suitable solution as they form a strong connection and can be soldered directly to the board [30]. This type of connector is commonly used in PCB design. Utilising the same connectors as much as possible keeps production costs low, as manufacturers offer discounts on larger orders.

# Storage requirements

To calculate the required storage for a two-hour test drive, the frequency of sample must be allocated for each sensor. Table six details the requirements for each sensor, the max values, resolution, and frequency of sample is used to calculate the total number of bytes required.  For the two-hour test drive, 5.2MB of storage is required.   As SD cards are relatively cheap, a larger card can be used such as a 2GB SD card from RS  [31]. The max values are based on the range of an average car. The resolution refers to the smallest increment the sensor is required to record [32]. The sample frequency is unique for each sensor and changes based on predicted rate of change for each variable.

As storage is so inexpensive, there is no need to align data efficiently in the SD card. Figure 2 details the colour coded data structure for each bit of sensor input in a byte. This can then be multiplied for the full sample rate.

Table 6 - Sample requirements

Figure 2 - Data structure diagram

# Power supply selection

As the design must be portable and reusable, a Lithium-ion battery pack is the most suitable option. Table 7 shows the voltage requirements for the chosen components.

| Component | Voltage requirements |
| --- | --- |
| PIC18F26J53 | 2-3.6V |
| Honeywell SNGS-S series | 8-16v |
| Calex PC21MT-0 | 6-28Vdc |
| Gefran linear transducer | Max 60 V |
| LSM6DSV16BXTR | 1.7 - 3.6V |

Table 7 - Voltage requirements for chosen components [26] [9] [16] [20] [24].

Figure 3 - Voltage divider [35, p. 7]

An 11.1V battery pack from Cellpack solutions offers 5200mAh, which would be suitable for powering all components for the two-hour duration [33]. To lower voltage consumption and deliver a constant supply, a voltage regulator can be used [34]. An L7809CV delivers a steady 9V which can be used to power all components [34]. The PIC and LSM6DSV16BXTR (gyro) require a voltage divider circuit to deliver 3.3V. Figure 3 shows the voltage divider circuit and equation [35].

Vout/Vin = R2/R1+R2 	(Let R2= 3.3KΩ)

3.3/9 = 3.3KΩ / R1 + 3.3KΩ

0.3667 * R1 + 3.3KΩ = 3.3KΩ

R1= (3.3KΩ / 0.3667) – 3.3KΩ

R1 = 5.7KΩ

# Signal conditioning and overvoltage protection

Figure 4 - Passive low pass filter [38, p. 137]

The data from sensor inputs may include noise which requires filtering. Using a resistor and capacitor a passive low pass filter can be constructed allowing only the sample data to input the MCU [35, p. 50]. Figure 4 shows a standard passive low pass filter.

The values for R and C can be calculated using the formula:

RC=1/(2πfc)

Where fc is the desired frequency cut off [35, p. 408]. RC can be calculated for each sensor using the sample frequency as the frequency cut off.

RPM:

Sample rate = 50Hz

Let R=10KΩ

10KΩ x C = 1 / (2π50Hz)

C=1/(2π*50*10KΩ)

=0.3183µF

=0.33µF closest standard value [36]

Engine temperature:

Sample rate = 1Hz

Let R=10KΩ

10KΩ x C = 1 / (2π1Hz)

C=1/(2π*1*10KΩ)

=15.915µF

=16µF closest standard value [36]

Suspension travel:

Sample rate = 100Hz

Let R=10KΩ

10KΩ x C = 1 / (2π100Hz)

C=1/(2π*100*10KΩ)

=0.159µF

=0.16µF closest standard value [36]

Road speed:

Sample rate = 20Hz

Let R=10KΩ

10KΩ x C = 1 / (2π20Hz)

C=1/(2π*20*10KΩ)

=0.795µF

=0.82µF closest standard value [36]

In addition to signal conditioning, over voltage protection is required to protect the MCU from voltage spikes. For this a Zener diode can be used as connected in figure 5 below. Once the voltage surpasses the rating of the Zener diode it allows current to flow to ground [35, p. 12].

Figure 5 - Over voltage protection using Zener diode. [35, p. 12]

# Circuit design

Figure 6 - Power supply circuitry

Figure 6 shows the power supply circuitry as designed in the previous section. The sensor supply is taken before the voltage divider circuit, which supplies the PIC and Gyro IC. Two capacitors are connected to the voltage regulator as recommended in the datasheet (See appendix 1) [34]. The input capacitor is reccomended as a bypass to ensure stable operation. The output capacitor improves transient response [34, p. 25].

Figure 7 - Microcontroller circuitry [26]

Figure 7 shows the microcontroller circuit set up as recommended in the data sheet found in appendix 2. Decoupling capacitors are added to all VDD and VSS pins and the memory clear pin is set up with pull up resistors [26].

Figure 8 - Gyro IC circuitry

Figure 8 shows the gyro IC which is connected as recommended in the data sheet in appendix 3 [24]. Decoupling capacitors are utilised on supply pins and pull up resistors are required on the data pins for I2C communication [24]. I2C is a serial interface communcating through two wires, either of which can be the master or slave [35, p. 1034].

Figure 9 shows the circuitry for the RPM sensor as recommended in the data sheet in appendix 4. Signal conditioning is included as calculated and an overvoltage Zener diode rated 3.3V. The sensor is connected to a digital input of the MCU.

Figure 9 - RPM sensor input circuit

Figure 10 - Temperature sensor input

Figure 10 shows the temperature sensor input with the calculated signal conditioning and over voltage protection diode. The sensor is connected to an analogue input of the MCU.

Figure 11 - Suspension sensor input circuit

Figures 11 and 12 show the suspension sensor input with the calculated signal conditioning and over voltage protection diode. The sensors are connected to analogue inputs of the MCU.

Figure 12 - Suspension sensors continued

Figure 13 shows the circuitry for the RPM sensor as recommended in the data sheet in appendix 4. With the calculated signal conditioning and over voltage protection added. The sensor is connected to a digital input of the MCU.

Figure 13 - Road speed sensor input circuit

Figure 14 - USB and SD card connection.

Figure 14 shows the connection of the USB and SD card connectors to the PIC. The SD card is connected using its recommended interface, utilising pull up resistors to VDD on all data pins [37]. The SD is connected to the PIC using SPI as specified in the data sheet [26]. SPI is a master and slave communication protocol and so the SD card is connected to corresponding SPI slave pins of the MCU [35, p. 1032] [26, pp. 142,14] [37]. The USB is connected as recommended in the PIC datasheet, with added over voltage protection diodes on the data pins [26, pp. 14,15].

Figure 15 - Full circuit schematic diagram

Figure 15 shows the full circuit schematic.

# Block diagram

The circuit operation can be displayed in a block diagram, as seen in figure 16.

Figure 16 - Circuit block diagram

# Operation flow chart

Figure 17 - Device initialisation flowchart

The operation of the program can be visualised using flowcharts. Figure 17 shows what happens when the device is switched on. If the usb is connected it will go into data upload mode, otherwise it will start data acquisition.

Figure 18 - Data acquisition flowcharts

Figure 18 shows the flowcharts for data acquisition mode for each sensor. The timer is set to the sample frequency selected for each sensor. After 2 hours the program ends.

# PCB design

Figure 19 - PCB design

The schematic diagram can be routed onto PCB as seen below in figure 19. The green outline represents the PCB and the red box represetns the area for copper pour. The copper pour layer connects all ground connections. Connections are routed on different layers to avoid crossing. The connectors are placed on the left had side of the board for easy access to the side of the enclosure. The power supply connector is located in the opposite side for easy access to the battery pack . The SD card is mounted in an accessable location.

Figure 20 - Top layer connections

Figure 21 - Mid layer connections

Figures 20-23 show each layer’s connections, figure 24 shows the copper pour and figure 25 shows a 3D rendering of the PCB.

Figure 22 - Bottom layer connection

Figure 23 - 3D render.

Figure 24 - Copper pour

# Enclosure selection

The designed PCB measures 119mm x 104.14mm and the battery pack measures 56.5mm x 37mm x 70mm. Thus, resulting in a required internal measurement of 119mm x 160.64 x 37 mm. The enclosure must allow easy access to the board as the SD card is to be removed periodically. For this reason, a removable lid is required. The enclosure displayed in figure 25 is a suitable solution [38] [39]. Its large enough to fit the components, with enough room for connectors. It has a suitable IP rating of 65 and a latching lid for easy access. It’s plastic casing allows holes to be drilled for the sensor connectors wherever required.

Figure 25 - Chosen enclosure [39]

# Build costs.

Figure 26 - PCB quotation [54]

The Gerber files from the PCB design allowed for a quotation to be generated for manufacturing of the PCB, as seen in figure 26. The rest of the build costs can be found in table 8.

| Component list | Quantity | Price Each | Price |
| --- | --- | --- | --- |
| PIC18F26J53 | 1 | £5.04 | £5.04 [40] |
| Honeywell SNGS-S series | 2 | £68.04 | £136.08 [41] |
| Calex PC21MT-0 | 1 | £207.00 | £207.00 [42] |
| Gefran linear transducer | 4 | £239.23 | £956.92 [43] |
| LSM6DSV16BXTR | 1 | £5.48 | £5.48 [44] |
| Voltage regulator | 1 | £0.62 | £0.62 [45] |
| Li Ion battery pack 11.1V | 1 | £75.23 | £75.23 [33] |
| SD card | 1 | £7.33 | £7.33 [46] |
| SD card connector | 1 | £6.06 | £6.06 [47] |
| Molex Kk female 3 pin wire to board | 6 | £1.93 | £11.58 [48] |
| Molex Kk male 3 pin | 6 | £0.29 | £1.72 [49] |
| Molex Kk female 2 pin | 2 | £1.87 | £3.74 [50] |
| Molex Kk male 2 pin | 2 | £0.24 | £0.47 [51] |
| Amp Superseal 1.5 282087 connector kit | 6 | £1.50 | £9.00 [27] |
| Superseal Male & Female Connector 2-Way 1PR connector kit | 1 | £1.83 | £1.83 [29] |
| Battery charger | 1 | £57.60 | £57.60 [52] |
| On/Off switch | 1 | £8.56 | £8.56 [53] |
| Data cable | 100m | £129.00 | £129.00 [28] |
| passive components | N/A | £30.00 | £30.00 |
| PCB board | 1 | £23.22 | £23.22 [54] |
| Enclosure | 1 | £43.66 | £43.66 [39] |
|  |  | Total cost | £1,690.14 |

Table 8 - Build costs

# Recommendations

Improvements on the design could include a more advanced control panel, featuring LED indicators and buttons to start or stop the test drive. A better battery design could also be implemented allowing the battery to be charged via USB. Currently an off switch is to be connected to the battery pack, this would be better placed on a user interface.

Compared to the systems analysed at the start of the report, the designed system could benefit from having modular components like the Dewetron, or an LED display screen like the Dewesoft [4] [2].

# Conclusion

In this project a data acquisition system was designed for use in an automotive testing. Multiple sensors were compared to determine the most suited for such a harsh environment. A power supply was designed to supply the sensors and ICs with their required voltages respectively. Signal conditioning was utilized to ensure clean; noise free data was recorded from the sensors. Overvoltage protection was implemented to protect delicate components from high voltage spikes.

The designs operation was outlined with the use of operational flowcharts and circuit block diagrams. The circuit schematic was designed and used to produce a prototype PCB for the system.

Overall, the completed design would be capable of recording data from various parts of a vehicle during a two-hour test drive, as required.

# Bibliography

| [1] | G. M. Smith, “Data Acquisition (DAQ) - The Ultimate Guide,” Dewesoft, 18 06 2022. [Online]. Available: https://dewesoft.com/blog/what-is-data-acquisition. [Accessed 13 11 2023]. |
| --- | --- |
| [2] | DEWESoft, “SIRIUS® R1DB/R2DB,” DEWESoft, 15 11 2023. [Online]. Available: https://dewesoft.com/products/sirius-r1db-r2db. [Accessed 15 11 2023]. |
| [3] | Campbell Scientific, “GRANITE 10,” Campbell Scientific, 15 11 2023. [Online]. Available: https://www.campbellsci.co.uk/granite10. [Accessed 15 11 2023]. |
| [4] | DEWETRON, “AUTOMOTIVE TESTING SOLUTIONS,” DEWETRON, 15 11 2023. [Online]. Available: https://www.dewetron.com/solutions/automotive-industry/. [Accessed 15 11 2023]. |
| [5] | J. Seybold, A. Bülau, K. P. Fritz, . A. Frank, C. Scherjon, J. Burghartz and A. Zimmermann, “Miniaturized Optical Encoder with,” Applied sciences, vol. 9, no. 3, 29 01 2019. |
| [6] | The Editors of Encyclopaedia Britannica, “Hall effect,” Encyclopedia Britannica, 27 10 2023. |
| [7] | Speed Sensors Information, “Speed Sensors Information,” Global Spec, 15 11 2023. [Online]. Available: https://www.globalspec.com/learnmore/sensors_transducers_detectors/velocity_sensing/magnetic_speed_sensors. [Accessed 15 11 2023]. |
| [8] | Honeywell, “LCZ series,” North Golden Valley, 2011. |
| [9] | Honeywell, “SNG-S series speed sensors,” Honeywell, Richardson, 2022. |
| [10] | Honeywell, “General purpose industrial VRS magnetic speed sensors,” Honeywell, Minneapolis,, 2007. |
| [11] | Honeywell, “ZH10 series Hall-effect zero speed sensors,” Minneapolis, 2007. |
| [12] | Honeywell, “Speed sensor comparison,” Honeywell, Richardson, 2022. |
| [13] | how a car works, “How car electrical systems work,” 15 11 2023. [Online]. Available: https://www.howacarworks.com/basics/how-car-electrical-systems-work#:~:text=Modern%20cars%20have%20a%2012,is%20measured%20in%20amp%2Fhours. [Accessed 15 11 2023]. |
| [14] | Honeywell, “Installation instructions for the SNG-S series speed sensors,” Richardson, 2021. |
| [15] | RS, “Platinum Resistance Temperature Sensors,” Corby, 2023. |
| [16] | Calex electronics LTD, “Pyro couple,” Calex electronics LTD, Bedfordshire, 2020. |
| [17] | RS, “Infrared Temperature Sensor with Voltage Output and UART,” RS , Northants, 2022. |
| [18] | Optris, “optris CS LT,” Optris, Berlin, 2023. |
| [19] | RS, “RS Pro WT22 Series, Linear Potentiometer,” RS, Northants, 2023. |
| [20] | Gefran, “RECTILINEAR DISPLACEMENT TRANSDUCER,” Gefran, Sebina, 2019. |
| [21] | SP2, “Compact String Pot,” Measurment specialists, Chatsworth, 2013. |
| [22] | Analog devices, “High Performance, Digital output gyroscope,” Analog devices, Norwood, 2013. |
| [23] | Tronics, “GYPRO3300 Datasheet MCD014-F,” Tronics, Crolles, 2019. |
| [24] | ST Microelectronics, “LSM6DSV16BX,” ST Microelectronics, Budapest, 2022. |
| [25] | Microchip, “Microchip advanced part selector,” Microchip, Chandler, 2023. |
| [26] | Microchip, “PIC18F47J53 FAMILY,” Chandler, 2016. |
| [27] | 3 way components, “TE Connectivity 3 Way AMP Superseal 1.5 Series Connector Kit to Accept a Wire Size of 0.30-0.50mm²,” 3 way components, 15 11 2023. [Online]. Available: https://www.3waycomponents.co.uk/product/amp-superseal-waterproof-electrical-3-way-connector-kit-green-seals/. [Accessed 15 11 2023]. |
| [28] | RS components, “RS PRO Multi-Conductor Data Cable, 0.34 mm², 3 Cores, 22 AWG, Screened, 100m, Grey Sheath,” RS components, 15 11 2023. [Online]. Available: https://uk.rs-online.com/web/p/twisted-pair-multicore-data-cable/1368207?gb=s. [Accessed 15 11 2023]. |
| [29] | 3 way components, “TE Connectivity 2 Way AMP Superseal 1.5 Series Connector Kit to Accept a Wire Size of 0.30-0.50mm²,” 3 way components, 15 11 2023. [Online]. Available: https://www.3waycomponents.co.uk/product/amp-superseal-waterproof-electrical-2-way-connector-kits-with-green-seals/. [Accessed 15 11 2023]. |
| [30] | RS components, “Molex,” RS components, 15 11 2023. [Online]. Available: https://uk.rs-online.com/web/b/molex?cm_mmc=UK-PPC-DS3A-_-bing-_-2_UK_EN_Molex_Phrase-_-Molex_Pure-_-molex&gclsrc=ds&kwd-71125355697034%3Aloc-188=&matchtype=p&msclkid=c9053517e35316c05ca550402fc93f66. [Accessed 15 11 2023]. |
| [31] | RS components, “Integral Memory 2 GB SD SD Card,” RS components, 15 11 2023. [Online]. Available: https://uk.rs-online.com/web/p/sd-cards/1805789?gb=s. [Accessed 15 11 2023]. |
| [32] | Instrumentation Tools, “Definition of Accuracy, Precision, Resolution, Range,” Instrumentation Tools, 15 11 2023. [Online]. Available: https://instrumentationtools.com/accuracy-resolution-range/. [Accessed 15 11 2023]. |
| [33] | Cell pack solutions, “Ansmann Standard Li-ion 3S2P 11.1V / 5200mAh Battery Pack,” Cell pack solutions, 2023. [Online]. Available: https://cellpacksolutions.co.uk/products/battery-packs/ansmann-standard-li-ion-3s2p-11-1v-5200mah-battery-pack/. [Accessed 15 11 2023]. |
| [34] | STMicroelectronics, “Positive voltage regulator ICs,” STMicroelectronics, 2014. |
| [35] | P. Horowitz and W. Hill, The art of electronics, 3rd ed., New York , New York: Cambridge University Press, 2016. |
| [36] | Bourns, “Standard Values Used in Capacitors, Inductors and Resistors,” Bourns, 2023. [Online]. Available: https://www.bourns.com/support/technical-articles/standard-values-used-in-capacitors-inductors-and-resistors. [Accessed 15 11 2023]. |
| [37] | I. Ayyub, “Interfacing with a Secure Digital (SD) card using pic-microcontroller,” PIC Microcontroller, 24 09 2022. [Online]. Available: https://pic-microcontroller.com/interfacing-secure-digital-sd-card-using-pic-microcontroller/. [Accessed 15 11 2023]. |
| [38] | Takachi, “IP65 Latch-hinge ABS plastic box,” Takachi, Saitama, 2018. |
| [39] | MiSumi, “Enclosures - Plastic, Water/Dust proof, Hinged Latch, BCAP series,” Misumi, 15 11 2023. [Online]. Available: https://us.misumi-ec.com/vona2/detail/222000227108/?HissuCode=BCAP162113G&curSearch=%7b%22field%22%3a%22%40search%22%2c%22seriesCode%22%3a%22222000227108%22%2c%22innerCode%22%3a%22%22%2c%22sort%22%3a1%2c%22specSortFlag%22%3a0%2c%22allSpecFlag%22%3a0%2c%22. [Accessed 15 11 2023]. |
| [40] | Farnell, “MICROCHIP PIC18F26J53-I/SP,” Farnell, 2023. [Online]. Available: https://uk.farnell.com/microchip/pic18f26j53-i-sp/microcontrollers-mcu-8-bit/dp/3633859. [Accessed 15 11 2023]. |
| [41] | Mouser, “SNG-SPSC-001,” Mouser electronics, 2023. [Online]. Available: https://www.mouser.co.uk/ProductDetail/Honeywell/SNG-SPSC-001?qs=u4fy%2FsgLU9Pz3EGqFQjS0w%3D%3D. [Accessed 15 11 2023]. |
| [42] | RS components, “Calex PC21MT-0 mA Output Signal Infrared Temperature Sensor, 1m Cable, 0°C to +250°C,” RS components, 15 11 2023. [Online]. Available: https://uk.rs-online.com/web/p/infrared-temperature-sensors/0553349. [Accessed 15 11 2023]. |
| [43] | RS components, “Gefran Transducer Linear Transducer,” RS components, 15 11 2023. [Online]. Available: https://uk.rs-online.com/web/p/motion-control-sensors/3908008?cm_mmc=UK-PLA-DS3A-_-google-_-CSS_UK_EN_PMAX_Catch+All-_--_-3908008&matchtype=&&gad_source=1&gclid=EAIaIQobChMIr8Wk-bqyggMVCdrtCh1A4A2yEAQYASABEgLXgPD_BwE&gclsrc=aw.ds. [Accessed 15 11 2023]. |
| [44] | RS componets, “STMicroelectronics 3-Axis Surface Mount Accelerometer & Gyroscope, LGA-14L, SPI, 14-Pin,” Rs components, 15 11 2023. [Online]. Available: https://uk.rs-online.com/web/p/motion-sensor-ics/2615021?gb=s. [Accessed 15 11 2023]. |
| [45] | RS components, “STMicroelectronics L7809CV, 1 Linear Voltage, Voltage Regulator 1.5A, 9 V 3-Pin, TO-220,” RS components, 15 11 2023. [Online]. Available: https://uk.rs-online.com/web/p/voltage-regulators/7140666?gb=s. [Accessed 15 11 2023]. |
| [46] | RS components, “Integral Memory 2 GB SD SD Card,” RS components, 15 11 2023. [Online]. Available: https://uk.rs-online.com/web/p/sd-cards/1805789?gb=s. [Accessed 15 11 2023]. |
| [47] | RS components, “Molex, 501189 9 Way Right Angle SD Card Memory Card Connector With Solder Termination,” RS components, 15 11 2023. [Online]. Available: https://uk.rs-online.com/web/p/memory-card-connectors/4879327?gb=s. [Accessed 15 11 2023]. |
| [48] | RS components, “Molex 3 Way Female KK 254 Unterminated Wire to Board Cable, 150mm,” RS components, 15 11 2023. [Online]. Available: https://uk.rs-online.com/web/p/wire-to-board-cables/2264428?gb=s. [Accessed 15 11 2023]. |
| [49] | RS components, “Molex KK 254 Series Straight Through Hole Pin Header, 3 Contact(s), 2.54mm Pitch, 1 Row(s), Unshrouded,” RS components, 15 11 2023. [Online]. Available: https://uk.rs-online.com/web/p/pcb-headers/4838477?gb=s. [Accessed 15 11 2023]. |
| [50] | RS components, “Molex 2 Way Female KK 254 to 2 Way Female KK 254 Wire to Board Cable, 150mm,” RS components, 15 11 2023. [Online]. Available: https://uk.rs-online.com/web/p/wire-to-board-cables/2264374?gb=s. [Accessed 15 11 2023]. |
| [51] | RS components, “Molex KK 254 Series Straight Through Hole Pin Header, 2 Contact(s), 2.54mm Pitch, 1 Row(s), Unshrouded,” RS components, 15 11 2023. [Online]. Available: https://uk.rs-online.com/web/p/pcb-headers/4838461?gb=s. [Accessed 15 11 2023]. |
| [52] | Cell pack solutions, “Ansmann 11.1V 0.7A Lithium-Ion Battery Charger,” Cell pack solutions, 15 11 2023. [Online]. Available: https://cellpacksolutions.co.uk/products/battery-chargers/ansmann-11-1v-0-7a-lithium-ion-battery-charger/. [Accessed 15 11 2023]. |
| [53] | RS components, “Arcolectric (Bulgin) Ltd Illuminated DPST, On-Off Rocker Switch Panel Mount,” RS components, 15 11 2023. [Online]. Available: https://uk.rs-online.com/web/p/rocker-switches/3779759?gb=s. [Accessed 15 11 2023]. |
| [54] | JLC PCB, “Instant quote,” JLC PCB, 23 11 2023. [Online]. Available: https://jlcpcb.com/. [Accessed 23 11 2023]. |
| [55] | A. S. Morris and R. Langari, Measurement and instrumentation, 1st ed., Waltham, Massachusetts: Academic press (Elsevier), 2012. |
| [56] | P. J. vis, “Standard capacitor value,” 2023. [Online]. Available: https://www.petervis.com/electronics/standard-capacitor-values-conversion-chart-pf-nf-uf/standard-capacitor-values-conversion-chart-pf-nf-uf.html. [Accessed 15 11 2023]. |

# Appendix 1 – Voltage regulator data sheet

Figure 27 - voltage regulator, recommended circuit [34].

Figure 28 - Micro controller recommended circuit [26]

Appendix 2 – Microcontroller data sheet

Figure 29 - Gyro IC recommended connections [24, p. 42]

Appendix 3 – Gyro IC data sheet

Figure 30 - SNG-SPSC-001 recommended circuitry [9]

Appendix 4 – SNG-SPSC-001 data sheet

# Appendix 5 – Enclosure data sheet

Figure 31 - Enclosure datasheet [38]

# Appendix 6 – Suspension sensor data sheet

Figure 32 - Suspension travel sensor datasheet 1/2 [20]

Figure 33 - Suspension travel data sheet 2/2 [20]

# Appendix 7 – temperature sensor data sheet

Figure 34 - Temperature sensor data sheet 1/2 [16]

Figure 35 - Temperature sensor datasheet 2/2 [16]