Final Configuration:

R1#sh run

Building configuration...

Current configuration : 1809 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname R1

ip dhcp excluded-address 192.168.21.1 192.168.21.20

ip dhcp excluded-address 192.168.23.1 192.168.23.20

!

ip dhcp pool ACCT

network 192.168.21.0 255.255.255.0

default-router 192.168.21.1

dns-server 10.10.10.10

ip dhcp pool ENGNR

network 192.168.23.0 255.255.255.0

default-router 192.168.23.1

dns-server 10.10.10.10

!

!

no ip cef

ipv6 unicast-routing

!

no ipv6 cef

!

!

interface GigabitEthernet0/0

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

!

interface GigabitEthernet0/1.21

description Accounting LAN

encapsulation dot1Q 21

ip address 192.168.21.1 255.255.255.0

!

interface GigabitEthernet0/1.23

description Engineering LAN

encapsulation dot1Q 23

ip address 192.168.23.1 255.255.255.0

!

interface GigabitEthernet0/1.99

description Management LAN

encapsulation dot1Q 99

ip address 192.168.99.1 255.255.255.0

!

interface Serial0/0/0

description Connection to R2

ip address 172.16.1.1 255.255.255.252

ipv6 address 2001:DB8:ACAD:1::1/64

clock rate 128000

!

interface Serial0/0/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router rip

version 2

passive-interface GigabitEthernet0/1.21

passive-interface GigabitEthernet0/1.23

passive-interface GigabitEthernet0/1.99

network 172.16.0.0

network 192.168.21.0

network 192.168.23.0

network 192.168.99.0

no auto-summary

!

ip classless

!

ip flow-export version 9

!

ipv6 route ::/0 Serial0/0/0

!

line con 0

!

line aux 0

!

line vty 0 4

login

!

!

end

R2#sh run

Building configuration...

Current configuration : 1804 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname R2

!

username webuser privilege 15 secret 5 $1$mERr$SvWFW3CBz5vTzcBF6nT5u0

!

interface Loopback0

description Simulated Web Server

ip address 10.10.10.10 255.255.255.255

ip nat inside

!

interface GigabitEthernet0/0

description Conneciton to ISP

ip address 209.165.200.225 255.255.255.248

ip nat outside

duplex auto

speed auto

ipv6 address 2001:DB8:ACAD:A::1/64

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

description Connection to R1

ip address 172.16.1.2 255.255.255.252

ipv6 address 2001:DB8:ACAD:1::2/64

!

interface Serial0/0/1

description Conneciton to R3

ip address 172.16.2.2 255.255.255.252

ipv6 address 2001:DB8:ACAD:2::2/64

clock rate 128000

!

interface Vlan1

no ip address

shutdown

!

router rip

version 2

passive-interface Loopback0

network 10.0.0.0

network 172.16.0.0

default-information originate

no auto-summary

!

ip nat pool INTERNET 209.165.200.225 209.165.200.228 netmask 255.255.255.248

ip nat inside source list 1 pool INTERNET

ip nat inside source static 10.10.10.10 209.165.200.229

ip classless

ip route 0.0.0.0 0.0.0.0 GigabitEthernet0/0

!

ip flow-export version 9

!

ipv6 route ::/0 GigabitEthernet0/0

!

ip access-list standard ADMIN-MGT

permit host 172.16.12.1

access-list 1 permit 192.168.21.0 0.0.0.255

access-list 1 permit 192.168.23.0 0.0.0.255

access-list 1 permit 192.168.4.0 0.0.3.255

line con 0

!

line aux 0

!

line vty 0 4

access-class ADMIN-MGT in

login

transport input telnet

end

R3#sh run

Building configuration...

Current configuration : 1296 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname R3

!

interface Loopback4

ip address 192.168.4.1 255.255.255.0

!

interface Loopback5

ip address 192.168.5.1 255.255.255.0

!

interface Loopback6

ip address 192.168.6.1 255.255.255.0

!

interface Loopback7

no ip address

ipv6 address 2001:DB8:ACAD:3::1/64

!

interface GigabitEthernet0/0

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/0/1

description Conneciton to R2

ip address 172.16.2.1 255.255.255.252

ipv6 address 2001:DB8:ACAD:2::2/64

!

interface Vlan1

no ip address

shutdown

!

router rip

version 2

passive-interface Loopback4

passive-interface Loopback5

passive-interface Loopback6

network 172.16.0.0

network 192.168.4.0

network 192.168.5.0

network 192.168.6.0

no auto-summary

!

ip classless

!

ip flow-export version 9

!

ipv6 route ::/0 Serial0/0/1

line con 0

!

line aux 0

!

line vty 0 4

login

end

S1#sh run

Building configuration...

Current configuration : 2003 bytes

!

version 12.2

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname S1

!

!

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport mode access

shutdown

!

interface FastEthernet0/2

switchport mode access

shutdown

!

interface FastEthernet0/3

switchport mode trunk

!

interface FastEthernet0/4

switchport mode access

shutdown

!

interface FastEthernet0/5

switchport mode trunk

!

interface FastEthernet0/6

switchport access vlan 21

switchport mode access

!

interface FastEthernet0/7

switchport mode access

shutdown

!

interface FastEthernet0/8

switchport mode access

shutdown

!

interface FastEthernet0/9

switchport mode access

shutdown

!

interface FastEthernet0/10

switchport mode access

shutdown

!

interface FastEthernet0/11

switchport mode access

shutdown

!

interface FastEthernet0/12

switchport mode access

shutdown

!

interface FastEthernet0/13

switchport mode access

shutdown

!

interface FastEthernet0/14

switchport mode access

shutdown

!

interface FastEthernet0/15

switchport mode access

shutdown

!

interface FastEthernet0/16

switchport mode access

shutdown

!

interface FastEthernet0/17

switchport mode access

shutdown

!

interface FastEthernet0/18

switchport mode access

shutdown

!

interface FastEthernet0/19

switchport mode access

shutdown

!

interface FastEthernet0/20

switchport mode access

shutdown

!

interface FastEthernet0/21

switchport mode access

shutdown

!

interface FastEthernet0/22

switchport mode access

shutdown

!

interface FastEthernet0/23

switchport mode access

shutdown

!

interface FastEthernet0/24

switchport mode access

shutdown

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

mac-address 0001.9776.6501

ip address 192.168.99.2 255.255.255.0

!

ip default-gateway 192.168.99.1

!

!

!

!

line con 0

!

line vty 0 4

login

line vty 5 15

login

!

!

!

end

S1#sh vlan

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Fa0/1, Fa0/2, Fa0/4, Fa0/7

Fa0/8, Fa0/9, Fa0/10, Fa0/11

Fa0/12, Fa0/13, Fa0/14, Fa0/15

Fa0/16, Fa0/17, Fa0/18, Fa0/19

Fa0/20, Fa0/21, Fa0/22, Fa0/23

Fa0/24, Gig0/1, Gig0/2

21   Accounting                       active    Fa0/6

99   Management                       active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

VLAN Type  SAID       MTU   Parent RingNo BridgeNo Stp  BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

1    enet  100001     1500  -      -      -        -    -        0      0

21   enet  100021     1500  -      -      -        -    -        0      0

99   enet  100099     1500  -      -      -        -    -        0      0

1002 fddi  101002     1500  -      -      -        -    -        0      0

1003 tr    101003     1500  -      -      -        -    -        0      0

1004 fdnet 101004     1500  -      -      -        ieee -        0      0

1005 trnet 101005     1500  -      -      -        ibm  -        0      0

VLAN Type  SAID       MTU   Parent RingNo BridgeNo Stp  BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

Remote SPAN VLANs

------------------------------------------------------------------------------

Primary Secondary Type              Ports

------- --------- ----------------- ------------------------------------------

S1#

S3#sh run

Building configuration...

Current configuration : 2076 bytes

!

version 12.2

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname S3

!

!

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport mode access

shutdown

!

interface FastEthernet0/2

switchport mode access

shutdown

!

interface FastEthernet0/3

switchport mode trunk

!

interface FastEthernet0/4

switchport mode access

shutdown

!

interface FastEthernet0/5

switchport mode access

shutdown

!

interface FastEthernet0/6

switchport mode access

shutdown

!

interface FastEthernet0/7

switchport mode access

shutdown

!

interface FastEthernet0/8

switchport mode access

shutdown

!

interface FastEthernet0/9

switchport mode access

shutdown

!

interface FastEthernet0/10

switchport mode access

shutdown

!

interface FastEthernet0/11

switchport mode access

shutdown

!

interface FastEthernet0/12

switchport mode access

shutdown

!

interface FastEthernet0/13

switchport mode access

shutdown

!

interface FastEthernet0/14

switchport mode access

shutdown

!

interface FastEthernet0/15

switchport mode access

shutdown

!

interface FastEthernet0/16

switchport mode access

shutdown

!

interface FastEthernet0/17

switchport mode access

shutdown

!

interface FastEthernet0/18

switchport access vlan 23

switchport mode access

!

interface FastEthernet0/19

switchport mode access

shutdown

!

interface FastEthernet0/20

switchport mode access

shutdown

!

interface FastEthernet0/21

switchport mode access

shutdown

!

interface FastEthernet0/22

switchport mode access

shutdown

!

interface FastEthernet0/23

switchport mode access

shutdown

!

interface FastEthernet0/24

switchport mode access

shutdown

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan23

mac-address 0005.5e76.4d01

no ip address

!

interface Vlan99

mac-address 0005.5e76.4d02

ip address 192.168.99.3 255.255.255.0

!

ip default-gateway 192.168.99.1

!

!

!

!

line con 0

!

line vty 0 4

login

line vty 5 15

login

!

!

!

end

S3#sh vlan

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Fa0/1, Fa0/2, Fa0/4, Fa0/5

Fa0/6, Fa0/7, Fa0/8, Fa0/9

Fa0/10, Fa0/11, Fa0/12, Fa0/13

Fa0/14, Fa0/15, Fa0/16, Fa0/17

Fa0/19, Fa0/20, Fa0/21, Fa0/22

Fa0/23, Fa0/24, Gig0/1, Gig0/2

23   Enigeering                       active    Fa0/18

99   Management                       active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

VLAN Type  SAID       MTU   Parent RingNo BridgeNo Stp  BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

1    enet  100001     1500  -      -      -        -    -        0      0

23   enet  100023     1500  -      -      -        -    -        0      0

99   enet  100099     1500  -      -      -        -    -        0      0

1002 fddi  101002     1500  -      -      -        -    -        0      0

1003 tr    101003     1500  -      -      -        -    -        0      0

1004 fdnet 101004     1500  -      -      -        ieee -        0      0

1005 trnet 101005     1500  -      -      -        ibm  -        0      0

VLAN Type  SAID       MTU   Parent RingNo BridgeNo Stp  BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

Remote SPAN VLANs

------------------------------------------------------------------------------

Primary Secondary Type              Ports

------- --------- ----------------- ------------------------------------------

S3#

S3#