AN INVESTIGATION TO HIGHLIGHT THE MISUSE OF MACHINE PREFORMANCE WITHIN RENISHAW’S SURFACE MOUNT TECHNOLOGY (SMT) DEPARTMENT.

By

ANEURIN PENDERYN RHYS BAMSEY

(Student No. 2014505)

19th of May 2023

Dissertation

University Wales Trinity Saint David

BACHELOR OF ENGINEERING IN MECHANICAL AND MANUFACTURING ENGINEERING

# Abstract

With the expansion to the Renishaw Miskin facility site, new and existing product lines are expecting to double in capacity. This as a result forecasts a dramatic increase for the need in integrated PCB’s. As there is only one SMT line within Miskin this forecast comes at great stress to the department as demands could be expected to double. To ensure these demands are met it is crucial that the process is running at it most optimised. This investigation focused on the SX2 pick and place machine as it was highlighted as the process bottleneck. The purpose of this study was to identify and understand why the SX2 had been underperforming compared to other inline equipment. The goal to come up with improvements for senior management to consider. Through using literacy and the help of stakeholders along with six sigma tools it was made clear that the reason the SX2 had been underperforming was down to its incorrectly setup for the current state of production demand. Using Data and OEE calculations a baseline for the machine performance was made it was then after improvements suggested was another data collection and OEE calculation conducted. This difference in data indicated the improvements made to the setup had worked which resulted in decreasing its overall run time by 16%.

# Declaration

I declare that this thesis is the result of the author’s original research. It has been composed by the author and has not been previously submitted for examination which has led to the award of a degree.

Printed Names:                                                                        ANEURIN BAMSEY

Signature:

Date:

19th of May 2023

# Acknowledgments

I would like to acknowledge and give my warmest thank you to my supervisors Andrew Thorn, Kelvin Lake, and Peter Charlton. I would like to Thank Peter for his support and guidance through the initial stages of my project. I would also like to give a Big thank you to Andrew and Kelvin for both stepping in when I was lost, even with their busy schedule still made time for me and will always appreciate that and give thanks for that.

I would also like to give a special thanks to my follow course mates and colleagues Ben Shear and Ieuan Morgan for their continuous support and friendship throughout my time both within the company and the course to help me always want to work harder.

Last but not least, I would like to give the biggest thank you to my family (Mum, Dad and Reiny) for their continuous love and support throughout this project and course in ensuring I never felt alone or didn’t have anyone to go to when I needed.

# List of Contents

Abstract	2

Declaration	3

Acknowledgments	4

List of Contents	5

List of Figures	8

List of Tables	10

List of Abbreviations	11

Chapter 1: Introduction	12

1.1   Company	12

1.2   The Project	12

1.3   Methodology	13

1.3.1   DMAIC Approach	13

1.3.2   Why DMAIC?	14

1.4   Gantt chart	15

Chapter 2: Literacy Review	16

2.1   Surface Mount Technology, SMT	16

2.2   Overall Equipment Effectiveness	16

2.3   Optimisation Pick and Place Machine	18

CHAPTER 3: DEFINE	19

3.1   Problem statement	19

3.2   Aims & Objectives	19

3.2.1	Aims	19

3.2.2   Objectives	19

3.3   Project Charter/Scope	20

3.4   Background Research	21

3.4.1   SMT Process	21

3.4.2   Pre SMT-Process	21

3.4.3   Solder Paste Printing (ASMPT Dek Horizon)	22

3.4.4   Component Placement (ASMPT SIPLACE SX1 & SX2) -	23

3.4.5   Reflow Soldering (BTU International) -	24

3.4.6   Inspection (Cyber Optics SX600) -	25

3.5   Process Bottleneck	26

3.5.1   Process Map	26

3.5.2   Zebra Effect Justification	27

3.6   ASMPT SIPLACE SX2	28

3.6.1   Transport system	28

3.6.2   The Head and Gantry	28

3.6.3   Nozzle Bank	28

3.6.4   X-Feeders	29

3.6.5   Feeder Trolley	29

3.7 Overall Equipment Effectiveness (OEE)	30

4.7.1   Overall Equipment Effectiveness	30

3.7.2   OEE Justification	31

3.7.3   Conclusion	32

3.8   Initial Research/Voice of the customer	33

3.8.1   CTQ Tree	33

CHAPTER 4: MEASURE	34

4.1   Data Collection Plan	34

4.1.1   Checklist plan	34

4.2   Initial data collected.	35

4.2.1 Initial availability data	35

4.2.2   Initial performance data	37

4.2.3   Initial quality data	39

4.3    Calculate initial OEE Value	40

4.3.1   Availability Calculation	40

4.3.2   Performance Calculation	40

4.3.3   Quality Calculation	40

4.3.4   Total OEE Calculation	40

4.3.5   Availability Calculation Results	41

4.3.6   Performance Calculation Results	42

4.3.7   Quality Calculation Results	43

4.3.8   OEE Calculation Results	44

4.3.9   OEE Run Charts Results.	45

4.3.10   Initial OEE Results Discussion	46

4.3.11   Updated Initial OEE Run Chart	47

CHAPTER 5: ANALYSE	48

5.1   Fishbone Diagram	48

5.2   The 5 Why’s	49

5.3   Random Job Setup (Cluster)	50

5.4   The current Setup using Clusters.	50

CHAPTER 6: IMPROVE	52

6.1   Pre-Improvements	52

6.1.1   Process Map for Turret head stages	52

6.2   Improvement Solution	53

6.2.1   Demonstration Diagram	54

6.3   New process (optimised)	55

6.3.1   Changes made.	55

6.4   New Optimised Data Collected	59

6.4.1   Availability data	59

6.4.2   Performance data	59

6.4.3   Quality data	59

6.5   Calculate Optimised OEE Value for each aspect.	60

6.5.1   Availability Calculation	60

6.5.2   Performance Calculation	60

6.5.3   Quality Calculation	60

6.5.4   Total OEE Calculation	60

6.6   Compare Initial and Optimised Data	61

6.6.1   Comparison Table	61

6.6.2   Updated Run Chart Results including Optimisation Values	61

6.7   Implications of changes	62

6.7.1   Data Surrounding Changeover	63

6.7.2   Pre-change (Cluster set up) Calculations.	64

6.7.3   Post-change (Single set up) Calculations	64

6.7.4   Final Calculations	65

6.8   Cost Benefit Analysis	66

6.9   Discuss Results shown in CBA.	67

CHAPTER 7: CONTROL	68

7.1   Control Measures	68

Chapter 8: Conclusion	69

8.1   Results	69

8.2   Discussion	69

8.3   Conclusion/summary	70

8.3   Future improvements	71

Bibliography	72

Appendices	74

# List of Figures

Figure 1. 1 : Drone Images of the Miskin site development for hall 3 & 4 (Renishaw, 2023)	12

Figure 1. 2 : DMAIC Lifecycle, Aneurin Bamsey	13

Figure 1. 3 : Gantt Chart	15

Figure 3. 1 : SMT line diagram within Renishaw Department, Miskin	21

Figure 3. 2 : PCB with a 2D barcode engraved on surface.	21

Figure 3. 3 : Outside of the Horizon Dek Machine at Miskin	22

Figure 3. 4 : Inside the Horizon Dek Machine, including squeegee, paste and stencil.	22

Figure 3. 5 : Pick & Place SX1 and SX2 side profile at Miskin	23

Figure 3. 6 : SX2 Turret Head (SIPLACE)	23

Figure 3. 7 : SX1 Twin Head (SIPLACE)	23

Figure 3. 8 : Temperature section diagram on Reflow oven.	24

Figure 3. 9 : International BTU Reflow Oven at Miskin	24

Figure 3. 10 : CyberOptic Machine at Miskin	25

Figure 3. 11 : Process Map Identifying Process Bottleneck (SX2)	26

Figure 3. 12 : Zoomed out Zebra Effect screenshot from SX1.	27

Figure 3. 13 : Zoomed in Zebra Effect screenshot from SX1.	27

Figure 3. 14 : SX Machine (Systems)	28

Figure 3. 15 : SX single conveyor system (Systems)	28

Figure 3. 16 : Head fitted on the SX machine Gantry.	28

Figure 3. 17 : Birds eye of Nozzle banks inside SX2 machine	28

Figure 3. 18 X-Feeder image (Systems)	29

Figure 3. 19 : Feeder trolley image (Systems)	29

Figure 3. 20 : OEE Calculation, Aneurin Bamsey	30

Figure 3. 21 : OEE Time Diagram, (Overall Equipment Effectiveness, 2022)	30

Figure 3. 22 : CTQ Tree, Aneurin Bamsey	33

Figure 4. 1 : Screenshot displaying all the data files.	35

Figure 4. 2 : Syteline Screenshot displaying expected Run Duration	37

Figure 4. 3 : Screenshot displaying all the data files 2.0.	39

Figure 4. 4 : Run Chart of all 62 Initial OEE values compared to World class Standards.	45

Figure 4. 5 : Bar Chart to displaying number of boards ran per board type.	46

Figure 4. 6 : New Updated Initial OEE Run chart consisting top 6 ran jobs.	47

Figure 5. 1 : Fishbone Diagram of Potential Causes for this project , Aneurin Bamsey	48

Figure 5. 2 : 5 Why exercise in practice	49

Figure 5. 3 : Screenshot of Siplace Pro Line Control Gui.	50

Figure 5. 4 : Parts List sheet for kitting job	51

Figure 6. 1 : Process Map for Turret head stages	52

Figure 6. 2 : Diagram of SX2 head turret movement (Systems)	52

Figure 6. 3 : Birds eye view diagram of the SX2 machine.	53

Figure 6. 4 : Birds eye diagram of SX2 with demonstrate to difference in travel distance	54

Figure 6. 5 : Screenshot of Siplace cluster job Generator tab	55

Figure 6. 6 : Screenshot of ‘SX Blank setup Miskin’ tab	56

Figure 6. 7 : Screenshot of restriction tab	56

Figure 6. 8 : Screenshot of Optimization Parameters	57

Figure 6. 9 : Screenshot of Final checks on Siplace toolkit	57

Figure 6. 10 : Screenshot with PHC-10-3 PLUS new generated cycle time.	58

Figure 6. 11 : Final Run chart with initial, optimised and World class values	61

Figure 6. 12 : Support pins on SX2 machine bed	62

Figure 6. 13 : Docking Station for Offline Kitting	62

Figure 6. 14 : Bar Chart displaying number of changeovers by duration.	63

Figure 6. 15 : Trend line Graph showing the cross between Cost and Benefits	67

Figure 8. 1 :ASM Gridlock system, (Systems)	71

Figure 8. 2 : ASM Smart Pin support dispenser	71

# List of Tables

Table 1. 1 : Project Charter	20

Table 4. 1 : Data Collection Plan	34

Table 4. 2 : First 32 Initial Availability Data taken from SX2 machine.	36

Table 4. 3 : Second 32 Initial Availability Data taken from SX2 machine.	36

Table 4. 4 : Initial Information Surrounding 62 Cluster Performance Data	38

Table 4. 5 : 1-28 Initial Quality Data taken from the SX Machine	39

Table 4. 6 : 29-62 Initial Quality Data taken from the SX Machine	39

Table 4. 7 : Displaying all the Availability calculated percentage results.	41

Table 4. 8 : Displaying all the Performance calculated percentage results.	42

Table 4. 9 : Displayed below are all the Quality calculated percentage results.	43

Table 4. 10 : Showing the initial total OEE Value for each of the 62 clusters.	44

Table 4. 11 : Highlighting top 6 most frequent runs.	47

Table 6. 1 : Table with Collected Optimised Availability Data	59

Table 6. 2 : Table with Collected Optimised Performance Data	59

Table 6. 3 : Table with Collected Optimised Quality Data	59

Table 6. 4 : Displaying OEE comparison results	61

Table 6. 5 : showing 41 different cluster change over durations.	63

Table 6. 6 : Displaying additional time gained.	65

Table 6. 7 : Cost Benefit Analysis for project	66

| Abbreviation: | Meaning: |
| --- | --- |
| AOI | Automated Optical Inspector |
| CBA | Cost Benefit Analysis |
| CMM | Coordinate Measuring Machine |
| CSP | Component sequence problem |
| CTQ | Critical to Quality |
| DMADV | Define, Measure, Analyse, Design and Verify |
| DMAIC | Define, Measure, Analyse, improve and control |
| FAB | Feeder assignment problem |
| IDEAL | Initiating, Diagnosing, Establishing, Acting and Learning |
| I/O | Inputs/Outputs |
| MTBF | Mean Time Between Failure |
| NAP | Nozzle assignment problem |
| OEE | Overall Equipment Effectiveness |
| OIS | Operator Information System |
| OPE | Overall Production Effectiveness |
| PCB | Printed Circuit Board |
| P&P | Pick and Place |
| ROI | Return On Investment |
| SEI | Software Engineering Institution |
| SMED | Single-Minute Exchange of Die |
| SMT | Surface Mount Technology |
| SOP | Standard Operation Procedures |
| TEEP | Total Effective Equipment Performance |
| THT | Through-hole Technology |
| TPM | Total Production Maintenance |
| VOC | Voice of Customer |

# List of Abbreviations

# Chapter 1: Introduction

## 1.1   Company

Renishaw is a global, high precision metrology and healthcare technology group. The company has over 5000 employees covering 77 offices in 36 different countries. It was started in 1973 by the two founders Sir David McMurtry and John Deer and since then the company has become one of the world leaders in fields such as additive manufacturing and encoder produces. (Renishaw, 2023)

## 1.2   The Project

The manufacturing industry has always been viewed as competitive and therefore, companies find themselves continuously looking to improve in as many aspects possible with the hopes of separate themselves from others and to meeting their customer needs. This project was used as an investigation into the Surface Mount Technology (SMT) Department within Renishaw. The investigation utilises Overall Equipment Effectiveness (OEE) as a metric to help highlight weaknesses within the current process, allowing improvements to be introduced and measured for future departmental development. Majority of Renishaw’s products sold to customers require an inbuilt Printed circuit board (PCB), which are manufactured in-house at both Miskin and Woodchester’s SMT Departments. This project had been undertaken at Miskin’s SMT department due to a greater flexibility access without disruptions to project. It was decided if successful, along with senior management approval, the project would be permanently implemented into both Miskin and Woodchester’s SMT Department.

As growth within the company continues to excel so does Miskin’s site as a £50 million investment for an additional two production halls (400,000 ft²) had been announced in June 2022 as seen in Figure 1.1, This allows Renishaw to continue meeting significant demand for new and existing products arises within the market. This includes Encoders, Machine tool probes and a new line of Agility (CMM). These increased demands had forecasted to cause tremendous stress onto the SMT Departments, consequently converting it from its low/medium-volume to a high-volume production state. This transformation gives purpose for undergoing this investigation into improve output productivity using lean manufacturing techniques to ensure the company keeps customers interest. This project discusses how the Pick and Place machines performance had been improved through optimising its previous method of setting up and running the machine using a cluster (multiple assembly job within a single run) to running a single assembly job only.

Figure 1. 1 : Drone Images of the Miskin site development for hall 3 & 4 (Renishaw, 2023)

## 1.3   Methodology

### 1.3.1   DMAIC Approach

For the project to succeed the correct methodology had to be used to ensure a clear and effective professional structure. Being a Six-Sigma project looking into process improvement a DMAIC Process was chosen on the reasons highlighted in section 1.3.2. DMAIC is an acronym which stands for the 5 phases Define, Measure, Analyse, Improve and Control. The DMAIC is a data driven quality strategy that can resembles a roadmap. Within each phase, steps must be completed and understood to move onto the next. (THE DEFINE MEASURE ANALYZE IMPROVE CONTROL (DMAIC) PROCESS, 2022)

Define is where the project is selected and a scope is given outlining its project problem, goals, limitation, timeframe (Gantt Chart) and voice of customers. Measure usually consists of creating a plan for reliable data collection and understanding/recording current process creating a collective reliable data baseline. Analysis is the stages in which the potential causes are investigated and examined with the use of graphically displayed to aid process of elimination to verify main cause. The improve phase is where solutions are generated through brainstorming exercise and then tested to measure level of improvement (cycle-time and costs). The final phase is Control, this is where a successful solution has been generated and tested, it is now important that the solution is maintained at the same level using plans put in place such as 5’s and others. This phase also gives opportunity to investigate further improvement to better again.

Define

Control

Measure

Analyse

Improve

!!!

Figure 1. 2 : DMAIC Lifecycle, Aneurin Bamsey

### 1.3.2   Why DMAIC?

Before deciding on a DMAIC process alternatives such as DMADV (Define, Measure, Analyse, Design and Verify) and IDEAL (Initiating, Diagnosing, Establishing, Acting and Learning) were considered. DMADV less as its focus surrounds designing new products and processes rather than existing ones. IDEAL also referred as the transition process was introduced by individuals within the (SEI) with the idea of focusing on improvement ideas (ideal state) and proceeding to make them reality for a greater customer satisfaction, this approach is known as a Strength-based. Conversely to that DMAIC inheritance a weakness-based approach which focuses more on reducing costs and cycle time, using data to expose causes within a process to then generate solutions (Hallowell, 2022). Regarding this project honestly will provide best results, Strength-based approach tends to display only its most eye appealing data missing small, troubled details which has potential to migrate issues to a larger scale later. Whereas organisations that embrace the weakness values highlight the micro issues to discover its fundamental causes and drivers to utilise all opportunities. Although the goal is primary to ensure customer satisfaction is met, constraints must first be reduced which is why DMAIC was chosen over the next best suited Methodology.

## 1.4   Gantt chart

A Gantt Chart was created to ensure the project deadline would be closely monitored to meet required Milestones and deadlines. This would act as a rough roadmap to what steps was required to arrive at the destination, The goal was to finish a week or two prior the Final deadline to ensure everything could be checked over before final submission. The Gantt chart was structure using the DMAIC approach also to keep consistency and clear milestone within the project. The start date for the Gantt chart started on Tuesday 2nd of August 2022 and finished on May 14th giving a 5-day grace period. Find full size Gantt Chart in Appendices (Apx.1).

Figure 1. 3 : Gantt Chart

Control

Define

Improve

Analyse

Measure

Closing

Displayed above is the completed Gantt chart, which shows the projects entire process from start to finish. Unfortunately, other than the initial viva there is no history of the initial Gantt Chart. Find Presentation Viva in (Apx.2) for comparison. Date had been moved around as deadlines did not allow enough time for Optimised data collection which can be seen in Measure phase.

# Chapter 2: Literacy Review

## 2.1   Surface Mount Technology, SMT

Surface Mount is no recent concept, the process allows modern day assembly of electronic print circuit boards to be manufactured. Over the past decades, the world has pushed for miniaturisation within the electronic industry, with the goal of using smaller more compact products (Prasad, 2013). Although the idea of electronic assembly has remained the same the method of assembly has changed several times over the years. Between 1920-1950’s had the Vacuum tube era, from then until the 1960 had the transistor era and a couple years after around mid-1960 arrived the integrated circuit era, which is what is seen used today (HUI, 1996). Within this integrated circuit era there have been many types of assembly methods with the most common being Through-hole Technology (THT) and SMT. Through hole is still used today but is nowhere near as popular as once was. This is due to high costing and restrictions. THT is the construction process in which components are place through drilled holes located on the board and then mounted together using a soldering machine such as a wave or selective (Seide, Kastle, Ockel, & Franke, 2022). As technology advanced so did the complexity of the PCB design. Boards have gone from a basic single sideboard to double-sided and can now even find multi-layered boards. These advancements left THT struggling to meet assembly requirements, therefore presenting the opportunity for SMT to exceed (Amole, Adewuy, & Akinde, 2021). Using SMT can achieve low cost, reliable, high packing density PCB production due to its ability to solder components onto the boards surface in one process through using a solder print machine, a pick and place machine and a reflow oven (HUI, 1996). Whereas THT would have to run through 2 or more due to its nature. Using surface mount technology also allows the use of smaller components which furthermore reduces the area needed for a given function and moreover allows greater room to add quiet I/O areas and fit suppression and filtering components where needed (T. Williams, 2010). Although many think technology will rapidly change in years to come causing another industrial change but (Houston, 2001) suggests that the PCB markets will still have a massive growth and demand as well as the use of SMT will continue to increase as the expectations for even smaller size PCB to be produced.

## 2.2   Overall Equipment Effectiveness

Today’s business world suffers from an extremely fast-evolving economy where only the best succeeds. However, even the best within their industry can find struggle. This all stems from what is known as a competitive market, where business sales, profits and customer growth base are constantly observed, the only way for businesses to continue to stay afloat is by establishing superiority over others and meeting customers’ demands. (Jürgen Fleischer, 2006) Stresses that the competitiveness within manufacturing companies is down to their availability and productivity towards production facility. (Samuel H. Huang, 2003) further stresses to perpetuate this, organisations must oversee and maintain production facilities closely and correctly. (Samuel H. Huang, 2003) also states because of the markets intense competitive characteristic, companies are forced to improve and optimise their way of production to compete in the industry, meaning finding more meticulous ways to identify factors including equipment availability, performance, and quality. This is where OEE comes into play. OEE was first used as a quantitative metric by Nakajima in 1980s when Nakajima introduced the Total Production Maintenance (TPM) concept (Nakajima, 1988). In which equipment is individual measured highlighting and recording losses surrounding its availability, performance, and quality with the goal of achieving zero breakdown/defects, improving productivity and reduced cost. The goal of OEE is to calculate the individual piece of equipment performance to ensure it is doing what it is supposed to do (Williamson, 2006). The calculation then produces three analytical baseline percentages (Availability, performance, and quality). These baselines provide a basis for improvement priorities and initiates root cause analysis for uncovering hidden capacity within a process. (Dal, Tugwell, & Greatbanks, 2000) Adds onto this by saying OEE also tracks and traces improvements or decline in equipment’s effectiveness over time. Oppose to this a paper on OEE’s flexibility highlights why it believes OEE may not be best for uncovering hidden capability. Highlighting as today’s customer demands is catered more personalised result in equipment, requiring greater flexibility capacity to meet frequent product changeovers and therefore decreasing equipment’s availability. Slowing down cycles and introducing learning- forgotten effect on operators which all can lead to an increase in defects/ quality issues. But this theory seems to only apply if OEE is not accurate collecting data or being watched closely. Flexibility is important but to achieve successful OEE measurements data is crucial, not only any data but valid data. Data collection is the most important factor into any OEE project as without data no measurements mean no improvements. The worst thing to do within a business is to mask true data as it leads to no true results (Corp, 2023). This can be done by inadequate data collection or inaccuracy in data recording. (Jonsson & Lesshammar, 1999) Suggests obtaining correct ‘true’ data companies must sacrifices enough resources without restrictions as restrictions could later cause losses (activities that absorb more resources which provide no value). Examples of OEE within industry has shown that the tool delivered positive results and satisfaction among manufacturers. A case study found (Chiarini, 2015) discusses how an Italian injection mould company was able to improve their thermoplastic-injection presses machine performance using OEE which as a result increasing its OEE value from 40% to 60%. Another injection moulding company that manufactures plastic parts for the automobile industry had also incorporated OEE into their continuous production practices in which they also improved their OEE value from 61% to 81%. (S.R.Vijayakumar & S.Gajendran, 2014 ). Unfortunately using the tool does not always provide positive results. As mentioned, without accurate descriptive data the tool is useless. (Hedman, Subramaniyan, & Almström, 2016) had carried out a study in which data was acquired from 23 different companies and 884 machine and found the average OEE as calculated around 65% with almost half of the recorded losses unclassified due to lack/poor descriptions. This arises the question are most companies measuring OEE or just equipment’s availability as speed efficiency can be difficult to record and quality rates can vary per machine. However, if individual companies invest both time and effort into technical systems for automatic measurements these issues can be handed to produce defined core data.

## 2.3   Optimisation Pick and Place Machine

The SMT pick and place machine has evolved rapidly throughout the years with the goal of increasing speed, accuracy, and usability. The machine is responsible for picking and placing components correctly onto a PCB. (He, Li, & Yoon, 2017) Discusses that although the machine is possibly the most important on the SMT line it usually found to be the processes bottleneck. This has caused plethora of studies and investigation in recent years all with the goal to increase the machines speed performance. Component sequence problem (CSP), Feeder assignment problem (FAB) and Nozzle assignment problem (NAP) are all aspects within a setup that affect a machine performance. (Hs, 2022) Highlights how FAP and CSP have been historically the prime focus, leaving NAP to be ignored due to its complexity. Therefore, resulting in underachieving total potential output of optimisation. (Hs, 2022) Paper introduces a two-stage hybrid framework to overcome these problems on a multi-head system using both metaheuristic and dynamic programming methods. It was made clear, through using different metaheuristics it would allow to overcome CSP and FAP optimisation issues within stage one. Following this by using dynamic programming in stage two found a solution for NAP. Another study which looked at the same issue, but used a different approach was (Lee, Lee, Lee, & Lee, 1999)  where a generic algorithm was used to solve the problem also on a multi-head system. The method was to group the reels (component) together, this was shown to perform better than the heuristic algorithm which is currently used in industry. It was found through using computer simulation that it had saved an average 10.7% of assembly time. One noticeable similarity among all researched papers although solutions where uncovered, if not all authors would conclude discussing how this area required more investigating and research, insinuating that there are still areas surrounding this problem that need addressing and is not yet fully understood. These papers include (Hs, 2022), (Ahmadi & Kouvelis, 1994) and (Li, Qiang, & Yoon, 2017).

# CHAPTER 3: DEFINE

## 3.1   Problem statement

As production is forecasted to increase significantly due to site expansion over the next 5 years, it has been identified that the SMT production line which is responsible for producing all Renishaw integrated PCB boards will likely struggling to keep up with inhouse demand. As a result, this investigation had been carried out, analysing the whole SMT line and identifying the Pick and Place machine underperforming and slowing down the processes output capacity which has been made clear later within the define phase though bottleneck process mapping.

## 3.2   Aims & Objectives

### Aims

- Highlight the misuse of the pick and place machines’ performance. Using OEE as a metric value to compare machine capability, before and after improvements to produce a recognised baseline.

- Ensure a Return on investment (ROI) can be made from the project.

### 3.2.2   Objectives

- Utilising data collection plan to collect accurate data for OEE calculation values which would be used as a machine’s baseline.

- Using Six sigma industrial tools to correctly identify the route cause.

- Comparing Initial OEE value with Optimised OEE value to display improvements to the machine is performing.

- Using Cost benefit analysis to estimate ROI.

## 3.3   Project Charter/Scope

Table 1. 1 : Project Charter

A project charter had been the first piece work done on the project, this highlight the projects justification, what is expected, its goals, its customer, any initial assumptions that was made as well as any constrains that would be expected to face. It had also included deadline taken from the university to help create an expected timeframe. This was created to help ensure project succession for the companies benefit. The Project team included only me with the aid of stakeholders in a few research and analysis activities.

## 3.4   Background Research

### 3.4.1   SMT Process

Figure 3. 1 : SMT line diagram within Renishaw Department, Miskin

The Surface Mount Technology Production process is an assembly process for placing electronic components onto a circuit board surface to create a working board that can be used for a massive variety of application. The Process involves many sub-processes to manufacture the final integrated printed circuit board (PCB). The image below shows the SMT process line that can be found on both Renishaw sites.

### 3.4.2   Pre SMT-Process

Before starting the SMT process, panels (Multiple board located on a single board) and individual boards are required identification this is done through engraving 2D barcodes onto the surface using an ASYS Laser Marker. This is crucial as it allows engineers to record traceability of each PCB.

Figure 3. 2 : PCB with a 2D barcode engraved on surface.

### 3.4.3   Solder Paste Printing (ASMPT Dek Horizon)

This process uses a solder paste mixed with sticky flux (which act as both a glue holding parts in place as well as cleansing soldering surface of impurities/oxidation) in the form of a putty which is applied onto the board. This is achievable using a stencil and squeegee (Fig.8).

Solder paste is first placed by operators on top of the stencil, when the machine operates, a board is sent under the stencil using convert belts. The board is then pressed against the bottom side stencil leaving no space between the two. The squeegee then runs over the top side stencil spreading the solder paste evenly across and filling the stencil apertures (Printing over the copper soldering pads on the board) with the correct quality of paste needed taken from the stencil thickness to achieve appropriate soldering.

Figure 3. 3 : Outside of the Horizon Dek Machine at Miskin

Squeegee blade

Stencil

Solder Paste

Figure 3. 4 : Inside the Horizon Dek Machine, including squeegee, paste and stencil.

### 3.4.4   Component Placement (ASMPT SIPLACE SX1 & SX2) -

The board is transported into the Pick and Place machines SX2 and SX1 (Fig.6), where components are placed accurately onto the solder paste locations using either a twin or turret heads. The turret heads are found fitted on the SX2 and have a collective 12 nozzles that pick parts up from loaded feeders on either end of the machine. These components tend to be small whereas the Twin head only able to hold two nozzles, located on the SX1 is used for larger components collected from feeders and wafer trays. All nozzles use vacuum to pick, hold and place each part alongside high-speed cameras within the gantry head and machine bed to ensuring correct positioning when picking at high speeds.

Figure 3. 5 : Pick & Place SX1 and SX2 side profile at Miskin

Figure 3. 6 : SX2 Turret Head (SIPLACE)

Figure 3. 7 : SX1 Twin Head (SIPLACE)

### 3.4.5   Reflow Soldering (BTU International) -

The reflow process is done in stages. The newly placed boards are first sent in at room temperature entering a preheat zone (highlighted yellow in Fig.8). This zone temperature gradually increases simultaneously and evenly to around 140°C-160°C. This is done to prevent damage from thermal shock to components. The board then continues to the thermal soak zone (highlighted orange) this is where the flux is activated ensuring oxidisation is reduced around pads and improve soldering. In the reflow zone (highlighted red) temperatures around 210°C-230°C can be expected, depending on size and number of components on the board. The solder paste becomes liquid creating a bond between the component termination and the boards pads. The final stage is known as the cooling stage (highlighted blue) this is where the board, solder and components are cooled gradually again to avoid heat shock. As the solder cools it creates a solder joint between the component and the board. It is crucial to get the oven profiles correct to eliminate poor wetting of solder joints or damage to components.

Figure 3. 8 : Temperature section diagram on Reflow oven.

Figure 3. 9 : International BTU Reflow Oven at Miskin

### 3.4.6   Inspection (Cyber Optics SX600) -

Finally, the board is sent through an Automated Optical Inspector (AOI) machine (Fig.10) where a camera analysis each component placed on the board and flagging any defect, then directs the board to either pass or fail stack to be checked and reworked by trained Technicians.

Figure 3. 10 : CyberOptic Machine at Miskin

## 3.5   Process Bottleneck

A process bottleneck was first used to help dilute the searching process to help speed up the process of identifying potential under preforming or hidden capacity issues.

A bottleneck is the stage in the process where its maximum throughput capacity is not meeting the required work capacity as others, therefore causing an interruption to a process flow, and creating delays to the final output time. (What Is a Bottleneck and How to Deal With It?, 2023)

For accurate identification of a processes bottleneck it is crucial the data is clear and reliable. If this is not done correctly it can mislead the project in the wrong direction from the start, focusing on the wrong machine and wasting time and resources. Although initial research within the literacy review found that the pick and place machines tend to be the common SMT processes’ bottleneck. Before jumping to conclusions additional justification had to be done to verify this theory applies.

Figure 3. 11 : Process Map Identifying Process Bottleneck (SX2)

3.5.1   Process Map

To identify a bottleneck each machine had an upstream and downstream percentage flow which would determine how long a machine is on standby waiting for its upstream flow of board this was done on all machine within the process and found SX1 to display a zebra effect due to a lack of oncoming boards in time.

### 3.5.2   Zebra Effect Justification

This theory was verified by looking at the SX1 data software that highlights an extreme case of a ‘Zebra effect’. As can see in the figures below the process flow is similar to zebra strips in which it shows the SX1 machine waiting on standby for the upstream flow of boards.

Figure 3. 12 : Zoomed out Zebra Effect screenshot from SX1.

Purple represents the SX1 on standby which totals 70%+ while green (productive) accounts for under less than 30%, this is a very severe example but displays the potential seriousness of the issue. Figure 3.13 is the same Figure 3.12, just zoomed in a shorter duration of the run.

Figure 3. 13 : Zoomed in Zebra Effect screenshot from SX1.

When researched if anything been down about this, the supplier ASM have stated in (KG, 2023) that they have tried to do everything possible to eliminate this machine from becoming a process bottleneck this includes.

- Using a wider component spectrum

- Optimum performance utilisation

- More feeder tracks for the Twin Star

- Improved throughput due to perfect line utilisation

Figure 3. 14 : SX Machine (Systems)

3.6   ASMPT SIPLACE SX2

The ASMPT SIPLACE SX2 is a high precision pick and place machine designed for PCB assembly. The machine is made from the ground up to deliver modern mix of production. To better understand how the machine operates this section will discuss is key areas.

### 3.6.1   Transport system

Figure 3. 15 : SX single conveyor system (Systems)

The SX2 machine uses a single conveyor system that runs through the centre of the machine where boards can be loaded to the build position as well as unloaded. To prevent board movement while building, clamps activate from below holding it securely. The conveyor system also considers various board sizes using automatic width adjustment to fit all between 50mmx50mm to 450mmx560mm.

### 3.6.2   The Head and Gantry

Figure 3. 16 : Head fitted on the SX machine Gantry.

The machine is built with two multi-Star heads that travel in both X and Y Directions using encoding scale gantry systems. This allows for high-speed precision movement down to a micrometre. This is essential for meeting accuracy requirements. The heads use a 12-segment turret design to allow 12 separate nozzles to operate at once. These nozzles are then used to pick, hold and place required components using vacuum. When picking up components the turret positions the appropriate nozzle for the component size at the 6 o’clock position. Once obtained the turret head will rotate a total 180 degrees so that the component is at the 12 o’clock position where a camera is located above to check components and its positioning as a check is satisfied it will continue doing the same for the other 11 segments before placing onto boards.

### 3.6.3   Nozzle Bank

Figure 3. 17 : Birds eye of Nozzle banks inside SX2 machine

The Nozzle bank is where all the required nozzles are kept within the machine. These allow ease when a nozzle changeover is required mid-run or at the start of a new run. The banks are located on the bed on either side of the machine behind the entrance door. Each bank holds a total 12 nozzles and there are 8 banks on either side of the machine in total. The location for each physical nozzle is matched with the software interface to ensure the machine and user are aware what is where if required changing.

Figure 3. 18 X-Feeder image (Systems)

3.6.4   X-Feeders

The tape feeders are responsible for feeding components accurately to the heads. Components arrive in reels which can be fed into feeder manually. The machine can recognise each feeder by its module ID to prevent mix-up and confusion. Once the feeder has been set up correctly (Aligning the components to its required pitch pick up window) the feeder then uses a brushless motor to continuously feed components to the correct pick-up window when needed by the machine. The benefits using these feeders allow to minimise stoppage as feeders can be easily swapped in and out of the machine, and by using its contactless feature which when seated against the feeder control unit, data and power can transmit back and forth making its very user friendly. The feeders also display a traffic light system to help signal its status.

### 3.6.5   Feeder Trolley

This is also known as a Cot; it is responsible for holding all the feeders and component reels to allow easy/quick loading and unloading of the machine during changeovers. The trolly can hold up to 60 feeders in total and has an empty drop trey to collect all tape excess from reels to control mess. Positioning and height are crucial as all feeders depend on it as a fixed position.

Figure 3. 19 : Feeder trolley image (Systems)

## 3.7 Overall Equipment Effectiveness (OEE)

### 4.7.1   Overall Equipment Effectiveness

Overall Equipment Effectiveness is a best practice tool that is used in lean manufacturing as a standard for measuring the process/equipment’s productivity. Productivity is then used to assess how well a process is running in the eyes of the manufacturer. So, by using OEE it identifies the TRUE productivity of the process. The tool looks over Availability (no stop time), Performance (runs fast as possible) and Quality (producing only good parts). It would then calculate giving a percentage for each aspect and then use the equation below (Fig.1) to calculate the total OEE. Therefore, highlighting any underlining losses to be exploited and ameliorated.

Figure 3. 20 : OEE Calculation, Aneurin Bamsey

The diagram below (Fig.2) gives a visual example of the potential productive time that could be lost. By being able to identify where the losses occur and the size of the losses is crucial for engineers, as it saves time and money searching blind.

Figure 3. 21 : OEE Time Diagram, (Overall Equipment Effectiveness, 2022)

### 3.7.2   OEE Justification

As well as OEE there are several other tools that operate in similar fashion. It is crucial that the correct tool is selected for the project. Other tools that can be found in the industry are Overall Production Effectiveness (OPE) also known as Total Effective Equipment Performance (TEEP) and Mean Time Between Failure (MTBF).

- OPE

  - This is the ratio of total production time to all time – looking at the wider manufacturing capacity of a plant.

  - Whereas OEE measures the percentage of planned production time that is true productivity, OPE measures the percentage of All time that is true productive.

  - OPE looks against 24/7, 365 days, after discussing with management they are not concerned about time lost when equipment planned to go down as they have already taken consideration to expecting weekends, holidays, and planned maintenance.

  - TEEP = OEE x Utilization

(Klaess, 2023)

- MTBF

  - Tells how often on average (Mean), it should be expected to see an equipment fail to fulfil its function. Higher MTBF performance is better as its expected to run as expected without interruptions and attributes to failure for longer.

  - Does not consider situation where equipment is running at reduced rate nor the size or duration of the run.

  - are metrics that indicate the equipment’s reliability and durability?

(Which is right for you? OEE vs. MTBF, 2023)

### 3.7.3   Conclusion

These tool all have one thing in common they are proven very helpful if used correctly but if not, they can lead to worsening a situation. Initially OPE seemed to be the ideal choice as it not only looks at the machine it also investigates what can affect the effectiveness around the machine, therefore not isolating the search area. Unfortunately, the tool looks at measuring production time against 24/7, 365 days. This would not make sense to do as the SMT line can only run a maximum of 74hrs (2 x 37hour week) before Holidays, breaks and planned maintenance. If compared this to the total 168hrs a week the availability will give poor result. MTBF is a tool focusing more on failure aspect and not of the machines performance running rate or quality. OEE was chosen on these reasons along with the availability of additional knowledge from Phil Wilson (Senior Electronics Engineer) with his experience using OEE for other similar projects within SONY and Panasonic SMT lines.

## 3.8   Initial Research/Voice of the customer

Now that the problem has been highlighted along with the required tool needed. It next important to consider Voice of the customer (VOC). This allowed the opportunities for customers (management, engineers and those working on the line) to provide what quality drivers are important to them as well as performance requirements expected post-improvement. This information was gathered through walking on the shopfloor and questioning key member on what was important to them. Each requirement was followed with measures in place to maintain customer satisfaction (Critical-to-Quality).

### 3.8.1   CTQ Tree

Figure 3. 22 : CTQ Tree, Aneurin Bamsey

This CTQ was then compared to the project results to ensure customer needs are met and if not why? Followed by ensuring any demands missed can be implemented into further improvements stages, More on this in the Conclusion.

# CHAPTER 4: MEASURE

## 4.1   Data Collection Plan

Within this chapter, data has been collected to create a substantial baseline to display the current process and its capability. The data collected focuses on the ASM SX2 machine. To best achieve results a basic check list plan was created to ensure goals are identified and that the data collected is as accurate as possible.

A confirmation checklist sheet structure allows the ability to ensure no requirements are left out and data is done in an orderly and systematic manner. All data that is gathered had been done through an automatic data collection system with descriptions generated from the machine error log to save loss of description. Utilising this method provides a high accuracy rate, without human interaction which was uncovered to be a concern within (Hedman, Subramaniyan, & Almström, 2016) research.

### 4.1.1   Checklist plan

Table 4. 1 : Data Collection Plan

Using this plan allowed to speed up the process of obtaining data which therefore permitted greater time within other areas of the project as data collection can traditionally be a long-winded process.

All data collected are taken from cluster jobs as single run jobs are less commonly run thus not satisfying sufficient data to create a clear understanding of how the machine performs.

## 4.2   Initial data collected.

The initial data has been obtained between the dates 1st of June 2022 up until 22nd of December 2022, giving 7 months, ranging 62 sufficient total cluster jobs. This did not include any run shorter than 6 hours as data would be impacted by start-up and cool down and therefore give a false indication of the machine’s steady performance. The data is in order of most recent onwards.

### 4.2.1 Initial availability data

The initial availability data is looking at the total planned time the machine is given to run, then comparing the actual time the machine ran considering documented and undocumented delays. This indicates its utilisation value to help understand where time is being lost and why.

The data gathered is directly from the SX2 machines auto data history log and is then exported onto an excel spreadsheet that can be easily read for clear and understanding presenting purposes.

Figure 4. 1 : Screenshot displaying all the data files.

When open Elec Eng Review chart the excel spreadsheet shown below appears, this shows all time values in decimal hours. This spreadsheet details the job names along the top followed with planned time given to run, its utilised time and all its documented and undocumented delay data to help see where the time was lost. To the bottom of the table there is a Utilisation target of 85% as well as its true Utilization. For reference the OEE World class standard is 90% for availability.

Table 4. 2 : First 32 Initial Availability Data taken from SX2 machine.

Table 4. 3 : Second 32 Initial Availability Data taken from SX2 machine.

### 4.2.2   Initial performance data

Inintial preformance data looks at how well the machine preforms taking into account the efficiency of its cycle speed and wether or not its running at its most optimum potential. This was done through comparing its most optimum value with its actual run value.

Unlike the avalibility the data, the preformance data showed to not be as accessable. This is down to the fact management have historically only shown intrest in the machines utilization (avalibility) as means of measuring the machines preformance. On investigating why this is the case, engineering have stated preformace is still measured but not as regular as quality and avalibility as management just need a clear idea on how well production is running and the quality delivered within the soft reports.

Therefore, to overcome this data had to be collected manually from both the machines database and Syteline (cloud software used in Renishaw). This involved gathering a large number of real-value ran cluster jobs which was accessable on ‘SIPLACE EXPLORER’ to then compare with sytelines ‘Expected Run Duration’ which would also be expected to see on a job sheet.

This example is of the Duration time for PHC10-3 PLUS, note value displays in decimal hours.

Figure 4. 2 : Syteline Screenshot displaying expected Run Duration

To combined and display all nessacery data clearly all information was entered into a table.

Presented below is all 62 cluster jobs including the machines actual preformance values of each job, manually taken from the data history log (inc Job name, Cluster build, datas, quanity, and Min/Max/Ave Values). Along side on the far left it displayes the data collected from the example above contains the time given on the sheet which was taken from the Syteline Run Duration box.

Table 4. 4 : Initial Information Surrounding 62 Cluster Performance Data

The tables 4.4 uses the use of colours to help pair up duplicated cluster jobs that are of similar build, while the non-duplicated jobs rows are left in white. As the clusters can be made up of several jobs, to help identify which one job the information in the table 4.4 relates to it uses font colour (red/white).

4.2.3   Initial quality data

The Initial Quality data looks at all the boards that pass through the AOI machine. For this project it’s only required to look at the components placed on the board by the SX2 and not SX1 to get a true result of the SX2 machine alone. Much like the initial availability data, this quality data can be gathered on an excel spread sheet exported from the AOI machine history log.

Figure 4. 3 : Screenshot displaying all the data files 2.0.

When opened AOI Review the spreadsheet would display the following data below, includes all 62 cluster jobs along with the total boards passed through the AOI with its number of passes and its number of failures.

Table 4. 5 : 1-28 Initial Quality Data taken from the SX Machine

Table 4. 6 : 29-62 Initial Quality Data taken from the SX Machine

## 4.3    Calculate initial OEE Value

In this section all the initial data collected had be calculated to produce their OEE value, to achieve this the SIPLACE SX2 Availability, Performance, and Quality had to be calculated first, then using the percentage value given from each job allowed to produce it final OEE value. These OEE values had been used and compared with the post optimised results to help distinguish a positive or negative change in the results. All 62 jobs had undergone this calculation but for this example, PHC10-3 PLUS (PP-5863-0550-01-A) was used.

### 4.3.1   Availability Calculation

AVAILABILITY=Operating TimePlanned Production Time x 100

A=11.6712.63 x 100

A=92.4%

### 4.3.2   Performance Calculation

PREFORMANCE=(Ideal Speed x Boards Produced)(Run time)   x 100%

P=(0.0319449 x 306)(11.67) x 100

P= 83.78%

### 4.3.3   Quality Calculation

QUALITY=Acceptable BoardsTotal  Boards (Acceptable+Scrap) x 100%

Q=297306 x 100

Q=97.0%

### 4.3.4   Total OEE Calculation

OEE=Avalibility, A x Preformance,P x Quality ,(Q)

Initial OEE=0.924x 0.8378 x 0.97

Initial OEE=75%

### 4.3.5   Availability Calculation Results

Table 4. 7 : Displaying all the Availability calculated percentage results.

### 4.3.6   Performance Calculation Results

This is a result due to potentially a corrupt run which as a resulted caused the minimum run time to equal ‘0’.

PHC10-3 Percentage (Lower its Availability and quality)

Table 4. 8 : Displaying all the Performance calculated percentage results.

### 4.3.7   Quality Calculation Results

Table 4. 9 : Displayed below are all the Quality calculated percentage results.

### 4.3.8   OEE Calculation Results

Invalid value continued into the Initial OEE value and can also be seen on the run chart as an empty gap.

Table 4. 10 : Showing the initial total OEE Value for each of the 62 clusters.

### 4.3.9   OEE Run Charts Results.

To better display this data all OEE values were inputted into a Run chart, doing this does not only allow better understanding to how the data of the machine is preforming but it also allows to analyse for trends and patters within the data that may not be possible through a list form.

The chart displays all the cluster jobs against their initial OEE value, this can be seen coloured in blue. To help compare this data with a standard the OEE world class standards (85%) was also included (RED).

Conclusion

Figure 4. 4 : Run Chart of all 62 Initial OEE values compared to World class Standards.

### 4.3.10   Initial OEE Results Discussion

Unfortunately, as predicted from the initial project scope along with the Gantt chart. Due to a lack of time given this projects deadline, a large amount of comparable data would not be realistically possible to collect and match the initial data collected thus far. Fortunately, this was expected and as this is only an initial investigation to highlight the issue to senior management the decision was made to target a pragmatic data collection to achieve an unbiased comparison. The decision was to aim and identify several jobs to continue the investigation with. This was done through identifying the most frequent runners over the period. Using a bar chart and a trendline would highlight which jobs would choose for continuing the project. Benefits of this would be (1.) Most likely to have resources to create an optimise run and, (2.) to have the opportunity to utilize majority of the initial data collected.

Shown below is the used bar chart, the decision was to extract the top 6 most frequently ran jobs to continue for the remainder of the project. This value was chosen on the reasoning that it would cover 10% of the total variations of clusters while accounting for a total 27 clusters out of 62.

A red dash line illustrates where the separation line between the top 6 most ran jobs from the others.

Figure 4. 5 : Bar Chart to displaying number of boards ran per board type.

### 4.3.11   Updated Initial OEE Run Chart

Therefore, the jobs that have been selected with continuing of the project are as shown below.

Table 4. 11 : Highlighting top 6 most frequent runs.

To help keep things tidy a new table was created exaclty the same but only displaying the top 6 jobs to keep what data was necessary all together and clear to save from confusion when comparison later in the project.

Figure 4. 6 : New Updated Initial OEE Run chart consisting top 6 ran jobs.

# CHAPTER 5: ANALYSE

Within this phase of the DMAIC process the potential root cause of a problem is identified using different world-class Six Sigma tools. This was done by taking the data gathered within the measurement phase and trying to understand where the problem originates from.

This section required the aid from all relevant stakeholders including kitters/operators, engineers, maintenance members and management and myself (project engineer) to ensure coverage for key identification for all potential causes. It’s crucial to revisit both define and measure phase to reenforce clear understanding of the problem.

As highlighted on Table 4.8, The average value of performance is lower than both its Availability and Quality. This helps going into analysis by narrow down potential causes as is it indicates relation to the performance of the machine and not the other areas.

## 5.1   Fishbone Diagram

Fishbone diagram also known by cause-and-effect diagram or Ishikawa diagram. This analysis tool is among the most popular in the word for identifying the cause of a known problem within a process, in this case poor performance percentage. The reason for its popularity is due to its ability to assess all aspects. As can see in Fig.5.1, The diagram is in the shape of a fishbone with each bone targeting a specific aspect involved in a process (Environment, Equipment, Method, Materials, Measures and People). It is then brainstormed among all stake holders which were identified in Section 3.8 to come up with as many possible causes though possible. Once all potential causes have been added its then required to ask, “Why is this happening?” which is then branched off from the cause. Find zoomed in Fishbone in Appendices, (Apx.3).

Figure 5. 1 : Fishbone Diagram of Potential Causes for this project, Aneurin Bamsey

The process of asking ‘why?’ would continue until an answer would appear that all stakeholders could clearly see would be the cause of the problem this technique is called the 5 Why’s which is also a six-sigma industry popular tool.

## 5.2   The 5 Why’s

For this given project hours were spent questioning every potential cause of why the SX2 was underperforming, discussing lack of maintenance, inexperience operators or poor output targets. It was finally agreed after questioning why on everything on the fishbone diagram that the most realistic reason was down to the point of incorrect set up of machine. The 5 why process for incorrect setup can be found; illustrated below.

Why?

Why?

Why?

Why?

Why?

Why?

Why?

Why?

Why?

Why?

Why?

Why?

SX2 performance aspect is under preforming (higher utilization) compared to other inline equipment and therefore congesting (Bottleneck) the lines flow downstream.

SX2 performance aspect is under preforming (higher utilization) compared to other inline equipment and therefore congesting (Bottleneck) the lines flow downstream.

The recorded real minimum build time for each board took longer than the time given on job sheet (ideal duration). Not include machine stoppage.

The recorded real minimum build time for each board took longer than the time given on job sheet (ideal duration). Not include machine stoppage.

The Pick and Place head is traveling inefficiently around its working area causing the travel time to increase affecting build time.

The Pick and Place head is traveling inefficiently around its working area causing the travel time to increase affecting build time.

It has been setup (feeder and nozzle configuration) to run without consideration a best optimum route.

It has been setup (feeder and nozzle configuration) to run without consideration a best optimum route.

Random job set up algorithm does not consider optimum route as its priority is to fit as many jobs on a single cluster

Random job set up algorithm does not consider optimum route as its priority is to fit as many jobs on a single cluster

Renishaw have always been a low/medium volume production this was the best fit method of running.

Renishaw have always been a low/medium volume production this was the best fit method of running.

Figure 5. 2 : 5 Why exercise in practice

As can see the process required 6 Why’s to uncover the Cause of the problem, which has highlighted the use of a cluster setup as it was the best method to run low to medium production volume. Moving into a larger production volume running longer and bigger jobs will clearly be preventing the machine from preform to its best capacity.

## 5.3   Random Job Setup (Cluster)

A random job setup is process that’s been implemented into SMT production line from the start. This process was suggested by ASM, SIPLACE in the early stages of Renishaw production to keep efficiency high and stoppage low. The Idea behind a cluster setup it is to improve efficiency of the line by reducing the number of changeovers when frequently running smaller volume jobs. The random Setup has been designed to maximise flexibility by simplifying and accelerating the process. It is carried out through using a software called ‘SiClusterPro’ where engineers/technicians input several jobs into a generator where the software uses an algorithm to produce a possible run that accommodates all job requested in one random setup. The restriction using a cluster algorithm to set up a machine is that it does not consider best optimal travel route when building boards as its job is to fit as many build jobs into one cluster. Once cluster has been generated there is no option for user interaction to make changes to better route/setup (feeder and nozzle configuration). This is a great tool for low volume but not suitable for Renishaw’s current needs.

## 5.4   The current Setup using Clusters.

The setup process for the pick and place machines is a mixture of PC operations and manual operations which are carried out by technicians and Operators. To carry out the setup the machine must first be powered up and a desktop PC must be running ‘SIPLACE PRO LINE CONTROL GUI’ (Fig.11) to transfer build setup over.

Figure 5. 3 : Screenshot of Siplace Pro Line Control Gui.

Days in advance of the run, technicians would receive several job sheets of the products needed to run in the coming days or weeks and would be required to create ‘Clusters’. Clusters are setups that combined several different assembly jobs (usually around 3 each cluster) to all run at once without needing to changeover after each product. This was because products may be running for no longer than half an hour. In the ‘SIPLACE PRO LINE CONTROL GUI’ software there is a process function called ‘SiCluster’. Technician would select this function and then add several assemblies into a single cluster which is known as a ‘Recipe’. The software takes all inputted assemblies and uses an algorithm to create a ‘random job setup’ which accommodated all assemblies in the recipe for that run. This involves machine factors such as the required nozzles needed in the nozzle configuration bank and which electronic component parts needed for all feeder locations.

Once Recipe is created a Parts List Sheet (Fig.12) will be available for Kitters to use in pre-setting up the feeder trolleys for the run.

Figure 5. 4 : Parts List sheet for kitting job

On the day of the run the Cluster recipe will need to be open on ‘SIPLACE PRO LINE CONTROL GUI’ to allow the program to be transferred directly to the machine. Once program is transferred to the machine the operators and technicians can carry out the manual operation setup side involving arranging nozzle configuration, change transport width, locate board magnetic support pins, load larger parts on waffle trays, align barcode readers and to scan the feeder trolleys into the machine. After a final check a board is sent through to run (First Off) it is monitored on Operator Information Systems (OIS) where it would highlight any errors surrounding pickup or vision. If no errors occur, it will exit the machine onto the Telescopic conveyor where technicians initially check correct parts are placed and is placed correctly on the board before thoroughly checked by the AOI machine.

# CHAPTER 6: IMPROVE

In this section of the DMAIC process, a solution had been generated and made fit for the problem highlighted within the analysis stage which pointed at the machines incorrect set up for running longer and larger jobs.

## 6.1   Pre-Improvements

Before looking into how best to improve the cause highlighted its crucial to better understand the machine behaviour to generate solutions. To help visualise what happens during the pick and place decision making/movement a Process flow chart has been created. This flow chart shows the three crucial decision stages, Nozzle change, component pick up and component placement.

### 6.1.1   Process Map for Turret head stages

Figure 6. 1 : Process Map for Turret head stages, Aneurin Bamsey

It is clear many stages occur from the point of selecting the nozzles needed to having the components placed on the product board. Principally speaking by minimising the decisions (changes) made this should lower the time spend during the operation.

Figure 6. 2 : Diagram of SX2 head turret movement (Systems)

## 6.2   Improvement Solution

Having a better understanding on how the head move around its working area, as well as the literature found from passed investigation into optimising the pick and place machine to decrease build time. To best improve the machine performance with the increase in longer and larger jobs needed without having to run several different algorithms to see if they work or create a whole new algorithm from scratch which would prove very difficult as it would have to operate correctly with ASM interfaces. Is to use a similar approach to (Lee, Lee, Lee, & Lee, 1999) where a single assembly job would be used while considering strategic placement of Nozzles and components (Grouping up), to provide as short of a route possible. First consideration done was to ensure that components closer to together on the PCB will be grouped together to minimise moving distance of the gantry head. As well as this, minimisation of nozzle changes will also bring the time down, however nozzle change may not be avoided due to the limitations of nozzle capacity on the heads. If nozzle changes are required then to help minimise time considerations surrounding the configuration of these nozzles were crucial. Placing more frequent components closer to the cameras also cut down time as the travel distance for these more frequent components were shortened. All these considerations aided to decrease it travel time and bring the total build time down.

The diagram below is a bird’s eye view of the SX2 working area and was used when coming up with considerations as these would be drawn on to visualise distance (time) being cut out.

Figure 6. 3 : Birds eye view diagram of the SX2 machine, Aneurin Bamsey.

### 6.2.1   Demonstration Diagram

Shown below is to examples demonstrate the difference in travel distance for the same job but different setup. One setup as a cluster job and the other as a strategic single assembly job. As can see the clusters additional travel time can result in a lot of unwanted waste.

Example 1 – Cluster Job

Example 2 – Strategic Single Job

Figure 6. 4 : Birds eye diagram of SX2 with demonstrate to difference in travel distance, Aneurin Bamsey

## 6.3   New process (optimised)

### 6.3.1   Changes made.

Now that the required optimisation improvement changes had been identified. It was then implemented into a new set up strategy. In this section the suggestions/considerations made had been put in a real setup for the machine to receive and run. This was done for the six clusters jobs (TSI-2, HIS, Tonic Readhead Alt limit, PI200-3 Standalone, RGH24 Base Eris and PHC10-3 PLUS) as they are the most recorded frequently ran assemblies with the required resources to run on site.

Firstly, a new cluster had been generated, for this exercise the PHC10-3 PLUS has been chosen job. By entering the correct Scan Job Number which is found on the job sheet (2683317) it gathered the required recipe and information for the job including what the parts and nozzles are needed.

NOTE: Arrows are used to help point out what is important keys taken from the description above each figure.

Figure 6. 5 : Screenshot of Siplace cluster job Generator tab

Once the information had been entered it could then be generated which allowed the users to open on CrusterPRO, this is where the modifications were made to the machine setup.

Once opened ClusterPRO it was important to first set the SX machine setup to a blank default setting, this ensures no previous saved settings could be missed and effect the setup desired. NOTE: None of the 5 boards chosen are larger than 450mm.

Figure 6. 6 : Screenshot of ‘SX Blank setup Miskin’ tab

Next was the feeder setup. As this is not a cluster there will be no restrictions on feeder locations, this was enabled by selecting Feeder as shown in Figure 6.7. By doing this it ensured the feeders will be set strategically neighboring (grouping) from one another and not scattered randomly over the entire Cart, delivering minimum travel distance.

Figure 6. 7 : Screenshot of restriction tab

Figure 6. 8 : Screenshot of Optimization Parameters

As well as the feeder setup, Nozzles had also been optimised, this was done by, opening Optimisation Parameters using Optimisation level (Experts). There is a a Box titled ‘Optimize nozzle changer configuration’ by ticking this is allowed to find best setup for the nozzles and remove any that are not required for the assembly.

Figure 6. 9 : Screenshot of Final checks on Siplace toolkit

Finally, before it was generated and sent over to the machine, all changes were double checked and that all the parts list and nozzle configurations match up with the new paperwork.

Once generated the Recipe gave the ideal run time to be 99.53s = 1minutes 40seconds. This value can be used to calculate the Machine performance when it come back to calculating a new OEE value.

Figure 6. 10 : Screenshot with PHC-10-3 PLUS new generated cycle time.

This was repeated for the other 5 jobs, and then left over the next few months to trial and run on the line when these specific jobs were called on.

## 6.4   New Optimised Data Collected

Once again data had to be collected to calculate the new OEE values this time the only data collected will be taken from trial runs that use the new improved optimised setup. This data was taken Between February and March 2023.

### 6.4.1   Availability data

Table 6. 1 : Table with Collected Optimised Availability Data

### 6.4.2   Performance data

Table 6. 2 : Table with Collected Optimised Performance Data

### 6.4.3   Quality data

Table 6. 3 : Table with Collected Optimised Quality Data

## 6.5   Calculate Optimised OEE Value for each aspect.

To correctly compare the old and new preformance, new optimised calculations must be conducted with the new OEE values gained. These results can then be put into a compareison table and runchar. Once again for this Exercise (PHC10-3 PLUS) was used.

### 6.5.1   Availability Calculation

AVAILABILITY=Operating TimePlanned Production Time x 100

A=14.9515.67

A=95.4%

### 6.5.2   Performance Calculation

PREFORMANCE=(Ideal Speed x Boards Produced)(Run Time)  x 100%

P=(0.027647222hrs x 456)(14.95hrs) x 100

P=84.3%

### 6.5.3   Quality Calculation

QUALITY=Acceptable BoardsTotal  Boards (Acceptable+Scrap) x 100%

Q=441456 x 100

Q=96.7%

### 6.5.4   Total OEE Calculation

OEE=Avalibility, A x Preformance,P x Quality ,(Q)

Optimised OEE= 0.954 x 0.843 x 0.967

Optimised OEE=77.8%

## 6.6   Compare Initial and Optimised Data

Shown below is a table comparing both the Initial OEE value and the New Optimised OEE value. A clear difference in both values, the Newly optimised values are slightly higher than those taken before changes. Averaging a 5.03% percent increase.

### 6.6.1   Comparison Table

Table 6. 4 : Displaying OEE comparison results

### 6.6.2   Updated Run Chart Results including Optimisation Values

The new OEE percentage values were then added to the same run chart as before to visualise and see if it had followed a similar trend or if there were any substation changes. It was good to see gain was made and the machine is closer to The OEE world class standards. There are clearly still improvements there to be made but is a step into the correct directions regarding machine performance.

Figure 6. 11 : Final Run chart with initial, optimised and World class values

## 6.7   Implications of changes

Being aware of implication to a current process is crucial when introducing/suggesting new improvements to a process. As changes could affect other aspects within the process negatively. Therefore, within this section the expected implications due to the change made are covered and analysis to distinguish if running the machine using a single assembly job is beneficial with respect to costs and practicality.

Prior to this project the machine would run using majority a cluster job setup which entails using between 2-4 jobs assemblies. Now that it is proposed better in terms of machine performance to run single assembly jobs instead its clear doing this has increased the number of changeovers required. Therefore, A Cost Benefit Analysis (CBA) was required, to calculate and weigh what might benefit the department and company more instead of solely improving the machines’ performance. To gather the data to create a fair CBA certain data needed obtaining.

- Number of changeovers

- Average duration of a changeover

- Cost Per changeover

To calculate whether this is practical or not, data surrounding changeovers can be found in the following pages. This data was then given a monetary value to forecast future costs and benefits to determine if the project is feasible as well as practical.

The current change over process can be found in the Appendices (Apx.4), it is important to note that the major time consumption of a changeover uses Single-Minute Exchange of Die (SMED). This is used on the build of the feeder trolly offline at a docking station. Depending on the size of the job a trolley can take anywhere between 0.5-2hrs to build. This way it impacts no additional time during a changeover. This just leaves small adjustments to be made within a changeover such as arranging nozzle configuration, change transport width, locate board magnetic support pins, load larger parts on waffle trays, align barcode readers which would take minutes to do.

Figure 6. 12 : Support pins on SX2 machine bed

Figure 6. 13 : Docking Station for Offline Kitting

### 6.7.1   Data Surrounding Changeover

The first step was identifying roughly the current number of changeovers done on a quarterly basis using a clustering setup. Show below is data collected on the number of cluster changeovers that have occurred between the months of February and March 2023.

Figure 6. 14 : Bar Chart displaying number of changeovers by duration.

It was found that a total of 41 clusters changeovers were carried out in this 2-month period, this would give approximately 246 expected changeovers per year. It was then required to identify the average time a changeover would take. This average was taken from the same 41 changeovers.

Table 6. 5 : showing 41 different cluster change over durations.

By adding all the time together then dividing it by the number of jobs the overall average for a changeover was timed at 5.27min = 5 minutes 16 seconds. All that was left was to obtain the costing value for having the SMT production line down for an hour. This was obtained through discussions with Senior engineer Phil Wilson.

The total current SMTY labour rate is approximately £412.45/hr. This value considers facility costs, light, heating, machine depreciation and labour cost of the people running the line. This value is used when considering when looking into speed and downtime of line.

### 6.7.2   Pre-change (Cluster set up) Calculations.

Number of changeovers yearly=246

Avarage duration for aaverage duration in minutes1 hr=5.260=0.086hrs

Total duration of changeover for the year=246 x 0.086=21.156 (21 hours 9 minutes)

Yearly cost to have the line down for changeover only=21.156 x 412.45=£8,725.79

### 6.7.3   Post-change (Single set up) Calculations

Although Custer jobs can vary between 2 or more assembly jobs, it is worked out to have an average of three assemblies within a cluster therefore the Number of yearly changeovers will consider this.

Number of changeovers yearly=246 x 3=738

Avarage duration for aAverage duration in minutes1 hr=5.260=0.086hrs

Total duration of changeover for the year=738 x 0.086=63.468 (63 hours 28 minutes)

Yearly cost to have the line down for changeover only=63.468 x 412.45=£26,177.37

Additional time spent in a year carrying out changeover=42 hours 19 minutes

Total Additional cost per year to carry out 3 x current changeover=£17,451.34

### 6.7.4   Final Calculations

As the site is doubling in size it is expected that the demand for boards to double its current output capacity. But for this example, the original number of boards will be used.

Table 6. 6 : Displaying additional time gained.

Shown in the Table above is the total hours that has been cut out through using the optimised setup over the initial setup, which accounts to a total 14 hour and 33 minutes.

To calculate the cost saving the 13hrs 31min will be multiped by the hourly SMT line labour costs. This gives the total cost saving of running the 6 jobs alone.

14.5hrs x 412.45=£5,980.52

Now to calculate the monthly saving if it was to just run chosen 6 jobs throughout the month.

Initial time totals 90.34hrs

Optimised time total 75.73hrs

It took 90hrs 20 min to compete though 6 jobs in initial state which account to (90.34/74hrs two shift working week = 1.22) 1.22 Weeks running. Therefore, within a month there is a potential to save up to £5,980.52 x 3.27 (4week/1.22weeks) = £19,556.30

To calculate the yearly saving the monthly value is multiplied by 12 = £234,675.60

Now, that the figures are obtained they can be inserted into the benefits section within the CBA table show in Table 6.7.

## 6.8   Cost Benefit Analysis

The purpose of the CBA tool is to help identify how long it will take for the benefit to outweigh the cost of the project. Shown below is the CBA tool that has been segregated into cost and benefits, The Cost are what is expected cost being fed into the project in this it includes.

- The cost of having the machine down for an additional 42hrs and 19min a year

- The cost for initial training everyone on the new procedure causing the line to be down for two 8-hour shifts.

- The cost to carry out OEE audits which requires the line down 4hrs a month.

- Due to more changeover a new kitters will be required on each shift to help current operators spread the workload.

- It would also require an initial cost for two new trolleys to help kit jobs offline ready for changeovers, feeders will not be necessary as less will be used on a trolley as it only runs single jobs.

In terms of benefit which is what cost expected to come from the project, this will include.

- Increased running speed therefore needing less time on the line to produce same output at before optimisation, introducing more time to run more products.

As can see from the top of the Table the Benefits outweigh the cost 1.56 to 1 in just a 5-month period.

Table 6. 7 : Cost Benefit Analysis for project

## 6.9   Discuss Results shown in CBA.

To help make clear of how this project would perform it respect to cost a line graph was created using the Cost data entered the CBA to discover if benefits would outweigh cost and if so how long it would be expected to take.

Figure 6. 15 : Trend line Graph showing the cross between Cost and Benefits

As displayed in the Graph Fig.6.15, initially the costing of the project dramatically tops the benefit gained, this is due to all the initial costing including training new kitters and new equipment (feeder trolley). But as the month pass the distance between the two closes, until around the 2 months both lines cross indicating benefits coming out is starting to outweigh the costs going in. From Table 6.7, it forecasts the Net benefit (cost) to give £33,636.44.

If this was look at over the entirety of the first year it can be estimated £119,402.05 after all deductions. This value will increase in the second year as initial cost introduced to the project will not need to be carried over to the second.

This proves that the project is not only better as the goal was met in improving the machine performance, but it had also become a huge potential cost saving with the increase in availability to run or jobs.

# CHAPTER 7: CONTROL

Following from the improvement stage, the process has now been optimised and is performing at a greater level than prior. The goal moving forward ensuring that this level is maintained and not returning to its original underperforming state. This change in performance is something that could only been seen given a long period of time, making control to be a very difficult to forecast its behavior. Thus, to ensure that control does succeed is all down to its policing process.

## 7.1   Control Measures

To ensure that this is the case there are several measures that could be put into place to help sustain the level desired. These measures are a mix of minor and major implementations.  It is important to note these rules/measures have not yet been implemented and are only suggestions on what to do if the project is agreed upon by senior management and planned to be used on the line.

An example of a minor measure that would need to be taken is an update to the current Standard Operation Procedures (SOP) on how to setup for a single assembly run for anyone new who was to come into the role. This would be followed up with a refresher course for all existing operators and technicians who will be responsible for setting up future runs. Like other operational procedures they will need to be revisited on a yearly basis to be reviewed.

While updating SOP might be a good place to start there is no guarantee that it will enforce control over how the policing toward the improvement made. Therefore, a Major measure that needs putting in place is a preventative OEE audit, this will be conducted monthly at first but may change to a quarterly basis if deemed not needed so frequently. This entails initially assessment of all equipment within the SMT process to gain an OEE baseline for future references as this has not yet been done withing the company as of prior this project. The data will be referenced from month to month as changes are constantly introduced to the line. Using a moving average moving range chart will help to display the data collected for monetary value when investigating trends and patterns. The moving average moving range chart will uphold tolerances that are used as a safeguard area. If the OEE value was to exit this safeguard area it would trigger a Reactive plan, where another investigation will be conducted in the same manner.

# Chapter 8: Conclusion

## 8.1   Results

After conducting both a comparison on the machine’s performance (Difference in OEE) and its ROI, it’s clear that this investigation was a success, the average percentage of the machine OEE had increase by 5.03% and from the example used has estimated to cut out 14.5 hours from a the total initial hours it took to run the 6 jobs within a cluster set up (90.34hrs) this equates to 16% less time the job is spent running on the machine when using a single assembly setup over a cluster assembly job. The ROI on the project is not what would have expected going into this project, potentially earning £119K within the first year or running. This figure is significantly high and will require more investigating. The CBA has calculated to benefits to outweigh the cost 1.59 to 1 while earning a net value of £33K within the first 5 months. Overall, the machine performance has improved with no resulting side effect had been caused towards the availability or quality of the machine as can see in Figure 6.11.

## 8.2   Discussion

The results indicate that by having the machine set up to run single assembly jobs provide more benefit including speed efficiency and greater output capacity. This can be seen in Figure 6.11 where both OEE trend line are in side by side displayed on a run chart.  The data showed that by increasing its performance it was able to increase its overall equipment effectiveness value. The finding of this project can similarly relate to existing research, for example the method used to optimised the process was inspired by (Lee, Lee, Lee, & Lee, 1999) in which the grouping concept was utilised, which resulted on averaged save 11% of assembly time whereas this project did a similar strategy and also saved an average 16% assembly time. This could be down to the use of similar multi-head equipment or algorithm. Although the investigation may seem a success there are many limitations that have come with this research. Time management and sample sizes where two factors that had to be overcome. Due to such a short deadline for the given size this project covers not a large amount of data was able to be collected post optimisation this therefore could provide a bias representation of the result, however as the initial OEE value had 62 sufficient cluster the OEE surrounding it would be reliable and when compared with the optimised result they are not too far apart indicating consistent data results. Due to the size of the project its best to continue to investigate the project, looking into larger sample sizes of boards including different build time and number of components. Another direction in which the investigation could lead is by looking at is because of these improvements does it create potential bottlenecks further down the process and what can be done to eliminate these.

Looking at this investigation sort term I believe there is a lot more work involved to ensure management is persuaded and on board to start over and to change their current approach to hoe the pick and place machine is setup. With the recommended investigation suggested the project will be able to answer more question through providing a larger scope. Long term with the expansion of the Miskin site, by having this project implemented would reduce the stress on the line and secure customer demands are met as the results show.

## 8.3   Conclusion/summary

To conclude, this project has displayed great results meeting all its aims and objective stated at the start and satisfying customer needs taken from the CTQ. The investigation managed to identify that the SX2 pick and place machine in fact was underperforming, this was made aware during the analysis phase where through using cause and effect techniques as well as the 5 whys, highlighted that the machine was being set up incorrectly. Thus, resulting in insufficiently running and becoming a bottleneck.

This was caused by setting the machine to run using a clusters setup on long hour jobs.  The cluster setup is designed to cutdown on changeovers when running smaller jobs.  The goal of algorithm used is to fit as many assembly jobs into 1 cluster as possible, to achieve this it takes no consideration to optimal travel route and therefore components and nozzles are scattered around the working area which as a result increases both the travel distance of the head and the number of nozzle changes made. This is the cause of longer build time. To verify this was the case initial data pre optimisation underwent an OEE calculation to provide the machine Effectiveness. It was then through research and understanding of the cause was an improvement generated, which would focus on running a single assembly job to ensure all components and nozzle where strategically altered to keep the travel time to as minimum as possible. It was then undergone trains over two months in which data was collected and entered the same OEE calculation which showed improvement in machines performance. It was then investigated to see what impacts this improvement would have. The main change would be an increase in changeovers needed. Although the project showed promising performance result it needed a ROI as well as being practical. It was at this point where a Cost benefit analysis was conducted to determine if it would cost money or save it. Unexpectedly the CBA highlighted a significant cost saving. Finally control measures where then recommended to ensure standards of level would be maintained. There is still a lot more that need to be covered regarding this investigation with respect to moving away from clusters to single assembly runs. But this project does a perfect job of highlighting the true route cause and depending on senior management decision will depend on the future of this project.

## 8.3   Future improvements

Further improvement into this project surrounding improving the SMT line set up outside of the recommendations made in the dissection following the investigation, would be to investigate decreasing the time spend during changeovers.

It was made apparent during the research withing the current SMT line set up that apparat from the kitting of the trolleys which are done offline the second most time consuming activity was found to be placement of the support pins on the SX machine bed. It would involve operators manually placing individual support pins in locations where it would not meet components underneath the board. This proved to be a very tedious and time-consuming process. These are used to prevent bowing of the board while building.

Therefore, suggestions id make for the future would be looking into solutions to overcome this. This could include.

- Using an automatic support gridlock system like the one on the Dek Horizon machine. Figure 8.1

- ASM Smart Pin support dispenser Figure 8.2

Figure 8. 1 :ASM Gridlock system, (Systems)

Figure 8. 2 : ASM Smart Pin support dispenser

# Bibliography

Ahmadi, R. H., & Kouvelis, P. (1994). Staging problem of a dual delivery pick-and-place machine in printed circuit card assembly. Operations research, Vol 42, Iss 1, 81-91.

Amole, A., Adewuy, O. B., & Akinde, O. (2021). Fabrication of Electronic Circuits Using Through- Hole PCB Technology: An Appraisal. 1st ICEECE & AMF 2021 (pp. 170-174). Ibadan: Oludamilare Bode Adewuyi Department of Electrical & Electronic and Biomedical Engineering First Technical University.

Chiarini, A. (2015). Improvement of OEE performance using a Lean Six Sigma approach: An Italian manufacturing case study. International Journal of Productivity and Quality Management, 416-433.

Corp, P. A. (2023, December 21). The Hidden Pitfalls Of Doing 'Back Of The Envelope' Calculations To Determine Your Current OEE. Retrieved from SILO.TIPS: https://silo.tips/download/the-hidden-pitfalls-of-doing-back-of-the-envelope-calculations-to-determine-your

Dal, B., Tugwell, P., & Greatbanks, R. (2000). Overall equipment effectiveness as a measure of operational improvement – A practical analysis. International Journal of Operations & Production Management, Volume 20, No. 12, 1488-1502.

Hallowell, D. L. (2022, December 10). Comparing and Contrasting IDEAL and DMAIC. Retrieved from ISIXSIGMA: https://www.isixsigma.com/dmaic-methodology/ideal-and-dmaic/

He, T., Li, D., & Yoon, S. W. (2017). A multi-phase planning heuristic for a dual-delivery SMT placement machine optimization. Robotics and Computer–Integrated Manufacturing 47, 85-94.

Hedman, R., Subramaniyan, M., & Almström, P. (2016). Analysis of critical factors for automatic measurement of OEE. Analysis of critical factors for automatic measurement of OEE, 128-133.

Houston, D. S. (2001). Surface Mount Technology Market Forecasting. Journal of Industrial Technology vol17 no.1, 2-7.

Hs, H.-P. (2022). Solving the feeder assignment, component sequencing, and nozzle assignment problems for a multi-head gantry SMT machine using improved firefly algorithm and dynamic programming. Advanced Engineering Informatics 52, 1-2.

HUI, I. K. (1996). ANALYSIS OF SURFACE MOUNT TECHNOLOGY SOLDER JOINTS. London: Department of Materials Engineering, Brunel University.

Jonsson, P., & Lesshammar, M. (1999). Evaluation and improvement of manufacturing performance measurement systems – the role of OEE. International Journal of Operations & Production Management, Vol. 19 No. 1, 55-78.

Jürgen Fleischer, U. W. (2006). Calculation and optimisation model for costs and effects of availabilityrelevant service elements. Proceedings of LCE, 675-680.

KG, A. A. (2023, 1 5). SIPLACE SX1/SX2 - Capacity on Demand. Retrieved from banner-ever.com: http://www.banner-ever.com/upfiles/201844144411__A10011-ASM-T46-EN-Spec-SX-V2_DMS%2020151103.pdf

Klaess, J. (2023, 1 20). OEE vs. OPE – Toward a Holistic Understanding of Your Machine Data. Retrieved from Tulip: https://tulip.co/blog/oee-vs-ope-toward-a-holistic-understanding-of-your-machine-data/

Lee, W., Lee, S., Lee, B., & Lee, Y. (1999). Journal of Universal Computer Science, vol. 5, no. 12 , 833-854.

Li, D., Qiang, R., & Yoon, S. W. (2017). Balanced Adaptive Tabu Search Algorithm to Optimize Dual-Gantry Pick-and-Place Assembly. Procedia Manufacturing 11, 1892-1899.

Nakajima, S. (1988). Introduction to TPM: Total Productive Maintenance. Cambridge: Productivity Press.

Overall Equipment Effectiveness. (2022, August 14). Retrieved from OEE: https://www.oee.com/

Prasad, R. (2013). Surface Mount Technology: Principles and Practice. Portland: Springer Science & Business Media.

Renishaw. (2023, March 22). About Renishaw. Retrieved from Renishaw.com: https://www.renishaw.com/en/about-renishaw--6432

S.R.Vijayakumar, & S.Gajendran. (2014 ). IMPROVEMENT OF OVERALL EQUIPMENT EFFECTIVENESS (OEE) IN INJECTION MOULDING PROCESS INDUSTRY. IOSR Journal of Mechanical and Civil Engineering (IOSR-JMCE), 47-60.

Samuel H. Huang, J. P. (2003). Manufacturing productivity improvement using effectiveness metrics and simulation analysis. International Journal of Production Research, Volume 41, No. 3, 513-527.

Seide, R., Kastle, C., Ockel, M., & Franke, J. (2022). Impact of THT-hole dimensioning on manufacturability in selective wave soldering. Microelectronic Reliability 137.

SIPLACE, A. (n.d.). SX Heads. Munich.

Systems, A. (n.d.). SIPLACE X-Series. Munich.

T. Williams. (2010). Chapter 45 - EMC. In W. Boyes, Instrumentation Reference Book (Fourth Edition) (pp. 797-871). Butterworth-Heinemann.

THE DEFINE MEASURE ANALYZE IMPROVE CONTROL (DMAIC) PROCESS. (2022, December 10). Retrieved from ASQ: https://asq.org/quality-resources/dmaic#:~:text=DMAIC%20is%20an%20acronym%20that,(internal%20and%20external)%20requirements

What Is a Bottleneck and How to Deal With It? (2023, 01 6). Retrieved from Kanbanize: https://kanbanize.com/lean-management/pull/what-is-bottleneck#:~:text=In%20the%20simplest%20definition%2C%20a,delays%20across%20the%20production%20process.

Which is right for you? OEE vs. MTBF. (2023, 01 20). Retrieved from Allied Reliability: https://www.alliedreliability.com/blog/which-is-right-for-you-oee-vs-mtbf

Williamson, R. M. (2006). Using Overall Equipment Effectiveness: the Metric and the Measures. Strategic Work Systems, Inc, 1-6.

# Appendices

Axp.1 – This is a file link to access the Final Gantt Chart to see the Projects process more clearly with respect to dates.

Axp. 2 – This is a file link to access the screenshot to the Initial Gantt Chart if interested in comparison between the two.

Axp.3 – This is a file link to access the spreadsheet for the zoomed in project fishbone diagram.

Axp.4 – This is a file link to access a written step by step guide to the current SMT changeover procedure.