| Device | Interface | Device Type | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- | --- |
| PC1 | NIC | Host | 192.168.1.153 | 255.255.255.0 | 192.168.1.1 |
| PC2 | NIC | Host | 192.168.3.50 | 255.255.255.0 | 192.168.3.1 |
| PC3 | NIC | Host | 192.168.4.115 | 255.255.255.0 | 192.168.4.1 |
| PC4 | NIC | Host | 192.168.5.83 | 255.255.255.128 | 192.168.5.1 |
| PC5 | NIC | Host | 192.168.5.227 | 255.255.255.128 | 192.168.5.129 |
| PC6 | NIC | Host | 192.168.2.48 | 255.255.255.224 | 192.168.2.33 |
| PC7 | NIC | Host | 192.168.2.67 | 255.255.255.224 | 192.168.2.65 |
| Hub | G0/0/0 | router | 192.0.2.1 | 255.255.255.252 | N/A |
| Hub | S0/1/0 | router | 192.168.0.1 | 255.255.255.252 | N/A |
| Hub | S0/1/1 | router | 192.168.0.5 | 255.255.255.252 | N/A |
| Hub | S0/2/0 | router | 192.168.0.9 | 255.255.255.252 | N/A |
| Hub | S0/2/1 | router | 192.168.0.13 | 255.255.255.252 | N/A |
| Branch-1 | G0/0/0 | router | 192.168.1.1 | 255.255.255.0 | N/A |
| Branch-1 | S0/1/0 | router | 192.168.0.2 | 255.255.255.252 | N/A |
| Branch-2 | G0/0/0 | router | 192.168.2.33 | 255.255.255.224 | N/A |
| Branch-2 | S0/1/0 | router | 192.168.0.6 | 255.255.255.252 | N/A |
| Factory | G0/0/0 | router | 192.168.3.1 | 255.255.255.0 | N/A |
| Factory | G0/0/1 | router | 192.168.4.1 | 255.255.255.0 | N/A |
| Factory | S0/1/0 | router | 192.168.0.14 | 255.255.255.252 | N/A |
| HQ | G0/0/0.1 | router | 192.168.6.1 | 255.255.255.0 | N/A |
| HQ | G0/0/0.5 | router | 192.168.5.1 | 255.255.255.128 | N/A |
| HQ | G0/0/0.10 | router | 192.168.5.128 | 255.255.255.128 | N/A |
| HQ | S0/1/0 | router | 192.168.0.10 | 255.255.255.252 | N/A |
| SW-B1 | VLAN 1 | switch | 192.168.1.252 | 255.255.255.0 | 192.168.1.1 |
| SW-B2 | VLAN 1 | switch | 192.168.2.62 | 255.255.255.0 | 192.168.2.1 |
| SW-F1 | VLAN 1 | switch | 192.168.3.252 | 255.255.255.0 | 192.168.3.1 |
| SW-F2 | VLAN 1 | switch | 192.168.4.252 | 255.255.255.0 | 192.168.4.1 |
| SW-HQ1 | VLAN 1 | switch | 192.168.6.252 | 255.255.255.0 | 192.168.6.1 |
| SW-HQ2 | VLAN 1 | switch | 192.168.6.253 | 255.255.255.0 | 192.168.6.1 |
| SW-HQ3 | VLAN 1 | switch | 192.168.6.254 | 255.255.255.0 | 192.168.6.1 |

Initial Connectivity tests.

| Source | Destination | Status |
| --- | --- | --- |
| PC1 | PC2 | Successful |
| PC1 | PC3 | Failed |
| PC1 | PC4 | Successful |
| PC1 | PC5 | Failed |
| PC1 | PC6 | Failed |
| PC1 | PC7 | Failed |
| PC1 | Internet Server | Failed |
| PC2 | PC3 | Failed |
| PC2 | PC4 | Successful |
| PC2 | PC5 | Failed |
| PC2 | PC6 | Failed |
| PC2 | PC7 | Failed |
| PC2 | Internet Server | Successful |
| PC3 | PC4 | Failed |
| PC3 | PC5 | Failed |
| PC3 | PC6 | Failed |
| PC3 | PC7 | Failed |
| PC3 | Internet Server | Failed |
| PC4 | PC5 | Failed |
| PC4 | PC6 | Failed |
| PC4 | PC7 | Failed |
| PC4 | Internet Server | Successful |
| PC5 | PC6 | Failed |
| PC5 | PC7 | Failed |
| PC5 | Internet Server | Failed |
| PC6 | PC7 | Successful |
| PC6 | Internet Server | Failed |

| Device | Issue | Action |
| --- | --- | --- |
| PC1 | Problem 1: PC1 failed to ping Internet server. Issue likely to be connection between pc and Internet server as other devices have successfully pinged the internet server. A ping from Branch 1 failed to reach the internet server. A ping from the Hub was a success. Issue may lay with the Hub router. after running a show running config command and inspected interface S0/1/0 i see that ip nat inside configuration is missing. | add the ip nat inside configuration to the hub routers serial int s0/1/0 |
| PC3 | Problem 2: PC3 cannot ping internet server or any other PC on the network. A ping from PC3 and its gateway failed, it seems the issue is likely to be with the gateway router. After inspecting the running config of the factory router, I identified that interface G0/0/1 is shutdown. After resolving this, PC3 can only ping PC2 which has the same gateway router. After analysing the running config again i noticed that ospf config did not have network 192.168.4.0/24. | Running a no shutdown command on int G0/0/1. Adding network 192.168.4.0 0.0.0.255 area 0 to the ospf statement. |
| PC5 | Problem 3: unreachable to all hosts and cannot ping internet server. After looking at the running config, I saw no issue. I looked at the ip interface brief and saw nothing wrong there and I attempted to ping the internet server and it was a success. I then looked on PC5 checked the ip config and saw that the gateway was blank. | on the command line of PC5 I run 'ipconfig 192.168.5.227 255.255.255.128 192.168.5.129' in line with my documentation |
| PC6 and PC7 | Problem 4: PC6 and PC7 can ping each other but can't ping any other host or internet server. After seeing the running config i saw nothing wrong so I checked ip routes. It seems that ospf routes have not been shared. Then I noticed that int s0/1/0 on Branch 2 did not match the documentation i had. | changing the int Ip address to 192.168.0.6 255.255.255.252 to match the documentation. |

| Source | Destination | Status |
| --- | --- | --- |
| PC1 | PC2 | Successful |
| PC1 | PC3 | Successful |
| PC1 | PC4 | Successful |
| PC1 | PC5 | Successful |
| PC1 | PC6 | Successful |
| PC1 | PC7 | Successful |
| PC1 | Internet Server | Successful |
| PC2 | PC3 | Successful |
| PC2 | PC4 | Successful |
| PC2 | PC5 | Successful |
| PC2 | PC6 | Successful |
| PC2 | PC7 | Successful |
| PC2 | Internet Server | Successful |
| PC3 | PC4 | Successful |
| PC3 | PC5 | Successful |
| PC3 | PC6 | Successful |
| PC3 | PC7 | Successful |
| PC3 | Internet Server | Successful |
| PC4 | PC5 | Successful |
| PC4 | PC6 | Successful |
| PC4 | PC7 | Successful |
| PC4 | Internet Server | Successful |
| PC5 | PC6 | Successful |
| PC5 | PC7 | Successful |
| PC5 | Internet Server | Successful |
| PC6 | PC7 | Successful |
| PC6 | Internet Server | Successful |