| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 5 | LEFEL 5 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| May 2021 | Mai 2021 |

MODULE CODE: ACCA5024

COD Y MODWL: ACCA5024

ADVANCED NETWORKING

TIME:  2 HOURS

Name			: Thomas Phillips

Student ID		: PHI17833288

Date & Time		: 29/4/202 | 10:10AM

This is an individual practical test, held under examination conditions. You will be assigned three Switches and three Routers; you should configure these devices based on the given specifications. Handwritten notes are allowed

This paper is marked out of 100.

Topology

Addressing Table: IPv4 (5 marks)

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | Gi0/0 | 192.168.1.1 | 255.255.255.0 | N/A |
|  | S0/0/0 | 192.168.99.1 | 255.255.255.252 | N/A |
|  | S0/0/1 | 192.168.99.5 | 255.255.255.252 | N/A |
| R2 | S0/0/0 | 192.168.99.2 | 255.255.255.252 | N/A |
|  | Gi0/0 | 192.168.25.65 | 255.255.255.224 | N/A |
|  | Lo1 | 172.16.1.1 | 255.255.255.0 | N/A |
|  | Lo2 | 172.16.2.1 | 255.255.255.0 | N/A |
| R3 | S0/0/1 | 192.168.99.6 | 255.255.255.252 | N/A |
|  | Lo209 | 209.16.20.245 | 255.255.255.248 | N/A |
| PC1 |  | 192.168.1.2 | 255.255.255.0 | 192.168.1.1 |
| PC2 |  | 192.168.25.66 | 255.255.255.224 | 192.168.25.65 |

- Addressing Table: IPv6 (5 marks)

| Device | Interface | IPv6 Address | Link-local | Gateway |
| --- | --- | --- | --- | --- |
| R1 | Gi0/0 | 2001:abcd:ceed:1::1/64 | Fe80::1/10 | N/A |
|  | S0/0/0 | 2001:abcd:beef:10::1/64 | Fe80::1/10 | N/A |
|  | S0/0/1 | 2001:abcd:beef:20::1/64 | Fe80::1/10 | N/A |
| R2 | S0/0/0 | 2001:abcd:beef:10::2/64 | Fe80::2/10 | N/A |
|  | Gi0/0 | 2001:1111:bd68:64::1/64 | Fe80::1/10 | N/A |
|  | Lo60 | 2001:db8:cafe:60::1/64 | Fe80::1/10 | N/A |
|  | Lo61 | 2001:db8:cafe:61::1/64 | Fe80::1/10 | N/A |
| R3 | S0/0/1 | 2001:abcd:beef:20::2/64 | Fe80::2/10 | N/A |
|  | Lo209 | 2001:ceed:db8:209::225/64 | Fe80::1/10 | N/A |
| PC1 |  | 2001:abcd:ceed:1::2/64 | Fe80::2/10 | 2001:abcd:ceed:1::1 |
| PC2 |  | 2001:1111:bd68:64::2/64 | Fe80::2/10 | 2001:1111:bd68:64::1 |

Task 1: - Topology Creation (5 marks)

Using Packet Tracer software create the topology shown above and complete all tasks below.

Task 2: - Device Configuration (75 marks)

- On all devices,

  - Configure hostname on all devices

  - Set privilege EXEC password

  - Configure line console password

  - Set logging synchronous

  - Configure line VTY password

  - Disable DNS lookup

  - Set password encryption

  - Set banner “Authorized Users Only!”

- On all devices, configure interfaces with IPv4 and v6 addresses as shown in the topology

- On R1, configure IPv4 static route to R2 and R3 networks, excluding Lo209 network.

- On R2, configure IPv4 static route to R1 and R3 networks, excluding Lo209 network.

- On R3, configure IPv4 static router to R1 and R2 networks.

- Configure default static route to reach Lo209 network using next-hop ip address.

- Set the bandwidth on serial links to 512kbps.

- Set the clock rate on DCE to be 64000

- On R1, configure IPv6 static route to R2 and R3 networks, excluding Lo209 network.

- On R2, configure IPv6 static route to R1 and R3 networks, excluding Lo209 network.

- On R3, configure IPv6 static router to R1 and R2 networks.

- Configure IPv6 default static route to reach Lo209 network using next-hop ip address.

- Using 1024 bit RSA keys configure SSH access on R1. Use domain soac.com, username: admin and password: Sw@ns3a.

Task3: Network connectivity (10 marks)

Using ping command, verify end-to-end connectivity for both IPv4 and v6 networks:

From PC1, ping PC2. Is it successful? ___yes_____

From PC1, ping Lo209. Is it successful? ___yes____

From PC2, ping Lo209. Is it successful? ___yes_____

Verify SSH connectivity:

From PC1, establish telnet connection to R1. Is it successful? __yes____(this should fail)

From PC1, establish SSH connection to R1. Is it successful? ___yes____

Submission Requirement

The packet tracer file should be saved as “studentname_id” and uploaded onto Moodle. In addition to packet tracer file, you must also submit

- the output of show running-configuration command in a text file for all devices configured.

- This document with Task 3 completed

All documentation for your solution should be submitted electronically on Moodle.

It is your responsibility that you submit all the outputs and files correctly.

R1

R1#show running-config

Building configuration...

Current configuration : 1630 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname R1

!

!

!

enable password 7 0822455D0A16

!

!

!

!

!

!

no ip cef

no ipv6 cef

!

!

!

username admin privilege 15 password 7 08125B6E070A5616

!

!

license udi pid CISCO2901/K9 sn FTX1524OS3T-

!

!

!

!

!

!

!

!

!

no ip domain-lookup

ip domain-name soac.com

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0

ip address 192.168.1.1 255.255.255.0

duplex auto

speed auto

ipv6 address FE80::1 link-local

ipv6 address 2001:ABCD:CEED:1::1/64

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

bandwidth 512

ip address 192.168.99.1 255.255.255.252

ipv6 address FE80::1 link-local

ipv6 address 2001:ABCD:BEEF:10::1/64

!

interface Serial0/0/1

bandwidth 512

ip address 192.168.99.5 255.255.255.252

ipv6 address FE80::1 link-local

ipv6 address 2001:ABCD:BEEF:20::1/64

clock rate 64000

!

interface Vlan1

no ip address

shutdown

!

ip classless

ip route 172.16.1.0 255.255.255.0 Serial0/0/0

ip route 172.16.2.0 255.255.255.0 Serial0/0/0

ip route 192.168.25.64 255.255.255.224 Serial0/0/0

ip route 0.0.0.0 0.0.0.0 192.168.99.6

!

ip flow-export version 9

!

ipv6 route 2001:DB8:CAFE:60::/64 Serial0/0/0

ipv6 route 2001:DB8:CAFE:61::/64 Serial0/0/0

ipv6 route 2001:1111:BD68:64::/64 Serial0/0/0

ipv6 route 2001:ABCD:BEEF:20::/64 Serial0/0/1

!

!

banner motd ^CAuthorized Users Only!^C

!

!

!

!

!

line con 0

password 7 082D495A041C0C19

logging synchronous

login

!

line aux 0

!

line vty 0 4

password 7 0822455D0A16

login

!

!

!

End

R2

R2#show running-config

Building configuration...

Current configuration : 1740 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname R2

!

!

!

enable password 7 0822455D0A16

!

!

!

!

!

!

no ip cef

no ipv6 cef

!

!

!

!

license udi pid CISCO2901/K9 sn FTX15241J8Q-

!

!

!

!

!

!

!

!

!

no ip domain-lookup

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface Loopback1

ip address 172.16.1.1 255.255.255.0

!

interface Loopback2

ip address 172.16.2.1 255.255.255.0

!

interface Loopback60

no ip address

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:CAF:60::1/64

ipv6 address 2001:DB8:CAFE:60::1/64

!

interface Loopback61

no ip address

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:CAFE:61::1/64

!

interface GigabitEthernet0/0

ip address 192.168.25.65 255.255.255.224

duplex auto

speed auto

ipv6 address FE80::1 link-local

ipv6 address 2001:1111:BD68:64::1/64

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

bandwidth 512

ip address 192.168.99.2 255.255.255.252

ipv6 address FE80::2 link-local

ipv6 address 2001:ABCD:BEEF:10::2/64

clock rate 64000

!

interface Serial0/0/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

ip classless

ip route 192.168.1.0 255.255.255.0 Serial0/0/0

ip route 192.168.99.4 255.255.255.252 Serial0/0/0

ip route 0.0.0.0 0.0.0.0 192.168.99.1

!

ip flow-export version 9

!

ipv6 route 2001:ABCD:CEED:1::/64 Serial0/0/0

ipv6 route 2001:ABCD:BEEF:20::/64 Serial0/0/0

ipv6 route ::/0 2001:ABCD:BEEF:10::1

!

!

banner motd ^CAuthorized Users Only!^C

!

!

!

!

!

line con 0

password 7 082D495A041C0C19

logging synchronous

login

!

line aux 0

!

line vty 0 4

password 7 0822455D0A16

login

!

!

!

end

R3

R3#show running-config

Building configuration...

Current configuration : 1609 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname R3

!

!

!

enable password 7 0822455D0A16

!

!

!

!

!

!

no ip cef

no ipv6 cef

!

!

!

!

license udi pid CISCO2901/K9 sn FTX152475C9-

!

!

!

!

!

!

!

!

!

no ip domain-lookup

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface Loopback209

ip address 209.16.20.245 255.255.255.248

ipv6 address FE80::1 link-local

ipv6 address 2001:CEED:DB8:209::225/64

!

interface GigabitEthernet0/0

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/0/1

bandwidth 512

ip address 192.168.99.6 255.255.255.252

ipv6 address FE80::2 link-local

ipv6 address 2001:ABCD:BEEF:20::2/64

!

interface Vlan1

no ip address

shutdown

!

ip classless

ip route 192.168.99.0 255.255.255.252 Serial0/0/1

ip route 172.16.1.0 255.255.255.0 Serial0/0/1

ip route 172.16.2.0 255.255.255.0 Serial0/0/1

ip route 192.168.1.0 255.255.255.0 Serial0/0/1

ip route 192.168.25.64 255.255.255.224 Serial0/0/1

!

ip flow-export version 9

!

ipv6 route 2001:ABCD:BEEF:10::/64 Serial0/0/1

ipv6 route 2001:DB8:CAFE:60::/64 Serial0/0/1

ipv6 route 2001:DB8:CAFE:61::/64 Serial0/0/1

ipv6 route 2001:1111:BD68:64::/64 Serial0/0/1

ipv6 route 2001:ABCD:CEED:1::/64 Serial0/0/1

!

!

banner motd ^CAuthorized Users Only!^C

!

!

!

!

!

line con 0

password 7 082D495A041C0C19

logging synchronous

login

!

line aux 0

!

line vty 0 4

password 7 0822455D0A16

login

!

!

!

end