#include <main.h>

#define LCD_ENABLE_PIN PIN_D2

#define LCD_RS_PIN PIN_D0

#define LCD_RW_PIN PIN_D1

#define LCD_DATA4 PIN_D4

#define LCD_DATA5 PIN_D5

#define LCD_DATA6 PIN_D6

#define LCD_DATA7 PIN_D7

#include <lcd.c>

// When changing task stop the motor, clear the display and clear the LEDs

void handle_task_change(unsigned int* current_task, unsigned int* prev_task)

{

if(*current_task != *prev_task)			// If the task has changed

{

// Stop the motor

output_low(PIN_C0);

output_low(PIN_C1);

printf(lcd_putc, "\f");				// Clear the display

output_b(0);					// Turn off the LEDs

*prev_task = *current_task;			// Update previous task

}

}

// Read the desired ADC channel and return its value

unsigned int read_adc_channel(unsigned int channel)

{

set_adc_channel(channel);

delay_us(30);

return(read_adc());

}

// When SW1-A is selected, the value of RC1 will drive Motor A, if the value is below 75 the motor will drive anti-clockwise

// If the value is above 175 the motor will drive clockwise. If the value is between 76 and 174 the motor will stop

// The value of the reading from the ADC will be displayed centred on LCD Line 1 along with the direction of rotation centred on line 2 of the LCD

void task_1()

{

unsigned int pot_1 = read_adc_channel(0);		// Read the ADC, store in pot_1

lcd_gotoxy(6, 1);

printf(lcd_putc, "POT1 = %3u  ", pot_1);		// Output the pot_1 value to the display

lcd_gotoxy(1, 2);					// Go to line 2 in preparation for each of the directions being printed

// If the potentiometer is below 75 spin the motor anti-clockwise

// If the potentiometer is above 175 spin the motor clockwise

// Else, stop the motor

// For each state, print the direction of the motor

if(pot_1 <= 75)

{

output_low(PIN_C0);

output_high(PIN_c1);

printf(lcd_putc, "   Anti-Clockwise"); 		// Print centered

}

else if(pot_1 >= 175)

{

output_high(PIN_C0);

output_low(PIN_C1);

printf(lcd_putc, "     Clockwise  ");		// Print as centered as possible

}

else

{

output_low(PIN_C0);

output_low(PIN_C1);

printf(lcd_putc, "      Stopped    ");		// Print as centered as possible

}

}

// When SW1-B is selected, the digital output of the ADC reading RV1 will be shown on the LEDs

// The value will be displayed centred on line 1 of the LCD and echoed onto the virtual terminal

void task_2()

{

unsigned int pot_1 = read_adc_channel(0);		// Read the ADC, store in pot_1

output_b(pot_1);					// Output the pot_1 value to the LEDs.

printf("POT1 = %3u\n\r", pot_1);			// Print pot_1 to the virtual terminal

lcd_gotoxy(6, 1);

printf(lcd_putc, "POT1 = %3u  ", pot_1);		// Output the pot_1 value to the display

}

// When SW1-C is selected, pressing the button will make Motor A rotate clockwise as long as the button is pressed, and the motor will stop when released

// The status of the button will be displayed on the LCD on centred on line 3 and echoed once per button press or release on the virtual terminal

void task_3()

{

unsigned state_changed = input_change_a() & 2;	// State the ensure virtual terminal is only printed to once

// Check if the button is pressed

if(input(PIN_A1))

{

if(state_changed) printf("Button Pressed\n\r");	// Print to the virtual terminal once when button is pressed

// Spin the motor clockwise

output_high(PIN_C0);

output_low(PIN_C1);

lcd_gotoxy(22, 1);

printf(lcd_putc, "  Button Pressed   ");		// Display button state on the display

}

else

{

if(state_changed) printf("Button Released\n\r");	// Print to the virtual terminal once when button is released

// Spin the motor anti-clockwise

output_low(PIN_C0);

output_low(PIN_C1);

lcd_gotoxy(22, 1);

printf(lcd_putc, "Button Not Pressed ");		// Display button state on the display

}

}

// When SW1-D is selected the LEDs will flash from left to right and back again in knight rider fashion changing @ 200mS intervals

// The message "Kitt mode" will be displayed centred on line 4 of the LCD display and echoed once on the virtual terminal

void task_4()

{

//printf(lcd_putc, "\f");

if(input_change_a() & 16) printf("Kitt mode\r\n");	// Check if the switch has changed to the D state, if so, print "Kitt mode"

lcd_gotoxy(21, 2);

printf(lcd_putc, "      Kitt mode");		// Print to the display

unsigned int led_direction = 1;			// Direction for the LEDs to light, 1 is L->R, 0 is L<-R

unsigned int current_led = 128;			// Start on left-most LED

// Print the animation in this loop

for(unsigned int i = 0; i <= 14; i++)

{

output_b(current_led); 				// Turn on & off LEDs

if(i != 14) delay_ms(200);			// Delay for 200ms,

// but not on the 14th iteration to avoid the first LED being on for 400ms if task_4() is recalled

// Change the current_led value depending on the direction

current_led = led_direction ? current_led >> 1 : current_led << 1;

if(i == 6) led_direction = 0;			// When reaching the right-most LED, change direction

}

}

void main()

{

unsigned int current_task = 0, prev_task = 0;	// Variables for task changed event

// ADC setup

setup_adc_ports(sAN0);

setup_adc(ADC_CLOCK_INTERNAL|ADC_TAD_MUL_0);

lcd_init();						// Initialise the LCD

// If the program starts on switch state D, print "Kitt mode" to the display

// If this case is not handled and the switch is in state D, "Kitt mode" will not be printed

if(input(PIN_A5))

{

printf("Kitt mode\r\n");

}

while(TRUE)

{

// Input portA, shift right by 2 to ignore button and potentiometer input then mask off top 2 unused pins

// Depending the switch value, perform each of the assigned tasks

// handle_task_change() must be called within each case instead of checking the state each iteration of the loop

// This is because otherwise there may be a state change within the small timeframe of checking the task change and switch state

// It may not be as tidy, but its functionally safer

switch(input_a() >> 2 & 0xf)

{

case 1:

current_task = 1;

handle_task_change(&current_task, &prev_task);

task_1();

break;

case 2:

current_task = 2;

handle_task_change(&current_task, &prev_task);

task_2();

break;

case 4:

current_task = 3;

handle_task_change(&current_task, &prev_task);

task_3();

break;

case 8:

current_task = 4;

handle_task_change(&current_task, &prev_task);

task_4();

break;

default:

break;

}

}

}

Comments

2.. 20   3.. 30    4.. 40   5.. 10

***************************************

2..Your code has a number of structures and all work except you don't put the value of RV1 on the LEDS in section 1. Your statements indicate what most lines do 68%

3.. Your program flows nicely 75%

4.. Your program achieves the desired results. 75%

5.. Your program is nicely laid out and well indented but could have been remarked better. 70%