Cerys Versey

PIC 1 Code

main.c

#include <main.h>

#include <2408.C>

#define LCD_ENABLE_PIN PIN_B2

#define LCD_RS_PIN PIN_B0

#define LCD_RW_PIN PIN_B1

#define LCD_DATA4 PIN_B4

#define LCD_DATA5 PIN_B5

#define LCD_DATA6 PIN_B6

#define LCD_DATA7 PIN_B7

//#define HS PIN_B3  // DEFINES THE HAND SHAKE PIN FOR Hand shake used in Read and write protocol *** sart of 7 ran out of time

#include <lcd.c>

char read_buttons(void);

char buts = 0;

unsigned int value=0x00;

char ep_r(char);

char valueb = 0;

void ep_w(char where,char what);

char txbyte;

char dvalue =0;

char cvalue =0;

void main()

{

setup_adc_ports(NO_ANALOGS, VSS_VDD);

init_ext_eeprom();

lcd_init();

while(TRUE)

{

buts = read_buttons(); 		// reads the value of the button press in *** task 1 ***

ep_w(0x00,buts);		 // writes the value of the button press to mem 0x00

delay_ms(30); 			// 20 ms delay

printf(lcd_putc,"\ftask 2  %3u",ep_r(0x00)); // prints the value of 0x00 onto the LCD screen an states the task number 2 ** task 2**

delay_ms(30); 			// 20 ms delay

valueb = ep_r(0x01); 		// reads the value held in 0x00 into a variable

lcd_gotoxy(1,2); 		// writes it to the lcd second line

printf(lcd_putc,"task 6 %3u",valueb); 		//prints the task number and value held in position 0x01 ** task 6**

//ep_w(0x0,ep_r(0x00)); // writes the value from mem 0x01 to 0x00

//dvalue= ep_r(0x01); // reads the value held in 0x00

//cvalue = ep_r(0x01); // reads the value held in 0x01 and assigns it to a variable

//lcd_gotoxy(1,2);

//printf(lcd_putc,"task 6 %3u",dvalue);

}

}

char read_buttons(void)

{

char value=0;                   // setup a variable for the buttons

while(!input_a());                  // wait for button press

value=input_a();              // get the value of port b

//		while(input_a());             // wait for button release

delay_ms(200);                      // time button release

return (value);                     // return the value of the buttons pressed

}

void ep_w(char where,char what)

{

while(!input(hs));						// wait for mem chip to be available

output_low(hs);							// set hs line low

i2c_start();							// issue i2c start pulse

i2c_write(0xA0);						// issue chip type and number

i2c_write(where);						// send the address to write to

i2c_write(what);						// send the data to be written

delay_ms(20);

i2c_stop();								// issue an i2c stop pulse

delay_ms(20);							// allow i2c chip to store data

input(hs);								// release hs line

}

char ep_r(char where)

{

char data=0;

while(!input(hs));						// wait for mem chip to be available

output_low(hs);							// set hs line low (mem busy)

i2c_start();							// issue i2c start pulse

i2c_write(0xA0);						// issue chip type and number /// needs to be b1 on both pins ran out of time to implement

i2c_write(where);						// send address to be read from

delay_ms(20);							// wait for chip to set its for read

i2c_start();							// issue another start pulse

delay_ms(20);							// wait for chip to be ready

i2c_write(0xA1);						// issue chip type and number read command

data=i2c_read();							// read data from the chip

delay_ms(20);

i2c_stop();

delay_ms(20);

input(hs);

return(data);

}

// wirte 1 and

/*code bank

//printf(lcd_putc,"\f welcome to the");

//lcd_gotoxy(1,2);									// display on line 2

//printf(lcd_putc," for Distributed ");

//while(true);

//TODO: User Code

//printf(lcd_putc," for Distributed ");

//printf(lcd_putc," for Distributed ");

//ep_w(0x00,buts);

//lcd_gotoxy(2,1);

// printf(lcd_putc,"%3u",dvalue);

//printf(lcd_putc,"\fbutton%3u",buts);

//lcd_gotoxy(1,2);									// display on line 2

*/

main.h

#include <18F2525.h>

#device ADC=10

#FUSES NOWDT                 	//No Watch Dog Timer

#FUSES FCMEN                 	//Fail-safe clock monitor enabled

#FUSES IESO                  	//Internal External Switch Over mode enabled

#FUSES PUT                   	//Power Up Timer

#FUSES NOMCLR

#FUSES BROWNOUT              	//Reset when brownout detected

#FUSES BORV21                	//Brownout reset at 2.1V

#FUSES NOPROTECT             	//Code not protected from reading

#FUSES NOWRT                 	//Program memory not write protected

#FUSES NOEBTR                	//Memory not protected from table reads

#use delay(internal=8MHz)

#use STANDARD_IO( C )

#use i2c(Master,Fast,sda=PIN_C4,scl=PIN_C3,force_hw)

#define HS pin_B3

PIC 2 Code

main.c

#include <main.h>

#include <2408.C>

#define LCD_ENABLE_PIN PIN_B2

#define LCD_RS_PIN PIN_B0

#define LCD_RW_PIN PIN_B1

#define LCD_DATA4 PIN_B4

#define LCD_DATA5 PIN_B5

#define LCD_DATA6 PIN_B6

#define LCD_DATA7 PIN_B7

//#define HS PIN_B3

#include <lcd.c>

char ep_r(char);

void ep_w(char where,char what);

char bvalue = 0;

char cvalue = 0;

char mem01 =0;

char read_buttons(void);

char buts=0;

void main()

{

setup_adc_ports(NO_ANALOGS, VSS_VDD);

init_ext_eeprom();

lcd_init();

while(TRUE)

{

bvalue = ep_r(0x00);				 // reads the value held in mem 0x00 and assigns it to a variable

lcd_gotoxy(1,1);									// display on line 1

printf(lcd_putc,"\ftask 3 %3u",bvalue);		// prints the task number and the value held in mem location  0x00  *** task 3**

ep_w(0x01,bvalue);				// writes the value held in mem 0x00 to 0x01

cvalue = ep_r(0x01);				// reads the value from 0x01 to a variable

lcd_gotoxy(1,2);					// prints it on the second line of the lcd

printf(lcd_putc,"task 4 %3u",cvalue); 		// prints the task number and the value held in 0x01 to confirm **task 4**

output_a(cvalue);  				// outputs the last button press to the LED's ** task 5**

}

}

void ep_w(char where,char what)							// name of the subroutine and two specified outputs and there catagorisation

{

while(!input(hs));						// wait for mem chip to be available

output_low(hs);							// set hs line low -- write hs low

i2c_start();							// issue i2c start pulse -- starts the 12c stream

i2c_write(0xA0);						// issue chip type and number -- writes the value in

i2c_write(where);						// send the address to write to -- sleects where to send the value

i2c_write(what);						// send the data to be written -- and what to write to it

delay_ms(30);							// delay of 20ms is added

i2c_stop();								// issue an i2c stop pulse

delay_ms(30);							// allow i2c chip to store data

input(hs);								// release hs line

}

char ep_r(char where)								// name of the subroutine so it can be called elsewhere in the script

{

char data=0;							// assigns the data a value

while(!input(hs));						// wait for mem chip to be available - while loop set a conditional statment

output_low(hs);							// set hs line low (mem busy)

i2c_start();							// issue i2c start pulse

i2c_write(0xA0);						// issue chip type and number

i2c_write(where);						// send address to be read from

delay_ms(30);							// wait for chip to set its for read

i2c_start();							// issue another start pulse

delay_ms(30);							// wait for chip to be ready

i2c_write(0xA1);						// issue chip type and number read command

data=i2c_read();							// read data from the chip

delay_ms(30);							// adds a 20ms delay

i2c_stop();							// stops the i2c subroutine

delay_ms(30);							// adds a 20 ms delay

input(hs);							// allws the hs input back into play by allowing it to have another input

return(data);							// returns the value in data

}

char read_buttons(void)

{

char value=0;                   // setup a variable for the buttons

while(!input_a());                  // wait for button press

value=input_a();              // get the value of port b

//          while(input_b());             // wait for button release

delay_ms(200);                      // time button release

return (value);                     // return the value of the buttons pressed

}

/*

code bank

printf(lcd_putc,"\f 2hr Practical test");

lcd_gotoxy(1,2);									// display on line 2

printf(lcd_putc," intelligent systems ");

*/

main.h

#include <18F2525.h>

#device ADC=10

#FUSES NOWDT                 	//No Watch Dog Timer

#FUSES FCMEN                 	//Fail-safe clock monitor enabled

#FUSES IESO                  	//Internal External Switch Over mode enabled

#FUSES PUT                   	//Power Up Timer

#FUSES NOMCLR

#FUSES BROWNOUT              	//Reset when brownout detected

#FUSES BORV21                	//Brownout reset at 2.1V

#FUSES NOPROTECT             	//Code not protected from reading

#FUSES NOWRT                 	//Program memory not write protected

#FUSES NOEBTR                	//Memory not protected from table reads

#use delay(internal=8MHz)

#use STANDARD_IO( C )

#use i2c(Master,Fast,sda=PIN_C4,scl=PIN_C3,force_hw)

#define HS pin_B3

Comments

Versey Cerys

1.. The simulation runs and puts the button number in the top of lcd 1 and writes the data to the address 0x00 in the memory,

2.. the value is recieved by pic 2 and the value is displayed on lcd 2 and written to address location 0x01 intermittantly, there appears to be a timing issue somewhere.

3  your program does not display the key value on the 2nd led but it does put it into a feedback position in the i2c mem.

4. your program does not display the key value on the 2nd led but it does put it into a feedback position in the i2c mem.

5.. Some lines on the displays agree with the displayed leds.

6.. the data is not returned to chip 1 so cannot display on lcd 1

your hand shaking works correctly

the error warning does not work correctly.