| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| MAY 2024 | 2024 |

MODULE CODE: ACDA4006

Electrical and Electronic Principles

Programme: BEng. Electrical and Electronics Engineering

TIME:  2 HOURS

Maximum Marks : 100

answer All the questions

Equipment or materials required – Calculator, Formula sheet, Graph sheet,

Adnoddau neu ddefnyddiau sydd eu hangen - Cyfrifiannell

ANSWER ALL THE QUESTIONS

Question 1 [25 Marks]

- A power supply requires 25W of input power and produces an output power of 20W. Calculate its efficiency in percentage and the total power loss.		[5 Marks]

- A solar yard light has a solar collector that can provide 1.0W of power for charging the 3.0V batteries. What is the maximum charging current the solar collector could supply to the fully discharged 3.0V batteries?				[4 Marks]

- For the circuit shown in fig.1, calculate the following.

- Equivalent Resistance,						[6 Marks]

- Current flowing through 20Ω Resistor				[3 Marks]

Fig:1

- Determine the voltage across each resistor in the circuit shown in fig.2.

[4 Marks]

Fig.2

- Define Kirchoff’s Current Law.						[3 Marks]

Question 2 [25 Marks]

- Determine the current across each resistor in the circuit shown in fig.3, using mesh analysis (Kirchhoff’s Voltage Law)		[16 Marks]

Fig.3

- For the circuit shown in fig 4, determine the following

- Total impedance						[5 Marks]

- Circuit current							[2 Marks]

- Phase angle							[2 Marks]

Fig.4

Question 3 [25 Marks]

- Define the following terms

- Resistance								[2 Marks]

- Impedance								[2 Marks]

- Reactance									[2 Marks]

- Voltage Rating								[2 Marks]

- The alternating current is given by i=40 sin314t. Determine the following

- Maximum Value of current		[1 Mark]

- RMS Value of Current		[2 Marks]

- Frequency		[2 Marks]

- What is the use of the circuit shown in fig.5?  Describe the functions of each stage of the circuit with necessary diagrams.		[12 Marks]

Fig.5

Question 4 [25 Marks]

- A 12 V Battery has an internal Resistance of 12Ω, Calculate the power dissipated by various load resistances given in the below table and complete the table.		[15 Marks]

| Load Resistance (Ω) | Load power (W) |
| --- | --- |
| 4 |  |
| 8 |  |
| 12 |  |
| 16 |  |
| 20 |  |

- Plot a graph of the load Power against the load resistance,.		[10 Marks]

Diwedd/End