Harry Mcquillan-Butt (P146303) 29/01/2018

Advanced engine technology

Cylinder head analysis

M2X9479

Contents

Introduction	2

Research	2

Methodology	3

Safety regulations	3

Preparing the flow bench for testing	3

Results	4

Rover cylinder head inlet	4

Rover cylinder head exhaust	6

Ford cylinder head inlet	8

Ford cylinder head exhaust	10

Analysis	12

Conclusion	13

Bibliography	15

# Introduction

In order to complete this assignment the students put themselves into groups in which they carried out flow test’s on a ford cylinder head and a rover cylinder head in order to compare the two. The students carried out the flow tests using a super flow sf600 flow bench, other measuring equipment was also used during the tests of the two heads. The students tested the flow of both the inlet valves and the exhaust valves.

# Research

A flow bench is a simple device which is used to measure and quantify the amount of air flow through cylinder head runners and intake manifolds. Different types of flow bench are available for certain purposes such as increasing engine performance and fuel economy, developing catalytic converters and certifying their flow rates at the end of a production line as well as a variety of other air flow test applications across multiple industries. (Superflow.com, 2017)

During a flow test, the component is mounted on the flow bench with a blue-tac gasket and secured to the test fixture using vise-grips and G-clamps. The flow bench is then used to suck or blow air through the component at a constant fixed pressure, the manufacturers recommended test pressure is 25 inches of water which is read on the manometer. The machine displays the air flow through the component. (Bohacz, 2017)

Test piece

Component

Test port

A cylinder heads port is the passage which air travels down to get to the combustion chamber, these ports are flow tested in order to make adjustments to the port and improve the engines performance.

Cylinder bore adapter

A flow bench often requires an adaptor to be fabricated to adapt the test object to the flow bench surface with an air tight seal.

Bench surface

This is the surface which the cylinder bore adapter is connected to, between the adapter and the bench surface is a makeshift gasket in order to create the airtight seal.

Settling chamber

The settling chamber Is the pipe section between the bench discharge port and the device which measures the airflow. The job of the settling chamber is in its name, it settles the turbulent air that has passed

Flow element

This is the element that measures the velocity of the air passing through the component being tested. From the velocity, it is possible to calculate the volume of air passing through the component.

Air supply

A bi-directional air source that causes air to flow through the component that is being tested in both directions.

(Flowperformance.com, 2017)

The flow bench in the university laboratory features an orifice inside of a larger settling chamber as opposed to usual flow benches which use a flow element, despite this the design and functionality of the flow bench is very much the same. Other methods that flow benches use to measure air flow are via a hot wire anemometer, a vane air meter, a pitot tube as well as a laminar air flow element. The benefit of using an orifice is that it works by measuring the pressure drop across a “hole” (the component being tested) and comparing it to another hole (the orifice), this is beneficial because both “holes” react the same as each other to changes in the atmosphere due to weather which means that the systems functionality is not altered by the outside weather. (Performancetrends.com, 2017)

Volume air flow

The volume flow rate “Q” of a fluid (a liquid or gas) is simply the volume of a fluid passing through a cross section of the given test piece over a test period. In essence the volume flow rate is the amount of air fuel mixture/ exhaust gasses which pass through the head of the engine over a given amount of time. (Physics LibreTexts, 2018)

Since volume air flow measures the amount of volume that passes through a volume over time, the equation below is how it is found.

Volume air flow = volume / time so therefore Q=V/T

# Methodology

## Safety regulations

In order to carry out the test process safely, precautions have to be taken by the students. The students are required to be wearing the correct ppe in order to enter the workshop, the correct ppe consists of steel toe capped boots, dust coat and ear plugs. It is also important to be aware of your surroundings and any potential hazards. It is recommended by lecturers that each student carries out an individual risk assessment prior to the test in order for them to become aware of all the possible hazards that they will be facing.

## Preparing the flow bench for testing

Firstly the students selected two cylinder heads from the universities parts room and brought them over to the flow bench. The students decided to choose two similar head designs by two different manufacturers in order to see the differences in performance and efficiency between the two cylinder heads.

Once the students had selected their cylinder heads their next task was to remove an inlet valve and an exhaust valve from each cylinder head. Once the valves were removed the students used a Vernier calliper to measure the diameter of the head of each valve and then to measure the valve stem diameter for each valve. The measurements that the students took were tabulated using Microsoft excel.

Now the student are ready to setup one of the heads on the super flow sf-600 flow bench, to do so they first had to select a suitable Cylinder bore adapter, it is important that the bore of the adapter is the same as the bore of the cylinder head as this simulates the combustion chamber. A blue-tac gasket was fabricated by the students and then with the gasket in place the cylinder bore adapter was bolted onto the flow bench.

Next the students placed the cylinder head over the adapter using blue-tac to act as a gasket between them, the head was then clamped down using a series of g-clamps. The students used a plate with 4 bolts in it which was fabricated for the students prior to the test by the workshop technician, this plate was used to wind the valves in and out during the test. Initially the plate was held on by a vise-grip and a g-clamp but this was found to be ineffective due to the plate twisting under pressure, the students were able to overcome this problem by bolting the plate directly onto the cylinder head.

A flat steel plate was placed on each side of the valve winding plate, these plate act as a good, flat surface for the dti gauges to be placed on. The dti sensors were placed on the top of the valve compressing bolts in order for the user to see how open the valves are.

Once the students have got the cylinder head setup on the flow bench, the system has to be checked for leaks which is done by turning the machine on with the valves closed and seeing if it is able to hold pressure. If the system held the pressure it is leak free and ready to start testing.

During the test the valves were opened in increments of 1.5 mm until they were completely open. After each 1.5mm increment the flow bench is switched on and the pressure is ranked up to 25 inches of water (flow range increased if necessary) and then the flow range and the flow scale are recorded. The flow scale is the rate of airflow though the valves. This test was repeated with the flow bench motor in reverse for the exhaust valve, ensuring that the inlet valve had been closed completely.

The same process was carried out on both cylinder heads.

# Results

## Rover cylinder head inlet

Effective valve area:

= π41.092-0.2342=0.8901262961

Ans x 2 (2 inlet valves) = 1.780252592

| range | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Inlet (E) | 16.5 | 33.2 | 69.7 | 138.2 | 207.8 | 279.5 |

| 1. test number | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. test pressure | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| 3. valve lift (A) | 1.4 | 2.8 | 4.1 | 5.5 | 6.9 | 8.3 | 9.6 | 11 |
| 4. flow range (1-6) | 1 | 2 | 3 | 3 | 4 | 4 | 4 | 4 |
| 5. flow scale (%) | 82.4 | 84.2 | 56.5 | 67 | 36 | 39 | 40.5 | 41.5 |
| 6. flowscale (5./100) | 0.824 | 0.842 | 0.565 | 0.67 | 0.36 | 0.39 | 0.405 | 0.415 |
| 7. raw test flow l/s (6.x E.) | 13.596 | 27.9544 | 39.3805 | 46.699 | 49.752 | 53.898 | 55.971 | 57.353 |
| cfm= l/s x 2.12 | 28.82352 | 59.26333 | 83.48666 | 99.00188 | 105.4742 | 114.2638 | 118.6585 | 121.5884 |
| 8. leakage (flowscale % when valves are shut) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 9. test cfm (7.-8.) | 28.82352 | 59.26333 | 83.48666 | 99.00188 | 105.4742 | 114.2638 | 118.6585 | 121.5884 |
| 10. test cfm/sqin (9./Effective area) | 16.19069 | 33.28928 | 46.89596 | 55.61114 | 59.24678 | 64.18402 | 66.65263 | 68.29838 |
| 11. potential flow (p30-31 of manual) | 25.3 | 47.3 | 65.7 | 80.2 | 91.1 | 98.2 | 101.5 | 101.1 |
| 12. % potential flow (10./11.) | 63.99483 | 70.37903 | 71.37894 | 69.34058 | 65.03489 | 65.3605 | 65.66762 | 67.55527 |

## Rover cylinder head exhaust

Effective valve area:

= π40.9512-0.2342=0.6673096225

Ans x 2 (2 exhaust valves) = 1.334619245

| range | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Exhaust (F) | 19.5 | 33.2 | 69.7 | 138.2 | 219.6 | 284.6 |

| 1. test number | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. test pressure | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| 3. valve lift (B) | 1.2 | 2.4 | 3.6 | 4.8 | 6 | 7.2 | 8.4 | 9.6 |
| 4. flow range (1-6) | 1 | 2 | 3 | 3 | 4 | 4 | 5 | 5 |
| 5. flow scale (%) | 47 | 54 | 38 | 47.5 | 27 | 29 | 21 | 22 |
| 6. flowscale (5./100) | 0.47 | 0.54 | 0.38 | 0.475 | 0.27 | 0.29 | 0.21 | 0.22 |
| 7. raw test flow cfm (6.x f.) | 9.165 | 20.034 | 28.5 | 35.625 | 40.095 | 43.065 | 46.116 | 48.312 |
| cfm= l/s x 2.12 | 19.4298 | 42.47208 | 60.42 | 75.525 | 85.0014 | 91.2978 | 97.76592 | 102.4214 |
| 8. leakage (flowscale % when valves are shut) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 9. test cmf (7.-8.) | 19.4298 | 42.47208 | 60.42 | 75.525 | 85.0014 | 91.2978 | 97.76592 | 102.4214 |
| 10. test cfm/sqin (9./Effective area) | 14.55830947 | 31.82337 | 45.27134 | 56.58917 | 63.68963 | 68.40738 | 73.25379 | 76.74207 |
| 11. potential flow (p30-31 of manual) | 25.3 | 47.3 | 65.7 | 80.2 | 91.1 | 98.2 | 101.5 | 101.1 |
| 12. % potential flow (10./11.) | 57.5427252 | 67.27985 | 68.90615 | 70.56007 | 69.91177 | 69.66128 | 72.17122 | 75.90709 |

## Ford cylinder head inlet

Effective valve area:

= π41.22-0.22=1.099557429

Ans x 2 (2 inlet valves) =2.199114858

| range | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Inlet (E) | 16.5 | 33.2 | 69.7 | 138.2 | 207.8 | 279.5 |

| 1. test number | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. test pressure | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| 3. valve lift (A) | 1.5 | 3 | 4.5 | 6 | 7.5 | 9 | 10.5 | 12 |
| 4. flow range (1-6) | 2 | 3 | 3 | 3 | 4 | 4 | 4 | 4 |
| 5. flow scale (%) | 54 | 50.5 | 80.4 | 94 | 49 | 50 | 50.5 | 51 |
| 6. flowscale (5./100) | 0.54 | 0.505 | 0.804 | 0.94 | 0.49 | 0.5 | 0.505 | 0.51 |
| 7. raw test flow cfm (6.x E.) | 17.928 | 35.1985 | 56.0388 | 65.518 | 67.718 | 69.1 | 69.791 | 70.482 |
| cfm= l/s x 2.12 | 38.00736 | 74.62082 | 118.8023 | 138.8982 | 143.5622 | 146.492 | 147.9569 | 149.4218 |
| 8. leakage (flowscale % when valves are shut) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 9. test cfm (7.-8.) | 38.00736 | 74.62082 | 118.8023 | 138.8982 | 143.5622 | 146.492 | 147.9569 | 149.4218 |
| 10. test cfm/sqin (9./Effective area) | 17.28303 | 33.93221 | 54.02276 | 63.16094 | 65.28179 | 66.61407 | 67.28021 | 67.94636 |
| 11. potential flow (p30-31 of manual) | 21.3 | 47.3 | 65.7 | 80.2 | 91.1 | 98.2 | 101.5 | 101.1 |
| 12. % potential flow (10./11.) | 81.14097 | 71.73828 | 82.22642 | 78.75429 | 71.65949 | 67.83511 | 66.28593 | 67.20708 |

## Ford cylinder head exhaust

Effective valve area:

π40.912-0.22=0.6189722926

Ans x 2 (2 exhaust valves) = 1.237944585

| range | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Exhaust (F) | 19.5 | 33.2 | 69.7 | 138.2 | 219.6 | 284.6 |

| 1. test number | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. test pressure | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| 3. valve lift (B) | 1.2 | 2.4 | 3.6 | 4.8 | 6 | 7.2 | 8.4 | 9.6 |
| 4. flow range (1-6) | 1 | 2 | 2 | 3 | 3 | 3 | 3 | 3 |
| 5. flow scale (%) | 53.5 | 60 | 86.4 | 39 | 56 | 57.5 | 59 | 60.5 |
| 6. flowscale (5./100) | 0.535 | 0.6 | 0.864 | 0.39 | 0.56 | 0.575 | 0.59 | 0.605 |
| 7. raw test flow cfm (6.x f.) | 10.4325 | 22.26 | 32.0544 | 29.25 | 42 | 43.125 | 44.25 | 45.375 |
| cfm= l/s x 2.12 | 22.1169 | 47.1912 | 67.95533 | 62.01 | 89.04 | 91.425 | 93.81 | 96.195 |
| 8. leakage (flowscale % when valves are shut) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 9. test cfm (7.-8.) | 22.1169 | 47.1912 | 67.95533 | 62.01 | 89.04 | 91.425 | 93.81 | 96.195 |
| 10. test cfm/sqin (9./Effective area) | 17.86582 | 38.12061 | 54.89368 | 50.0911 | 71.92568 | 73.85226 | 75.77884 | 77.70542 |
| 11. potential flow (p30-31 of manual) | 25.3 | 47.3 | 65.7 | 80.2 | 91.1 | 98.2 | 101.5 | 101.1 |
| 12. % potential flow (10./11.) | 70.6159 | 80.59325 | 83.55202 | 62.45772 | 78.95244 | 75.20596 | 74.65895 | 76.85996 |

# Analysis

The first step of an analysis is spotting anomalies in the student’s results, an anomaly is an abnormality which stands out. There are two anomalies in the student’s results that become obvious once the results have been put into graphs.

The student’s second anomaly is in their results for the Ford cylinder head exhaust values and is a lot more blatant than the first anomaly that the students came across. Many factors could have affected the student’s results in order to give them these faults but they were both most likely down to human error

The student’s first anomaly is in their results for the Rover cylinder head inlet values and is fairly easy to spot on the graph below.

# Conclusion

tuning potential – to obtain the true tuning potential the students must subtract their measured maximum cfm/square inch from the maximum calculated potential that can be obtained from the super flow manual.

Rover cylinder head inlet tuning potential

Maximum calculated potential = 80.2 cfm/square inch

Measured value = 55.61114 cfm/square inch

80.2 – 55.61114 = 24.6 cfm/square inch

Tuning potential = 24.6 cfm/square inch

Ford cylinder head inlet tuning potential

Maximum calculated potential = 80.2 cfm/square inch

Measured value = 63.16094 cfm/square inch

80.2 – 63.16094  = 17.03906 cfm/square inch

Tuning potential = 17.03906 cfm/square inch

Rover cylinder head exhaust tuning potential

Maximum calculated potential = 91.1 cfm/square inch

Measured value = 63.68963 cfm/square inch

91.1 – 63.68963 = 27.41037 cfm/square inch

Tuning potential = 27.41037 cfm/square inch

Ford cylinder head exhaust tuning potential

Maximum calculated potential =  91.1 cfm/square inch

Measured value = 71.92568 cfm/square inch

91.1 – 71.92568 = 19.17432 cfm/square inch

Tuning potential = 19.17432 cfm/square inch

From reviewing the results which have been obtained from this investigation, I am able to see that the ford cylinder head has been able to achieve a volume flow rate for its inlet and outlet valves which are both closer to the engines maximum calculated volume flow rate when compared to the results obtained from the rover engine. Having a flow rate closer to the maximum calculated flow rate means that in its current state the cylinder head for the ford engine will perform better than the rover engine.

Both of the cylinder heads which have been tested have fallen short of their maximum potential, there are many factors which can cause this. The first reason for the cylinder heads underperforming when compared to their maximum calculated potential is that the equation used to calculate the maximum potential of the heads does not take into to account limiting factors such as friction, which slows down and disrupt the flow of the gasses through the cylinder head. Another factor which can affect the flow of gasses through the head is the valve design, putting it simply the valve is in the gasses path which slows the gases down substantially by creating a high pressure on one side of the valve and low pressure on the opposite side of the valve. This is a large problem when it comes to volume flow rate which is why there is a large variation of valve designs to suit different uses.

Modifying an engines head is not the only way in which its volumetric efficiency can be improved, an alternative to this is forced induction. The two most widely used methods of forced induction in the motor industry are turbo chargers and super chargers, both are essentially compressors which force more air into the engine. The difference between the two is how they are powered, a supercharger is powered by a pulley connected directly to the crank shaft, whereas a turbo charger is driven by scavenging energy from the exhaust gasses. Both of these methods are proven to be effective when improving volumetric efficiency.

For optimum performance, a motorsport team would run a ported cylinder hear along with a method of forced induction, but in the real world race teams work on set budgets. Due to being run on a budget, parameters are set on what they can afford to do to improve the volumetric efficiency, this is why most low budget teams, when given the choice will opt for forced induction as opposed to cylinder head porting due to the fact that for the similar costs forced induction offers a larger improvement to an engines volumetric efficiency.

# Bibliography

Bohacz, R. (2017). Cylinder Head Flow Testing - Tech Articles - High Perfromance Pontiac. [online] Hot Rod Network. Available at: http://www.hotrod.com/articles/0210hpp-cylinder-head-flow-testing/

Performancetrends.com. (2017). Cite a Website - Cite This For Me. [online] Available at: http://performancetrends.com/ez_flow_system.htm

Flowperformance.com. (2017). Flow Performance | Flow Performance Flow Bench | Flow Bench 101. [online] Available at: http://www.flowperformance.com/flowbench.html

Superflow.com. (2017). Flowbenches | Flow Bench | SuperFlow. [online] Available at: http://www.superflow.com/aspx/cat.aspx?catid=4&navid=10

n, g. (1990). Diesel Engine Cylinder Head Design: The Compromises and the Techniques. 1st ed. SAE.

Mattarelli, e. and Valentini, a. (2000). On The Flow Modeling Through The Valve Assembly In Engine Cycle Simulations. SAE 2000 World Congress, Simulation and Modeling.

Singh, a., Gadekar, s. and Agarwal, a. (2016). In-Cylinder Air-Flow Characteristics Using Tomographic PIV at Different Engine Speeds, Intake Air Temperatures and Intake Valve Deactivation in a Single Cylinder Optical Research Engine. 1st ed. United States: SAE International.

Physics LibreTexts. (2018). 12.1: Flow Rate and Its Relation to Velocity. [online] Available at: https://phys.libretexts.org/TextMaps/Introductory_Physics_TextMaps/Map%3A_College_Physics_(OpenStax)/12%3A_Fluid_Dynamics_and_Its_Biological_and_Medical_Applications/12.1%3A_Flow_Rate_and_Its_Relation_to_Velocity