ACDA6020 LAB SESSION #4

DETERMINATION OF MELTING POINT

AIM OF THE EXPERIMENT: Determine the melting point of four methyl-3-nitrobenzoate samples synthesised in Cranfield to evaluate the purity.

INTRODUCTION

The melting point of a substance is the temperature at which it changes from a solid to a liquid. It is an important physical property used to help identify a compound and assess its purity. A pure organic solid typically melts over a very narrow temperature range, often within 2–3 °C of its literature value.

The presence of impurities, even in very small quantities, affects how a sample melts. Impure solids usually:

- melt at a lower temperature than the pure compound, and

- show a broader melting range.

Melting begins when the first droplets of liquid appear among the crystals and ends when the sample becomes fully liquid.

Recrystallisation is a common purification method for solid organic compounds. The process involves:

- Dissolving the impure compound in a suitable hot solvent.

- Cooling the solution slowly so that only the pure compound crystallises.

- Separating the pure crystals from the solvent and any remaining impurities.

This technique relies on differences in solubility between the desired product and its impurities: the pure compound crystallises out, while most impurities either stay dissolved or are removed by filtration.

For this experiment, methyl-3-nitrobenzoate was purified by recrystallisation from hot ethanol. This removed impurities such as unreacted starting materials and minor side-products formed during the nitration reaction. As the solution cooled, clean crystals of the product formed and impurities remained in the solvent.

You will measure the melting points of 5 samples of methyl-3-nitrobenzoate synthesised at Cranfield. By comparing the melting behaviour of each sample, you will assess the purity of the final nitration product.

POTENTIAL HAZARD

|  | Hazards |
| --- | --- |
| ### Health Hazards | Methyl-3-nitrobenzoate - may be harmful if swallowed, inhaled, or absorbed through the skin. - can cause irritation to the skin, eyes, and respiratory tract. - prolonged exposure may lead to sensitisation or allergic reactions in susceptible individuals. |
| Flammability and Combustibility | While not highly flammable, methyl-3-nitrobenzoate is combustible and can release toxic fumes, such as carbon monoxide, carbon dioxide, and nitrogen oxides, when burned |
| Environmental Hazards | It may pose a risk to aquatic environments if released improperly, as nitroaromatic compounds can be toxic to aquatic life. |
| Reactivity | Under certain conditions, nitro compounds can act as oxidising agents, potentially posing a hazard when in contact with strong reducing agents or combustible materials. |

DATA SHEET to RECORD OBSERVATIONS, CALCULATIONS and RESULTS

STUDENT’S NAME: ____________________________________________________ PARTNERS’ NAMES: ____________________________________________________

REPORT IS DUE DECEMBER 5TH, 2025

submitted via MOODLE as ONE .doc file

PROCEDURE

Put on the lab coat, safety goggles, and gloves. Make sure that long hair is tied.

IMPORTANT: Record all your observations, calculations, and conclusions in the data sheets at the end of this script.

Lab tools and supplies:

Cole-Parmer M200 Melting Point Microscope (MPM)

Glass capillaries

Samples of methyl-3-nitrobenzoate synthesised and purified in Cranfield.

Operating procedure will be discussed in the lab however the main points are:

- Set up the MPM device by placing it on a stable surface, connecting to power, and switching it on. The display will show the block temperature, and the sample chamber light will activate.

- Prepare samples for the analysis by filling a capillary tube with each sample .

- Set Plateau Temperature:

    - Press and hold the 'Plateau Set' button; adjust the temperature with arrow keys to 10°C below the expected melting point.

    - Release the button to confirm the setting.

- Place the capillary tube into a sample hole in the block and make sure that it is visible through the lens.

- Press 'Start' to heat quickly to the plateau temperature. The plateau light will turn on once stabilized.

- To observe melting point, press 'Start' again to begin slow ramping (2°C/min). Observe through the lens and record the temperature at the first sign of melting.

- Press 'Stop' after observation is completed. Allow the device to cool before the next use.

Safety Notes:

- Handle with care; the block becomes very hot and the glass capillaries can break.

- Use correct PPE.

REPEAT THE MEASUREMENTS TWO TIMES FOR EACH SAMPLE.

Literature values of melting points:

|  | melting point, °C |
| --- | --- |
| methyl benzoate |  |
| methyl-3-nitrobenzoate |  |

Experimental values of melting points

| Sample | appearance | trial 1 | trial 2 |
| --- | --- | --- | --- |
|  |  | mp, °C | mp, °C |
| A |  |  |  |
| B |  |  |  |
| C |  |  |  |
| D |  |  |  |
| E |  |  |  |

OBSERVATIONS AND OTHER RELEVANT INFORMATION

Guidance for lab report:

- Title page, TOC, etc

- Purpose (Aim) of the experiment

- Hypothesis: based on the experience and knowledge, what do you expect to observe? Why?

- Variables: independent, dependent, controlled

- Theory (background information)

  - Basic concepts, principles and laws, definitions

  - Chemical equation and formulas for calculations  (if relevant)

  - Examples of applications/importance of the method in OME supported by literature

- Observations/Data (as of the Data Sheet provided)

- Calculations (where relevant)

- Results and Discussion: How do the observations compare to your hypothesis  — do they match? How do the observations compare to the literature values?  What are primary sources of experimental errors?

- Conclusion

- References (IEEE)

- Appendix

  - Chemicals and equipment

  - Procedure

END OF LAB REPORT