Safety Risk Assessment Form

This form must be distributed to all persons named in the Owner column, where possible both as a hard copy and electronically

|  | Three Sisters 2019 – Show running Risk Assessment |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date: | 02/12/2019 | Assessor(s): | Michael Ager / Richard Eustace |  |  |  | Department/Area: |  |  | Lyttelton Theatre |  |
| Activity Describe the activity and the ways in which it could pose a risk | Hazards e.g. personal injury, electric shock, people or objects falling from height, crushing | Persons at risk | Risk before control measures |  |  | Controls Describe controls used to reduce risk to a tolerable or preferably acceptable rating | Residual Risk after control measures |  |  | Additional controls? e.g. monitor and review, suggestions for future activities | Owner Person responsible for the controls |
|  |  |  | Severity | Likeli-hood | SxL |  | Severity | Likeli-hood | SxL |  |  |
| Moving furniture and props | Back + shoulder strains, dropping items on feet, catching fingers | Stage management | 4 | 3 | 12 | - Work in pairs when moving large items. - Manual handling practices observed | 3 | 3 | 9 | Set furniture as close as possible to acting area. | SM and Crew |
| Drinking and eating during the show. | Allergies and choking. Spillages. Cuts from breakages. Warm liquids | Full Company + staff | 3 | 3 | 9 | - All drinks and food to be passed by actors to check for food allergies and intolerances. - All SM aware of above. - Cloths in the room for mopping up. - Minimum liquid in receptacles. | 2 | 2 | 4 | Scenes rehearsed. Dustpan + brush standing by. Plasters available | SM |
| Walking up and down stairs | Trips + falls | Company + stage management + visitors to the room | 4 | 3 | 12 | - Stairs and access treads will be securely fixed. - Tape on off stage stairs to indicate each step. - Company will be given an induction to working on the set | 3 | 3 | 9 | Company in sensible footwear | SM |
| Cables for appliances | Trips. Electric shock | Company + stage management | 4 | 3 | 12 | - When not in use, cables will be removed where possible. - All equipment PAT tested. - Company will be reminded when appliances are plugged in | 3 | 2 | 6 | Scenes rehearsed | LX |
| Turning electrical appliances on | Shocks | company + stage management | 4 | 3 | 12 | - All equipment PAT tested - company familiarised with switches and mechanisms | 3 | 2 | 6 | Scenes rehearsed | LX |
| Dancing and choreography | Slips, trips and falls | Cast | 3 | 3 | 9 | - Floor surface to be slip tested and appropriate surface treatments applied if required. - All dancing and choreography to be rehearsed with professional instructor. - Appropriate footwear to be worn at all times | 3 | 2 | 6 |  | SM |
| Burning Nigerian flag | Fire Burns | All staff and audience Cast and crew | 5 | 4 | 20 | - Flag to be constructed from Fire retardant fabric with a small untreated section to burn. - The flag will be dropped into a large bucket with 30mm of water in the bottom to extinguish the flames - All surrounding scenic materials are flame retardant. - Crew and fire officer will be on standby while the flag is being burned. - The flag burning will be rehearsed with the armourer to ensure that the burning flag is held safely - The actors costume will be FR treated to prevent risk of ignition. | 5 | 2 | 10 | Fire officer on standby | SM |
| Wearing Stiletto Shoes | Slips, trips and falls | Cast | 3 | 4 | 14 | - Cast members will have the opportunity to rehearse wearing similar shoes. - Gaps and trip hazards will be minimised but cannot be eliminated. - Cast to be shown all potential hazards, these include: - Gap around perimeter of revolve. - Spade lines in stage floor. - Steps up to house truck levels. | 3 | 2 | 6 |  | SM |
| Use of blank firing gun | Shrapnel injuries Hearing Damage | Cast Cast | 5 4 | 3 3 | 15 12 | - “ABTT Code of Practice 06 Weapons in Stage Productions 2019” to be followed at all times. - Cast and stage management to be trained in safe use of the blank firing gun by the armourer. - All heat and shrapnel exits from the side of the gun, not the barrel. - Cast member who is firing the gun and others in close proximity will be issued with hearing protection which must be worn during the scene | 5 4 | 1 1 | 5 4 |  | SM / Armourer |
| Wrestling scene | Collisions | Cast | 4 | 3 | 12 | - The wrestling scene will be rehearsed and blocked with a professional fight director from RC Annie. - Fight rehearsal will take place in the rehearsal room and on stage in full show conditions with the fight director. - A fight call will be scheduled onstage prior to every performance. | 4 | 2 | 8 |  | SM |
| Use of revolve (show & non-show modes) | Trips, slips, falls, off balance | Cast & technical team | 3 | 3 | 9 | - Revolves are operated by Automation Department. - All revolve cues are pre-programmed. - Revolves have been in rehearsals so cast are familiar with them and show has been choreographed around the use of revolves. - Cues have ramp up/down speeds to ensure jolting is kept to a minimum. - E-stops are located around stage area and Automation desk. - Use of revolves outside of show conditions (eg. Tech or reset): moves to be called by SM or Automation Department. | 3 | 1 | 3 |  | Stage Manager/Automation operator |
| Forestage elevators dropped lower than stage level | Fall from height | Cast and crew | 5 | 4 | 20 | - The drop to the forestage elevators will be kept to the minimum possible - The front edge of the mainstage elevator will be clearly marked and a kick rail will be added if required. - Adequate lighting will be provided. - Scenes will be rehearsed in full show lighting - A strip of EL wire will be routed into the downstage edge of the show floor to ensure that the edge of the stage can be seen at all times - When the elevators are dropped down to substage level, barriers will be placed along the stage egde. | 5 | 2 | 10 |  | Stage Manager/Automation operator |
| Moving truck with friction drives | Crushing and collision injuries | Cast and Crew | 4 | 3 | 12 | - Friction drive moves are operated by the automation department and called by the DSM - Friction drive moves outside of show conditions (eg. Tech or reset): moves to be called by SM or Automation Department. - The friction drives will be controlled locally from a control pickle on the back of the house truck. - All friction drive moves will be rehearsed in good lighting and in full show conditions so cast are familiar with them and show has been choreographed around the moves. | 4 | 2 | 8 |  | Stage Manager/Automation operator |
| Splitting truck during interval change | Crushing and collision injuries | Crew | 4 | 3 | 12 | - One member of stage crew will lead on the splitting of the truck. - Any items attached to the walls or ceiling of the house truck will be securely attached to ensure that they do not fall when the truck is split. - The split in the roof beam will be unbolted first before the catches in the truck base are released. - Any moves will be called - The section of truck containing friction drive units will be driven upstage using the drives. - The remaining section of house truck will be towed upstage into the rearstage using the electric tug unit (Crazy Horse), whilst two members of crew guide and spot it. | 4 | 2 | 8 |  | Stage Supervisor |
| Interval change | Crushing and collision injuries | Crew | 4 | 3 | 12 | - All staff will wear steelcap boots and hard hats during the interval change - An appropriate number of staff will move any Items that need to be moved manually - Crew members who are moving the trucks of dried grass will be required to wear safety goggles to prevent eye injuries | 4 | 2 | 8 |  | Stage Supervisor |
| Natural dried grasses on stage | Fire | All staff and audience | 5 | 3 | 15 | - All grasses will be FR treated to BS476 Class 0 by Gorts Services | 5 | 1 | 5 |  | PM |
| Flown curtains constructed from 4mm polyester cord | Fire | All staff and audience | 5 | 3 | 15 | - The Curtains will be constructed using inherently flame retardant cord. | 5 | 1 | 5 |  | PM |
| Strobe Lighting Effect | Potential for audience members to fit | All staff and audience | 4 | 2 | 8 | - Warning signs posted in the foyer - Short effect - Flash rate per ABTT guidelines | 4 | 1 | 4 |  | LX team |

| Risk Scoring Key | Severity |  | Likelihood |  |
| --- | --- | --- | --- | --- |
|  | Fatality | 5 | Certain or imminent | 5 |
|  | Major injury, disabling illness, major damage | 4 | Very likely | 4 |
|  | Lost time injury, illness, damage | 3 | May happen | 3 |
|  | Minor injury, minor damage | 2 | Unlikely | 2 |
|  | Delay only | 1 | Very unlikely | 1 |

| Risk Rating Categories | 10 to 25 | Unacceptable | Do not proceed; seek immediate guidance from the safety team |
| --- | --- | --- | --- |
|  | 6 to 9 | Tolerable | Proceed with caution but seek to reduce risk further if possible |
|  | 1 to 5 | Acceptable | Proceed |

| Order of Control Measures | Most effective | Eliminate | Ask yourself if the activity needs to be carried out |
| --- | --- | --- | --- |
|  |  | Substitute | Ask yourself if the same effect can be achieved with something less risky |
|  |  | Reduce | Ask yourself if you can use less of something, or limit the time etc |
|  |  | Isolate | Make sure that the risk is contained to the smallest possible area |
|  |  | Enclose | Make sure that no-one can get to the hazard |
|  |  | Other Engineering Controls | Emergency stop buttons, automated controls etc |
|  |  | Safe System of Work | Carry out the work according to a specific step by step programme with training |
|  |  | Training/Communication | Safety team can advise |
|  |  | PPE | Use of ear defenders, hard hat, toetectors etc |
|  | Least effective | Discipline and Enforcement | Telling people to be careful |