PRACTICAL 8:  Assessment of Body Composition

By the end of this practical session and continued practice you should be able to:

- Conduct a basic body composition assessment using Tanita bio-electrical impedance scales

- conduct simple field based test of body composition using skin fold calipers

  - Data Collection Procedures

- Bioelectrical Impedence (Tanita)

Follow the instructions on the monitor of the Tanita scale to conduct an assessment of body composition. From the print out, record the value for body fat %

- Skinfolds

- Obtain a pair of skinfold calipers

- Use the images and instructions below to take skinfold measurements at four sites: abdomen, suprailium, tricep and thigh.

- To measure the skin fold, follow the following instructions:

  - Lift skinfolds two or three times before placing the skinfold calliper and taking the measurement.

  - Place the calliper below the thumb and fingers and perpendicular to the fold so that the dial can be easily read. Release the caliper grip completely and read the dial 1 to 2 seconds later.

  - Repeat the process at least three times: the measure should not vary by more than 1mm. The median value should be used as the measure.

  - Use the equations below to convert the sum of skinfolds into body fat %

- Data Management

Input your raw data into the data collection proforma below:

Practical 8 Data Collection Proforma

| Tanita Body Fat % | Skinfold Body Fat % |
| --- | --- |