| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 7 | LEFEL 7 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| JANUARY 2017 | IONAWR 2017 |

MODULE CODE: M7X01384

Structural Integrity and Materials Evaluation

TIME:  2 HOURS

answer all questions

Equipment or materials required – Calculator and Ruler

Formulae and Material Properties on last 3 pages

ANSWER ALL QUESTIONS

- Give a brief discussion regarding the Risk Based Inspection approach for the management of equipment and plant integrity. 10 Marks

- The table below provides hit-miss data for complimentary NDT methods used to inspect an aircraft component.  The technicians were required to report defects detected or not on a location by location basis.  Carry out a full PoD analysis using the ROC method. 10 Marks

- With reference to the eddy current method:

- With reference to induced eddy currents in a conductor, explain the skin effect and effective depth of penetration. 4 Marks

- A test coil is used to inspect a copper component 1mm in thickness.  Assuming 0.5mm as the standard depth of penetration required and that the conductivity of copper is 98.6 %IACS:

- Calculate the required test frequency.

2 Marks

- What would be the maximum phase difference between the induced near and far surface eddy currents?

2 Marks

- If we were to increase the test frequency by an order of 2, what would the effects be regarding the depth of penetration and near and far surface phase differences?

2 Marks

- With reference to the ultrasonic method, where a 2.5MHz conventional ultrasonic transducer is to be used to gauge thickness in mild steel

- Determine the thickness required of a PZT crystal to produce a resonant frequency of 2.5MHz, given that the compression wave velocity for PZT is 3000 m/s.

2 Marks

- Assuming for the aforementioned crystal that it has a diameter of 10mm, determine the crystals near field distance when transmitting into Perspex.

2 Marks

- Assuming the crystal is in contact with a mild steel surface.  Calculate the 6dB beam width for the crystal at a thickness in mild steel of 25mm.

2 Marks

- If a Perspex delay line is used to compensate for the near field zone, what would the resulting effect be in terms of the beam width?

2 Marks

- What are the implications with regard defect sensitivity in this application?

2 Marks

- With reference to radiographic methods

- A micro-focus 2D x-ray inspection is to be undertaken on a square (8mm x 8mm) mild steel plate of thickness 20mm.  The source to detector distance is fixed at 0.5m.  The focal spot size of the x-ray tube is 30µm.  A geometric unsharpness value (Ug) of better than 0.01mm is required.  Calculate the required source to object distance and resulting magnification factor.

5 Marks

- A technician is in an area for 10 minutes and the reading on an exposure meter is 5mR/hr.  Determine the actual radiation dose received by the technician.

1 Mark

- A technician should not receive a radiation dose greater than 1mR.  Based upon Q5 b) conditions determine the maximum time the technician can stay in the area.

1 Mark

- The intensity of an unshielded I-131 source is 32mR/hr. If the source is put into a lead shield 30mm thick what would be the intensity on the outside of the shield? Assume your distance from the source has not changed. The HVL of lead for I-131 is 0.178cm.

3 Marks

| Snell’s Law: sin iV1=sin rV1=sin RV2 | Reflected Energy Equation: RE=Z1-Z2Z1+Z22×100% |
| --- | --- |
| Ultrasonic Beam Formulae: NF=d24λ Where: NF = Near Field Distance d = Crystal diameter or aperture width λ = wavelength | Far Field Approximation: Aθ=sin(α)α Where α=kd2sin⁡(θ) K is the wave number (2π/λ) and d is the aperture width. |
| Q Factor Q=f0f2-f1 Where: f0 = the centre frequency f2 = the upper -6dB frequency f1 = the lower -6dB frequency |  |
| Frequency thickness crystal constant: f x t = V/2 Where, - f = desired frequency - t = crystal thickness, and - V = compression wave velocity in the crystal material |  |

General Formula and Constants

δmm=661fµrσ

θ=xδ×57.3

B=µH

µr=µµo

µo=4π x 10-7

Current Loop Centre Flux Density

B=µrµoI2R

Solenoid Centre Flux Density

B=μnI

n=Nh

Magnetic Circuit Equations

μoJ.∂s=B.∂l

VL=-L∂I∂t                 VL=-N∂Φ∂t

mmf=H.l       Rm=lμA        Fm=N.I          Rm=mmfΦ    Ug=Uff2-f1f1

I=Ioe-μx

Material Properties.

Diwedd/End