//------------------------------------------------------------------------------------------------------

#include <main.h>                                                        // Include the main header file

#define LCD_ENABLE_PIN PIN_D2                                            // Define the pin for LCD enable

#define LCD_RS_PIN PIN_D0                                                // Define the pin for LCD Register Select

#define LCD_RW_PIN PIN_D1                                                // Define the pin for LCD Read/Write

#define LCD_DATA4 PIN_D4                                                 // Define the pin for LCD data line 4

#define LCD_DATA5 PIN_D5                                                 // Define the pin for LCD data line 5

#define LCD_DATA6 PIN_D6                                                 // Define the pin for LCD data line 6

#define LCD_DATA7 PIN_D7                                                 // Define the pin for LCD data line 7

#include <lcd.c>                                                         // Include the LCD driver code

//------------------------------------------------------------------------------------------------------

// Function declarations for tasks

//------------------------------------------------------------------------------------------------------

unsigned int lights[16]={128,64,32,16,8,4,2,1,1,2,4,8,16,32,64,128};     // Array to control the light pattern for "Knight Rider" effect

void task1(void);                                                        // variable for task 1

void task2(void);				                         // variable for task 2

void task3(void);				                         // variable for task 3

void task4(void);				                         // variable for task 4

int RV1;                                                                 // Variable to store ADC result

int but;                                                                 // Variable to store button state

unsigned int switches;                                                   // Variable to store the state of switches

int disp = 0;                                                            // initial variable state for the LCD display

int last_button_state;                                                   // variable for the state of the button

//------------------------------------------------------------------------------------------------------

// Function to read the ADC value from a specified port

//------------------------------------------------------------------------------------------------------

unsigned adc_read(unsigned port){

set_adc_channel(port);                              // Select the ADC channel

delay_us(10);                                       // Small delay to stabilize ADC reading

return read_adc();                                  // Return the ADC result

}

//------------------------------------------------------------------------------------------------------

// Main function - execution starts here

//------------------------------------------------------------------------------------------------------

void main(){

setup_adc_ports(sAN0);                             // Initialize the ADC channel

setup_adc(ADC_CLOCK_INTERNAL|ADC_TAD_MUL_0);       // Set up the ADC operation

lcd_init();                                        // Initialize the LCD display

while(1){                                          // Infinite loop to continuously check for switch input

switches = input_a();                           // Read the input from switches

switches = switches & 0x3C;                    // Mask the relevant bits

switches /= 2;                                 // Divide by 2 to shift to correct position

RV1 = adc_read(0);                             // Read ADC value from channel 0

but = pin_a1;                                  // Store the state of pin A1 (button)

switch(switches){                              // Switch case based on the switch position

case 2:                                    // Switch in the first position

task1();  			       // Call task1

task2(); 			       // Call task2

disp = 0;                              // Reset display variable

break;                                 // adds a break

case 4:                                    // Switch in the second position

task2();                               // Call task2

disp = 0;                              // Reset display variable

break;                                 // adds a break

case 8:                                    // Switch in the third position

task3();                               // Call task3

disp = 0;                              // Reset display variable

break;                                 // adds a break

case 16:                                   // Switch in the fourth position

disp = disp + 1;                       // Increment display variable

task4();                               // Call task4

break;				       // adds a break

default:break;                             // adds a break

}

}

}

//------------------------------------------------------------------------------------------------------

// Task 1 - Controls C0 and C1 based on ADC value and displays on LCD

//------------------------------------------------------------------------------------------------------

void task1(){

printf(lcd_putc, "\f");                            // Clear the LCD screen

if (RV1 > 175){                                    // If ADC value is greater than 175

output_high(PIN_C0);                           // Set PIN C0 high

output_low(PIN_C1);                            // Set PIN C1 low

lcd_gotoxy(9, 1);                              // Set cursor position on LCD

printf(lcd_putc, "%3u", RV1);                  // Display ADC value on LCD

printf( "%3u\r\n", RV1);                       // Display ADC value on LCD

delay_ms(5);

lcd_gotoxy(8,2);                               // Set cursor to second line of LCD

printf(lcd_putc, "right");                     // Display "right" on LCD

printf("right \r \n");                         // Display "right" on LCD

delay_ms(300);

}

else if (RV1 < 75){                                // If ADC value is less than 75

output_low(PIN_C0);                            // Set PIN C0 low

output_high(PIN_C1);                           // Set PIN C1 high

lcd_gotoxy(9, 1);                              // Set cursor position on LCD

printf(lcd_putc, "%3u", RV1);                  // Display ADC value on LCD

printf( "%3u\r\n", RV1);                       // Display ADC value on LCD

delay_ms(5);

lcd_gotoxy(8,2);                               // Set cursor to second line of LCD

printf(lcd_putc, "left");                      // Display "right" on LCD

printf("left \r \n");                          // Display "right" on LCD

delay_ms(300);

}

else{                                              // If ADC value is between 75 and 175

output_low(PIN_C0);                            // Set PIN C0 low

output_low(PIN_C1);                            // Set PIN C1 low

lcd_gotoxy(9, 1);                              // Set cursor position on LCD

printf(lcd_putc, "%3u", RV1);                  // Display ADC value on LCD

printf( "%3u\r\n", RV1);                       // Display ADC value on LCD

delay_ms(5);

lcd_gotoxy(8,2);                               // Set cursor to second line of LCD

printf(lcd_putc, "stop");                      // Display "right" on LCD

printf("stop \r \n");                          // Display "right" on LCD

delay_ms(300);

}

}

//------------------------------------------------------------------------------------------------------

// Task 2 - Display ADC value on LCD and output the value to LEDs

//------------------------------------------------------------------------------------------------------

void task2(){

printf(lcd_putc, "\f");                            // Clear the LCD screen

lcd_gotoxy(9, 1);                                  // Set cursor to display the ADC value

printf(lcd_putc, "%3u", RV1);                      // Display ADC value

printf( "%3u\r\n", RV1);                           // Display ADC value on LCD

output_b(RV1);                                     // Output the ADC value to PORT B

}

//------------------------------------------------------------------------------------------------------

// Task 3 - Check button press and display "right", 1 and turn motor or 0 on LCD

//------------------------------------------------------------------------------------------------------

void task3(void){

int current_button_state = input(pin_a1);                //read the curret button state

if(current_button_state != last_button_state){	         //cheks if the current button state is not the same as its previous state

if (current_button_state){

printf("Button pressed \r\n");	         //print to the virutal terminal

output_low(PIN_C1);		         //set pin C1 low for clockwise motor movement

output_high(PIN_C0);		         //set pin C0 high for clockwise motor movement

lcd_gotoxy(24,1);			         //go to center of line 3 on the LCD display

printf(lcd_putc,"Button pressed ");         //print to the LCD display

}

else{

printf("Button released \r\n");	         //print to the virtual terminal

output_low(PIN_C1);	                 //set pin C1 low to turn the motor off

output_low(PIN_C0);			 //set pin C1 low to turn the motor off

lcd_gotoxy(24,1);				 //go to center of line 3 on the LCD display

printf(lcd_putc,"Button released ");	 //print to the LCD display

}

last_button_state = current_button_state;	 //update last button state

}

}

//------------------------------------------------------------------------------------------------------

// Task 4 - Implement the "Knight Rider" light pattern and print KITT Mode

//------------------------------------------------------------------------------------------------------

void task4(void){

printf(lcd_putc, "\f");                           // Clear the LCD screen

lcd_gotoxy(27, 2);                                // Set position for the "KITT Mode" label

printf(lcd_putc, "Kitt Mode");                    // Display "Kitt Mode"

if (disp == 1)                                    // If display variable is 1

printf("Kitt Mode \r\n");                     // Display "Kitt Mode" on the console

unsigned int q;                                   // Local variable for loop

for (q = 0; q < 16; q++){                         // Loop from 0 to 16

output_B(lights[q]);                          // Output the current light pattern to PORT B

delay_ms(200);                                // Wait for 200ms

}

output_b(0x00);                                   // Turn off all lights

printf(lcd_putc, "\f");                           // Clear the LCD screen

}

//------------------------------------------------------------------------------------------------------

//              super cool and epic art. A++

//	                 -----------!-----------

//	 -----------!---------   /=====\

//	          |===\_________/_  o  |

//	         /_]    o o  o o____  /

//	        <_]___[]_______<____>/

//	            o              o

//          ---------------------------------------

Comments

2.. 20   3.. 30    4.. 40   5.. 10

**************************************

2.. Your code has a number of structures and all work except you don't test the switch position properly meaning you don't call the right function for each switch position.  Your statements indicate what each line does. 65%

3.. Your program flows nicely 75%

4.. Your program mostly achieves the desired results. 68%

5.. Your program is nicely laid out and well indented. 75%