Data Entry Derived from  Example 2

| Data |  |  |
| --- | --- | --- |
| MR fl | Front Left Motion Ratio | 0.546 |
| MR fr | Front Right Motion Ratio | 0.546 |
| MR rl | Rear Left Motion Ratio | 0.625 |
| MR rr | Rear Right Motion | 0.625 |
| M fl | Front Left total measured static corner mass [kg] | 194 |
| M fr | Front Right measured static corner mass [kg] | 194 |
| M rl | Rear Left total measured static corner mass [kg] | 208 |
| M rr | Rear Right total measured static corner mass [kg] | 208 |
| Mus fl | Front Left unsprung mass [kg] | 38 |
| Mus fr | Front Right total unsprung mass [kg] | 38 |
| Mus rl | Rear Left unsprung mass [kg] | 44 |
| Mus rr | Rear Right unsprung mass [kg] | 44 |
| Ks fl | Front Left Spring rate [lb/in] | 314.06 |
| Ks fr | Front Right Spring rate [lb/in] | 314.06 |
| Ks rl | Rear Left Spring rate [lb/in] | 342.61 |
| Ks rr | Rear Right Spring rate [lb/in] | 342.61 |
| Tf | Front Track [m] | 1.27 |
| Tr | Rear track [m] | 1.335 |
| L | Wheel base [m] | 2.351 |
| Rrf | Rolling Radius front [m] | 0.275 |
| Rrr | Rolling Radius rear [m] | 0.275 |
| Htot | Height of CG of total mass above ground [m] | 0.354 |
| Hs | Height of CG of sprung mass above ground [m] | 0.374 |
| hf | Height of Front roll centre above ground | 0.03 |
| hr | Height of Rear roll centre above ground | 0.06 |