Extreme Physiology Environmental Labs Protocols

Equipment Required:

- Seca Scales

- Servomex gas analyser

- Harvard Dry Gas Meter

- Calibration gases

- Polar H7 Bluetooth HR strap

- ParQ

- Douglas bags x 3

- Hans-Rudolph mouthpiece, hose and noseclip.

- Timer

- Wattbike Atom

- Infra-Red ear thermometer and probe covers

- Borg RPE scale

- Indoor Weather station

Procedure

- Ensure all equipment is in clean working order and has been calibrated.

- Check for any changes to ParQ and ensure subject is safe to participate.

- Weigh subject in minimal clothing prior to commencing.

- Inform subject that they are not to consume any food or fluid during the test, and should aim to be in a similar state of hydration prior to each test.

- Ask subject to put on Polar H7 Bluetooth chest strap.

- Set up the Wattbike for the subject as outlined in previous lab sessions.

- Determine power @ 60% VO2max.

- Record core body temperature prior to commencing.

- Ensure HR chest strap is paired to the wattbike app.

- Select ‘Just Ride’ on the wattbike app, and set target power on ‘ERGO’ mode.

- Ask the subject to commence cycling at the pre-determined effort level and start the timer.

- At 5:00 min, and every 5 min after, press ‘lap’ on the wattbike app.

- At 8:30 min, ask the subject to insert the Hans-Rudolph mouthpiece and wear the nose clip.

- At 9:00 min commence the collection of a 60s sample of air in the douglas bag.

- At 9:50 min ask the subject for an RPE.

- After collection of the air sample, record core body temperature.

- Record all of these values on the data sheet alongside the 10 min time.

- Repeat steps 13-16 at 18:30 min and 28:30 min, for the 20 min and 30 min times.

- After completing the test, record weight once again, wearing the same clothing as in the previous measurement.

- Evacuate bags recording all relevant data for calculating VO2, VCO2 and RER.

- Access the wattbike hub to determine the mean HR at 10 min, 20 min and 30 min (take the mean for the 5 min section leading up to that time e.g. 5-10 min mean for 10min)

- Record data onto master spreadsheet.