A Strontium- and Chlorine-Free Pyrotechnic Illuminant of High Colour Purity

CONTEXT

This paper was written by authors Johann Gluck, Thomas M. Kaplotke, Magdalena Rusan, Jesse J. Sabatini and Jorg Stierstorfer. These authors are all well established researchers in energetics and pyrotechnics who discovered a need to research Strontium and Chlorine free compositions because of a recent report from the US Environmental Protection Agency stating that Strontium may be hazardous to health. Chlorine containing compositions can produce highly carcinogenic polychlorinated biphenyl, dibenzofuran, and dibenzo-p-dioxin derivatives. The quantities are unknown but due to the hazard, chlorine free compositions are desired. The article was published in Weinheim, Germany in 2017 in the journal Angewandte Chemie (International ed.) which focusses on Environmental Law and Energetic Materials, particularly Strontium and Chlorine compositions. In the article is a chromaticity diagram comparing the colour of these compositions compared to the Strontium and Chlorine compositions along with synthetic routes to obtain these compounds. Keywords are well defined and the layout and flow are very easy to follow and logical.

ANALYSIS

The article states that lithium-based compositions have been considered as replacements but have had problems of high hygroscopicity and low colour purity giving pink or orange flames. The red-emitter in lithium based compositions is atomic lithium which needs to be kept in strict concentrations and burnt in a reductive flame environment to reduce the amount of oxidation on the outside edges of the flame. This reduces the atomic lithium and lowers colour purity. One suitable compound established was the dilithium salt of 3,3’-diamino-4,4’-dinitramino-5,5’-bi-1,2,4-triazolate, Li2ANAT. It has favourable hygroscopic properties and high decomposition temperatures. Two compositions containing this salt and varying quantities of binder, Magnesium, hexamine and nitro-cellulose as were created and the burn time, dominant wavelength, luminous intensity, spectral purity, friction, impact, and electrostatic discharge sensitivity were determined with the decomposition temperature. From the chromaticity diagram composition B was closer to the colour of the Strontium control than A so a suitable halogen free composition can be developed.

SIGNIFICANCE

This article provides a significant advancement in halogen and Strontium free pyrotechnics and shows that sufficient hygroscopic properties can occur in Lithium compositions. It proves that Lithium compositions can provide greater colour purity than strontium compounds and greater stability due to lower sensitivity to friction, impact and ESD. Therefore, Strontium and halogen compositions can be removed from pyrotechnics with further research to improve the luminosity of lithium compounds.