Surveying Problems

Module code

ACBA4002

Prepared for

A.Abraham and H. Williams

Submission

November 2023

Created by

Lloyd Wilson

## Question 1.

- 1.465m

- 2.179m

- 3.975m

- 0.820m

- 1.502m

## Question 2.

| Back Sight | Intermediate | Fore Sight | Height of Collimation | Reduced Level | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1.850 |  |  | 51.850 | 50.000 | BM |
| 1.720 |  | 1.500 | 52.070 | 50.350 | A |
|  | 1.640 |  |  | 50.430 | B |
|  | 1.590 |  |  | 50.480 | C |
| 2.100 |  | 1.550 | 52.620 | 50.520 | D |
|  |  | 1.240 |  | 51.380 | E |

- 50 + 1.85 = 51.850

- 50.350 + 1.720 = 52.070

- 51.850 – 1.500 = 50.350

- 52.070 – 1.640 = 50.430

- 52.070 – 50.480 = 1.590

- 52.620 – 50.520 = 2.100

- 52.070 – 1.550 = 50.520

- 52.620 – 1.240 = 51.380

## Question 3.

### 3.1 Area calculation process

Point A 	100E 200N

Point B	200E 300N

Point C 	350E 225N

Point D 	325E 125N

Point E 	175E 50N

Area A = 150 x 75 /2 = 5625

Area B = 100 x 100 /2 = 5000

Area C = 150 x 75 /2 = 5625

Area D = 100 x 25 /2 = 1250

Area E = 150 x 75 /2 = 5625

Small box = 25 x 75 = 1875

Add all the triangle and small square areas together.

5625 + 5000 + 5625 + 1250 + 5625 + 1875 = 25000

Large box (overall area) = 250 x 250 = 62500

Take away the triangle and small square from the large square box to find the area.

62500 – 25000

Area = 37500

Source: Lloyd Wilson 19/11/2023

3.2 Perimeter

| Points | Easting (X) | Northing (Y) |
| --- | --- | --- |
| A | 100 | 200 |
| B | 200 | 300 |
| = | 100 | 100 |
| B | 200 | 300 |
| C | 350 | 225 |
| = | 150 | 75 |
| C | 350 | 225 |
| D | 325 | 125 |
| = | 25 | 100 |
| D | 325 | 125 |
| E | 175 | 50 |
| = | 150 | 75 |
| E | 175 | 50 |
| A | 100 | 200 |
| = | 75 | 150 |

A-B:

Pol(100,100)

r = 141.4213562

B-C:

Pol(75,150)

r = 167.7050983

C-D:

Pol(25,100)

r = 103.0776406

D-E:

Pol(150,75)

r = 167.7050983

E-A:

Pol(75,150)

r = 167.7050983

141.4213562 + 167.7050983 + 103.0776406 + 167.7050983 + 167.7050983

Perimeter = 747.6142917

Perimeter =747.614

## Question 4

Source: Lloyd wilson 20/11/2023

### Question 5

An Auto Level is a type of surveying implement that is used in surveying raise and falls in the landscape or on building sites. The equipment is used by being placed on top of a tripod and looking down the lens to a chain person using a staff. The Auto level must be levelled on the tripod by using the adjustment dials by visually centring the bubble to precisely set it level. The lens can also be adjusted using the focus dial and the chain person can rock the staff back and forth to identify the exact height to be recorded manually into a handbook or tablet device. The auto level is securely placed into the carrier box that is cushioned with foam to prevent any damage in storage or transport to locations of use.