Williams, Matthew

BEng, Yr. 1 - 2023

electronic systems design

BEng Electrical & Electronic Engineering

14 Day Weather and Air Quality Measurement Logging System

Table of Contents

Abstract	3

Design Specification	3

Output Requirements	3

Existing Systems	4

The Design	5

Design Parameters	6

Equipment Requirements	6

Sensors	7

Temperature	7

Humidity	8

Wind Speed	9

Wind Direction	10

Airborne Particle Count	11

Sunlight Brightness	12

UV Level	13

GPS Position	14

VOC Levels	15

Microcontroller Selection	17

Storage	21

USB & SD Card Modules	28

Batteries	29

Enclosure	31

The Hardware Layout	32

Connectors (Panel Mounted to Casing)	36

Connectors (PCB Mounted)	37

ASSUMPTION	37

Circuit and PCB Design	38

Power Supply Schematic	40

12v Supply Schematic	41

5v Supply Schematic	41

3.3v Supply Schematic	42

LED Flashing Indicator Schematic	43

Temperature & Humidity Sensor Schematic	45

Wind Speed & Wind Direction Sensor Schematic	46

GPS Sensor Schematic	47

Air Quality & VOC Sensor Schematic	48

UV Sensor Schematic	49

Sunlight Sensor Schematic	50

SD Card Circuit Schematic	51

USB Circuit Schematic	52

PIC Microcontroller Schematic	53

PIC Decoupler & PIC MCLR Schematic	53

PCB Schematic	56

PCB Schematic (with Copper Pour)	57

PCB Schematic 3D View (Top)	58

PCB Schematic 3D View (Bottom)	59

The Software	62

Costs and Figures	66

Summary	69

Conclusions	69

Recommendations for Improvements	69

References	70

Appendix	77

Appendix 1 – yEd Software File	77

Appendix 2 – Links to all Components	77

# Abstract

The requirement of this assignment is to design a microcontroller based Weather and Air Quality Logger. The Data logger is designed to be fully portable and adaptable for any site, and is intended for the amateur weather forecasters market.

# Design Specification

Our Weather and Air Quality Logger needs to be powered by an independent power supply, and designed to withstand the varied environmental forces that this type of system needs to endure, e.g. Heat & Cold, Water & Dust Ingress etc. As well as this, it needs to be capable of recording data for a 14 day period.

Our Weather and Air Quality Logger needs to be capable of obtaining the following data:

- Temperature (°C)

- Humidity (%)

- Wind Speed (mph)

- Wind Direction (°)

- Airborne Particle Count (f3/min)

- Sunlight Brightness (W/m2)

- UV Levels (0 to 11)

- GPS Position (NMEA) - National Marine Electronics Association

- Volatile Organic Compound (VOC) Level (μg/m3)

# Output Requirements

To achieve the design specification, the Weather and Air Quality Logger must be powered by an independent power supply, be portable and adaptable. This is to ensure it will continue monitoring even when the site is remote and unpowered.

The microcontroller will need to be capable of not only connecting to a variety of sensors and measuring tools, but will need the capacity to store the data during the 14 day cycle on non-volatile memory and also have the capability of connecting to an external computer via USB to process the information gathered. An LED will need to be programmed to Blink every 10 seconds for one second to indicate operation.

How the microcontroller and sensors are housed and the connectors used will be a major consideration. IP Rating (also known as Ingress Protection Rating) will need to be adequate.

# Existing Systems

Before we start designing our Weather and Air Quality Logger, we need to look at what is already on the market, and how we can improve on the existing systems available. After extensive research, the following systems have been identified:

- Davies Wireless VP2 Station 6152 from https://shop.weatherstations.co.uk/

- SEN-15901 from https://cpc.farnell.com/

- Sencorp Weather Station from https://www.marketplace.farm/

- BRESSER MeteoChamp 7-in-1 HD Wi-Fi Weather Station from https://www.bresseruk.com/en/home/

- Davies Vantage Vue 6250 from https://shop.weatherstations.co.uk/

Below is a detailed tabulated comparison of three of the existing systems that are on the market:

| Existing Data Acquisition Systems |  |  |  |
| --- | --- | --- | --- |
| System: | SEN-15901 | Davies Vantage Vue 6250 | Sencorp Weather Station |
| Company: | Spark Fun | Davies Instruments | Sencorp |
| Photo: |  |  |  |
| Input Voltage Range: | 5v | 3v | 3.6 v |
| Power: | Battery | Battery & Solar | 19Ah Battery |
| Data Logging: | N/A | 1 Minute | Every 15 minutes |
| Memory: | N/A | Real Time | Real Time to Application |
| Measures: | Wind Speed, Wind Direction, Rainfall, | Wind Speed, Wind Direction, Temperature, Humidity, Pressure, Rainfall, Dew Point | Rainfall, Temperature, Humidity, Dew Point, Wind Speed, Wind Direction, Gusts, Solar Irradiance |
| GPS: | No | No | Yes |
| Communication: | Wired | Wi-Fi | Sigfox LPWAN |
| Other Connections: | RJ11 | N/A | N/A |
| USB: | No | No | No |
| Rainfall |  |  |  |
| Measurement Range: | - | - | 0 to 1039 mm/h |
| Resolution: | 0.2794 mm | 0.2 mm | 0.254 mm/h |
| Wind Speed |  |  |  |
| Measurement Range: | - | 0 to 320 km/h (0 to 200 mph) | 0 to 160 km/h |
| Resolution: | 2.4 km/h | - | 0.5 km/h |
| Wind Direction |  |  |  |
| Measurement Range: | Compass Baring | Compass Baring | 0° to 359° |
| Resolution: | 16 Point | 8 Point | 1° |
| Temperature |  |  |  |
| Temperature Range: | N/A | -40 to 65 °C | -40 to 125 °C |
| Temperature Accuracy: | N/A | - | ± 0.2°C |
| Humidity |  |  |  |
| Humidity Range: | N/A | 1% to 100% | 0% to 100% |
| Humidity Accuracy: | N/A | - | ± 2% |
| Differences |  |  |  |
| Advantages: | Sensors can be used by other logging systems Much cheaper than the competition | Solar panel included in the design which extends battery life Logging rate of every 1 second gives accurate data | The Sencorp Weather Station measures many more parameters compared to its competitors on the list GPS |
| Disadvantages: | Limited amount of parameters measured | Needs to be within Wi-Fi range | It’s one the most expensive on the market |
| Price: | £69.77 | £515.00 | £1,042.80 |
| Referencing: | [1] | [2] | [3] |

Table 1: Existing Systems Comparison

The SEN-15901 is an example of a very basic weather station. It measures Wind Speed, Direction and Rainfall only. This would require a microprocessor or logging device in order to record the readings. Compared to the SEN-15901, the Davies Vantage Vue 6250 is a big upgrade, as it measures Wind Speed, Wind Direction and Rainfall like the above, but also measures Temperature, Humidity, Pressure and Dew Point onto a logging device. For someone who is looking for instant access to sensor readings, the Sencorp Weather Station is the ideal solution. It measures Wind Speed, Wind Direction, Rainfall, Temperature, Humidity, Dew Point like the Davies Vantage, and also logs Gusts & Solar Irradiance. All this comes at double the cost however. This can be justified due to the facility to read real time results on a mobile phone app.

# The Design

Below is the information required to build our Weather and Air Quality Logger. The starting point is to work out the parameters we are working with. From the information we already know, i.e. what parameters we need to be able to measure, we can work through in a methodical way to build the required information required to choose our Microcontroller. Starting with the number of Analogue/Digital Inputs required to connect our sensors and working through to the memory required to store our data.

## Design Parameters

Parameters

Temperature

Humidity

Wind Speed

Wind Direction

Air Borne Particle Count

Sunlight Brightness

UV Levels

GPS Position

VOC Level

Table 2: Parameters to Measure

Table 2 shows the parameters to be measured by our Weather Station as set out in the brief.

## Equipment Requirements

The equipment that will be required to create our Weather Station will be:

- 8bit PIC Microcontroller

- Sensors:

  - Temperature Sensor

  - Humidity Sensor

  - Wind Speed Sensor

  - Wind Direction Sensor

  - Airborne Particle Count Sensor

  - Sunlight Brightness Sensor

  - UV Levels Sensor

  - GPS Sensor

  - VOC Sensor

- Enclosure

- PCB

- USB Module

- SD Card Module

- LED Indicator Light

- Battery

- Panel Connectors

- PCB Connectors

- SD Card

## Sensors

### Temperature

| Temperature Sensors |  |  |  |
| --- | --- | --- | --- |
| Model: | BME280 Breakout Board | HPP809A031 Temperature & Humidity Sensor | HX93BC |
| Company: | SB Components | TE Connectivity | OMEGA |
| Photo: |  |  |  |
| Interface: | I2C / SPI | Analogue | Analogue |
| Temperature Measurement: | -40 to 85 °C | -40 to 85 °C | -30 to 75°C |
| Operating Voltage: | 3.3 v | 5 v | 9 v to 30 v |
| Current: | 1.8 μA | 1.2 mA | 4 - 20 mA |
| Dimensions: | 22 mm x 23 mm | 11.6 mm x 76.3 mm | 3.15 inch x 3.24 inch x 2.19 inch |
| Weight: | 2g | - | 352 g |
| Price: | £7.99 | £99.23 | £491.52 |
| Pros: | Cheap | Low Current Draw, Can record Humidity | Highly Accurate |
| Cons: | Difficult to adapt to a “Mobile” Unit. Needs separate enclosure to rec. air temperature and keep dry etc. and difficult to integrate. | Will require bracket to mount. | Expensive |
| Chosen Sensor: |  |  |  |
| Website: | https://shop.sb-components.co.uk/ | https://uk.rs-online.com/web/p/temperature-humidity-sensor-ics/8937373?gb=s | https://uk.farnell.com/ |
| Referencing: | [4] | [5] | [6] |

Table 3: Temperature Sensors Comparison

Figure 1: Chosen Temperature Sensor

Datasheet - https://docs.rs-online.com/fcd5/0900766b8142cdce.pdf

### Humidity

| Humidity Sensors |  |  |  |
| --- | --- | --- | --- |
| Model: | HPP809A031 Temperature & Humidity Sensor | TM1SHC4 Sensor | DHT20 Sensor |
| Company: | TE Connectivity | Schneider | Grove |
| Photo: |  |  |  |
| Interface: | Analogue | Analogue | I2C |
| Humidity Measurement: | -40 to 85 °C | 0% to 100% | 0% to 100% |
| Operating Voltage: | 5 v | 28 v | 2.3 v to 5.5 v |
| Current: | 1.2 mA | 4 - 20 mA | 980 μA |
| Dimensions: | 11.6 mm x 76.3 mm | 20 mm x 40 mm | 20 mm x 40 mm |
| Weight: | - | 24 g | 10 g |
| Price: | £99.23 | £99.60 | £5:16 |
| Pros: | Low Current Draw, Can also record Temperature |  | Cheap |
| Cons: | Will require bracket to mount. | High Voltage Requirements | Difficult to adapt to a “Mobile” Unit. Would need separate enclosure to access air temperature and keep dry etc. and difficult to integrate. |
| Chosen Sensor: |  |  |  |
| Website: | https://uk.rs-online.com/web/p/temperature-humidity-sensor-ics/8937373?gb=s | https://www.kempstoncontrols.com/ | https://www.seeedstudio.com/ |
| Referencing: | [5] | [7] | [8] |

Table 4: Humidity Sensors Comparison

Figure 2: Chosen Humidity Sensor

Datasheet - https://docs.rs-online.com/fcd5/0900766b8142cdce.pdf

### Wind Speed

| Wind Speed Sensors |  |  |  |
| --- | --- | --- | --- |
| System: | A100L2 | WindMaster 3D | Gill |
| Company: | Vector Instruments | Omni Instruments | WindSonic 75 Anemometer |
| Photo: |  |  |  |
| Wind Speed Range: | 0.2 to 75 m/s | 0 to 50 m/s | 0 to 146 knots (146 Knots = 168 MPH) |
| Resolution: | 0.15 m/s | 0.01 m/s | 1 |
| Operating Temperature Range: | -30 to 70 °C | -40 to 70 °C | -35 to 70 °C |
| Interface: | Analogue | RS232, 422, 485, Binary | RS232 |
| Operating Voltage: | 6.5 v to 28 v | 9 v to 30 v | 12v or 30v |
| Current: | 2 mA | 55 mA | 44mA |
| Dimensions: | 195 mm x 152 mm | 750 mm x 240 mm | 142 mm x160 mm |
| Weight: | 500 g | 1 kg | 500 g |
| Price: | £537.70 | £2,413.00 | £1,443.00 |
| Pros: | - | Also Measures Direction | Also Measures Wind Direction |
| Cons: | Limited Speed Measuring | Expensive | - |
| Chosen Sensor: |  |  |  |
| Website: | https://www.windspeed.co.uk/ws/index.php?Itemid=1 | https://www.omniinstruments.co.uk/ | https://r-p-r.co.uk/product/gill-windsonic-75-anemometer/ |
| Referencing: | [9] | [10] | [11] |

Table 5: Wind Speed Sensors Comparison

Figure 3: Chosen Wind Speed Sensor

Datasheet - https://r-p-r.co.uk/app/uploads/2019/01/WindSonic75-1405-0035-Iss-5.pdf

### Wind Direction

| Wind Direction Sensors |  |  |  |
| --- | --- | --- | --- |
| System: | Gill | S200 | W200P |
| Company: | WindSonic 75 Anemometer | SenseCAP | Vector Instruments |
| Photo: |  |  |  |
| Measurement Range: | 0° to 360° | 0° to 360° | 1.15° to 358.85° |
| Resolution: | 1° | 0.1° | 0.2° |
| Operating Temperature Range: | -35 to 70 °C | -40 to 85 °C | -50 to 70 °C |
| Interface: | RS232 | RS485 / SDI | Analogue |
| Operating Voltage: | 12v or 30v | 24v | 1 v to 5 v |
| Current: | 44mA | - | 20 mA |
| Dimensions: | 142 mm x160 mm | - | 225 mm x 219 mm |
| Weight: | 500 g | 2 kg | 500 g |
| Price: | £1,443.00 | £287.26 | £587.93 |
| Pros: | Also Measures Wind Speed | Accurate | Accurate |
| Cons: | - | High Operating Voltage | - |
| Chosen Sensor: |  |  |  |
| Website: | https://r-p-r.co.uk/product/gill-windsonic-75-anemometer/ | https://www.distrelec.be/en/ | https://www.windspeed.co.uk/ws/index.php |
| Referencing: | [11] | [12] | [13] |

Table 6: Wind Direction Sensors Comparison

Figure 4: Chosen Wind Direction Sensor

Datasheet - https://r-p-r.co.uk/app/uploads/2019/01/WindSonic75-1405-0035-Iss-5.pdf

### Airborne Particle Count

| Airborne Particle Count Sensors |  |  |  |
| --- | --- | --- | --- |
| System: | Pro-Plus | Next PM Sensor | SEN54-SDN-T |
| Company: | ISOAir | Tera | Sensirion |
| Photo: |  |  |  |
| Sampling Interval: | 1 s | 1 s | 1 s |
| Mass Concentration Range: | From 1μm | 1 to 1,000 μg/m3 | 0 to 1,000 μg/m3 |
| Operating Temperature: | 0 to 50 °C | -10 to 60 °C | -10 to 50 °C |
| Operating Voltage: | 100 - 240v AC | 5v | 4.5v to 5.5v |
| Current: | 4 - 20 mA | 80 - 300 mA | 100mA |
| Interface: | Digital | UART | I2C |
| Weight: | 4 Kg | - | 36.4 g |
| Price: | £949.00 | £68.24 | £27.32 |
| Pros: | Accurate | - | Also measures VOC, Cheap |
| Cons: | Expensive, High Operational Voltage | Only measures Particle Count | Difficult to integrate |
| Chosen Sensor: |  |  |  |
| Website: | https://www.alphacontrols.com/ | https://uk.rs-online.com/web/ | https://uk.farnell.com/sensirion/sen54-sdn-t/particle-sensor-dust-digital-5v/dp/3924519 |
| Referencing: | [14] | [15] | [16] |

Table 7: Airborne Particle Count Sensors Comparison

Figure 5: Chosen Airborne Particle Count Sensor

Datasheet - https://www.farnell.com/datasheets/3689381.pdf

### Sunlight Brightness

| Sunlight Brightness |  |  |  |
| --- | --- | --- | --- |
| Model: | SP Lite2 | MPY54C569 | CS320 |
| Company: | Omni Instruments | Photonix | Campbell Scientific |
| Photo: |  |  |  |
| Interface: | Digital | Analogue | Digital |
| Operating Temperature: | -30 to 70 °C | -30 to 75 °C | -30 to 75 °C |
| Operating Range: | 0 to 2000 W/m2 | - | 0 to 2000 W/m2 |
| Operating Voltage: | - | 3v to 5v | 6 - 24 Vdc |
| Current: | - | 75mA | 25 mA |
| Price: | £405.00 | £0.61 | £265.00 |
| Pros: | - | Cheap | Operates at the correct Voltage |
| Cons: | Expensive | Inaccurate | - |
| Chosen Sensor: |  |  |  |
| Website: | https://www.omniinstruments.co.uk/ | https://www.switchelectronics.co.uk/ | https://www.campbellsci.com/cs320 |
| Referencing: | [17] | [18] | [19] |

Table 8: Sunlight Brightness Sensors Comparison

Figure 6: Chosen Sunlight Brightness Sensor

Datasheet - https://s.campbellsci.com/documents/us/manuals/cs320.pdf

### UV Level

| UV Level Sensors |  |  |  |
| --- | --- | --- | --- |
| Model: | VEML6075 | PMA1107 Analog Non-Weighted UVA+B Sensor | MS - 212W |
| Company: | Adafruit | Solar Light | Atmos |
| Photo: |  |  |  |
| Interface: | I2C | Analogue | Analogue |
| Operating Range: | 0 - 11 | 280-400nm | 1 |
| Operating Voltage: | 3v or 5v | 5 - 12 Vdc | 100 to 249 v AC |
| Current: | 100 mA | 30 mA |  |
| Dimensions: | 17 mm x 16 mm | 343 mm x 396 mm | 120 mm x 160 mm |
| Weight: | 0.8 g | 200g | 1.1 kg |
| Price: | £4.74 | £425.00 | £583.00 |
| Pros: | Cheap | Measures UV A & B | - |
| Cons: | Not suitable for external use | - | High Operating Voltage |
| Chosen Sensor: |  |  |  |
| Website: | https://www.adafruit.com/ | https://solarlight.com/product/uvauvbsensorpma1107/ | www.atmos-meteo.com/sensors/precision-uv-radiometer.html |
| Referencing: | [20] | [21] | [22] |

Table 9: UV Level Sensors Comparison

Figure 7: Chosen UV Level Sensor

Datasheet - https://solarlight.com/wp-content/uploads/AnalogSensors_PMA1107.pdf

Wiring Diagram - https://solarlight.com/wp-content/uploads/2013/04/Wiring-Diagram-for-Analog-PMA11XX-Detectors.pdf

### GPS Position

| GPS Position Sensors |  |  |  |
| --- | --- | --- | --- |
| Model: | PA1010D Breakout | M10Q-5883 GNSS | GPS Logger |
| Company: | Pimoroni | MATEKSYS | MAV Sense |
| Photo: |  |  |  |
| Interface: | I2C / UART | UART | GPS Logger |
| Operating Range: | GPS / GLONASS / GALILEO | GPS / GLONASS / GALILEO / BeiDou | GPS / GLONASS / GALILEO |
| Operating Voltage: | 3v to 5v | 4v to 9v | 3.5v to 8.4v |
| Current: | 36 mA | 13 mA | 50 mA |
| Dimensions: | 10 mm x 10 mm | 20 mm x 20 mm | 28 mm x 29.5 mm |
| Weight: | - | 8 g | 15 g |
| Price: | £29.70 | £31.10 | £98.27 |
| Pros: | Cheap | Cheap | - |
| Cons: | - | - | Expensive |
| Chosen Sensor: |  |  |  |
| Website: | https://shop.pimoroni.com/ | www.yourfpv.co.uk/product/mateksys-m10q-5883-gnss-compass/ | https://esoaringgadgets.co.uk/ |
| Referencing: | [23] | [24] | [25] |

Table 10: GPS Sensors Comparison

Figure 8: Chosen GPS Sensor

### VOC Levels

| VOC Levels Sensors |  |  |  |
| --- | --- | --- | --- |
| System: | AQDTF Duct Air Quality VOC Sensor | SEN54-SDN-T | SEN0134 |
| Company: | MSI | Sensirion | DF Robot |
| Photo: |  |  |  |
| Sampling Interval: | 1 s | 1 s | - |
| Mass Concentration Range: | 450 to 2,000 ppm | 0 to 1,000 μg/m3 | 10 to 1,000 ppm |
| Operating Temperature: | 0 to 50 °C | -10 to 50 °C | -40 to 85 °C |
| Operating Voltage: | 20v to 28v | 4.5v to 5.5v | 0.3 - 2.16 Vdc |
| Current: | 35 mA | 100mA | 100mA |
| Interface: | Analog | I2C | I2C |
| Weight: | 290 g | 36.4 g | 1.1g |
| Price: | £310.00 | £28.54 | £22.99 |
| Pros: | High Operating Voltage | Also measures Particle Count | Cheap |
| Cons: | Expensive | Difficult to integrate | - |
| Chosen Sensor: |  |  |  |
| Website: | www.measurementsystems.co.uk/sensors_and_meters/air_quality_and_gas_sensors/aqdtf-duct-air-quality-voc-sensor | https://uk.farnell.com/sensirion/sen54-sdn-t/particle-sensor-dust-digital-5v/dp/3924519 | https://coolcomponents.co.uk/products/sgp30-air-quality-sensor-breakout-voc-and-eco2 |
| Referencing: | [26] | [16] | [27] |

Table 11: VOC Levels Sensors Comparison

Figure 9: Chosen VOC Levels Sensor

Datasheet - https://www.farnell.com/datasheets/3689381.pdf

| Review of Sensors to be Used |  |  |
| --- | --- | --- |
| SENSOR | MAKE & MODEL | PARAMETER MEASURED |
|  | TE Connectivity HPP809A031 Temperature & Humidity Sensor | Temperature & Humidity |
|  | Gill WindSonic 75 Anemometer | Wind Speed & Wind Direction |
|  | Sensirion SEN54-SDN-T | VOC Levels & Airborne Particle Count |
|  | Campbell Scientific CS320 | Sunlight Brightness |
|  | Solar Light PMA1107 Analog Non-Weighted UVA+B Sensor | UV Level |
|  | MATEKSYS M10Q-5883 GNSS | GPS |

Table 12: Review of Sensors to be used

Table 12 above shows that even though the design requirement is to measure 9 different parameters, by careful selection of sensors, we actually only need to incorporate 6 different devices into our design build.

## Microcontroller Selection

| Parameters | Type of Output from Sensor |
| --- | --- |
| Temperature Humidity | Analogue |
| Wind Speed Wind Direction | Analogue |
| Air Borne Particle Count VOC Level | I2C |
| Sunlight Brightness | Analogue |
| UV Levels | Analogue |
| GPS Position | UART |

Table 13: Type of signal from sensor

Table 13 shows the type of output signal our sensors will produce. This complete with the tabulated view below in Table 14 of how many Analogue / Digital IO pins, Communication Devices and peripherals our Microcontroller will need will give us the information we need to select our PIC.

| Parameters | Type of Input Pin required on our Microcontroller |
| --- | --- |
| Temperature & Humidity | 2x Analogue Pins |
| Wind Speed Wind Direction | 2x Analogue Pins |
| Air Borne Particle Count & VOC Level | 1x I2C |
| Sunlight Brightness | 1x Analogue Pin |
| UV Levels | 1x Analogue Pin |
| GPS Position | 1x UART |
| USB Module | 1x USB |
| SD Card Module | 1x SPI |
| LED On Indicator | 1x Digital Output |

Table 14: Number of IO Pins required for PIC Microcontroller

From the above tables, we see that at a minimum, the microcontroller requirements are as follows:

- 8 Bit

- Minimum 17 Pins

- 6x Analogue Inputs

- 1x Digital Output

- 1x UART

- 1x SPI (4 I/O pins)

- 1x I2C (2 I/O pins)

- 1x USB (2 I/O pins)

- 1x Real Time Clock

We took this list of requirements for our Microcontroller, and went to the Microchip parametric part selector website tool, and used the parameter search to shortlist suitable Microcontrollers. [28]

When the above criteria was used to filter Microchip products, 19 suitable Pic18 Microcontrollers were identified as suitable. These were as follows:

- PIC18F97J94

- PIC18F87J94

- PIC18F67J94

- PIC18F47J53

- PIC18F27J53

- PIC18F96J94

- PIC18F86J94

- PIC18F66J94

- PIC18F46J50

- PIC18F46J53

- PIC18F26J50

- PIC18F26J53

- PIC18F95J94

- PIC18F85J94

- PIC18F65J94

- PIC18F45J50

- PIC18F25J50

- PIC18F44J50

- PIC18F24J50

Table 15 on the next page is a tabulated comparison of the 5 shortlisted Microcontrollers…

|  | PIC18 F47J53 | PIC18 F26J50 | PIC18 F26J53 | PIC18 F25J50 | PIC18 F24J50 |
| --- | --- | --- | --- | --- | --- |
| Family | 8-bit MCU | 8-bit MCU | 8-bit MCU | 8-bit MCU | 8-bit MCU |
| CPU Type | PIC | PIC | PIC | PIC | PIC |
| Max CPU Speed (MHz) | 48 MHz | 48 MHz | 48 MHz | 48 MHz | 48 MHz |
| Operation Voltage | (2V-3.6V) | (2V-3.6V) | (2V-3.6V) | (2V-3.6V) | (2V-3.6V) |
| Temp Range | (-40 to 85) | (-40 to 85) | (-40 to 85) | (-40 to 85) | (-40 to 85) |
| Pin count | 28 / 44 | 28 | 28 | 28 | 28 |
| Program Memory Size (KB) | 128 Flash | 64 Flash | 64 Flash | 32 Flash | 16 Flash |
| SRAM | 3.7109 KB | 3.7109 KB | 3.7109 KB | 3.7109 KB | 3.7109 KB |
| Low Power | Yes | Yes | Yes | Yes | Yes |
| ADC Input | 13 | 10 | 13 | 10 | 10 |
| Max ADC Resolution (Bits) | 12 | 10 | 12 | 10 | 10 |
| UART | 2 | 2 | 2 | 2 | 2 |
| SPI | 2 | 2 | 2 | 2 | 2 |
| I2C | 2 | 2 | 2 | 2 | 2 |
| Number of USB Modules | 1 | 1 | 1 | 1 | 1 |
| USB Interface | FS Device | FS Device | FS Device | FS Device | FS Device |
| Max 8-Bit Digital Timers | 4 | 2 | 4 | 2 | 2 |
| Max 16-Bit Digital Timers | 4 | 3 | 4 | 3 | 3 |
| Hardware RTCC/RTC | Yes | Yes | Yes | Yes | Yes |
| Integrated full-Speed USB 2.0 | Yes | Yes | Yes | Yes | Yes |
| High memory density for more complex application | Yes | No | Yes | No | No |
| Peripheral Pin Select for design flexibility | Yes | Yes | Yes | Yes | Yes |
| Features nanoWatt XLP Technology, for battery applications | Yes | Yes | Yes | Yes | Yes |
| Deep Sleep mode for low power application | 9nA | 9nA | 9nA | 15nA | 15nA |
| Sleep mode (RAM retention) | 200nA | 60nA | 200nA | 60nA | 60nA |
| Real Time Clock & Calendar operation in Sleep modes | 700nA | 830nA | 700nA | 700nA | 830nA |
| Watch Dog Timer operation in Sleep modes | 330na | 780na | 330na | 830na | 780na |
| Integrated full speed USB 2.0 | Yes | Yes | Yes | Yes | Yes |
| Deep Sleep mode ideal for battery applications | Yes | Yes | Yes | Yes | Yes |
| Peripheral Pin Select for mapping digital peripherals | Yes | Yes | Yes | Yes | Yes |
| RTCC provides clock, calendar & alarm functions | Yes | Yes | Yes | Yes | Yes |
| Charge Time Measurement Unit (CTMU) | Yes | Yes | Yes | Yes | Yes |
| Enhanced Capture / Compare / PWM modules | 3 | 2 | 3 | 2 | 2 |
| MSSP serial ports for SPI or I2C™ communication | 2 | 2 | 2 | 2 | 2 |
| Enhanced USART modules | 2 | 2 | 2 | 2 | 2 |
| 8-bit Parallel port | Yes | No | Yes | No | No |
| Analog comparators | 3 | 2 | 3 | 2 | 2 |
| ADC | 13ch, 12-bit | 10ch, 10-bit | 10ch, 12-bit | 10ch, 10-bit | 10ch, 10-bit |
| Self-programming Flash supports 10k erase/write cycles & 20 years retention | Yes | Yes | Yes | Yes | Yes |
| Operating voltage 2.0 - 3.6V, 5.5V tolerant digital inputs | Yes | Yes | Yes | Yes | Yes |
| Chosen Microcontroller: |  |  |  |  |  |

Table 15: PIC Microcontroller Comparison

PIC Microcontroller selected is the PIC18F47J53

Figure 10: Chosen PIC Microcontroller

Datasheet - https://ww1.microchip.com/downloads/aemDocuments/documents/OTH/ProductDocuments/DataSheets/30009964C.pdf

## Storage

Memory Requirements

In order to store the data collected from our sensors, we need to calculate the required Non-Volatile memory. Non- Volatile memory will be data stored even when the device is powered down, this opposed to the Volatile memory which can only be stored when the device is powered up.

| Parameters | Possible Values |
| --- | --- |
| Temperature | -40° C --> +55° C |
| Humidity | 0% --> 100% |
| Wind Speed | 0 --> 155 MPH |
| Wind Direction | 0° --> 360° |
| Air Borne Particle Count | 0 --> 1,000 μg/m3 |
| Sunlight Brightness | 0 --> 1120 W/m2 |
| UV Levels | 280 --> 400nm |
| GPS Position | ≤ 65536 |
| VOC Level | 1 --> 500 |

Table 16: Sensor Value Ranges

Table 16 shows the ranges that each of the sensors will measure. These ranges need to be as such that any possible value is recorded.

Temperature - The lowest point of -40° C has been chosen as this is the average winter temperature in the Russian territory of Siberia according to ClimatesToTravel.com. [29]

The highest point of +55° C has been chosen as 56.7° C is the highest temperature recorded on earth. This was at Death Valley, California according to Arizona State University. [30]

Humidity - It was decided to measure the full spectrum from 0 to 100% humidity, as according to MostPopulationReview.com the humidity in Singapore ranges between 60% and 90%. This is classed as a very humid city. Therefore we can expect the lower figure to be considerably lower in Western Europe. [31]

Wind Speed - It was decided to measure only up to 155 MPH, as anything above this is classed as Category 5 hurricane. This would cause catastrophic damage to buildings to occur, and certainly damage a Weather Station, according to the National Hurricane Centre. [32]

Wind Direction	The full spectrum from 0° to 360° will be needed in order to detect wind direction. Shifts in degrees will determine the direction of the wind. This is explained in Figure 11.

Figure 11: Compass rose with 16 wind directions

Airborne Particle Count	 - The full spectrum of the sensor, 0 --> 1,000 μg/m3 is to be read. This is due to the fact that a conversion formula would then be needed to be applied in order to convert into ppb (Parts per Billion) which is the standard Air Particle Count Unit. [33]

Sunlight Brightness - The maximum recorded solar radiation recorded according to SynergyFiles.com is 1120 W/m2. [34]

UV Levels - As our UV Level sensor measures across UVA & UVB we need to record values between 280 --> 400nm. Wavelength can be calculated into UV Index. [35]

GPS - A GPS signal is received in a NEMA message structure of numbers and letters of up to 82 characters. [36]

VOC Level - According to Lane Regional Air Protection Agency, VOC levels can be grouped into 6 different categories. Figure 12 shows how these are broken down. This is the reason for measuring between 1 & 500. [37]

Figure 12: The Air Quality Index (AQI) categories and colours

| Parameters | Maximum Value | Resolution | Number of Possible Values | No. Bits Required |
| --- | --- | --- | --- | --- |
| Temperature | -40° C --> +55° C | 0.1° C | 950 | 10 |
| Humidity | 0% --> 100% | 1% | 100 | 7 |
| Wind Speed | 0 --> 155 MPH | 1 MPH | 155 | 7 |
| Wind Direction | 0° --> 360° | 1° | 360 | 9 |
| Air Borne Particle Count | 0 --> 1,000 μg/m3 | 10 μg/m3 | 100 | 7 |
| Sunlight Brightness | 0 --> 1120 W/m2 | 10 W/m2 | 112 | 7 |
| UV Levels | 280 --> 400nm | 1 nm | 120 | 7 |
| GPS Position | ≤ 65,536 | 1 | ≤ 65,536 | 16 |
| VOC Level | 1 --> 500 | 1 | 500 | 9 |

Table 17: Sensor Resolution, Number of Possible Values and No. Bits Required

Table 17 is showing what Resolution we are measuring in, that is in what increments. Also it breaks down the resolution to display the possible Number of Possible Values or increments we will be measuring. The final column then shows No of Bits Required in order to accurately measure our sensors data output.

|  | 1 Byte |  |  |  |  |  |  |  |  | 1 Byte |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| Humidity | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  |  |  |  |  |  |  |  |  |
| Wind Speed | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  |  |  |  |  |  |  |  |  |
| Wind Direction | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| Airborne Particle Count | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  |  |  |  |  |  |  |  |  |
| Sunlight Brightness | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  |  |  |  |  |  |  |  |  |
| UV Levels | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  |  |  |  |  |  |  |  |  |
| GPS | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| VOC Level | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |

Figure 13: No of 8 Bit Bytes Required

The Microcontroller I have selected is from the 8 Bit Pic family range, so to calculate the no of bytes required for our Non-Volatile Memory we need to convert the Bit requirement to a Bytes. We can do this by using the image in Figure 13. By drawing out 8 B Bits next to each of the sensors, we can understand the process better.

| Parameters | Number of Possible Values | No. Bits Required | No. Bytes Required |
| --- | --- | --- | --- |
| Temperature | 950 | 10 | 2 |
| Humidity | 100 | 7 | 1 |
| Wind Speed | 155 | 7 | 1 |
| Wind Direction | 360 | 9 | 2 |
| Air Borne Particle Count | 100 | 7 | 2 |
| Sunlight Brightness | 112 | 7 | 2 |
| UV Levels | 120 | 7 | 2 |
| GPS Position | ≤ 65,536 | 16 | 2 |
| VOC Level | 500 | 9 | 2 |
|  |  |  | Total 16 Bytes |

Table 18: No of Bytes Required

Table 18 shows the result of how many Bytes of data that could be required every time our weather station records data from our sensors.

| Parameters | Frequency of Measurement |
| --- | --- |
| Wind Speed | ½ second |
| Temperature | 1 second |
| Sunlight Brightness | 1 second |
| Wind Direction | 1 second |
| Humidity | 5 seconds |
| UV Levels | 5 seconds |
| Air Borne Particle Count | 5 seconds |
| VOC Level | 5 seconds |
| GPS Position | 60 seconds |

Table 19: Frequency of Measurement

Table 19 shows the frequency we will be collecting data from each of our sensors. This is visually easier to understand from Figure 14 below.

Figure 14: Frequency of measurement (Visual Representation)

Below are the Memory Requirements calculations:

Temperature

2 Bytes/cycle*1 (Frequency of Measurement)=2 Bytes/Second

2 Bytes/Second*60=120 Bytes/Minute

120 Bytes/Minute*60=7,200 Bytes/Hour

7,200 Bytes/Hour*24=172,800 Bytes/Day

172,800 Bytes/Day*14 Days Operation=2,419,200 Bytes/ 14 Days Operation

Humidity

1 Bytes/cycle*0.2 (Frequency of Measurement)=0.2 Bytes/Second

0.2 Bytes/Second*60=12 Bytes/Minute

12 Bytes/Minute*60=720 Bytes/Hour

7,200 Bytes/Hour*24=17,280 Bytes/Day

172,800 Bytes/Day*14 Days Operation=241,920 Bytes/ 14 Days Operation

Wind Speed

2 Bytes/cycle*1 (Frequency of Measurement)=2 Bytes/Second

2 Bytes/Second*60=120 Bytes/Minute

120 Bytes/Minute*60=7,200 Bytes/Hour

7,200 Bytes/Hour*24=172,800 Bytes/Day

172,800 Bytes/Day*14 Days Operation=2,419,200 Bytes/ 14 Days Operation

Wind Direction

2 Bytes/cycle*1 (Frequency of Measurement)=2 Bytes/Second

2 Bytes/Second*60=120 Bytes/Minute

120 Bytes/Minute*60=7,200 Bytes/Hour

7,200 Bytes/Hour*24=172,800 Bytes/Day

172,800 Bytes/Day*14 Days Operation=2,419,200 Bytes/ 14 Days Operation

Air Borne Particle Count

2 Bytes/cycle*0.2 (Frequency of Measurement)=0.4 Bytes/Second

0.4 Bytes/Second*60=24 Bytes/Minute

24 Bytes/Minute*60=1,440 Bytes/Hour

1,440 Bytes/Hour*24=34,560 Bytes/Day

34,560 Bytes/Day*14 Days Operation=483,840 Bytes/ 14 Days Operation

Sunlight Brightness

2 Bytes/cycle*1 (Frequency of Measurement)=2 Bytes/Second

2 Bytes/Second*60=120 Bytes/Minute

120 Bytes/Minute*60=7,200 Bytes/Hour

7,200 Bytes/Hour*24=172,800 Bytes/Day

172,800 Bytes/Day*14 Days Operation=2,419,200 Bytes/ 14 Days Operation

UV Levels

1 Bytes/cycle*0.2 (Frequency of Measurement)=0.2 Bytes/Second

0.2 Bytes/Second*60=12 Bytes/Minute

12 Bytes/Minute*60=720 Bytes/Hour

7,200 Bytes/Hour*24=17,280 Bytes/Day

172,800 Bytes/Day*14 Days Operation=241,920 Bytes/ 14 Days Operation

GPS Position

2 Bytes/cycle*0.01666667 (Frequency of Measurement)=0.03333333 Bytes/Second

0.03333333 Bytes/Second*60=2 Bytes/Minute

2 Bytes/Minute*60=120 Bytes/Hour

120 Bytes/Hour*24=2,880 Bytes/Day

2,880 Bytes/Day*14 Days Operation=40,320 Bytes/ 14 Days Operation

VOC Level

1 Bytes/cycle*0.2 (Frequency of Measurement)=0.2 Bytes/Second

0.2 Bytes/Second*60=12 Bytes/Minute

12 Bytes/Minute*60=720 Bytes/Hour

7,200 Bytes/Hour*24=17,280 Bytes/Day

172,800 Bytes/Day*14 Days Operation=241,920 Bytes/ 14 Days Operation

| Parameters | No. Bytes Required/Cycle | No. Bytes Required/Second | No. Bytes Required/Min | No. Bytes Required/Day | No. Bytes Required/14 Days |
| --- | --- | --- | --- | --- | --- |
| Temperature | 2 | 2 | 120 | 172,800 | 2,419,200 |
| Humidity | 1 | - | 12 | 17,280 | 241,920 |
| Wind Speed | 1 | 2 | 120 | 172,800 | 2,419,200 |
| Wind Direction | 2 | 2 | 120 | 172,800 | 2,419,200 |
| Air Borne Particle Count | 2 | - | 24 | 34,560 | 483,840 |
| Sunlight Brightness | 2 | 2 | 120 | 172,800 | 2,419,200 |
| UV Levels | 2 | - | 12 | 17,280 | 241,920 |
| GPS Position | 2 | - | 2 | 2,880 | 40,320 |
| VOC Level | 2 | - | 12 | 17,280 | 241,920 |
|  | Total 16 Bytes | Total 8 Bytes | Total 542 Bytes | Total 780,480 Bytes (781 kilobyte) | Total 10,926,720 Bytes (10.93MB) |

Table 20: Non-Volatile Memory required for 14 day Operation of Weather Station

Table 20 tabulates the calculated maximum Non-Volatile memory required for the operation of our Weather Station for a 14 day period as per our Output Requirements.

Although larger memory solutions are available. A 2GB card will be sufficient for approximately 180 x 14 day Operating Cycles of the completed Weather Station. This keeps the card below 4GB which can cause problems with addressing on some FAT32 file systems.

Datasheet - https://www.farnell.com/datasheets/3110845.pdf

## USB & SD Card Modules

| LED Indicators, USB Module & SD Card Module |  |  |  |  |
| --- | --- | --- | --- | --- |
| System: | 130-1096 | CH340 | Diligent 410-380 | PELR0233 |
| Company: | RS Pro | Everything Smart | Components 101 | Pro Elec |
| Photo: |  |  |  |  |
| Type: | Green Panel Mount Indicator | USB-C to UART Serial Adapter | SD Card Module | Switch |
| Operating Voltage: | 1.8 – 3.3 Vdc | Selectable 3.3 or 5V | 4.5 – 5.5 Vdc | <125 V |
| Current: | 20 mA | 200 mA | 200 mA | 250 V |
| Weight: |  |  |  |  |
| Price: | £15.36 | £5.00 | £6.96 | £0.86 |
| Website: | https://uk.rs-online.com/web/p/panel-mount-indicators/1301096?gb=s | https://shop.everythingsmart.io/products/usb-c-to-serial-adapter | https://uk.rs-online.com/web/p/memory-development-tools/1840482?gb=s | https://cpc.farnell.com/unbranded/abrr016/switch-rocker-spst-green-250vac/dp/SW02923?ost=pelr0233 |
| Referencing: | [38] | [39] | [40] | [41] |

Table 21: Miscellaneous Parts 1

LED Datasheet - https://docs.rs-online.com/8bc8/A700000009334440.pdf

USB Datasheet - https://shop.everythingsmart.io/products/usb-c-to-serial-adapter

SD Datasheet - https://docs.rs-online.com/9265/A700000006690481.pdf

Switch Datasheet - https://www.farnell.com/datasheets/1564449.pdf

## Batteries

Before deciding on a battery, we must calculate the required capacity.

| POWER CONSUMING COMPONENT: | CURRENT DRAW: |
| --- | --- |
| Microcontroller | 25 mA |
| Temperature & Humidity Sensor | 1.2 mA |
| Wind Speed & Wind Direction Sensor | 44 mA |
| VOC Levels & Airborne Particle Count Sensor | 100 mA |
| Sunlight Brightness Sensor | 25 mA |
| UV Level Sensor | 30 mA |
| GPS Sensor | 13 mA |
| USB Module | 200 mA |
| SD Card Module | 200 mA |
| LED Indicator Light | 20 mA |
| TOTAL CURRENT DRAW: | 658.20 mA |

Table 22: Battery Power Requirement

The information found in Table 22 has been taken from Datasheets

Reference Numbers:  [5] , [11] , [16] , [19] , [21] , [24] , [38] , [39] , [40]

Current (I) = 658.20 mA

Discharge Time (T) = 336 hours (24 hours x 14 days)

Amp hour Ah=Current I*Discharge Time T

658.20 *336=221155.20

221155.20=221.1552 A

Minimum Battery Size Required for 14 day Weather Station=222 Ah

| Battery Systems |  |  |  |
| --- | --- | --- | --- |
| System: | LED240 | 24-230SMART3 | EXP12-250C |
| Company: | Varta | Roamer | Expedition Plus |
| Photo: |  |  |  |
| Type: | Deep Cycle Leisure Battery | Leisure Battery | Lead Carbon Gel |
| Voltage: | 12 Volt | 24 Volt | 12 Volt |
| Current: | 240 Ah | 230 Ah | 250 Ah |
| Cycles: | 1,000 | 3,500 | 1,500 |
| Weight: | 56.8 Kg | 38 Kg | 63 Kg |
| Length x Width x Height: | 513 mm x 276 mm x 242 mm | 521 mm x 268 mm x 218 mm | 532 mm x 240 mm x 219 mm |
| Pros: | Cheapest | Large Cycle Lifespan | - |
| Cons: | Least Cycle Lifespan | Expensive | Heavy & Bigger |
| Price: | £262.29 | £1,800.00 | £389.98 |
| Chosen Battery: |  |  |  |
| Website: | https://www.powerland.co.uk/collections/marine-batteries/products/varta-led240-professional-dual-purpose-efb-leisure-battery-240ah-lfd230-930240120-1 | https://roamer.com/products/24-230smart?_pos=1&_sid=a28ebb3eb&_ss=r | https://www.alpha-batteries.co.uk/12v-250ah-expedition-plus-lead-carbon-gel-ultra-deep-cycle-battery-exp12-250c/ |
| Referencing: | [42] | [43] | [44] |

Table 23: Battery Comparison

Figure 15: Chosen Battery

Datasheet - https://www.alpha-batteries.co.uk/wp-content/uploads/K087-EXP12-250C-.pdf

## Enclosure

| Enclosures |  |  |  |
| --- | --- | --- | --- |
| System: | 960 Case | M14 Mini Screen | 3I-2424-10B-C |
| Company: | NANUK | MetSpec | SKB |
| Photo: |  |  |  |
| Material: | Synthetic Rubber | PVC Plastic | Moulded Plastic |
| Protection: | IP6X | - | - |
| No. of Sizes: | 34 | 1 | 40 |
| Length x Width x Height: | 559 mm x 432 mm x 328 mm | 275 mm x 315 mm x 405 mm | 610 mm x 610 mm x 254 mm |
| Weight: | 8.7 Kg | 7.2 Kg | - |
| Price: | £332.43 | SOLD OUT | £525.68 |
| Pros: | Cheapest | Ideal for Sensors | Large |
| Cons: | - | Sold Out | Expensive |
| Chosen Battery: |  |  |  |
| Website: | https://www.casesuk.com/nanuk-cases/nanuk-960-case.html | https://www.metcheck.co.uk/products/metspec-m14-mini-polycarbonate-stevenson-screen | https://www.protective-cases.co.uk/product/skb-3i-2424-10b-c-case-with-cubed-foam_3901 |
| Referencing: | [45] | [46] | [47] |

Table 24: Enclosure Comparison:

Figure 16: Chosen Enclosure

## The Hardware Layout

The enclosure will be adapted to house and create a mobile power supply.

The battery will be housed within the enclosure, which will then be fitted with a terminal fuse block c/w terminal fuse.

4 Gauge wire will then be connected to the terminals, and then to a battery power cut-off switch, which the enclosure will be adapted to house.

Another 4 Gauge wire will then be connected to the output of the switch to the Fuse Block.

The enclosure will then be further adapted to house a 12 Volt Socket. This will be wired into the Fuse Block via the supplied cable. The Weather Station will then be able to be powered from the 12 Volt socket via a 12V cord.  This method would also future proof the design by giving a direct 12v supply to any upgraded equipment.

Figure 17: Battery Enclosure Schematic

Figure 17 shows the schematic of the battery hardware pack.

| Terminal Block, Terminal Fuse & 4 Gauge Wire |  |  |  |
| --- | --- | --- | --- |
| System: | Terminal Fuse Block | BATTERY TERMINAL FUSE 30-AMP | 3M-9MQF-X7LI |
| Company: | Blue Sea Systems | Blue Sea Systems | LotFancy |
| Photo: |  |  |  |
| Seller: | Amazon | Amazon | Amazon |
| Weight: | 227g | 45g | 390g |
| Price: | £34.99 | £22.60 | £31.67 |
| Website: | https://www.amazon.co.uk/ | https://www.amazon.co.uk/ | https://www.amazon.co.uk/ |
| Referencing: | [48] | [49] | [50] |

Table 25: Miscellaneous Parts 2

| Battery Cut Off Switch, Fuse Block, 12V Socket & Lead |  |  |  |  |
| --- | --- | --- | --- | --- |
| System: | ‎Am-CBS01 | Fuse Block | 12 Volt Socket | IR-EC-A08 |
| Company: | ‎Ampper | Huazu | Nilight | ‎Irzaki |
| Photo: |  |  |  |  |
| Seller: | Amazon | Amazon | Amazon | Amazon |
| Weight: | 264g | 181g | 159g | 310g |
| Price: | £12.99 | £19.60 | £9.99 | £15.99 |
| Website: | https://www.amazon.co.uk/ | https://www.amazon.co.uk/ | https://www.amazon.co.uk/ | https://www.amazon.co.uk/ |
| Referencing: | [51] | [52] | [53] | [54] |

Table 26: Miscellaneous Parts 3

As well as an enclosure, we need a stand to hold the external sensors. This one can be secured down if necessary. And a mounting pole to ensure adequate mounting space. A plastic enclosure will also be fitted to the tripod to house the electronics via a pole mounting kit.

| Battery Cut Off Switch, Fuse Block, 12V Socket & Lead |  |  |  |  |
| --- | --- | --- | --- | --- |
| System: | Tripod | Mounting Pole Kit | Cabinet Enclosure | Pole Bracket |
| Company: | Davis | Davis | Evatron | SHENZHEN TECHNOLOGIES |
| Photo: |  |  |  |  |
| Seller: | Weather Shop | Weather Shop | Evatron | Amazon |
| Weight: | - | - | - | 460g |
| Price: | £172.30 | £60.00 | £31.13 | £26.99 |
| Website: | https://www.weathershop.co.uk/ | https://www.weathershop.co.uk/ | https://evatron.com/ | https://www.amazon.co.uk/ |
| Referencing: | [55] | [56] | [57] | [58] |

Table 27: Miscellaneous Parts 4

## Connectors (Panel Mounted to Casing)

| Case Mounted Connectors Systems |  |  |  |
| --- | --- | --- | --- |
| System: | SP2912 | PSG08744 | SP 13 Series |
| Company: | In2Connect | Pro - Signal | Pro - Elec |
| Photo: |  |  |  |
| Material: | Plastic | Nickel | Nylon |
| Protection: | IP68 | - | IP68 |
| Cap Fitted: | Yes | No | Yes |
| No. of Poles Available: | 2, 3, 4, 7, 8, 9, 10, 12, 16, 17, 20, 24, 26, 35 | 2, 3, 4, 5, 7, 9, 12 | 2, 3, 4, 5, 7, 9, 12 |
| Rated Voltage: | 500 V | 50 V | 250 V |
| Pros: | Lots of Variations | - | Cheaper than alternative |
| Cons: | - | Not IP Rated | Expensive |
| Price: (Plug & Socket – 3 Pole) | £9.00 / pair | £3.89 / pair | £8.28 / pair |
| Chosen Connector: |  |  |  |
| Website: | https://componentbuddy.com/product/sp29-panel-mount-connectors-female/ | https://cpc.farnell.com/neutrik/nc3faav2/xlr-socket-3p-pcb-vert/dp/AV15739?st=3%20pole%20xlr%20socket | https://cpc.farnell.com/mbs-connect/sp2112-s3-1c/circular-panel-socket-3-pole/dp/CN21385?st=pro%20elec%20sp%2013%20series |
| Referencing: | [59] | [60] | [61] |

Table 28: Panel Mounted Connectors Comparison

Figure 18: Chosen Panel Mounted Connectors

## Connectors (PCB Mounted)

| PCB Mounted Connectors Systems |  |  |  |
| --- | --- | --- | --- |
| System: | PCB Connector | Pin Header | Fixed Terminal Block |
| Company: | Reichelt | Molex | TE Connectivity |
| Photo: |  |  |  |
| Type: | Angled | Vertical | Screw |
| Max Voltage: | 250 V | 500 V | 150 V |
| Fixing: | Push Together | Locking Tang | Screw Clamp |
| Material: | Nylon | Nylon | Polyamide |
| No. of Poles Available: | 2, 3, 4, 7, 8, 9, 10, 12 | 2, 3, 4, 7, 8, 10, 12 | 2, 3, 4, 7, 8, 9, 10, 12 |
| Pros: | - | - | Good Mechanical Connection |
| Cons: | - | - | - |
| Price: | £0.34p | £0.83p | £1.44p |
| Chosen Connector: |  |  |  |
| Website: | https://www.reichelt.com/at/en/pcb-connector-angled-brown-10-pin-ps-25-10w-br-p47229.html?&trstct=pos_2&nbc=1 | https://uk.farnell.com/molex/26-60-4043/connector-header-4pos-1row-3-96mm/dp/3049003?st=molex%20pin%20header | https://www.mouser.co.uk/ProductDetail/TE-Connectivity/282851-4?qs=qTrdGJE%2F%252BQz%252BSpIukU4Srg%3D%3D |
| Referencing: | [62] | [63] | [64] |

Table 29: PCB Mounted Connectors Comparison

Figure 19: Chosen PCB Mounted Connectors

ASSUMPTION

An assumption has been made here that the factory connected cables on the sensors are adequate and long enough to get back to the Weather Station from the sensor.

## Circuit and PCB Design

Figure 20: Power Supply Block Diagram

The Block diagram in Figure 20 shows that the Weather Station three different voltages. That is 12V, 3.3V and 5V. The diagram also shows what voltages are required for which peripheral and sensor.

Figure 21: System Block Diagram

The Block diagram in Figure 21 shows how the sensors and peripherals in our Weather Station communicate with each other. We need to understand these paths before we design our sensor network.

## Power Supply Schematic

Figure 22: Proteus Power Supply Schematic

Figure 23: Easy PC Power Supply Schematic

The Proteus design in Figure 22 and the Easy PC Schematic in Figure 23 shows the step down from the 12 Volt Battery to 5V via a T7805 voltage regulator, and from 5V to 3.3V via a 7833 voltage regulator. This integrated circuit regulators maintain a constant output even if the input fluctuates. Note a Schottky Diode to protect against cross polarity of supply.

### 12v Supply Schematic

Figure 24: Proteus 12V Schematic

### 5v Supply Schematic

Figure 25: Proteus 5V Schematic

### 3.3v Supply Schematic

Figure 26: Proteus 3.3V Schematic

### LED Flashing Indicator Schematic

Figure 27: Easy PC LED Indicator

Figure 28: Proteus LED Indicator

R=Vs-VfIf

R=5V-1.8V20mA

R=3.2V20mA

R=160 Ω

In order to interface the sensors to our Microcontroller, a low pass filter must be designed into our circuit. The low pass filter only allows low frequency signals from 0Hz to its cut-off frequency, ƒc, to pass while blocking those any higher.

fc=12  R C

If we choose a cut off frequency of 1.5 MHz, and a resistor value of 100Ω, then we can determine the capacitor size by rearranging the equation:

C=12  R f

C=1(2) () (100) (1.5*106)=1.061*10-9

C=1.061*10-9

1(2) () (100) (1*10-9)=1.5 MHz

### Temperature & Humidity Sensor Schematic

Figure 29: Proteus Temperature & Humidity Sensor Schematic

Figure 30: Easy PC Temperature & Humidity Sensor Schematic

The Proteus design in Figure 29 and the Easy PC Schematic in Figure 30 shows how the Temperature & Humidity Sensor will interface with the PIC Microcontroller.

### Wind Speed & Wind Direction Sensor Schematic

Figure 31: Proteus Wind Speed & Direction Sensor Schematic

Figure 32: Easy PC Wind Speed & Direction Sensor Schematic

The Proteus design in Figure 31 and the Easy PC Schematic in Figure 32 shows how the Wind Speed & Wind Direction Sensor will interface with the PIC Microcontroller.

### GPS Sensor Schematic

Figure 33: Proteus GPS Unit Schematic

Figure 34: Easy PC GPS Unit Schematic

The Proteus design in Figure 33 and the Easy PC Schematic in Figure 34 shows how the GPS Unit will interface with the PIC Microcontroller.

### Air Quality & VOC Sensor Schematic

Figure 35: Proteus Air Quality & VOC Sensor Schematic

Figure 36: Easy PC Air Quality & VOC Sensor Schematic

The Proteus design in Figure 35 and the Easy PC Schematic in Figure 36 shows how the Air Quality & VOC Sensor will interface with the PIC Microcontroller. Note the pull up resistor, values are as per datasheet.

### UV Sensor Schematic

Figure 37: Proteus UV Sensor Schematic

Figure 38: Easy PC UV Sensor Schematic

The Proteus design in Figure 37 and the Easy PC Schematic in Figure 38 shows how the UV Sensor will interface with the PIC Microcontroller.

### Sunlight Sensor Schematic

Figure 39: Proteus Sunlight Brightness Sensor Schematic

Figure 40:  Easy PC Sunlight Brightness Sensor Schematic

The Proteus design in Figure 39 and the Easy PC Schematic in Figure 40 shows how the Sunlight Brightness Sensor will interface with the PIC Microcontroller.

### SD Card Circuit Schematic

Figure 41: Proteus SD Card Schematic

Figure 42: Easy PC SD Card Schematic

The Proteus design in Figure 41 and the Easy PC Schematic in Figure 42 shows how the SD Card Module will interface with the PIC Microcontroller.

### USB Circuit Schematic

Figure 43: Proteus USB Module Schematic

Figure 44: Easy PC USB Module Schematic

The Proteus design in Figure 43 and the Easy PC Schematic in Figure 44 shows how the USB Module will interface with the PIC Microcontroller.

### PIC Microcontroller Schematic

Figure 45: Proteus PIC Microcontroller Schematic

### PIC Decoupler & PIC MCLR Schematic

Figure 46: Proteus MCLR Schematic

Figure 47: Proteus PIC Decoupler

Figure 48: Easy PC PIC Microcontroller Schematic

The Proteus design in Figures 45 and the Easy PC Schematic in Figure 48 shows the interfacing of the PIC 18F47J53 Microcontroller to the peripherals and sensors. Note the Filter connected as well as the decoupling capacitors. All values and connections are as per datasheet.

Figure 49: Proteus Schematic

Figure 50: Easy PC Schematic

### PCB Schematic

Figure 51: Easy PC PCB Schematic

### PCB Schematic (with Copper Pour)

Figure 52: Easy PC PCB Schematic (Copper Pour)

### PCB Schematic 3D View (Top)

Figure 53: Easy PC PCB 3D View (Top)

### PCB Schematic 3D View (Bottom)

Figure 54: Easy PC PCB 3D View (Bottom)

Figure 55: JLCPCB Website

Figure 56: PCB Build Costs

## The Software

Figure 57: Software Flowchart

The block drawing in Figure 57 shows the complete software layout. A copy of the yEd file is included in the Appendix of this report for ease of view.

Figure 58: The Start of the Software

Figure 58 is the Start of the Software

Figure 59: Data Acquisition Mode

Figure 60: Acquisition Sub-Routines

Figures 59 and Figure 60 is the Weather Station Acquisition Mode, where Data is collected. Fig 59 shows the whole section, where Figure 60 concentrates on the subroutines. This method was decided on as the sensors collect data at different frequencies.

Figure 61: Flashing LED Indicator

The Output Requirement of this task requires the flashing LED indicator light to flash for 1 second every 10 seconds as shown in Figure 61. This part of the program runs simultaneously with the sub routines collecting data.

Figure 62: PC Download Mode

Figure 62 shows the software part that downloads the data from our Non Volatile Memory to the PC.

# Costs and Figures

| Component | Price/Item | Qty Required | Total Cost | Notes |
| --- | --- | --- | --- | --- |
| Hardware Parts |  |  |  |  |
| TE Connectivity - HPP809A031 Temperature & Humidity Sensor | £93.23 | 1 | £93.23 | Temp. & Humid. Sensor |
| Gill - WindSonic 75 Anemometer | £1,443.00 | 1 | £1,443.00 | Wind Speed & Dir. Sensor |
| Sensirion - SEN54-SDN-T | £27.32 | 1 | £27.32 | Air Part & VOC Level Sensor |
| Campbell Scientific - CS320 | £265.00 | 1 | £265.00 | Sunlight Sensor |
| Solar Light - PMA1107 Analog Non-Weighted UVA+B Sensor | £425.00 | 1 | £425.00 | UV Sensor |
| MATEKSYS - M10Q-5883 GNSS | £31.10 | 1 | £31.10 | GPS Pos Sensor |
| PIC18F47J53 - Microcontroller | £4.63 | 1 | £4.63 | PIC Microcontroller |
| RS PRO - Green Panel Mount Indicator | £15.36 | 1 | £15.36 | LED Indicator |
| Everything Smart - CH340 | £5.00 | 1 | £5.00 | USB Module |
| Components 101 - Diligent 410-380 | £6.96 | 1 | £6.96 | SD Card Module |
| Pro Elec - PELR0233 | £0.86 | 1 | £0.86 | 12V Panel Switch |
| Varta - LED240 | £262.29 | 1 | £262.29 | Battery |
| NANUK - 960 Case | £332.43 | 1 | £332.43 | Battery Enclosure |
| Blue Sea Systems - Terminal Fuse Block | £34.99 | 1 | £34.99 | Terminal Fuse Block |
| Blue Sea Systems - Battery Fuse 30A | £22.60 | 1 | £22.60 | Battery Fuse |
| LotFancy - 3M-9MQF-X7LI | £31.67 | 2 | £63.34 | 4 Gauge Battery Wire |
| Ampper - Am-CBS01 | £12.99 | 1 | £12.99 | Battery Cut-Off Switch |
| Huazu - Fuse Block | £19.60 | 1 | £19.60 | Fuse Block |
| Nilight - 12 Volt Socket | £9.99 | 1 | £9.99 | 12 Volt Socket |
| Davis - Tripod | £172.30 | 1 | £172.30 | Tripod |
| Davis - Mounting Pole Kit | £60.00 | 1 | £60.00 | Mounting Pole |
| Evatron - Cabinet Enclosure | £31.13 | 1 | £31.13 | Enclosure |
| Shenzhen Technologies - Pole Bracket | £26.99 | 1 | £26.99 | Pole Bracket |
| Pro – Elec - SP 13 Series (3 Pin, IP68) | £8.28 / pair | 2 | £16.56 | Panel Mount Connectors |
| Pro – Elec - SP 13 Series (4 Pin, IP68) | £8.28 / pair | 4 | £33.12 | Panel Mount Connectors |
|  | Sub Total (Hardware) |  | £3,415.79 |  |

Table 30: Hardware Costs

| Circuitry Parts (from mouser.co.uk) | Price/Item | Qty Required | Total Cost | Notes |
| --- | --- | --- | --- | --- |
| TE Connectivity - Fixed Terminal Block (2 Pin) | £0.78 | 1 | £0.78 | PCB Connectors |
| TE Connectivity - Fixed Terminal Block (3 Pin) | £1.14 | 2 | £2.28 | PCB Connectors |
| TE Connectivity - Fixed Terminal Block (4 Pin) | £1.44 | 4 | £5.76 | PCB Connectors |
| STMICROELECTRONICS L7805CV-DG | £0.74 | 1 | £0.74 | Voltage Regulator (5V) |
| Texas Instruments LM1085IT-3.3/NOPB | £1.59 | 1 | £1.59 | Voltage Regulator (5V) |
| STMicroelectronics 1N6263 | £0.35 | 2 | £0.70 | Schottky Diode |
| Panasonic 20SEP150M+T | £1.39 | 2 | £2.78 | Capacitor - 150μF |
| KEMET ESS105M050AB2AA | £0.17 | 4 | £0.68 | Capacitor - 0.1μF |
| Vishay / BC Components K102J15C0GF53L2 | £0.34 | 10 | £3.40 | Capacitor - 1nF |
| Zener Diodes 5.1 Volt 0.5W 10% | £0.15 | 6 | £0.90 | Zener Diode - 5.1V |
| TE Connectivity / Neohm EP10WSS100RJBB | £1.81 | 6 | £10.86 | Resistor - 100 Ω |
| TE Connectivity / Neohm EP10WSS160RJBB | £0.85 | 1 | £0.85 | Resistor - 160 Ω |
| TE Connectivity / Neohm EP10WSS470RJBB | £1.43 | 1 | £1.43 | Resistor - 470 Ω |
| TE Connectivity / Neohm EP10WSS10KJBB | £1.36 | 3 | £4.08 | Resistor - 10 kΩ |
| TE Connectivity / Holsworthy EP9W15KJ | £0.80 | 2 | £1.60 | Resistor - 15 kΩ |
| PCB Board from JLCBCB.com | 2.54 | 1 | £2.54 | PCB Board |
|  | Sub Total (Circuitry) |  | £40.97 |  |

Table 31: Circuitry Costs

| Miscellaneous Parts | Price/Item | Qty Required | Total Cost | Notes |
| --- | --- | --- | --- | --- |
| SD Memory Card, 2GB - INSD2GV2 | £4.85 | 1 | £4.85 | SD Card |
| Irzaki - IR-EC-A08 | £15.99 | 1 | £15.99 | 12V Cord |
|  | Sub Total (Miscellaneous) |  | £20.84 |  |

Table 32: Miscellaneous Costs

| Sub Total (Hardware) | £3,415.79 |
| --- | --- |
| Sub Total (Circuitry) | £40.97 |
| Sub Total (Miscellaneous) | £20.84 |
| TOTAL PPROTOTYPE COST | £3,477.60 |

Table 33: Total Weather Station Build Costs

Please Note

The above costs do not include engineer’s time.

# Summary

The project has been successful. Knowledge of Weather Stations, Data Acquisition, Microprocessors and Electronic Designs has been gained.

# Conclusions

Did it meet the design specification?

The task requested that we design a simple microcontroller based Weather and Air Quality Monitoring Station. The Weather Station is designed to be fully portable and adaptable for any reasonable site it may be located at. It has enough power and memory storage in order to monitor and take measurements for a 14 day period.

Our Weather Station has been designed in a way that it is capable of obtaining the following data:

- Temperature

- Humidity

- Wind Speed

- Wind Direction

- Air borne particle count

- Sunlight brightness

- UV levels

- GPS position

- VOC level

Given the above, I conclude that the design meets the requirements and is therefore successful.

# Recommendations for Improvements

Even though the system meets the design specification, if I was to replicate this project again, there are more features that could be added to the system in the future. The system could be improved by making component and software changes:

Component Changes:

- Wireless connectivity via 4G/5G technology for real time data transfer

- Replacing sensors for Electronic Micro Sensors. These would lower the cost and power requirement considerably. Also these would reduce the hardware setup. I chose not to use them during this project as it would rewire a lot of modifications to enclosure in order to maintain IP rating of the hardware.

Software Changes:

- The software could include a diagnostic option

- The software could include a program that warns the user via Mobile Device when weather conditions are above certain levels. I.e. Storm Warning

# References

| [1] | CPC, “Weather Meter / Weather Station Kit - SEN-15901,” 2024. [Online]. Available: https://cpc.farnell.com/sparkfun-electronics/sen-15901/weather-meter-kit/dp/MK00988. [Accessed 8 March 2024]. |
| --- | --- |
| [2] | Weather Shop, “Davis Vantage Vue 6250UK - Highly Accurate & Robust, Optional Mobile Phone App/Web/PC Connectivity,” Weather Shop, 2024. [Online]. Available: https://www.weathershop.co.uk/davis-vantage-vue-weather-station. [Accessed 8 March 2024]. |
| [3] | Farm Marketplace Ltd, “Sencrop Weather station - Irrigation stations + Pro plan subsc.,” Extra Digital, 2024. [Online]. Available: https://www.marketplace.farm/grain-storage/testing-and-monitoring/sencrop-irricrop-weather-station/. [Accessed 8 March 2024]. |
| [4] | SB-Components, “BME280 Breakout - Temperature, Pressure, Humidity Sensor,” SB-Components, 2024. [Online]. Available: https://shop.sb-components.co.uk/products/bme280-breakout-temperature-pressure-humidity-sensor. [Accessed 8 March 2024]. |
| [5] | RS Components Ltd, “TE Connectivity Temperature & Humidity Sensor,” RS Components Ltd, 2024. [Online]. Available: https://uk.rs-online.com/web/p/temperature-humidity-sensor-ics/8937373?gb=s. [Accessed 8 March 2024]. |
| [6] | Farnell Limited, “OMEGA HX93BC, TEMPERATURE/HUMIDITY TRANSMITTER, 9-30V,” Premier Farnell Limited, 2024. [Online]. Available: https://uk.farnell.com/omega/hx93bc/temperature-humidity-transmitter/dp/3410282?st=omega%20hx93bc. [Accessed 8 March 2024]. |
| [7] | Kempston Controls, “SCHNEIDER - TM1SHC4 HUMIDITY SENSOR,” Midland Automation Ltd, 2024. [Online]. Available: https://www.kempstoncontrols.com/TM1SHC4/Schneider/sku/1830029. [Accessed 8 March 2024]. |
| [8] | Seeed Technology Co.,Ltd, “Grove - Temperature & Humidity Sensor,” Seeed Technology Co.,Ltd, 2024. [Online]. Available: https://www.seeedstudio.com/Grove-Temperature-Humidity-Sensor-V2-0-DHT20-p-4967.html?queryID=2d79ea30cb5f5094f53e29364264c23c&objectID=4967&indexName=bazaar_retailer_products. [Accessed 8 March 2024]. |
| [9] | Vector Instruments, “Anemometers,” Windspeed Limited, 2024. [Online]. Available: https://www.windspeed.co.uk/ws/index.php?option=news&task=viewarticle&sid=5. [Accessed 8 March 2024]. |
| [10] | Omni instruments, “WindMaster 3D Ultrasonic Anemometer,” Omni instruments, 2018. [Online]. Available: https://www.omniinstruments.co.uk/windmaster-3d-ultrasonic-anemometer.html. [Accessed 8 March 2024]. |
| [11] | Richard Paul Russell Ltd, “Gill WindSonic 75 Anemometer,” Richard Paul Russell Ltd, 2024. [Online]. Available: https://r-p-r.co.uk/product/gill-windsonic-75-anemometer/. [Accessed 8 March 2024]. |
| [12] | Distrelec, “101991044 - SenseCAP S200 Wind Speed and Direction Weather Sensor, Seeed Studio,” Distrelec Schweiz, 2024. [Online]. Available: https://www.distrelec.be/en/sensecap-s200-wind-speed-and-direction-weather-sensor-seeed-studio-101991044/p/30386206. [Accessed 8 March 2024]. |
| [13] | Vector Instruments, “W200P/FC Potentiometer Windvane,” Windspeed Limited, 2024. [Online]. Available: https://www.windspeed.co.uk/ws/index.php?option=displaypage&op=page&Itemid=143. [Accessed 8 March 2024]. |
| [14] | Alpha Controls & Instrumentation Inc, “IsoAir Pro-Plus Remote Particle Counter,” Alpha Controls & Instrumentation Inc, 2024. [Online]. Available: https://www.alphacontrols.com/IsoAir-Pro-Plus-Remote-Particle-Counter/model/7085. [Accessed 8 March 2024]. |
| [15] | RS Components Ltd, “TERA Sensor Next-PM, Particle Matter / Dust Air Quality Sensor,” RS Components Ltd, 2024. [Online]. Available: https://uk.rs-online.com/web/p/environmental-sensor-ics/1953770?searchId=4b0398ca-2525-42ae-ac3c-da5b807a35e0&gb=s. [Accessed 8 March 2024]. |
| [16] | Premier Farnell Limited, “SENSIRION SEN54-SDN-T,” Premier Farnell Limited, 2024. [Online]. Available: https://uk.farnell.com/sensirion/sen54-sdn-t/particle-sensor-dust-digital-5v/dp/3924519. [Accessed 8 March 2024]. |
| [17] | Omni Instruments, “SP Lite2 Silicon Pyranometer,” Domain Design Agency, 2018. [Online]. Available: https://www.omniinstruments.co.uk/weather-stations-and-instruments/pyranometers-solar-irradiance/sp-lite2-silicon-pyranometer.html. [Accessed 8 March 2024]. |
| [18] | Switch Electronics, “Photosensitive LDR Resistor Module,” Switch Electronics, 2024. [Online]. Available: https://www.switchelectronics.co.uk/products/photosensitive-ldr-resistor-module. [Accessed 8 March 2024]. |
| [19] | Campbell Scientific, Inc, “CS320,” Campbell Scientific, Inc, 2024. [Online]. Available: https://www.campbellsci.com/cs320. [Accessed 8 March 2024]. |
| [20] | Adafruit, “Adafruit VEML6075 UVA UVB and UV Index Sensor Breakout,” Adafruit, 2024. [Online]. Available: https://www.adafruit.com/product/3964. [Accessed 8 March 2024]. |
| [21] | Solar Light, “Model PMA1107 Analog Non-Weighted UVA+B Sensor,” Solar Light Company, 2022. [Online]. Available: https://solarlight.com/product/uvauvbsensorpma1107/. [Accessed 8 March 2024]. |
| [22] | BLET-ATMOS, “Precision UV Radiometer,” BLET-ATMOS, 2024. [Online]. Available: https://www.atmos-meteo.com/sensors/precision-uv-radiometer.html. [Accessed 8 March 2024]. |
| [23] | PIMORONI LTD, “PA1010D GPS Breakout,” PIMORONI LTD, 2024. [Online]. Available: https://shop.pimoroni.com/products/pa1010d-gps-breakout?variant=32257258881107. [Accessed 8 March 2024]. |
| [24] | YourFPV, “MATEKSYS M10Q-5883 GNSS & Compass,” YourFPV, 2024. [Online]. Available: https://yourfpv.co.uk/product/mateksys-m10q-5883-gnss-compass/. [Accessed 8 March 2024]. |
| [25] | ESG-JETI-FIALA UK, “MAV Sense GPS sensor and logger,” ESG-JETI-FIALA UK, 2024. [Online]. Available: https://esoaringgadgets.co.uk/mavsence-gps-sensor-and-logger.html. [Accessed 8 March 2024]. |
| [26] | Measurement Systems Ltd, “AQDTF Duct Air Quality VOC Sensor,” Measurement Systems Ltd, 2024. [Online]. Available: https://www.measurementsystems.co.uk/sensors_and_meters/air_quality_and_gas_sensors/aqdtf-duct-air-quality-voc-sensor. [Accessed 8 March 2024]. |
| [27] | Cool Components, “SGP30 Air Quality Sensor Breakout - VOC and eCO2 (ID: 2458),” Cool Components, 2024. [Online]. Available: https://coolcomponents.co.uk/products/sgp30-air-quality-sensor-breakout-voc-and-eco2. [Accessed 8 March 2024]. |
| [28] | Microchip, “Microchip Advanced Part Selector,” Microchip, 21 November 2023. [Online]. Available: https://www.microchip.com/maps/microcontroller.aspx. [Accessed 2023 November 2023]. |
| [29] | Climates to travel, “Climate - Siberia,” Climates to travel, 2024. [Online]. Available: https://www.climatestotravel.com/climate/siberia. [Accessed 9 March 2024]. |
| [30] | Arizona State University, “World: Highest Temperature,” Arizona State University, 2024. [Online]. Available: https://wmo.asu.edu/content/world-highest-temperature. [Accessed 9 March 2024]. |
| [31] | World Population Review, “Most Humid Cities in the World 2024,” World Population Review, 2024. [Online]. Available: https://worldpopulationreview.com/world-city-rankings/most-humid-cities-in-the-world. [Accessed 9 March 2024]. |
| [32] | National Hurricane Centre and Central Pacific Hurricane Centre, “Saffir-Simpson Hurricane Wind Scale,” National Hurricane Centre and Central Pacific Hurricane Centre, 2024. [Online]. Available: https://www.nhc.noaa.gov/aboutsshws.php. [Accessed 9 March 2024]. |
| [33] | Breeze Technologies, “Air pollution – How to convert between mg/m3, µg/m3 and ppm, ppb,” Breeze Technologies, 20 August 2021. [Online]. Available: https://www.breeze-technologies.de/blog/air-pollution-how-to-convert-between-mgm3-%C2%B5gm3-ppm-ppb/. [Accessed 9 March 2024]. |
| [34] | H. Junaidi, “All you need to know about Solar Radiation,” Synergy Files, 2021. [Online]. Available: http://synergyfiles.com/2016/05/solar-radiation/#:~:text=The%20maximum%20recorded%20direct%20solar,recorded%20is%201120%20W%2Fm2.. [Accessed 9 March 2024]. |
| [35] | United States Enviromental Protection Agency, “Learn About the UV Index,” United States Enviromental Protection Agency, 5 February 2024. [Online]. Available: https://www.epa.gov/sunsafety/learn-about-uv-index. [Accessed 9 March 2024]. |
| [36] | E. Gakstatter, “https://www.gpsworld.com/what-exactly-is-gps-nmea-data/,” GPS World, 4 February 2015. [Online]. Available: https://www.gpsworld.com/what-exactly-is-gps-nmea-data/. [Accessed 9 March 2024]. |
| [37] | T. Knudsen, “The Air Quality Index (AQI) Basics & Beyond,” Lane Regional Air Protection Agency, 27 October 2022. [Online]. Available: https://www.lrapa.org/aqi101/. [Accessed 9 March 2024]. |
| [38] | RS Components Ltd, “RS PRO Green Panel Mount Indicator, 1.8 → 3.3V dc, 14mm Mounting Hole Size, Lead Wires Termination, IP67,” RS Components Ltd, 2024. [Online]. Available: https://uk.rs-online.com/web/p/panel-mount-indicators/1301096?gb=s. [Accessed 9 March 2024]. |
| [39] | Everything Smart Technology, “USB-C To UART Serial Adapter - CH340,” Everything Smart Technology, 2024. [Online]. Available: https://shop.everythingsmart.io/products/usb-c-to-serial-adapter. [Accessed 9 March 2024]. |
| [40] | RS Components Ltd, “Development Kit Pmod MicroSD Card Slot for use with Store and Access On System Board,” RS Components Ltd, 2024. [Online]. Available: https://uk.rs-online.com/web/p/memory-development-tools/1840482?gb=s. [Accessed 9 March 2024]. |
| [41] | Premier Farnell Limited, “Panel Mount Rocker Switch, Red, On / Off, SPDT, Green, 250V AC, 6A - PELR0233,” Avnet, 2024. [Online]. Available: https://cpc.farnell.com/unbranded/abrr016/switch-rocker-spst-green-250vac/dp/SW02923?ost=panel+mount+rocker+switch%2C+red%2C+on+%2F+off%2C+spdt%2C+green%2C+250v+ac%2C+6a+-&cfm=true&pm=true. [Accessed 9 March 2024]. |
| [42] | Powerland, “VARTA LED240 PROFESSIONAL DUAL PURPOSE EFB LEISURE BATTERY 240AH (LFD230) (930240120),” Shopify, 2024. [Online]. Available: https://www.powerland.co.uk/collections/marine-batteries/products/varta-led240-professional-dual-purpose-efb-leisure-battery-240ah-lfd230-930240120-1. [Accessed 9 March 2024]. |
| [43] | Roamer Batteries, “24-230SMART3,” Roamer Batteries, 2024. [Online]. Available: https://roamer.com/products/24-230smart?_pos=1&_sid=a28ebb3eb&_ss=r. [Accessed 9 March 2024]. |
| [44] | Alpha Batteries Limited, “12V 250AH Expedition Plus Lead Carbon Gel Ultra Deep Cycle Battery (EXP12-250C),” Alpha Batteries Limited, 2024. [Online]. Available: https://www.alpha-batteries.co.uk/12v-250ah-expedition-plus-lead-carbon-gel-ultra-deep-cycle-battery-exp12-250c/. [Accessed 9 March 2024]. |
| [45] | Cases UK, “NANUK 960 Case,” SAM Cases Ltd, 2024. [Online]. Available: https://www.casesuk.com/nanuk-cases/nanuk-960-case.html. [Accessed 9 March 2024]. |
| [46] | MetCheck, “MetSpec M14 Mini Polycarbonate Stevenson Screen,” MetCheck, 2024. [Online]. Available: https://www.metcheck.co.uk/products/metspec-m14-mini-polycarbonate-stevenson-screen. [Accessed 9 March 2024]. |
| [47] | Protective Cases, “SKB 3I-2424-10B-C Case With Cubed Foam,” Protective Cases, 2024. [Online]. Available: https://www.protective-cases.co.uk/product/skb-3i-2424-10b-c-case-with-cubed-foam_3901. [Accessed 9 March 2024]. |
| [48] | Amazon.com, “Blue Sea Systems MRBF Terminal Fuse Blocks,” Amazon.com, 2024. [Online]. Available: https://www.amazon.co.uk/Blue-Systems-MRBF-Terminal-Blocks/dp/B0019ZBTV4?th=1. [Accessed 2024 March 2024]. |
| [49] | Amazon.com, “Blue Sea Systems Terminal Fuses,” Amazon.com, 2024. [Online]. Available: https://www.amazon.co.uk/Blue-Sea-Systems-5187-Terminal/dp/B00198FP8Y/ref=pd_lpo_sccl_2/258-6860703-1721369?pd_rd_w=Xgbyx&content-id=amzn1.sym.448aab69-6fff-42b9-9d7a-f6325052a6f6&pf_rd_p=448aab69-6fff-42b9-9d7a-f6325052a6f6&pf_rd_r=7K7B16KG6RGYMPBVM0SZ&p. [Accessed 9 March 2024]. |
| [50] | Amazon.com, “LotFancy 4 AWG Battery Cables 5/16" in Lugs, 24 Inch Power Inverter Cables with Terminals for Car Marine Solar ATV Lawn Mower Motorcycle, (Positive and Negative),” Amazon.com, 2024. [Online]. Available: https://www.amazon.co.uk/LotFancy-Inverter-Terminals-Motorcycle-Positive/dp/B07YD4V5ZW?th=1. [Accessed 9 March 2024]. |
| [51] | Amazon.com, “Ampper Battery Switch, 12-48 V Battery Power Cut Master Switch Disconnect Isolator for Car, Vehicle, RV and Boat (On/Off),” Amazon.com, 2024. [Online]. Available: https://www.amazon.co.uk/Ampper-Battery-Switches-12-Disconnect/dp/B07413JWLD/ref=asc_df_B07413JWLD/?tag=googshopuk-21&linkCode=df0&hvadid=534929732553&hvpos=&hvnetw=g&hvrand=6170642811496463557&hvpone=&hvptwo=&hvqmt=&hvdev=c&hvdvcmdl=&hvlocint=&hvlocphy=1. [Accessed 9 March 2024]. |
| [52] | Amazon.com, “Fuse Block, Huazu 12V Fuse Box with LED Indicator Waterproof Cover 6 Circuits with Negative Bus Fuse Panel for DC 12V/24V Car Marine Boat RV Trailer Truck Van Vehicle,” Amazon.com, 2024. [Online]. Available: https://www.amazon.co.uk/Huazu-Indicator-Waterproof-Circuits-Negative/dp/B09VGCCD9R. [Accessed 9 March 2024]. |
| [53] | Amazon.com, “Nilight 2 Pack Car Cigarette Lighter Socket DC 12V Waterproof Power Outlet Adapter Replacement with Terminals Wires and Screws for Marine Boat Motorcycle, Boat, Car,Truck, RV, ATV,2 Years Warranty,” Amazon.com, 2024. [Online]. Available: https://www.amazon.co.uk/Nilight-50054B-Waterproof-Replacement-Motorcycle/dp/B082D8WKNW. [Accessed 9 March 2024]. |
| [54] | Amazon.com, “26FT 12V-24V Car Cigarette Lighter Extension Cord Heavy Duty 15A Fuse Extension Cable Plug to Socket for Air Compressor Pump 16AWG Male to Female Socket,” Amazon.com, 2024. [Online]. Available: https://www.amazon.co.uk/12V-24V-Cigarette-Lighter-Extension-Compressor/dp/B0C61F1V98/ref=sr_1_2?dib=eyJ2IjoiMSJ9.rstfy8PKCBeMBzT_c0O65WcLX5YSuOp1mDKckyKNgJcZl0wdTyhz4LarD1MHH33svmiRhjFMPJXCZ17irrrnSaiQTTKAJCmN0aANmBVqcxRMiO7LDrI6TJP4BO3vWUK4LdhWibKJAtgZ7. [Accessed 9 March 2024]. |
| [55] | Weather Shop, “Davis EZ-Mount Tripod with Lag Bolts (for in-ground mounting),” Weather Shop, 2024. [Online]. Available: https://www.weathershop.co.uk/davis-7716a-tripod-with-lag-bolts. [Accessed 9 March 2024]. |
| [56] | Weather Shop, “Davis Mounting Pole Kit,” Weather Shop, 2024. [Online]. Available: https://www.weathershop.co.uk/accessories/spares-by-brands/davis-spare-parts-accessories/davis-7717-mounting-pole-kit. [Accessed 9 March 2024]. |
| [57] | Evatron Plastic Enclosures Ltd, “DED001 – Cabinet Enclosure,” Evatron Plastic Enclosures Ltd, 2024. [Online]. Available: https://evatron.com/enclosures/ip-rated-enclosures/ip65-enclosures/ded001-cabinet-enclosure/?gad_source=1&gclid=CjwKCAiAi6uvBhADEiwAWiyRdomocI_G3FUUfu9i4-Q8F79cuZOwCXz05qOv8EK7T8NCcOKpN5iXdxoCT_IQAvD_BwE. [Accessed 9 March 2024]. |
| [58] | Amazon.com, “WiTi 2PACK Vertical Pole Mount for Security Camera, Stainless Steel Mounting Corner Bracket with 3 Loop Pipe Clamps, Suitable for CCTV Cameras LED Wall/Stage Lights, Solar Panels (small size),” Amazon.com, 2024. [Online]. Available: https://www.amazon.co.uk/dp/B0C3LPVFVF/ref=sspa_dk_detail_3?psc=1&pd_rd_i=B0C3LPVFVF&pd_rd_w=x25Go&content-id=amzn1.sym.84ea1bf1-65a8-4363-b8f5-f0df58cbb686&pf_rd_p=84ea1bf1-65a8-4363-b8f5-f0df58cbb686&pf_rd_r=405NP0TXF0MQM1YV0375&pd_rd_wg=Dw7ui&pd_rd_r=a. [Accessed 9 March 2024]. |
| [59] | Component Buddy, “SP2912/S Series,” Component Buddy, 2024. [Online]. Available: https://componentbuddy.com/product/sp29-panel-mount-connectors-female/. [Accessed 9 March 2024]. |
| [60] | Premier Farnell Limited, “XLR Socket, 3 Pole, PCB, Vertical - NC3FAAV2,” Avnet, 2024. [Online]. Available: https://cpc.farnell.com/neutrik/nc3faav2/xlr-socket-3p-pcb-vert/dp/AV15739?st=3%20pole%20xlr%20socket. [Accessed 9 March 2024]. |
| [61] | Premier Farnell Limited, “Circular Threaded Panel Mount Connector Socket, 3-Pole, IP68 - PELR0192,” Avnet, 2024. [Online]. Available: https://cpc.farnell.com/mbs-connect/sp2112-s3-1c/circular-panel-socket-3-pole/dp/CN21385?st=pro%20elec%20sp%2013%20series. [Accessed 9 March 2024]. |
| [62] | reichelt elektronik GmbH, “PS 25/10W BR PCB connector angled, brown, 10-pin,” reichelt elektronik GmbH, 2024. [Online]. Available: https://www.reichelt.com/at/en/pcb-connector-angled-brown-10-pin-ps-25-10w-br-p47229.html?&trstct=pos_2&nbc=1. [Accessed 9 March 2024]. |
| [63] | Premier Farnell Limited, “MOLEX 26-60-4043,” Avnet, 2024. [Online]. Available: https://uk.farnell.com/molex/26-60-4043/connector-header-4pos-1row-3-96mm/dp/3049003?st=molex%20pin%20header. [Accessed 9 March 2024]. |
| [64] | Mouser Electronics, “282851-4,” Mouser Electronics, 2024. [Online]. Available: https://www.mouser.co.uk/ProductDetail/TE-Connectivity/282851-4?qs=qTrdGJE%2F%252BQz%252BSpIukU4Srg%3D%3D. [Accessed 9 March 2024]. |

# Appendix

### Appendix 1 – yEd Software File

### Appendix 2 – Links to all Components

| Component | Link |
| --- | --- |
| TE Connectivity - HPP809A031 Temperature & Humidity Sensor | https://uk.rs-online.com/web/p/temperature-humidity-sensor-ics/8937373?gb=s |
| Gill - WindSonic 75 Anemometer | https://r-p-r.co.uk/product/gill-windsonic-75-anemometer/ |
| Sensirion - SEN54-SDN-T | https://uk.farnell.com/sensirion/sen54-sdn-t/particle-sensor-dust-digital-5v/dp/3924519 |
| Campbell Scientific - CS320 | https://www.campbellsci.com/cs320 |
| Solar Light - PMA1107 Analog Non-Weighted UVA+B Sensor | https://solarlight.com/product/uvauvbsensorpma1107/ |
| MATEKSYS - M10Q-5883 GNSS | www.yourfpv.co.uk/product/mateksys-m10q-5883-gnss-compass/ |
| PIC18F47J53 - Microcontroller | https://www.mouser.co.uk/ProductDetail/Microchip-Technology/PIC18F47J53-I-PT?qs=jecpP1bYcZSkJi3F6cR9zw%3D%3D |
| RS PRO - Green Panel Mount Indicator | https://uk.rs-online.com/web/p/panel-mount-indicators/1301096?gb=s |
| Everything Smart - CH340 | https://shop.everythingsmart.io/products/usb-c-to-serial-adapter |
| Components 101 - Diligent 410-380 | https://uk.rs-online.com/web/p/memory-development-tools/1840482?gb=s |
| Pro Elec - PELR0233 | https://cpc.farnell.com/unbranded/abrr016/switch-rocker-spst-green-250vac/dp/SW02923?ost=pelr0233 |
| Varta - LED240 | https://www.powerland.co.uk/collections/marine-batteries/products/varta-led240-professional-dual-purpose-efb-leisure-battery-240ah-lfd230-930240120-1 |
| NANUK - 960 Case | https://www.casesuk.com/nanuk-cases/nanuk-960-case.html |
| Blue Sea Systems - Terminal Fuse Block | https://www.amazon.co.uk/AUTOHAUX-250A-Battery-Terminal-Block/dp/B0BBFQ3W2H/ref=sr_1_58?crid=D9WZ26PGSLNP&dib=eyJ2IjoiMSJ9.1MBpac3Cke9Ap_LjhoZlJWCIpeMRMzHqmmRj84rFpks9fzKdHKemySwUDVCBQb0JnbxvD5mvtHE5hFMqTH5IS_-AAIRRS93tlO2xKG0kfey41DqvWxiMs2HcU05ftPqi5bDoR6StdKcv3iGSM1puOJdWkFtKwTcr-FXucuxOXv-RwvMUo15CRdRMPv_hJeAHBOiIAoLh3zPd3g5_ouPW8n-kiJvGgp1HFUdhaG89mAmw8weJwDbK1M8xZIKZBcF1TROtd2tQRT5hRd-5oZdj8VNcZ3ID8d2063MXRaDDpMs.7B94T3gD0FEq3D0EQHj9vdwFeEsp1JCtOeuCA55wLoc&dib_tag=se&keywords=Blue%2BSea%2BSystems%2Bterminal%2Bfuse%2Bblock&qid=1710674734&sprefix=blue%2Bsea%2Bsystems%2Bterminal%2Bfuse%2Bblock%2Caps%2C99&sr=8-58&th=1 |
| Blue Sea Systems - Battery Fuse 30A | https://www.amazon.co.uk/BLUE-SEA-5186-FUSE-TERMINAL/dp/B00198FP8Y/ref=sr_1_7?crid=1IY1QCBC1QAYM&dib=eyJ2IjoiMSJ9.1MBpac3Cke9Ap_LjhoZlJWCIpeMRMzHqmmRj84rFpks9fzKdHKemySwUDVCBQb0JnbxvD5mvtHE5hFMqTH5IS_-AAIRRS93tlO2xKG0kfey41DqvWxiMs2HcU05ftPqi5bDoR6StdKcv3iGSM1puOJdWkFtKwTcr-FXucuxOXv-RwvMUo15CRdRMPv_hJeAHBOiIAoLh3zPd3g5_ouPW8n-kiJvGgp1HFUdhaG89mAmw8weJwDbK1M8xZIKZBcF1TROtd2tQRT5hRd-5oZdj8VNcZ3ID8d2063MXRaDDpMs.7B94T3gD0FEq3D0EQHj9vdwFeEsp1JCtOeuCA55wLoc&dib_tag=se&keywords=Blue%2BSea%2BSystems%2BTerminal%2BFuse%2BBlock&qid=1710674642&sprefix=blue%2Bsea%2Bsystems%2Bterminal%2Bfuse%2Bblock%2Caps%2C103&sr=8-7&th=1 |
| LotFancy - 3M-9MQF-X7LI | https://www.amazon.co.uk/LotFancy-Inverter-Terminals-Motorcycle-Positive/dp/B07YD4V5ZW/?_encoding=UTF8&pd_rd_w=sfo2Z&content-id=amzn1.sym.f4bf6248-c25e-4846-ab32-07f8b3e1cc4d%3Aamzn1.symc.afd86303-4a72-4e34-8f6b-19828329e602&pf_rd_p=f4bf6248-c25e-4846-ab32-07f8b3e1cc4d&pf_rd_r=PRP1H6BWHTB3G1XPP60M&pd_rd_wg=z5VId&pd_rd_r=538c41a5-247e-4c2f-9f1c-58bb874acba2&ref_=pd_gw_ci_mcx_mr_hp_atf_m&th=1 |
| Ampper - Am-CBS01 | https://www.amazon.co.uk/Ampper-Battery-Switches-12-Disconnect/dp/B07413JWLD/ref=sr_1_12_sspa?crid=3K5F1TUOT9R4&dib=eyJ2IjoiMSJ9.LJvkofHxt-xYxu7II-egx5yi0hyOfkOCNCaW0WA0oLK2vsfNYjr3rSgNg9XZMzPI1m0vASR3nfEeL6NL7mX1y0P4cexT38g76tkO6ZtUOEGg0Ru0Q9dOZ28jWtQX19tvFsFz7M2YlKsVK1tAUkpYN_0oOrcm1LK1W99D6zum6fdYX6n9DY9oZEo26ue6sv9eh6YJ7Iox_gOPeXGe4m7mnwYSXAkx2NIdkkhH9NwO5EaLs30DUUqtikCDDLjRuaWRAlbdFn_qRuch_bd-CCbUGccjNDsJbG7RlZCTGcY47XA.rPn8R3yIjjXptrp7sNMT_wPP99XXWRW5egZIdU7-7MI&dib_tag=se&keywords=Blue+Sea+Systems&qid=1710674487&sprefix=blue+sea+systems%2Caps%2C172&sr=8-12-spons&sp_csd=d2lkZ2V0TmFtZT1zcF9tdGY&psc=1 |
| Huazu - Fuse Block | https://www.amazon.co.uk/KAOLALI-Negative-Indicator-Damp-Proof-Automotive/dp/B08HYD3JL7/ref=sr_1_12?crid=J2CILERSMNKH&dib=eyJ2IjoiMSJ9.5zXkeKAIJiHBSEQwQMfX3GBlxjQJ9RxZ-omImw-kggM1XgkU6SCr7IdXZz7XtlnQsZOPrOO-JPAYfrExIEM8JTCbBUVyid-Pnhd7LHlvsXQq2Va0pFVsMSjfhJ2sC8JktXpMNUfAUiFiGA58Dyy_FqV4MYtFe2mJiFguFzsIIZnMOVeu96_6nxt8A3Is6tOiZ4g7kSqKkhlNdlbvLPK09fdKBLnMft1fyP0Q_7o7OVI3D62jtaQv_CL-TfaKwxmEHDYYL3fLMbXJK9_frOVTmcbL1kh7JjmMrIRnVuGBIzg.e4VLZ3Iv-7ZXjSilCs3aI2qxOPLPXq-3pKx2F12ewWQ&dib_tag=se&keywords=Huazu%2B-%2BFuse%2BBlock&qid=1710674828&s=automotive&sprefix=huazu%2B-%2Bfuse%2Bblock%2Cautomotive%2C90&sr=1-12&th=1 |
| Nilight - 12 Volt Socket | https://www.amazon.co.uk/Digit-Tail-Cigarette-Lighter-Socket-Waterproof/dp/B08H1465P1/ref=sr_1_9?crid=9L7Y9N3MX9MI&dib=eyJ2IjoiMSJ9.Nl7TY-w0qYnubxJwziPulaOsJffIY-xkeFrsJCE1jepm8jiZPXWsy_LvUfnsUMPmp2znMvZQXP-zA2AQFuzijoZz_lunPfEnP0-rVLDsXylKcbZP0V0oATfRkUztSpJrceDRCAIGazHHP4gFM21RdipSQnd5HMezovY_WL1MjRkhSQ-S3Mi8qAkh0EJpLaC4sTHfptoxFZ3iVnW6K-QKDJfjWO71xlNnkqcUY-F8ll5Rh65bJBFUU-dWn8HbF1sO8ohn86b5-ZotdCnbWjKNVNmidXSZbE6xAgahE1H0Q-U.hI7rcTgkOggwr3aolqROj6ehovUuaCnfX2jiuMP7VDg&dib_tag=se&keywords=12v+socket&qid=1710674980&s=automotive&sprefix=12v+socket%2Cautomotive%2C92&sr=1-9 |
| Davis - Tripod | https://www.weathershop.co.uk/davis-7716a-tripod-with-lag-bolts |
| Davis - Mounting Pole Kit | https://www.weathershop.co.uk/accessories/spares-by-brands/davis-spare-parts-accessories/davis-7717-mounting-pole-kit |
| Evatron - Cabinet Enclosure | https://evatron.com/enclosures/ip-rated-enclosures/ip65-enclosures/ded001-cabinet-enclosure/ |
| Shenzhen Technologies - Pole Bracket | https://www.amazon.co.uk/WiTi-Vertical-Security-Stainless-Mounting/dp/B0C3LPVFVF/ref=sr_1_9?crid=2LH5CFE2JEUVV&dib=eyJ2IjoiMSJ9.Pavu3A_nO1sFsbl5f_g3iqj0t3ggIbN_b-ibOLzFbVBPENKl5_9xWvj89DV8UMIs9qNLwjkx1htbTLl6BeEVycdaHtnr7M9kTyF5pI-kdMtlJsQhZgcuD2ZK-qy7SjWId4ujVW1H4s2W8raXvClhQPri_d5iJ3IpFG31a0ouASDWivFTcH8j9XcYNXUgSDr4-YjlXG3A8-meCsd69eYwQmvjrPqkglMwCWT40Qw2j8c.N-rlPc8ezz_EC18lFychZrrWNqxyDp0XpmbM-BVyUG4&dib_tag=se&keywords=SHENZHEN+TECHNOLOGIES+Pole+Bracket&qid=1710675413&sprefix=shenzhen+technologies+pole+bracket%2Caps%2C89&sr=8-9 |
| Pro – Elec - SP 13 Series (3 Pin, IP68) | https://cpc.farnell.com/mbs-connect/sp2112-s3-1c/circular-panel-socket-3-pole/dp/CN21385?st=pro%20elec%20sp%2013%20series |
| Pro – Elec - SP 13 Series (4 Pin, IP68) | https://cpc.farnell.com/mbs-connect/sp2112-s4-1c/circular-panel-socket-4-pole/dp/CN21386 |
| Circuitry Parts |  |
| TE Connectivity - Fixed Terminal Block (2 Pin) | https://www.mouser.co.uk/ProductDetail/TE-Connectivity/282851-2?qs=qTrdGJE%2F%252BQwtK31S4gQKCQ%3D%3D |
| TE Connectivity - Fixed Terminal Block (3 Pin) | https://www.mouser.co.uk/ProductDetail/TE-Connectivity/282851-3?qs=qTrdGJE%2F%252BQygMkRDYDA4Uw%3D%3D |
| TE Connectivity - Fixed Terminal Block (4 Pin) | https://www.mouser.co.uk/ProductDetail/TE-Connectivity/282851-4?qs=qTrdGJE%2F%252BQz%252BSpIukU4Srg%3D%3D |
| STMICROELECTRONICS L7805CV-DG | https://www.mouser.co.uk/ProductDetail/STMicroelectronics/L7805CV-DG?qs=T%2FOtf55vL7c3O0as9P9fAw%3D%3D |
| Texas Instruments LM1085IT-3.3/NOPB | https://www.mouser.co.uk/ProductDetail/Texas-Instruments/LM1085IT-3.3-NOPB?qs=X1J7HmVL2ZHhXlc9GKCtqg%3D%3D |
| STMicroelectronics 1N6263 | https://www.mouser.co.uk/ProductDetail/STMicroelectronics/1N6263?qs=JV7lzlMm3yLnNWyC0f3Ojg%3D%3D |
| Panasonic 20SEP150M+T | https://www.mouser.co.uk/ProductDetail/Panasonic/20SEP150M%2bT?qs=OE1iw1LrrPERx7mXD5ZOgg%3D%3D |
| KEMET ESS105M050AB2AA | https://www.mouser.co.uk/ProductDetail/KEMET/ESS105M050AB2AA?qs=gt1LBUVyoHk47Kt2aLnHUQ%3D%3D |
| Vishay / BC Components K102J15C0GF53L2 | https://www.mouser.co.uk/ProductDetail/Vishay-BC-Components/K102J15C0GF53L2?qs=Mig2RDu1Bfp2x8pA52uzng%3D%3D |
| Zener Diodes 5.1 Volt 0.5W 10% | https://www.mouser.co.uk/ProductDetail/Vishay-Semiconductors/TZX5V1A-TR?qs=8Acm1RElxHJj95m7pOkYpg%3D%3D |
| TE Connectivity / Neohm EP10WSS100RJBB | https://www.mouser.co.uk/ProductDetail/TE-Connectivity-Neohm/EP10WSS100RJBB?qs=1Kr7Jg1SGW%2FT7P4Od60McQ%3D%3D |
| TE Connectivity / Neohm EP10WSS160RJBB | https://www.mouser.co.uk/ProductDetail/TE-Connectivity-Neohm/EP10WSS160RJBB?qs=HoCaDK9Nz5dOoiL4b18keA%3D%3D |
| TE Connectivity / Neohm EP10WSS470RJBB | https://www.mouser.co.uk/ProductDetail/TE-Connectivity-Neohm/EP10WSS470RJBB?qs=HoCaDK9Nz5f4Acc3ieTFZA%3D%3D |
| TE Connectivity / Neohm EP10WSS10KJBB | https://www.mouser.co.uk/ProductDetail/TE-Connectivity-Neohm/EP10WSS10KJBB?qs=HoCaDK9Nz5dV2DYUSjFZAQ%3D%3D |
| TE Connectivity / Holsworthy EP9W15KJ | https://www.mouser.co.uk/ProductDetail/TE-Connectivity-Holsworthy/EP9W15KJ?qs=gt1LBUVyoHkslqI78SgZVw%3D%3D |
| Miscellanious Parts |  |
| SD Memory Card, 2GB - INSD2GV2 | https://www.farnell.com/datasheets/3110845.pdf |
| Irzaki - IR-EC-A08 | https://www.amazon.co.uk/12V-24V-Cigarette-Lighter-Extension-Compressor/dp/B0C61FVSDD/ref=sr_1_1?crid=3REPTH7XH8SCM&dib=eyJ2IjoiMSJ9.Cib41AI_EeXKGxaPYX48enpV95lLLXokjbojbEpcNYFJRW_KxQHHT2yR8t7QnKL2zGXUgytWs3jzbPS6fgzZ9Q.E0VQl_oy2eTDD0Xr4sakRGcsgyJrqLU4i3blKnm0bHk&dib_tag=se&keywords=Irzaki%2B12v&qid=1710675037&sprefix=irzaki%2B12v%2B%2Caps%2C117&sr=8-1&th=1 |