Evaluation of Accelerated Rate Calorimetry (ARC) for Assessment of Explosives Compatibility

Research Proposal

Jordan Hayston

Contents

Figures	3

Tables	3

1. Introduction	4

2. Background to the Proposed Research	5

3. Aims & Objectives	6

4. Experimental Design	7

4.1 Outline of Experimental Procedure	7

1.2	Data Collection	7

4.3 Data Evaluation	7

1.3	Risks & Limitations	7

1.4	Ethical Issues	8

6. Resources & Planning	9

6.1 Resources Required	9

6.1.1 Physical Resources	9

6.1.2 Human Resources	9

6.1.3 Other Resources	9

6.2 Gantt Chart	10

7. References	11

# Figures

Figure 1 - temperature-time graph of a typical ARC experiment [1]	4

# Tables

Table 1 – Existing data of using ARC for testing of explosives compatibility [3]	5

# 1. Introduction

The purpose of this project is to determine whether the application of Accelerated Rate Calorimetry (ARC) is a suitable method of assessment for the compatibility of explosives with other substances. If successful, this could improve operator safety by taking the focus away from Vacuum Stability Testing (VST), which involves mixing and heating chemically adulterated explosives. The results from ARC will be analysed and compared against other recognised methods, Vacuum Stability Testing and Differential Scanning Calorimetry (DSC). ARC directly measures temperature change whilst VST relies on the volume of gas evolved, which is a secondary effect of the reaction.

In the 1970s, the Dow Chemical Company created the Accelerating Rate Calorimeter (ARC), which was then commercialised in 1980 following an explosion at Dow UK. The technology was developed to simulate exothermic runaway reactions from hazardous and reactive chemicals at safe laboratory scale. [1] The process consists of a heating chamber, in which a ‘bomb’ is situated; The ‘bomb’ contains the test material. The chamber temperature increases in defined increments, with a ‘sit time’ between each step. The chamber will also detect and match the temperature of the sample, which indicates if it is self-heating or reacting. The process will continue until a clear thermal runaway is registered, an example of which is demonstrated in Figure 1. [1]

Figure 1 - temperature-time graph of a typical ARC experiment [1]

The Vacuum Stability Test (VST) is used to measure and monitor changes to the thermal stability of explosives formulations. The VST is also the main method for carrying out explosives compatibility tests. It uses pressure transducers to measure gas pressure, which is recorded on data logger and converted to gas volume.

Differential Scanning Calorimetry is a technique used to measure the difference in the amount of heat required to increase the temperature of a sample. This is a secondary method of testing used by the company.

# 2. Background to the Proposed Research

Assessment of Explosives Chemical Compatibility is carried out to ensure that substances or materials that come into intimate contact with explosives do not cause any detrimental effect. There are several methods available for assessing compatibility as outlined in NATO STANAG 4147 [2], however each test is associated with its own limitations.

Routine assessment is carried out using the Vacuum Stability Test, which uses large samples (5g) heated to over 100°C for several hours. Compatibility is considered based on the quantity of gas evolved from a mixture of an explosive and adulterant material which has been intimately mixed. As such, liquids with low boiling points cannot be evaluated using this test. The test is carried out in standard laboratory glassware without any blast mitigation as the intention is that the experiments do not run to rapid decomposition. This in itself carries a risk to the operator.

Other smaller scall tests such as Differential Scanning Calorimetry (DSC) and Thermogravimetric Analysis (TGA) expect the experiment to run to decomposition, however they only operate on a 1-2mg scale. At this scale proper mixing of the samples and representation of bulk interactions are called into question.

This project poses to evaluate Accelerated Rate Calorimetry (ARC) which allows for up to 300mg of explosives to be mixed with an adulterant material in a purpose made “bomb”. The techniques has the potential to assess both liquids and solids and may run to decomposition at no risk to the operator.

A brief study of the same nature was carried out in 2015 which showed the possibility for further research in the subject matter. The study presented three sets of data taken from tests carried out using ARC. Figure 1 shows that the thermal runaway of the lone explosives took place at 135°C and 137°C in the first two experiments. The third experiment demonstrates that the addition of a second material to the process lowers the temperature of thermal runaway to 107°C, suggesting that the two may have reacted with one another. [3]

Table 1 – Existing data of using ARC for testing of explosives compatibility [3]

# 3. Aims & Objectives

The aim of this project is to evaluate the application of ARC to Explosives Compatibility Testing. Several key factors are important when establishing a new technique, these are:

- Ease of sample preparation

- Reproducibility of results

- Determination of a suitable “pass/Fail” criteria

- Data obtained is consistent with assessment by other techniques

The first objective for this project is to carry out each test twice at a minimum (18 in total) using ARC, within 8 weeks of practical time. This should ensure the results are somewhat reproducible if there are no setbacks with the experiments. Carrying out this volume of tests in the given timescale should be an easily achievable goal; if they are completed with enough time left, a third and possibly fourth round of tests will be carried out.

Due to the limited capability of the facility (only one Accelerating Rate Calorimeter and long test time), a second objective is to repeat each test using the Vacuum Stability Test and Differential Scanning Calorimetry whilst the ARC is operating. These testing methods are recognised in STANAG 4147 and therefore have existing pass/fail criteria. Therefore, any results produced using these techniques can be used to back up the outcome of the ARC experiments and further allow for cross analysis.

The third objective is to evaluate the ARC results and determine a pass/fail criteria. This can be aided by comparing them to results obtained from the other testing methods.

# 4. Experimental Design

## 4.1 Outline of Experimental Procedure

The procedure for this experiment will be to test the compatibility of a HMX-based explosive, with four other substances using Accelerated Rate Calorimetry. Two of the substances used will be known to react with the explosives and should therefore initiate thermal runaway at a lower temperature than the lone explosives. The explosives and each additional material will be tested twice on their own using ARC; this will produce a good baseline for initial analysis. The explosives will then be tested for compatibility with each material twice. These tests will involve the gradual heating of a ‘bomb’ filled with test material, which will increase in temperature by 5°C every 15 minutes.

Due to the capability of the facility only allowing one ARC test to be carried out at a time, other STANAG testing methods will be carried out simultaneously to allow for further analysis and comparison.  Using the same method, the explosives and adulterant materials will be tested using Vacuum Stability Testing (VST) and Differential Scanning Calorimetry (DSC), with each test being carried out twice in order to provide confidence in the results.

## Data Collection

The equipment used for each method of testing has the capability of collecting and storing the data that is produced using specialist software. The data can be extracted into excel format and can also be displayed graphically to allow for fast and easy interpretation/analysis. The ARC technology records temperature change over time, whilst VST records pressure and DSC records the temperature differential.

## 4.3 Data Evaluation

In order to evaluate the data produced by the ARC testing, it will be best to extract it to Microsoft Excel and visualise it in graph format. This will allow for ease of identifying the temperature at which each test begins to run away.

The same method will be useful for the other STANAG compatibility tests; pass/fail criteria already exist for these tests, and so it will be useful to evaluate and analyse any similar patterns between them and the ARC results.

## Risks & Limitations

Due to the work being carried out in a laboratory, there are going to be some generic lab-based risks as well as risks specific to the tasks being carried out. Some of the generic hazards could be slips and trips, chemical hazards, or electrical hazards. The more specific hazards will be associated with the equipment and materials used; for example, there is risk of burning when carrying out the vacuum stability tests as they use heating baths which reach hot temperatures. Another risk would be accidental detonation or deflagration of the explosives during handling.

The first limitation I encountered would be the limited access to the labs and equipment; the work would need to be planned in advance to ensure that both are available. In addition, training will need to be carried out before the work can be carried out.

Another limitation would be the timescale of the project. The study will have to be small and focused on one explosive (a HMX based explosive with energetic binder system). The ARC equipment has the ability to collect the gas from reactions, which could be looked at in any future research. Further to this, the temperature and time increments of the ARC could be altered to observe any deviations in the results.

## Ethical Issues

There are no significant ethical issues associated with this project, disregarding the generic ones related to carrying out explosives research for a MoD arms-length body.

The main ethical issue associated with this project regards the disposal methods of explosives waste. Suitable disposal routes are in place and will be overseen by the Work Supervisory Officer (WSO).

Aside from this, no other issues have been noted so far. Please see the attached ethical form at the end of this report.

# 6. Resources & Planning

## 6.1 Resources Required

### 6.1.1 Physical Resources

With regards to physical resources, the main item that will be required is the explosives. The process of acquiring these should be rather quick as the laboratory will have surplus from other research and development activities. Furthermore, due to the required amounts, the movement process should also be fast. The second physical resource that will be required is the labs and equipment for carrying out the tests. These will require booking in advance and the tests will need to be scheduled into the work plan. The last physical resource needed is the PPE (Personal Protective Equipment). PPE is readily available for use in the labs.

### 6.1.2 Human Resources

The first human resource required for these set of tests will be a Trials Conducting Officer (TCO). A TCO is responsible for the tests themselves (ARC, VST, DSC) and is considered similar to an explosives firing officer. A subject matter expert is also required but this is commonly the same person at the TCO. Finally, a Work Supervisory Officer (WSO) is needed as this is a company requirement. Company policy states that a WSO is required to supervise every process.

### 6.1.3 Other Resources

One other resource that will be required for the project will be a lab journal. This will be used to take logs and notes of each experiment as well as the results.

## 6.2 Gantt Chart

# 7. References

| [1] | Thermal Hazard Technology, “ARC® Accelerating Rate Calorimeter,” [Online]. Available: https://www.thermalhazardtechnology.com/contentfiles/downloads/38.pdf. [Accessed 12th May 2022]. |
| --- | --- |
| [2] | NATO, STANAG 4147 - Chemical Compatibility of Ammunition Components with Explosives (Non-Nuclear Applications), 2001. |
| [3] | R. Tunnel and A. MacDonald, “The use of Accelerating Rate Calorimetry for Chemical Compatibility Assessments of Explosives Discussion Session,” 2015. |

APPLICATION FOR ETHICAL APPROVAL

In order for research to result in benefit and minimise risk of harm, it must be conducted ethically.

The University follows the OECD Frascati manual definition of research activity: “creative work undertaken on a systematic basis in order to increase the stock of knowledge, including knowledge of man, culture and society, and the use of this stock of knowledge to devise new applications”. As such this covers activities undertaken by members of staff, postgraduate research students, and both taught postgraduate and undergraduate students working on dissertations/projects.

The individual undertaking the research activity is known as the “principal researcher”.

This form must be completed and approved prior to undertaking any research activity.

SECTION A: About You (Principal Researcher)

| 1 | Full Name: | Jordan Hayston |
| --- | --- | --- |
| 2 | Student Number: | 1910612 |
| 3 | Email address: | 1910612@student.uwtsd.ac.uk |
| 4 | Programme of Study: | Ordnance, Munitions and Explosives (OME) |
| 5 | Director of Studies/Supervisor: | David Holley |

SECTION B:  Internal and External Ethical Guidance Materials

|  | Please list the core ethical guidance documents that have been referred to during the completion of this form (including any discipline-specific codes of research ethics, location-specific dodes of research ethics, and also any specific ethical guidance relating to the proposed methodology). Please tick to confirm that your research proposal adheres to these codes and guidelines. You may add rows to this table if needed. |  |
| --- | --- | --- |
| 1 | UWTSD Research Ethics & Integrity Code of Practice | ☐ |
| 2 | UWTSD Research Data Management Policy | ☐ |
| 3 |  | ☐ |

SECTION C: Details of Research Activity

| 1 | Indicative title: | Evaluation of Accelerated Rate Calorimetry (ARC) for Assessment of Explosives Compatibility Research Proposal |  |  |
| --- | --- | --- | --- | --- |
| 2 | Proposed start date: | Sep 2023 | Proposed end date: | Jun 2024 |
|  | Introduction to the Research (maximum 300 words in each section) Ensure that you write for a Non-Specialist Audience when outlining your response to the three points below: - Purpose of Research Activity - Proposed Research Question - Aims of Research Activity - Objectives of Research Activity Demonstrate, briefly, how Existing Research has informed the proposed activity and explain - What the research activity will add to the body of knowledge - How it addresses an area of importance. |  |  |  |
| 3 | Purpose of Research Activity To determine the applicability of using Accelerating Rate Calorimetry (ARC) to determine explosives compatibility. (this box should expand as you type) |  |  |  |
| 4 | Research Question Can ARC be a suitable method of explosives compatibility? (this box should expand as you type) |  |  |  |
| 5 | Aims of Research Activity The aim of this project is to evaluate the application of ARC to Explosives Compatibility Testing. (this box should expand as you type) |  |  |  |
| 6 | Objectives of Research Activity - Carry out each test twice. If time left, three or four times. - Carry out same tests using other known methods of compatibility testing (this box should expand as you type) |  |  |  |
|  | Proposed data collection methods (maximum 600 words) Provide a brief summary of all the methods that may be used in the research activity to collect data, making it clear what specific techniques may be used. If methods other than those listed in this section are deemed appropriate later, additional ethical approval for those methods will be needed. You do not need to justify the methods here, but should instead describe how you intend to collect the data necessary for you to complete your project. |  |  |  |
| 7 | Experiment data will be collected by equipment software and logged on a computer. (this box should expand as you type) |  |  |  |

SECTION D: Scope of Research Activity

|  | Will the research activity include: | YES | NO |
| --- | --- | --- | --- |
| 1 | Use of a questionnaire or similar research instrument? | ☐ | ☒ |
| 2 | Use of interviews? | ☐ | ☒ |
| 3 | Use of focus groups? | ☐ | ☒ |
| 4 | Use of participant diaries? | ☐ | ☒ |
| 5 | Use of video or audio recording? | ☐ | ☒ |
| 6 | Use of computer-generated log files? | ☒ | ☐ |
| 7 | Participant observation with their knowledge? | ☐ | ☒ |
| 8 | Participant observation without their knowledge? | ☐ | ☒ |
| 9 | Access to personal or confidential information without the participants’ specific consent? | ☐ | ☒ |
| 10 | Administration of any questions, test stimuli, presentation that may be experienced as physically, mentally or emotionally harmful / offensive? | ☐ | ☒ |
| 11 | Performance of any acts which may cause embarrassment or affect self-esteem? | ☐ | ☒ |
| 12 | Investigation of participants involved in illegal activities? | ☐ | ☒ |
| 13 | Use of procedures that involve deception? | ☐ | ☒ |
| 14 | Administration of any substance, agent or placebo? | ☐ | ☒ |
| 15 | Working with live vertebrate animals? | ☐ | ☒ |
| 16 | Procedures that may have a negative impact on the environment? | ☒ | ☐ |
| 17 | Other primary data collection methods. Please indicate the type of data collection method(s) below. | ☐ | ☒ |
|  | Details of any other primary data collection method: (this box should expand as you type) |  |  |

If you have ticked NO to every question then the research activity is (ethically) low risk and you may skip section E and continue to section F.

If YES to any question, then no research activity should be undertaken until full ethical approval has been obtained.

SECTION E: Intended Participants

|  | Who are the intended participants: | YES | NO |
| --- | --- | --- | --- |
| 1 | Students or staff at the University? | ☐ | ☒ |
| 2 | Adults (over the age of 18 and competent to give consent)? | ☐ | ☒ |
| 3 | Vulnerable adults? | ☐ | ☒ |
| 4 | Children and Young People under the age of 18? (Consent from Parent, Carer or Guardian will be required) | ☐ | ☒ |
| 5 | Prisoners? | ☐ | ☒ |
| 6 | Young offenders? | ☐ | ☒ |
| 7 | Those who could be considered to have a particularly dependent relationship with the investigator or a gatekeeper? | ☐ | ☒ |
| 8 | People engaged in illegal activities? | ☐ | ☒ |
| 9 | Others. Please indicate the participants below, and specifically any group who may be unable to give consent. | ☐ | ☒ |
|  | Details of any other participant groups: N/A (this box should expand as you type) |  |  |

|  | Participant numbers and source Provide an estimate of the expected number of participants. How will you identify participants and how will they be recruited? |  |
| --- | --- | --- |
| 10 | How many participants are expected? | N/A (this box should expand as you type) |
| 11 | Who will the participants be? | N/A (this box should expand as you type) |
| 12 | How will you identify the participants? | N/A (this box should expand as you type) |

|  | Information for participants: | YES | NO | N/A |
| --- | --- | --- | --- | --- |
| 13 | Will you describe the main research procedures to participants in advance, so that they are informed about what to expect? | ☐ | ☐ | ☒ |
| 14 | Will you tell participants that their participation is voluntary? | ☐ | ☐ | ☒ |
| 15 | Will you obtain written consent for participation? | ☐ | ☐ | ☒ |
| 16 | Will you explain to participants that refusal to participate in the research will not affect their treatment or education (if relevant)? | ☐ | ☐ | ☒ |
| 17 | If the research is observational, will you ask participants for their consent to being observed? | ☐ | ☐ | ☒ |
| 18 | Will you tell participants that they may withdraw from the research at any time and for any reason? | ☐ | ☐ | ☒ |
| 19 | With questionnaires, will you give participants the option of omitting questions they do not want to answer? | ☐ | ☐ | ☒ |
| 20 | Will you tell participants that their data will be treated with full confidentiality and that, if published, it will not be identifiable as theirs? | ☐ | ☐ | ☒ |
| 21 | Will you debrief participants at the end of their participation, in a way appropriate to the type of research undertaken? | ☐ | ☐ | ☒ |
| 22 | If NO to any of above questions, please give an explanation |  |  |  |
|  | N/A (this box should expand as you type) |  |  |  |

|  | Information for participants: | YES | NO | N/A |
| --- | --- | --- | --- | --- |
| 24 | Will participants be paid? | ☐ | ☐ | ☒ |
| 25 | Is specialist electrical or other equipment to be used with participants? | ☐ | ☐ | ☒ |
| 26 | Are there any financial or other interests to the investigator or University arising from this study? | ☐ | ☒ | ☒ |
| 27 | Will the research activity involve deliberately misleading participants in any way, or the partial or full concealment of the specific study aims? | ☐ | ☐ | ☒ |
| 28 | If YES to any question, please provide full details |  |  |  |
|  | N/A (this box should expand as you type) |  |  |  |

SECTION F: Anticipated Risks

|  | # Outline any anticipated risks that may adversely affect any of the participants, the researchers and/or the University, and the steps that will be taken to address them. |  |
| --- | --- | --- |
| 1 | Risks to participants For example: sector-specific health & safety, emotional distress, financial disclosure, physical harm, transfer of personal data, sensitive organisational information. If you have identified in section D that there are no participants then enter N/A and go skip to question 3. |  |
|  | Risk to participants: N/A (this box should expand as you type) | How you will mitigate the risk to participants: (this box should expand as you type) |
| 2 | If research activity may include sensitive, embarrassing or upsetting topics (e.g. sexual activity, drug use) or issues likely to disclose information requiring further action (e.g. criminal activity), give details of the procedures to deal with these issues, including any support/advice (e.g. helpline numbers) to be offered to participants. Note that where applicable, consent procedures should make it clear that if something potentially or actually illegal is discovered in the course of a project, it may need to be disclosed to the proper authorities |  |
|  | N/A (this box should expand as you type) |  |
| 3 | Risks to the investigator For example: personal health & safety, physical harm, emotional distress, risk of accusation of harm/impropriety, conflict of interest |  |
|  | Risk to the investigator: Using explosives to carry out tests (this box should expand as you type) | How you will mitigate the risk to the investigator: Provide PPE (this box should expand as you type) |
| 4 | University/institutional risks For example: adverse publicity, financial loss, data protection |  |
|  | Risk to the University: N/A (this box should expand as you type) | How you will mitigate the risk to the University: (this box should expand as you type) |
| 5 | Environmental risks For example: accidental spillage of pollutants, damage to local ecosystems |  |
|  | Risk to the environment: Explosive Waste (this box should expand as you type) | How you will mitigate the risk to environment: Consult lab manager and waste management team (this box should expand as you type) |

SECTION G: Feedback, Consent and Confidentiality

If you have identified in section D that there are no participants then enter skip this section and continue to section H.

| 1 | Feedback What de-briefing and feedback will be provided to participants, how will this be done and when? |
| --- | --- |
|  | You don’t need to email your participants with your final report. A good alternative is to set up an email address that they will be able to contact for further details or results. (this box should expand as you type) |
| 2 | Informed consent Describe the arrangements to inform potential participants, before providing consent, of what is involved in participating. Describe the arrangements for participants to provide full consent before data collection begins. If gaining consent in this way is inappropriate, explain how consent will be obtained and recorded in accordance with prevailing data protection legislation. |
|  | If you are using a paper questionnaire then you should have the participants sign an appropriate consent form. These forms will count as personal data and should be noted as such in section J. If you are using an online questionnaire, then you should have a screen before the questions start that acts as a consent form, informing participants that by clicking on the NEXT button they are providing consent. (this box should expand as you type) |
| 3 | Confidentiality / Anonymity Set out how anonymity of participants and confidentiality will be ensured in any outputs. If anonymity is not being offered, explain why this is the case. |
|  | Do not collect names unless you really need them. Do not name participants or organisations in any research publications (including the thesis) without their explicit permission. (this box should expand as you type) |

SECTION H: Data Protection and Storage

|  | Does the research activity involve personal data (as defined by the General Data Protection Regulation 2016 “GDPR” and the Data Protection Act 2018 “DPA”)? | YES | NO |
| --- | --- | --- | --- |
| 1 | “Personal data” means any information relating to an identified or identifiable natural person (‘data subject’). An identifiable natural person is one who can be identified, directly or indirectly, in particular by reference to an identifier such as a name, an identification number, location data, an online identifier or to one or more factors specific to the physical, physiological, genetic, mental, economic, cultural or social identity of that natural person. Any video or audio recordings of participants is considered to be personal data. | ☐ | ☒ |
|  | If YES, provide a description of the data and explain why this data needs to be collected: |  |  |
| 2 | This includes audio/video data of participants, but can also include IP addresses and usernames. Names, addresses and emails also count, as do consent forms. (this box should expand as you type) |  |  |
|  | Does it involve special category data (as defined by the GDPR)? | YES | NO |
| 3 | “Special category data” means sensitive personal data consisting of information as to the data subjects’ – (a) racial or ethnic origin, (b) political opinions, (c ) religious beliefs or other beliefs of a similar nature, (d) membership of a trade union (within the meaning of the Trade Union and Labour Relations (Consolidation) Act 1992), (e) physical or mental health or condition, (f) sexual life, (g) genetics, (h) biometric data (as used for ID purposes), | ☐ | ☒ |
|  | If YES, provide a description of the special category data and explain why this data needs to be collected: |  |  |
| 4 | What counts as ‘sensitive’ will differ between cultures. Any information on behaviour that is not in accordance with cultural norms would count as sensitive personal data. (this box should expand as you type) |  |  |

|  | Will data from the research activity (collected data, drafts of the thesis, or materials for publication) be stored in any of the following ways? | YES | NO |
| --- | --- | --- | --- |
| 5 | Manual files (i.e. in paper form)? | ☒ | ☐ |
| 6 | University computers? | ☐ | ☐ |
| 7 | Private company computers? | ☒ | ☐ |
| 8 | Home or other personal computers? | ☐ | ☒ |
| 9 | Laptop computers/ CDs/ Portable disk-drives/ memory sticks? | ☐ | ☒ |
| 10 | “Cloud” storage or websites? | ☐ | ☒ |
| 11 | Other – specify: | ☐ | ☒ |
| 12 | For all stored data, explain the measures in place to ensure the security of the data collected, data confidentiality, including details of backup procedures, password protection, encryption, anonymisation and pseudonymisation: |  |  |
|  | Manual files will be stored in a secure cabinet or pedestal. Digital data will be stored on two separate computers: the equipment’s own dedicated computer, and my own work account (both password protected) All data will be kept in password protected cloud storage on the University Office 365 system which will not be shared. Audio/visual data will be transcribed and would be shown to participants to check accuracy of reporting. Any USB sticks used to store or transfer data will be password protected. All participants will be given a unique identifier to ensure confidentiality and this list will be kept securely in the password protected folder. (this box should expand as you type) |  |  |

|  | Data Protection |  |  |
| --- | --- | --- | --- |
|  | Will the research activity involve any of the following activities: | YES | NO |
| 13 | Electronic transfer of data in any form? | ☒ | ☐ |
| 14 | Sharing of data with others at the University outside of the immediate research team? | ☒ | ☐ |
| 15 | Sharing of data with other organisations? | ☐ | ☒ |
| 16 | Export of data outside the UK or importing of data from outside the UK? | ☐ | ☒ |
| 17 | Use of personal addresses, postcodes, faxes, emails or telephone numbers? | ☐ | ☒ |
| 18 | Publication of data that might allow identification of individuals? | ☐ | ☒ |
| 19 | If YES to any question, please provide full details, explaining how this will be conducted in accordance with the GDPR and Data Protection Act (2018) (and/or any international equivalent): |  |  |
|  | Data will be transferred electronically between the two computer systems specified above. This data will also be presented to the university in the final report. All data will be encrypted and kept in password protected cloud storage on the University Office 365 system which will not be shared. Any USB sticks used to store or transfer data will be password protected. All data transfers will be encrypted and password protected. All participants will be given a unique identifier to ensure confidentiality and this list will be kept securely in the password protected folder. The data will be stored until the completion of the project and then deleted. In accordance with the DPA2018, participants will have the right to ask to see what data is held relating to them, and this data will be deleted immediately if the participant requests this, in which case the data will not be used in the project. |  |  |
| 20 | List all who will have access to the data generated by the research activity: |  |  |
|  | Jordan Hayston, Jenny Roberts, Alan MacDonald (this box should expand as you type) |  |  |
| 21 | List who will have control of, and act as custodian(s) for, data generated by the research activity: |  |  |
|  | Jordan Hayston, Jenny Roberts, Alan MacDonald (this box should expand as you type) |  |  |
| 22 | Give details of data storage arrangements, including security measures in place to protect the data, where data will be stored, how long for, and in what form. |  |  |
|  | As already stated above. Data will be stored until final report is completed. All data will be encrypted and kept in password protected cloud storage on the University Office 365 system which will not be shared. Any USB sticks used to store or transfer data will be password protected, and will be reformatted at the end of the project in order to destroy the data. The data will be stored until the completion of the project and then deleted. (this box should expand as you type) |  |  |
| 22 | Confirm that you have read the UWTSD guidance on data management (see https://www.uwtsd.ac.uk/library/research-data-management/) | ☒ |  |
| 23 | Confirm that you are aware that you need to keep all data until after your research has completed or the end of your funding | ☒ |  |

SECTION I: Declaration

|  | The information which I have provided is correct and complete to the best of my knowledge. I have attempted to identify any risks and issues related to the research activity and acknowledge my obligations and the rights of the participants. In submitting this application I hereby confirm that I undertake to ensure that the above named research activity will meet the University’s Research Ethics and Integrity Code of Practice which is published on the website: https://www.uwtsd.ac.uk/research/research-ethics/ |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Signature of applicant: | J Hayston | Date: 22/05/2023 |  |  |
| 2 |  | Director of Studies/Supervisor: |  |  | Date: |
| 3 |  | Signature: |  |  |  |

FOR INTERNAL USE ONLY:

|  | Ethical approval given |  |  |
| --- | --- | --- | --- |
| 1 | Signature of assessor: |  | Date: |
| 2 | Name: |  |  |
| 3 | Role: |  |  |