CONSTRUCTION TECHNOLOGY 2 ACBA5019 ¦ Level 5 Assessment 2

Buildability and
Sustainability Analysis

| Student name | Mikey Scourfield |
| --- | --- |
| Student ID | 2410362 |
| Programme | HND/BSc Quantity Surveying |
| Module | Construction Technology 2 |
| Module code | ACBA5019 |
| Assessment title | Buildability and Sustainability Analysis |
| Word count | 2500 words +/- 10% |
| Submission deadline | 11/05/2026 |

Table of Contents

1. Introduction	3

2. AS1 Design Context Summary	3

2.1 Project / Building Overview	3

2.2 AS1-to-AS2 Expansion Tracker	3

3. Evaluation Framework: Buildability, Sustainability and Flexibility	4

3.1 Key Definitions	5

3.2 Evaluation Criteria	6

4. Buildability Analysis	7

4.1 Site Logistics and Access	7

4.2 Construction Sequence and Method	8

4.3 Plant, Labour and Temporary Works	8

4.4 Health, Safety, Quality and Programme	9

4.5 Buildability Risk Register	9

5. Sustainability Analysis	9

5.1 Materials and Embodied Carbon	10

5.2 Operational Energy and Building Performance	10

5.3 Waste, Water and Circular Economy	10

5.4 Durability, Maintenance and Lifecycle Value	10

5.5 Sustainability Evidence Table	11

6. Flexibility and Future Adaptability Analysis	11

6.1 Adaptability Checklist	12

7. Comparative Evaluation of Alternatives	13

7.1 Options to Compare	13

7.2 Decision Matrix	13

8. Recommendations	14

9. Conclusion	16

10. Reference List	17

# 1. Introduction

This report develops the AS1 concept for a proposed 8–12 storey hotel in Swansea, near Singleton Hospital. The project is carried forward as a medium/high-rise building in a coastal, operationally sensitive location, with the baseline concept comprising a steel composite frame with reinforced concrete core, masonry cavity wall envelope and a hybrid DfMA/MMC construction strategy. The AS1 proposal assumed piled foundations, but this report treats the substructure strategy as provisional until site-specific ground investigation confirms whether shallow foundations, a raft, a piled raft, ground improvement or full piling is technically justified (British Geological Survey, n.d.; British Standards Institution, 2020; BRE, 2005).

The aim of this AS2 report is to critically evaluate the proposal in terms of buildability, sustainability and flexibility, rather than simply describe the building form. Buildability is considered through site logistics, construction sequencing, cranage, ground conditions, envelope installation and fire-safety coordination. Sustainability is assessed using lifecycle thinking, with attention to embodied carbon, durability, material efficiency, waste reduction, operational performance and long-term maintenance. Flexibility is considered through structural grid choice, services integration, adaptability of hotel layouts and the implications of different construction methods.

The evaluation uses relevant literature, technical standards, lifecycle principles and comparative option appraisal to test the selected approach. The report concludes with practical recommendations for design development, procurement, construction planning and risk control.

# 2. AS1 Design Context Summary

## 2.1 Project / Building Overview

The Swansea AS1 project proposed a medium/high-rise hotel near Singleton Hospital and Mumbles Road. The 8–12-story hotel addresses medium/high-rise construction issues like vertical load transfer, wind stability, fire strategy, façade performance, site logistics, and construction sequencing.

Steel composite frames with reinforced concrete cores were chosen. The site context suggested made ground, groundwater uncertainty, and settlement risk, so AS1 assumed deep piled foundations with ground beams and a suspended slab. Please note that this assumption is not a final foundation commitment. Swansea evidence suggests variable ground and groundwater conditions, but it does not replace a boundary-specific GI for the hotel footprint. The foundation strategy should remain open until boreholes, in-situ testing, groundwater monitoring, contamination testing, and aggressive-ground testing determine whether piles are needed or if a lower-carbon shallow, raft, piled-raft, or ground-improvement solution is feasible. For fire compliance, acoustic performance, and durability in coastal exposure, masonry cavity walls with non-combustible insulation and cavity barriers are the main envelope strategy.

A hybrid DfMA/MMC construction strategy was carried over into AS2. This uses off-site manufacturing for repeatable elements like steelwork, windows, MEP modules, plant skids and possibly bathroom pods while keeping the frame and core on-site. AS2 evaluation is needed for coastal wind exposure, very severe wind-driven rain, marine corrosion risk, operational sensitivity near the hospital, restricted logistics, abnormal-load control, and fire-safety coordination.

## 2.2 AS1-to-AS2 Expansion Tracker

| AS1 point | Why it matters for AS2 | Buildability link | Sustainability / flexibility link | Source |
| --- | --- | --- | --- | --- |
| Proposed 8–12 storey hotel near Singleton Hospital | Establishes the building type, height range and operational context for a medium/high-rise evaluation. | Requires assessment of vertical logistics, craneage, emergency access, hospital-adjacent disruption and construction sequencing. | Hotel layouts use repetitive room modules, creating potential for standardisation, efficient servicing and future internal reconfiguration. | Swansea tall-building context and fire-safety requirements should inform the evaluation (City and County of Swansea, n.d.; Welsh Government, 2021; Welsh Government, 2025d). |
| Steel composite frame with reinforced concrete core | This is the main structural form carried forward from AS1 and must be tested critically rather than simply accepted. | Faster frame erection and lighter superstructure, but dependent on cranage, temporary stability, tolerance control and fire protection. | Regular grids can support internal adaptability; embodied carbon depends on efficient steel design, procurement and EPD-led specification. | Wind design, steel construction guidance and embodied-carbon principles should be applied (British Standards Institution, 2010; SteelConstruction.info, n.d.; Institution of Structural Engineers, 2020; RICS, 2024). |
| Provisional substructure strategy: AS1 assumed piles, but GI must confirm the final foundation form | Ground uncertainty is a major design risk for the Swansea site, but desk-study evidence alone is not enough to prove that full piling is the only justified option. | GI affects foundation choice, working-platform design, spoil handling, groundwater control, obstruction risk, concrete deliveries and programme certainty. | GI-led foundation selection can avoid unnecessary concrete and reinforcement if shallow foundations, a raft, piled raft or ground improvement are viable. | British Geological Survey (n.d.); British Standards Institution (2020); BRE (2005); British Standards Institution (2023); RICS (2024). |
| Masonry cavity wall with non-combustible insulation | The envelope is central to fire compliance, weather resistance, acoustic performance and durability. | Requires workmanship control, cavity trays, wall ties, cavity barriers, scaffold/access strategy and interface inspection. | Durable façades can reduce replacement cycles; good acoustic and thermal performance supports hotel use, but the wall build-up may reduce façade adaptability. | External wall fire guidance, exposure guidance and masonry detailing sources should be used (Welsh Government, 2025b; Welsh Government, 2025d; British Standards Institution, n.d.; LABC Warranty, n.d.; Marshalls, n.d.). |
| Hybrid DfMA/MMC construction strategy | AS1 identified off-site manufacture as beneficial but not suitable for every element. | Prefabricated steelwork, MEP modules, plant skids and possible bathroom pods can reduce site congestion, but wind and logistics constrain large modules. | Reduces waste and improves quality control; repeated components support standardisation, but full volumetric construction may reduce design adaptability. | MMC and lifting-risk evidence should support the comparison (UK Government, n.d.; SCI, n.d.; Construction Plant-hire Association, 2025; Construction Leadership Council, 2020). |

# 3. Evaluation Framework: Buildability, Sustainability and Flexibility

This section defines the criteria used to evaluate the AS1 proposal. The purpose is to create a consistent assessment framework before judging the selected structural frame, provisional foundation strategy, envelope and construction method. The assessment uses a qualitative approach, supported by technical guidance, standards, lifecycle thinking and project-specific constraints identified in AS1. The criteria are applied to the proposed 8–12 storey hotel in Swansea, where coastal exposure, hospital-adjacent logistics, ground uncertainty and fire-safety compliance are key design drivers.

## 3.1 Key Definitions

| Concept | Working definition for this report | How it will be assessed | Source |
| --- | --- | --- | --- |
| Buildability | The ease, safety, efficiency and reliability with which the proposed building can be constructed within the specific Swansea site context. | Assessed through site access, logistics, sequencing, craneage, temporary works, labour intensity, tolerance control, weather sensitivity, fire coordination and interface risk. | Construction logistics guidance, lifting guidance, site constraints, frame/envelope sequencing evidence and construction-phase risk controls (Construction Plant-hire Association, 2025; Transport for London and CLOCS, 2021; Health and Safety Executive, n.d.). |
| Sustainability | The environmental, economic and social performance of the building across construction, operation, maintenance and end-of-life stages. | Assessed through whole-life carbon, material efficiency, durability, waste, transport impacts, operational energy, maintenance burden and resilience to coastal exposure. | Whole-life carbon guidance, Planning Policy Wales, PAS 2080 principles, EPDs, durability standards and waste-reduction guidance (RICS, 2024; Welsh Government, 2024; Institution of Civil Engineers, 2025; Construction Leadership Council, 2020). |
| Flexibility | The ability of the building to accommodate future changes in use, layout, services, maintenance access or performance requirements without disproportionate cost, waste or disruption. | Assessed through structural grid regularity, floorplate adaptability, services distribution, façade maintainability, hotel-room repetition, plant replacement routes and capacity for future refurbishment. | Adaptability guidance, structural grid principles, MMC/modular guidance, maintenance-access planning and service-life considerations (International Organization for Standardization, 2020; SCI, n.d.; UK Government, n.d.; RICS, 2024). |

## 3.2 Evaluation Criteria

For this report, each criterion is rated by impact: High where it could significantly affect safety, compliance, programme, cost, carbon or long-term performance; Medium where it affects coordination, efficiency or future performance; and Low where the effect is limited or easily mitigated.

| Criterion | Questions to ask | Source | Impact rating: Low/Med/High |
| --- | --- | --- | --- |
| Site logistics and construction access | Can materials, plant, operatives and deliveries be managed without disrupting the hospital, road network or emergency access? Are abnormal loads or off-site holding areas required? | Construction logistics plan, delivery route review, abnormal-load checks, site access drawings, neighbour constraints and traffic-management proposals (Transport for London and CLOCS, 2021; National Highways, n.d.). | High |
| Structural frame buildability | Does the steel composite frame with RC core provide a practical balance between speed, stability, fire protection, tolerance control and floorplate repetition? | Frame option appraisal, stability strategy, lifting plan, fire-protection strategy, erection sequence and programme comparison (British Standards Institution, 2010; SteelConstruction.info, n.d.; Construction Plant-hire Association, 2025). | High |
| Ground and foundation risk | Is full piling justified by site-specific GI, or could shallow foundations, a raft, a piled raft or ground improvement provide adequate performance with lower carbon and reduced construction risk? | Ground investigation, BGS data, groundwater monitoring, contamination/aggressive ground testing, utility proving and foundation option appraisal (British Geological Survey, n.d.; British Standards Institution, 2020; BRE, 2005; British Standards Institution, 2023). | High |
| Envelope performance and installation risk | Can the masonry cavity wall resist very severe wind-driven rain, marine exposure and fire-safety requirements while remaining buildable? | Façade detailing, cavity barrier strategy, wall-tie specification, sample panel, inspection hold points, WDR exposure evidence and corrosion guidance (British Standards Institution, n.d.; Welsh Government, 2025b; Welsh Government, 2025d; International Organization for Standardization, 2017). | High |
| Fire safety and compliance | Does the selected structure, core, façade and construction method support a clear fire strategy for a sleeping-risk hotel? | Approved Document B review, fire strategy, compartmentation drawings, protected core/stair strategy, fire stopping inspections and temporary fire controls (Welsh Government, 2021; Welsh Government, 2025d; Health and Safety Executive, n.d.). | High |
| Whole-life carbon and resource efficiency | Are the structural frame, foundations, envelope and MMC strategy efficient in embodied carbon, waste, transport and replacement cycles? | Whole-life carbon screening, EPDs, material quantities, waste strategy, procurement data and durability assessment (RICS, 2024; Institution of Structural Engineers, 2020; Construction Leadership Council, 2020). | High |
| Flexibility, adaptability and maintainability | Can the hotel accommodate future layout changes, services replacement, façade maintenance and performance upgrades without major disruption? | Structural grid review, servicing strategy, plant replacement routes, façade access strategy, modular room/pod assessment and maintenance plan (International Organization for Standardization, 2020; SCI, n.d.; UK Government, n.d.). | Medium |

# 4. Buildability Analysis

This section evaluates whether the AS1 proposal is practical to construct. The assessment focuses on logistics, sequencing, plant, labour, temporary works, health and safety, quality and programme risk. The AS1 design is based on an 8–12 storey hotel near Singleton Hospital, with a steel composite frame, reinforced concrete core, provisional substructure strategy, masonry cavity wall envelope and hybrid DfMA/MMC strategy. The assessment specification requires practical application and critical evaluation of construction forms for medium/high-rise buildings, so this section tests the proposal against site and construction constraints rather than restating the design.

## 4.1 Site Logistics and Access

The site context creates a high-risk logistics environment because it is close to Singleton Hospital, the Mumbles Road corridor and university/open-space uses. The site should be treated as a live institutional frontage rather than a conventional construction plot, because construction traffic would interact with hospital users, university movement, bus-served frontage and constrained Mumbles Road access. AS1 already identifies limited tolerance for queuing, unmanaged deliveries and disruption to emergency or strategic routes. This makes a formal Construction Logistics Plan essential, with timed deliveries, off-site holding, banksmen, segregated pedestrian routes, off-corridor call-forward arrangements and no highway waiting (Swansea Bay University Health Board, n.d.; Swansea University, n.d.; Transport for London and CLOCS, 2021).

The construction method increases logistics pressure. If piles are confirmed by GI, the substructure will need rig access, working-platform control, spoil removal and concrete deliveries; if a raft or ground-improved shallow solution is viable, excavation, groundwater and material handling will still need careful coordination. The RC core needs reinforcement, formwork and pumping; the steel frame needs craneage and sequenced deliveries; and the masonry envelope needs scaffold or mast-climber access. A tower crane is likely to be required, but crane position must consider oversailing, hospital operations, wind limits and possible aviation safeguarding. Large prefabricated components should be limited unless transport routes, abnormal-load controls and lifting windows are proven (National Highways, n.d.; Construction Plant-hire Association, 2025).

## 4.2 Construction Sequence and Method

The likely sequence is enabling works, GI-confirmed substructure, RC core, steel frame, composite floors, envelope, MEP, fit-out and commissioning. This sequence is logical because the RC core can provide stability while the steel frame follows in repeated floor cycles. The hotel use also supports repetition through standardised rooms, risers and MEP layouts. If piles are confirmed, the programme must include piling mat design, obstruction probing, spoil classification and noise/vibration controls; if a shallow or raft option is viable, the emphasis shifts toward excavation control, bearing verification and groundwater management.

The main buildability weakness is interface risk. Core-to-steel connections require accurate cast-in plates and tolerance, while steel-to-façade interfaces must allow movement, deflection, cavity barriers and weathering. MEP penetrations must be coordinated before fabrication to avoid cutting, delay and fire-stopping defects. The programme risk is therefore concentrated at GI/substructure confirmation, core construction, wind-sensitive steel erection and envelope closure. Early contractor input, sample bays, coordinated BIM/service zones and clear inspection hold points should reduce this risk.

| Stage | Main work package | Main buildability risk | Control measure |
| --- | --- | --- | --- |
| 1. Enabling | Hoarding, welfare, temporary services, access setup | Conflict with hospital/public routes | Construction Logistics Plan; segregated routes; delivery booking system |
| 2. Substructure | GI-confirmed foundation solution: shallow, raft, piled raft, ground improvement or piles | Obstructions, groundwater, spoil, noise, overdesign risk | Intrusive GI; foundation option appraisal; working-platform/spoil plan; monitoring |
| 3. Core | RC core construction | Tolerance error, slow concrete cycle | Core survey checks; coordinated cast-in plates; reliable concrete supply |
| 4. Frame/floors | Steel frame, metal deck, topping | Wind stoppages, temporary instability | Engineered erection sequence; temporary bracing; wind limits |
| 5. Envelope | Masonry cavity wall, windows, fire barriers | Water ingress, fire-stopping defects | Sample bay; hold-point inspections; cavity/fire-stopping register |
| 6. MEP/fit-out | Risers, rooms, bathrooms, corridors | Trade congestion, rework | Standardised room zones; coordinated services; floor-by-floor handover |

## 4.3 Plant, Labour and Temporary Works

The main plant will include excavators, concrete pumps, tower crane, mobile crane, telehandlers, hoists, scaffold or mast climbers, MEWPs and cutting/fixing equipment. If GI confirms deep foundations, piling rigs and certified working platforms will also be required; if ground improvement or raft construction is selected, specialist treatment plant, compaction equipment or larger excavation plant may be needed instead. Specialist labour will include substructure contractors, steel erectors, slingers/signallers, formwork teams, rebar fixers, masonry/façade operatives, fire-stopping installers and MEP specialists.

Temporary works are significant. A piled option requires a designed and certified piling mat, while a raft or shallow solution still requires temporary excavation support, groundwater control and safe access. The RC core requires formwork platforms, edge protection and concrete pumping arrangements. Steel erection requires temporary bracing, exclusion zones and safe decking systems. The masonry envelope requires stable access and inspection before cavities are closed. Productivity depends on zoning the works so that structure, façade, MEP and fit-out trades do not congest the same floors.

## 4.4 Health, Safety, Quality and Programme

Working at height, lifting, falling materials, manual handling, noise, dust, hot work, and vehicle movement are the main health and safety risks. The building's height and hospital proximity exacerbate these. Program risks include coastal exposure: Mumbles Head climate data shows 1,277.37 mm average annual rainfall and 160.38 days with at least 1 mm rainfall, making it wetter and more exposed than an inland comparator (Met Office, n.d.). When lifting steelwork, pods, or plant skids, weather limits, exclusion zones, and competent supervision should be considered.

Quality control is key to buildability. Although factory-made steelwork, windows, MEP modules, and plant skids can improve consistency, the masonry cavity wall is still handmade. Poor cavity trays, weeps, wall ties, cavity barriers, or window interfaces can allow water in and require costly remediation in severe wind-driven rain. Mandate façade sample bays, photographic records, and hold-point inspections. The hotel is a sleeping-risk building, so fire stopping must be inspected before closing. Only with disciplined logistics, weather contingency, early coordination, and thorough inspection can the proposal be built.

## 4.5 Buildability Risk Register

| Buildability risk | Cause | Impact | Likelihood | Mitigation / design response | Evidence |
| --- | --- | --- | --- | --- | --- |
| Delivery congestion | Hospital-adjacent roads and limited holding space | High | High | Timed deliveries, off-site holding, banksmen, no highway queuing | Transport for London and CLOCS (2021); National Highways (n.d.) |
| Crane stoppages | Coastal wind, wet weather and high-sail loads | High | Medium/High | Wind limits, smaller loads, weather float, temporary weather protection, avoid over-reliance on volumetric modules | Met Office (n.d.); Construction Plant-hire Association (2025); SCI (n.d.) |
| Substructure delay or overdesign | Unconfirmed ground model, groundwater, obstructions or contamination | High | Medium/High | Boundary-specific GI, utility proving, foundation option appraisal, working-platform/spoil strategy | British Geological Survey (n.d.); British Standards Institution (2020); BRE (2005) |
| Frame tolerance issues | Core/steel interface errors | High | Medium | Survey control, early steel contractor input, coordinated cast-in plates | BS EN 1991-1-4; SteelConstruction.info (n.d.) |
| Façade leakage | Poor cavity detailing in wind-driven rain | High | Medium | Sample bay, cavity inspections, robust trays/weeps/ties | BS 8104; LABC Warranty (n.d.); Marshalls (n.d.) |
| Fire-stopping defects | Late coordination of MEP/façade penetrations | High | Medium | Fire-stopping register, inspections before closure, coordinated risers | Welsh Government (2021; 2025d); Health and Safety Executive (n.d.) |

# 5. Sustainability Analysis

This section evaluates the AS1 proposal using lifecycle sustainability criteria. The assessment considers embodied carbon, operational energy, waste, water, circular economy, durability and maintenance. As no detailed quantities, EPDs or energy model are currently available, the appraisal is qualitative. A formal whole-life carbon assessment should therefore be completed at the next design stage using RICS guidance (RICS, 2024).

## 5.1 Materials and Embodied Carbon

High-embodied-carbon areas include steel composite frame reinforced concrete core, provisional substructure, masonry cavity wall, and insulation. Steel composite frames are better than concrete frames for AS1 because they reduce superstructure weight and foundation demand. The frame should be rationalised through efficient grid design, standardised member sizes, and EPD-led procurement because steel has a high upfront carbon impact. RICS (2024; ISBE 2020) wants supply-chain data to prove recycled-content steel's impact.

Concrete dominates core and substructure. Overdesigning structure-critical elements increases embodied carbon. Not overcommitting to piles may boost sustainability most. Pads, rafts, piled rafts, or ground improvement may reduce concrete and reinforcement if investigation shows competent shallow strata or manageable settlement risk. Pile construction may be justified if the site has uncontrolled fill, soft deposits, high groundwater, or unacceptable settlement risk. Though not wrong, unproven piling is a carbon, cost, and programme risk (British Geological Survey, n.d.; BRE, 2005; British Standards Institution, 2020; RICS, 2024). When strength, program, and exposure requirements are met, consider lower-carbon concrete mixes. Coastline durability is supported by BS 8500:2023.

## 5.2 Operational Energy and Building Performance

Operational energy will depend mainly on the façade, airtightness, thermal bridging, glazing and building services. The masonry cavity wall can support good thermal and acoustic performance for hotel bedrooms, but only if insulation continuity, cavity barriers, window interfaces and slab-edge details are properly coordinated. Poor detailing would increase heat loss, condensation risk and long-term maintenance.

The design should avoid excessive glazing because this can increase solar gain, overheating and cooling demand. A balanced façade strategy should combine controlled glazing ratios, solar-control glass, airtight construction, efficient ventilation and careful thermal bridge detailing. At detailed design stage, CIBSE TM54 should be used to assess likely operational energy more realistically than compliance-only modelling, while the whole-life carbon assessment should test both upfront carbon and replacement cycles for façade, services and substructure options (CIBSE, 2022; RICS, 2024).

## 5.3 Waste, Water and Circular Economy

The hybrid DfMA/MMC strategy reduces waste by producing steelwork, windows, MEP modules, plant skids and possibly bathroom pods in a controlled factory. Offcuts, site congestion, and rework can decrease. The site-built masonry envelope may generate bricks, blocks, insulation, pallets, mortar, and packaging waste. Standard dimensions, material take-off accuracy, and supplier take-back are recommended.

Water sustainability should include construction and operation. Runoff, cement washout, fuel storage, and dust-suppression water must be controlled during construction to avoid pollution. The completed hotel should manage surface water using a SuDS strategy in accordance with Welsh drainage standards (Welsh Government, 2019). Because the Singleton/Mumbles Road corridor is coastal and flood/drainage screening is plot-dependent, SuDS, exceedance routing, and access resilience should be design issues, not late compliance items (Welsh Government, 2025a).

## 5.4 Durability, Maintenance and Lifecycle Value

In a coastal Swansea location, durability is central to sustainability. The lowest upfront-carbon option is not automatically the best whole-life option if it leads to corrosion, water ingress, early replacement or difficult maintenance. The masonry cavity wall has relatively high material mass, but it may be justified if it provides fire robustness, acoustic performance, weather resistance and long service life.

Steelwork, façade fixings, wall ties and support systems should be specified for marine-influenced corrosion risk. Maintenance access must also be considered early, because inaccessible façades and plant areas increase cost, disruption and carbon from repair works. Sealant replacement, façade inspection, cleaning access and rooftop plant replacement routes should be coordinated from the outset. The most sustainable AS1 outcome is therefore a durable, maintainable and material-efficient design supported by WLCA, energy modelling and EPD-based procurement.

## 5.5 Sustainability Evidence Table

| Issue | Current AS1 position | Sustainability concern | Possible improvement | Evidence/source | Expected benefit |
| --- | --- | --- | --- | --- | --- |
| Structural frame | Steel composite frame with RC core | High upfront embodied carbon from steel and concrete | Optimise grid, reduce member sizes, require EPDs, use verified recycled-content steel | RICS (2024); Institution of Structural Engineers (2020); SteelConstruction.info (n.d.) | Lower embodied carbon and improved adaptability |
| Provisional foundations | AS1 assumed deep piles with ground beams and suspended slab | Concrete and reinforcement volumes may be unnecessary if full piling is not GI-justified | Use GI to test shallow foundations, raft, piled raft, ground improvement and piles before design freeze | British Geological Survey (n.d.); British Standards Institution (2020); BRE (2005); RICS (2024) | Lower material use, reduced carbon and better cost/programme certainty |
| Masonry envelope | Masonry cavity wall with non-combustible insulation | High material mass and workmanship risk | Improve detailing, reduce waste, inspect cavities and use durable ties | British Standards Institution (n.d.); LABC Warranty (n.d.) | Longer service life and lower defect risk |
| Operational energy | Fabric-first approach assumed | Actual energy use not yet tested | Undertake CIBSE TM54 modelling and thermal bridge review | CIBSE (2022) | Better prediction of heating/cooling demand |
| Construction waste | Hybrid MMC plus site-built masonry | Offcuts, pallets, packaging and rework | Standardisation, take-back schemes and waste segregation | Construction Leadership Council (2020); WRAP (2011) | Less landfill and improved material recovery |
| Water and drainage | Drainage not yet developed | Runoff, flood screening and SAB approval may affect layout and programme | Develop SuDS, exceedance routing and construction water-control plan early | Welsh Government (2019); Welsh Government (2025a) | Better runoff control, resilience and compliance |

# 6. Flexibility and Future Adaptability Analysis

Flexible buildings are less likely to need early demolition, major structural changes, or carbon-intensive refurbishment. The steel composite frame with reinforced concrete core gives the AS1 hotel flexibility because internal partitions can be changed without changing the main structure. This is more flexible than full volumetric modular construction, which may limit future reconfiguration due to module widths, stacked load paths, and fixed service zones (SCI, n.d.; UK Government, n.d.). Hotel bedrooms, corridors, and risers are likely to repeat, increasing flexibility. Standard layouts, repeated MEP distribution, and easier refurbishment are possible with a regular grid and floorplate. Insufficient space for service upgrades in the final grid, floor-to-floor heights, and core position limits flexibility. Thus, the design should avoid tight ceiling voids and include accessible risers, coordinated service zones, and spare capacity for ventilation, electrical, and low-carbon technology upgrades.

Do not consider the reinforced concrete core background structure. It fixes main circulation, risers, vertical servicing, floorplate planning strategy, and fire-protected escape. Thus, future layout changes are most likely in the bedroom and public-area zones, not the core. Due to their cellular room layouts, they could be used as student housing, serviced apartments, or healthcare accommodations. Different daylight, servicing, floorplate, and occupancy requirements make office conversion harder.

Masonry cavity walls are durable but less flexible than panelized façades for major façade upgrades. Windows, insulation interfaces, and façade access should allow glazing units, seals, and internal linings to be replaced without disruption to improve adaptability. ISO 20887 principles of adaptability and design for future change are supported (ISO, 2020).

The AS1 proposal adapts moderately to well. The structural concept is less important than grid regularity, service capacity, façade access, riser space, and maintainable components, which determine its flexibility.

## 6.1 Adaptability Checklist

| Adaptability factor | Current provision | Limitation | Improvement | Priority |
| --- | --- | --- | --- | --- |
| Structural grid | Steel composite frame likely allows non-loadbearing internal partitions | Exact grid/span not confirmed | Use regular grid and avoid unnecessary transfer structures | High |
| Core position | RC core provides stability, stairs, lifts and service routes | Core fixes circulation, risers and major re-planning options | Test core location against future room, servicing and plant replacement options | High |
| Floor-to-floor height | Not confirmed at AS1 stage | Tight voids could restrict future MEP upgrades | Allow adequate ceiling/service zones | High |
| Internal layouts | Hotel bedrooms support repetition | Cellular layout less suited to open-plan office conversion | Design partitions as non-structural where possible | Medium |
| MEP risers | Hybrid MEP/module strategy assumed | Future capacity unknown | Include accessible risers and spare service capacity | High |
| Façade upgrade potential | Masonry cavity wall gives durability | Major façade retrofit may be disruptive | Use replaceable windows, accessible seals, maintainable fixings and inspection/cleaning access | Medium |
| Alternative future use | Could suit student accommodation, serviced apartments or key-worker accommodation | Less suited to open-plan office use | Keep room modules and service zones adaptable | Medium |

# 7. Comparative Evaluation of Alternatives

This section compares the AS1 preferred solution with reasonable alternatives. A weighted appraisal is used because the options cannot be judged on one criterion alone. Buildability/logistics and embodied carbon are weighted heavily because the site is constrained and the likely carbon hotspots are the frame, core and foundations. The scores are pre-GI and pre-contractor judgements, not final procurement decisions.

## 7.1 Options to Compare

| Option | Description | Why it is relevant | Key assumptions |
| --- | --- | --- | --- |
| A. AS1 preferred solution | Steel composite frame, RC core, provisional foundation strategy, masonry cavity wall and selective DfMA/MMC | Baseline proposal carried forward from AS1 | Steel frame reduces weight; RC core provides stability; foundations remain GI-dependent |
| B. RC flat slab frame | Reinforced concrete flat slab frame with RC core | Common hotel/residential medium-rise option | Better fire/acoustic mass and flat soffits, but heavier and slower than steel |
| C. Precast concrete frame | Factory-made concrete frame or hybrid concrete components | Tests whether more off-site structural manufacture improves quality and programme | Connection, lifting, transport and disproportionate-collapse design must be controlled |
| D. Volumetric modular | Factory-made hotel room modules with core/transfer structure | Relevant because hotels use repeated room layouts | Fast in theory, but transport, cranage, tolerance and early design-freeze risks are significant |
| E. CLT / hybrid timber | Mass timber or timber-hybrid frame with non-combustible compliant external wall strategy | Tests the low-carbon structural narrative | Requires specialist fire, moisture and acoustic strategy; external wall compliance is critical |
| F. Steel frame with panelised façade | Steel composite frame with more off-site envelope manufacture | Tests whether more prefabrication improves programme | Faster enclosure, but joints, lifting, fire-stopping and coastal durability need careful control |

## 7.2 Decision Matrix

| Option | Buildability/logistics | Programme and certainty | Embodied carbon | Fire/durability/flexibility | Weighted judgement |
| --- | --- | --- | --- | --- | --- |
| A. Steel composite frame + RC core | Strong: familiar frame, repeatable floors, selective MMC | Strong: fast dry frame and known UK supply chain | Moderate: steel/core carbon needs EPD-led optimisation | Good: adaptable floorplate; core fixes risers | 85/100 - preferred balanced baseline |
| B. Steel frame + panelised façade | Good but tolerance/jointing risk increases | Strong: faster enclosure if supplier is mature | Moderate: depends on panel and steel EPDs | Moderate: façade replacement and fire-stops need QA | 78/100 - strong alternative if programme dominates |
| C. RC flat slab + RC core | Moderate: slower wet trades and heavier frame | Moderate/low: longer curing and follow-on sequencing | Weaker: higher concrete volume and foundation demand | Strong: acoustic mass, fire resistance and flat soffits | 66/100 - viable fallback if robustness dominates |
| D. Precast concrete frame | Moderate: factory quality but lifting/connection risk | Good if route and craneage are proven | Weaker: concrete-intensive unless optimised | Moderate: robustness depends on connection design | 63/100 - conditional option |
| E. Volumetric modular | Weak/moderate: abnormal loads and craneage constrain site | Strong in theory, weaker if routeability fails | Moderate: less waste but transport/module limits matter | Lower: fixed module sizes and service routes | 62/100 - not preferred for this site |
| F. CLT / hybrid timber | Weak/moderate: specialist fire/moisture control needed | Moderate: supply-chain and approval complexity | Strong potential, but must be proven whole-life | Moderate/low: fire, acoustic and moisture strategy critical | 61/100 - low-carbon option but high approval risk |

AS1 has the best balance but isn't perfect. Steel composite frame with RC core is best for programme speed, superstructure weight reduction, service integration, and UK delivery routes. Steel with a panelized façade is the closest alternative for program-critical applications, but jointing, tolerance, and fire-stopping quality assurance are difficult. RC flat slab construction is heavier and slower but good for acoustic mass, fire resistance, and flat soffits. The transport, craneage, abnormal-load, and early design-freeze risks of volumetric modular make it unsuitable for coastal, hospital-adjacent environments. Because GI, contractor input, and supplier EPDs can change the ranking, use it as a framework.

# 8. Recommendations

| No. | Recommendation | Problem it addresses | Evidence from analysis | Expected benefit | Implementation issue |
| --- | --- | --- | --- | --- | --- |
| 1 | Keep the AS1 steel composite frame with RC core, but optimise the grid and member sizes before procurement. | Embodied carbon, steel tonnage and future layout constraints. | Steel composite frame performed best overall, but carbon and flexibility depend on efficient design (RICS, 2024; Institution of Structural Engineers, 2020). | Lower embodied carbon, better floorplate adaptability and reduced foundation demand. | Requires early structural coordination and contractor input. |
| 2 | Complete intrusive GI before finalising piles and substructure design. | Risk of overdesign, obstruction delays, uncertain groundwater and avoidable foundation carbon. | Piled foundations may be required, but they should not be treated as the only justified solution until boreholes, in-situ testing, monitoring, contamination/aggressive-ground testing and utility proving are complete (British Geological Survey, n.d.; British Standards Institution, 2020; BRE, 2005). | Reduced risk of overdesign, lower concrete and reinforcement quantities, better spoil strategy and improved programme certainty. | GI must be commissioned early enough to influence design before structural and cost plans are fixed. |
| 3 | Prepare a detailed Construction Logistics Plan. | Hospital-adjacent disruption, constrained access and delivery congestion. | The site is a live institutional frontage with hospital, university, bus and Mumbles Road movement (Swansea Bay University Health Board, n.d.; Swansea University, n.d.; Transport for London and CLOCS, 2021). | Safer deliveries, fewer delays and reduced disruption to surrounding users. | Requires agreement with local highway, hospital and university stakeholders. |
| 4 | Use a façade sample bay and inspection hold points before closing cavities. | Water ingress, fire-stopping defects and workmanship risk. | Masonry cavity wall is durable but quality-dependent in severe exposure (British Standards Institution, n.d.; LABC Warranty, n.d.). | Lower defect risk, stronger fire compliance and better weather resistance. | Adds early programme time but reduces later rework. |
| 5 | Apply selective DfMA/MMC rather than full volumetric modular construction. | Craneage, transport, wind, routeability and interface risk. | Full volumetric construction is less suitable unless abnormal-load routing, craneage and early design freeze are proven (SCI, n.d.; UK Government, n.d.; National Highways, n.d.). | Better quality and waste control without excessive logistics risk. | Needs early supplier coordination and standardised room/service layouts. |
| 6 | Undertake WLCA and CIBSE TM54 modelling at detailed design stage. | Current sustainability analysis is qualitative only. | Whole-life carbon and operational performance require quantified testing with measured quantities, supplier EPDs and realistic energy assumptions (RICS, 2024; CIBSE, 2022). | Evidence-based carbon reduction and more reliable energy-performance prediction. | Requires cost, quantity, product and services data. |

# 9. Conclusion

The AS1 approach is buildable, sustainable, and flexible if the main risks are managed. The steel composite frame with RC core supports efficient floor cycles and a logical sequence, but site logistics, craneage, substructure uncertainty, and façade workmanship are critical risks.

This proposal has sustainability potential due to its lighter structural frame, hybrid DfMA/MMC, and durable envelope. However, reducing steel and concrete quantities, avoiding unnecessary piling, specifying durable coastal materials, controlling waste, and WLCA and operational energy modelling determine its true performance.

Regular hotel layout, steel frame, and non-loadbearing internal partitions offer moderate-to-good flexibility. The fixed RC core, service risers, floor-to-floor height, and masonry façade upgrade constraints are the main constraints.

Only after evidence-based modification should the AS1 steel composite frame with RC core remain the preferred baseline. The foundation strategy should be provisional until GI decides whether full piling or a lower-carbon alternative can meet bearing, settlement, groundwater, and durability requirements. We recommend selective MMC for steelwork, MEP modules, plant skids, windows and possibly bathroom pods, but full volumetric modular construction should be rejected without routeability, craneage, tolerance control and early design freeze. The biggest improvement is moving from concept assumptions to evidence-based design using GI, coordinated logistics, façade testing, EPD-led procurement, WLCA, and energy modelling.

# 10. Reference List

- British Geological Survey (n.d.) GeoIndex onshore. Available at: https://www.bgs.ac.uk/map-viewers/geoindex-onshore/ (Accessed: 22 April 2026).

- British Standards Institution (2010) BS EN 1991-1-4:2005+A1:2010 Eurocode 1: Actions on structures. General actions. Wind actions. London: BSI.

- British Standards Institution (2018) BS 1377-3:2018 Methods of test for soils for civil engineering purposes. Chemical and electro-chemical tests. London: BSI.

- British Standards Institution (2020) BS 5930:2015+A1:2020 Code of practice for ground investigations. London: BSI.

- British Standards Institution (2023) BS 8500-1:2023 Concrete. Complementary British Standard to BS EN 206. Method of specifying and guidance for the specifier. London: BSI.

- British Standards Institution (n.d.) BS 8104: Code of practice for assessing exposure of walls to wind-driven rain. London: BSI.

- BRE (2005) Special Digest 1: Concrete in aggressive ground. Available at: https://bregroup.com/documents/311572/1990118/189849.pdf/8c07e3b0-0dc6-ebf3-c5c8-ea8b27a28db4 (Accessed: 22 April 2026).

- CIBSE (2022) TM54: Evaluating operational energy use at the design stage. Available at: https://www.cibse.org/knowledge-research/knowledge-portal/tm54-evaluating-operational-energy-use-at-the-design-stage-2022 (Accessed: 22 April 2026).

- City and County of Swansea (n.d.) Swansea Tall Buildings Strategy. Available at: https://www.swansea.gov.uk/media/8105/swansea-tall-buildings-strategy/pdf/Swansea_Tall_Buildings_Strategy.pdf (Accessed: 22 April 2026).

- Construction Leadership Council (2020) Zero avoidable waste in construction: how best to interpret it. Available at: https://www.constructionleadershipcouncil.co.uk/wp-content/uploads/2016/05/ZAW-Report-Final-Draft-25-February-2020.pdf (Accessed: 25 April 2026).

- Construction Plant-hire Association (2025) Planning and control of lifting operations in wind: CPA-CIG/TCIG TIN-110. Available at: https://cpa.uk.net/wp-content/uploads/2025/08/CPA-CIG_TCIG-TIN-110-Planning-Control-of-Lifting-Operations-in-Wind-V1-July-2025.pdf (Accessed: 25 April 2026).

- Health and Safety Executive (n.d.) Fire safety in construction. Available at: https://www.hse.gov.uk/construction/faq-fire.htm (Accessed: 25 April 2026).

- Institution of Civil Engineers (2025) What is PAS 2080? 2023 version. Available at: https://www.ice.org.uk/news-views-insights/inside-infrastructure/what-is-pas-2080-2023-version (Accessed: 01 May 2026).

- Institution of Structural Engineers (2020a) Carbon footprint benchmarking data for buildings. Available at: https://www.istructe.org/IStructE/media/Public/TSE-Archive/2020/Carbon-footprint-benchmarking-data-for-buildings.pdf (Accessed: 01 May 2026).

- Institution of Structural Engineers (2020b) How to calculate embodied carbon. 2nd edn. Available at: https://www.istructe.org/resources/guidance/how-to-calculate-embodied-carbon/ (Accessed: 01 May 2026).

- International Organization for Standardization (2017) ISO 12944-2:2017 Paints and varnishes - Corrosion protection of steel structures by protective paint systems - Part 2: Classification of environments. Geneva: ISO.

- International Organization for Standardization (2020) ISO 20887:2020 Sustainability in buildings and civil engineering works - Design for disassembly and adaptability - Principles, requirements and guidance. Available at: https://www.iso.org/standard/69370.html (Accessed: 01 May 2026).

- LABC Warranty (n.d.) Technical Manual V11: Section 6 external walls. Available at: https://www.labcwarranty.co.uk/hubfs/Technical%20Manual%20V11%20-%20Thumbnails/Technical%20Manual%20V11%20-%20Section%20PDF%20Downloads/Section%206%20External%20Walls.pdf (Accessed: 01 May 2026).

- Marshalls (n.d.) Brick Technical Bulletin 9: Resistance to rain penetration. Available at: https://media.marshalls.co.uk/image/upload/v1612169211/Resistance-To-Rain-Penetration-BTB9.pdf (Accessed: 04 May 2026).

- Met Office (n.d.) Mumbles Head climate averages. Available at: https://www.metoffice.gov.uk/research/climate/maps-and-data/location-specific-long-term-averages/gcjjm7j5g (Accessed: 11 May 2026).

- NATSPEC (n.d.) DES 010 Atmospheric corrosivity categories. Available at: https://www.natspec.com.au/images/TECHnotes/NTN-DES-010-Atmospheric-corrosivity-categories.pdf (Accessed: 04 May 2026).

- National Highways (n.d.) Abnormal loads and the ESDAL system. Available at: https://nationalhighways.co.uk/road-safety/abnormal-loads-and-the-esdal-system/ (Accessed: 04 May 2026).

- RICS (2024) Whole life carbon assessment for the built environment. 2nd edn. Available at: https://www.rics.org/profession-standards/rics-standards-and-guidance/sector-standards/construction-standards/whole-life-carbon-assessment (Accessed: 04 May 2026).

- SCI (n.d.) Building design using modules. SCI P348. Available at: https://www.steelconstruction.info/images/6/67/SCI_P348.pdf (Accessed: 04 May 2026).

- SteelConstruction.info (n.d.) Composite construction. Available at: https://www.steelconstruction.info/Composite_construction (Accessed: 04 May 2026).

- Swansea Bay University Health Board (n.d.) Singleton Hospital. Available at: https://sbuhb.nhs.wales/hospitals/our-hospitals/singleton-hospital/ (Accessed: 11 May 2026).

- Swansea University (n.d.) Location. Available at: https://www.swansea.ac.uk/the-university/location/ (Accessed: 11 May 2026).

- Transport for London and CLOCS (2021) Construction Logistics Planning Guidance. Version 1.2. Available at: https://content.tfl.gov.uk/clp-guidance-clocs.pdf (Accessed: 04 May 2026).

- UK Government (n.d.) Volumetric modular construction research. Available at: https://www.gov.uk/government/publications/volumetric-modular-construction-research/volumetric-modular-construction-research (Accessed: 04 May 2026).

- Welsh Government (2019) Statutory national standards for sustainable drainage systems. Available at: https://www.gov.wales/sites/default/files/publications/2019-06/statutory-national-standards-for-sustainable-drainage-systems.pdf (Accessed: 04 May 2026).

- Welsh Government (2021) Building regulations guidance: Part B fire safety. Volume 2: Buildings other than dwellinghouses. Available at: https://www.gov.wales/sites/default/files/publications/2021-12/building-regulations-guidance-part-b-fire-safety-volume-2-buildings-other-than-dwellinghouses.pdf (Accessed: 09 May 2026).

- Welsh Government (2024) Planning Policy Wales: Edition 12. Available at: https://www.gov.wales/sites/default/files/publications/2024-07/planning-policy-wales-edition-12.pdf (Accessed: 09 May 2026).

- Welsh Government (2025a) Technical Advice Note 15: Development, flooding and coastal erosion. Available at: https://www.gov.wales/sites/default/files/publications/2025-03/technical-advice-note-15-development-flooding-and-coastal-erosion.pdf (Accessed: 09 May 2026).

- Welsh Government (2025b) Approved Document 7: Materials and workmanship. Available at: https://www.gov.wales/sites/default/files/publications/2025-06/approved-document-7-to-support-regulation-materials-and-workmanship.pdf (Accessed: 09 May 2026).

- Welsh Government (2025c) Building (Amendment) (Wales) Regulations 2025: Welsh Government Circular WGC 004/2025. Available at: https://www.gov.wales/building-amendment-wales-regulations-2025-wgc-0042025-html (Accessed: 11 May 2026).

- Welsh Government (2025d) Amendments to Approved Document B: Fire safety. Volume 1 and Volume 2. Available at: https://www.gov.wales/sites/default/files/publications/2025-06/amendments-to-approved-document-B-fire-safety-volume-1-and-volume-2.pdf (Accessed: 11 May 2026).

- WRAP (2011) Using construction consolidation centres to reduce construction waste and carbon emissions. Available at: https://clocs.org.uk/wp-content/uploads/2025/06/Using-Construction-Consolidation-to-reduce-construction-waste-and-carbon-emissions-WRAP.pdf (Accessed: 11 May 2026).