| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| may 2019 | mai 2019 |

MODULE CODE: M4X00386

ENGINEERING SCIENCE 2

TIME:  2 HOURS

answer all questions

Equipment or materials required – Calculator, Thermodynamics Tables

FORMULAE BOOKLET FOR ELECTRICAL ENGINEERING

| Q=CV | Q=It | V=IR | V=V0e-tRC | I=I0e-tRC |
| --- | --- | --- | --- | --- |
| Q=Q0e-tRC | P=IV | P=I2R | P=V2R | E=12CV2 |
| E=12QV | ρ=RAl | RT=R1+R2+R3 | 1RT=1R1+1R2+1R3 | V=ε-Ir |
| Vrms=V02 | Irms=I02 | Vrms=IrmsR |  |  |

FORMULAE BOOKLET FOR THERMODYNAMICS

- BASIC CONCEPT:

The change in the energy content of a body or any other system is equal to the difference between the energy input and the energy output, and the energy balance is expressed as:

∆E=Ein-Eout

E2 – E1

WORK:

Work done by the system is considered: POSITIVE

Work done on the system is considered: NEGATIVE

HEAT:

Heat transfer into the system is considered: POSITIVE.

Heat transfer out of the system is considered: NEGATIVE.

- CLOSED SYSTEMS:

Non-flow energy equation:

∆U=Q-W

- ENTHALPY:

H=U+P.V           (J)

Where:

- U = internal energy

- P = pressure

- V = volume

- FLOW WORK:

The flow work, or flow energy, or again displacement energy:

Wflow=P×V

- OPEN SYSTEMS / Steady-flow systems

- Mass Balance:

inm=outm                       (kg/s)

- Energy Balance:

Q-W=outm.h+12.c2+g.z-inm.h+12.c2+g.z

- CONTINITY OF MASS FLOW:

m=mass flow in kgs                                     A=cross sectional area in m2

v=specific volume in m3kg                       c=velocity in ms

Consider the volume of substance passing the section per second = V in m3/s

V=m.v      also     V=A.c   (Continuity equation)

Therefore:

m.v=A.c

From which

m=A.cv

- OPEN SYSTEMS / Unsteady-flow systems

- Mass Balance:

mi-me=m2-m1CV

Where:

- i = inlet, and e = exit.

- 1 = initial state, and 2 = final state of the control volume.

- Often one or more terms in this equation are zero.

- Energy Balance: (when kinetic and potential energy changes associated with the control volume and fluid streams are negligible)

Q-W=outmh-inmh+m2u2-m1u1CV

- IDEAL GAS:

We defined an ideal gas as a gas whose temperature, pressure, and specific volume are related by:

P.v=R.T

where:

- R is the gas constant (kJ/kg.K) or (kPa.m3/kg.K)

- P the absolute pressure (kPa)

- T the absolute temperature (K)

- v is the specific volume (m3/kg)

The change in internal energy or enthalpy for an ideal gas during a process from state 1 to state 2 are defined as:

u2-u1=cvT2-T1       (kJkg)

h2-h1=cpT2-T1       (kJkg)

where:

- cv is the specific heat at constant volume

- cp is the specific heat at constant pressure

Reminder:

°C=K-273                                K=°C+273

ELECTRICAL ENGINEERING

- QUESTION ONE (10 marks)

- QUESTION TWO (20 marks)

Figure 2 shows an ac waveform that is displayed on an oscilloscope screen.

Figure 2

a)	The time base of the oscilloscope is set at 3ms per division and the y-gain at 3V per division.

i	Calculate the period of the waveform							[2]

ii	Calculate the peak to peak voltage							[2]

iii	Calculate the frequency								[2]

iv	Calculate the rms current if this voltage was across a 20kΩ resistor 		[2]

b)	State and explain the effect on the oscilloscope if the time base is switched off	[2]

c)	In the US, domestic users are supplied with mains electricity at a root mean square voltage of 110V

Figure 3

i	State what is meant by root mean square voltage					[2]

ii	Calculate the peak value of the supply voltage					[2]