University of Wales Trinity Saint David

School of Engineering

“An Assessment of Automotive Fuels from a Well to Wheel Perspective”

Victoria Greene

P142884

Automotive Engineering BSc

23/04/2018

Research Methods & Project M3X9486

# Abstract

“A technical comparison of the energy losses and emissions produced via various automotive fuels from a well to wheel life cycle perspective”

Contents

Abstract	2

Introduction	12

Aim 2	12

Layout of Report:	13

Refinery Stage	14

1.0 Research	15

1.1. Extraction of Crude Oil	15

1.2 Transportation of Crude Oil	15

1.3 Refineries in Britain	15

1.4 Distillation Process of Crude Oil:	16

1.5 Removal of Sulphur	17

1.6 Crude Oil Products	17

1.7 Previous and Current Electricity Mix	18

1.7.1 Future Electricity Mix	18

1.7.2 Assessing the United Kingdom’s General Energy Usage and transport Specific Usage	19

1.8 Renewable Energy Sources	19

1.9 Generating Electricity from Non-renewable sources	20

1.9.1 Producing Electricity from Coal	20

2.0 Methodology	21

2.1 Life cycle Assessments	21

3.0 Results	22

3.1 LCA Data Gathered from European Life Cycle Database	22

3.1.1 “Gasoline mix (regular) at refinery from crude oil and bio components”	22

3.1.2 “Diesel at refinery from crude oil and bio components”	23

3.1.3 Electricity – LCA Data	24

4.0 Analysis	25

4.1 Comparison of Gasoline, Diesel and Electricity	25

4.2 Efficiency Calculations:	26

4.3 Graphical Analysis of Energy Consumption (Energy Required)	27

4.4 Graphical Analysis of Energy Consumption (Carbon Dioxide Produced)	28

5.0 Validation data (Gabi)	29

5.1 Gasoline	29

5.2 Diesel	29

5.3 Electricity	30

5.4 Gabi Credibility	31

5.5 European Life Cycle Database Credibility	31

6.0 Conclusion	32

Refinery to Storage Distribution Points	33

(Via Pipeline)	33

1.0 Research	34

2.0 Methodology	35

2.1 Forming a Case Study:	36

2.2 Description of Case Study	36

2.3 Case Study Starting Point	37

2.4 Velocity	39

2.5 Reynolds Number (required for Darcy’s Friction Factor)	39

2.6 Two Methods for Calculating a Friction Factor (Turbulent Flow)	40

2.6.1 Blasius’ Law	40

2.6.2 Moody’s friction factor:	40

2.7 Specific Gravity/ Relative Density:	41

2.8 Length of Pipe, Diameter and Losses Due to Elevation	42

2.8.1 Elevation	42

2.8.2 Start and End Latitudinal/Longitudinal Coordinates:	43

2.8.3 Mapping Tools	44

2.8.4 Length of Pipeline:	44

3.0 Results	45

3.1 Excel Workbook of Collected Data	45

3.2 Results Applicable to Diesel and Gasoline	46

3.3 Gasoline Specific Result	47

3.4 Diesel Specific Result	48

3.5 Total Pressure Loss	49

3.6 Final Pressure and Pressure Change Values	49

3.7 Pipeline Efficiency	50

3.7.1 Hydraulic and Thermal Efficiency	50

3.7.1.1 Hydraulic Efficiency:	50

3.8 Pumping Power Required	51

3.9 Volumetric Flow Rate:	52

3.10 Mass Flow Rate:	52

3.11 Efficiency and Inefficiency	53

3.12 Gasoline Specific Result:	54

3.13 Diesel Specific Result:	55

3.14 Validation of result in comparison to Gabi data	56

3.15 Gabi Results	56

3.16 Summary of Pipeline Efficiencies	57

3.17 Thermal efficiency:	58

3.17.1 Efficiency Vs Pump Efficiency Curve	58

4.0 Analysis	59

4.1 Pipeline Loses – General Analysis	59

4.2 Gasoline and Diesel Results Compared	59

4.3 Elevation	59

4.4 Volumetric and Mass Flow Rate	59

4.5 Differences in Efficiency between Gasoline and Diesel	59

4.6 Roughness of the Pipe	60

4.7 Pressure Losses	60

4.8 Length of Pipe	60

4.9 Gabi Result Analysis	60

4.9.1 Efficiency Value	61

5.0 Conclusion	61

Refinery to Storage Distribution Points	62

(Via Ship)	62

1.0 Research	63

1.1 Fuel consumption of Oil Tankers, Ocean and Coastal	63

2.0 Methodology	64

3.0 Results	64

3.1 Efficiency calculations:	65

3.2 Values converted to capacity	66

4.0 Analysis	67

4.1 Co2  Results	67

4.2 Energy Consumption	67

4.3 Efficiency	67

4.4 Additional Transport	67

5.0 Conclusion	67

Distribution from Storage Points to Local Garage	68

(Via Lorry)	68

1.0 Research	69

1.1 Amount of fuel sold in the UK 2017:	69

2.0 Methodology	70

2.1 Amount of fuel a single large road tanker can carry at full capacity	70

2.2 Maximum Road Tanker Journeys	70

2.3 Average Road Tanker Journeys	71

2.4 Lorry Fuel Usage per Journey	72

2.5 Energy Usage per Journey	72

2.6 Co2 Produced per average Journey	72

2.7 Total energy within the fuel being transported	72

2.8 Efficiency	73

2.9 Amount of Co2 produced per 1Mj of fuel energy carried within average journey	73

2.10 Energy consumed per 1Mj of energy carried	73

3.0 Validation	74

3.1 Euro 5 Regulated Diesel Tanker Truck, Gross weight above 32t	74

3.2 Comparison to case study calculated data:	74

4.0 Analysis	75

5.0 Conclusion	76

Usage Phase	77

1.0 Research	78

1.1 Drive cycle testing	78

1.2 NEDC	78

1.2.1 Urban Phase	78

1.2.2 Extra Urban Phase	78

1.3 The Effect of Modern Dynamometers on NEDC Vehicle Inertia Categories	79

1.4 WLTP	80

1.5 Graphically described differences between NEDC and WLTP	81

1.5.1 Acceleration Variation	82

1.5.2 Velocity differences	83

1.5.4 General Differences	84

1.6 Differences in the vehicle efficiency predicted by the WLTP and NEDC	85

2.0 Methodology	86

3.0 Results Gathered from the UCSB Automotive Energy & GHG Model:	87

3.1 Gasoline Results:	88

3.1.2 Gasoline Continued:	89

3.2 Diesel Results:	90

3.2.1 Diesel Results Continued:	91

4.0 Analysis	92

4.1 Compact – NEDC	92

4.1.1 Compact - WLTP	93

4.1.2 ICE and BEV differences in energy consumption:	94

4.1.3 Diesel and Gasoline Energy Consumption Differences:	94

4.1.4 Decreased Greenhouse Gas Values for the Diesel Set Up	94

4.2 Mid-Size NEDC	95

4.2.1 Mid- Size WLTP	96

4.2.3 Mid-Size Vehicle Analysis	97

Comparison of Energy Consumption for Mid-Size Vehicles (NEDC + WLTP)	97

4.2.4 Differences between Gasoline and Diesel	97

4.2.5 WLTP Showing Lower Energy Consumption for Both Gasoline and Diesel	97

4.2.6 Default Fuel Energy Demand	98

4.2.7 Reasoning for Unexpected WLTP Result:	98

4.3 SUV – NEDC	99

4.3.1 SUV - WLTP	100

4.3.2 SUV Vehicle Analysis	101

4.4 Percentage Difference in Readings Between the NEDC and WLTP	102

4.5 Validation of Results from Ford compared to UCSB Automotive Energy & GHG Model:	103

4.5.1 Compact	103

4.5.2 Midsize	104

4.6 Efficiency Calculations:	105

5.0 Conclusions	106

Electricity Grid Losses (Electricity Transportation)	107

1.0 Electricity Grid Losses	108

1.1 Research	108

1.2 Types of Loses	108

1.3 Technical Loses – Causes	108

1.4 Electric Vehicles and the National grid	108

3.0 Analysis and Conclusion	109

Overall Analysis	110

1.0 Efficiency Comparison of Each Section	111

1.1 ICE Refinery Stage Calculations:	112

1.2 ICE Transportation Stage Calculations:	112

1.3 Electricity Refinery Value	112

1.4 Electricity Transportation Value (Overall Grid Losses):	112

1.5 Ice Usage Efficiency	112

1.6 Electricity Usage Efficiency	112

1.7 Overall Efficiency Calculation:	113

2.0 Graphical Comparison	114

2.2 Overall Efficiency	115

Overall Conclusion and Evaluation	116

Evaluation	117

Conclusion	117

Bibliography	118

Appendices	126

1.0 Renewable Energy Sources	126

1.1 Solar Power	126

1.2 Wind Power	127

1.3 Geothermal	128

1.4 Hydroelectricity	129

1.5 Biofuels	130

1.6 Tidal	131

Figure 1- Fractional Distillation Tower	16

Figure 2- Temperature of Distillation	16

Figure 3- Previous Electricity Grid Mix - United Kingdom	18

Figure 4- Projection of Electricity Mix - United Kingdom (GaBi, Database, 2013)	18

Figure 5- Energy Usage by Various Sectors	19

Figure 6- Input Flows to Make 1Kg of 'Gasoline Mix Regular'	22

Figure 7- Output Flows to Make 1Kg of 'Gasoline Mix Regular'	22

Figure 8- Summary Table of Flows for 'Gasoline Mix Regular'	22

Figure 9- Input Flows to Make '1Kg of Diesel Mix Regular’'	23

Figure 10-Output Flows to Make 1Kg of 'Diesel Mix Regular'	23

Figure 11- Summary Table of Flows for 'Diesel Mix Regular'	23

Figure 12- Input Flows to Make 3.6MJ of 'EU 27 Electricity Grid Mix'	24

Figure 13-Output Flows to Make 3.6MJ of 'EU 27 Electricity Grid Mix'	24

Figure 14- Summary Table of Flows to Make 3.6MJ of 'EU 27 Electricity Grid Mix'	24

Figure 15-Electiricy Summary Table of Energy Required and Produced	25

Figure 16-Gasoline Summary Table of Energy Required and Produced	25

Figure 17- Diesel Summary Table of Energy Required and Produced	25

Figure 18- Electricity Table - Per MJ	25

Figure 19- Gasoline Table - Per MJ	25

Figure 20- Diesel Table - Per MJ	25

Figure 21- Efficiency Summary Table at Refinery Stage	26

Figure 22-Graph of Energy Required at Refinery Stage to Create 1 MJ of Fuel	27

Figure 23- Graph of Carbon Dioxide Produced to Create 1MJ of Fuel	28

Figure 24- Validation Data of Input Flows to Make 1MJ of 'Gasoline Mix Regular'	29

Figure 25-Validation Data of Output Flows to Make 1MJ of 'Gasoline Mix Regular'	29

Figure 26-Validation Data of Input Flows to Make 1MJ of 'Diesel Mix Regular'	29

Figure 27- Validation Data of Output Flows to Make 1MJ of 'Diesel Mix Regular'	29

Figure 28- Validation Data of Input Flows to Make 3.6MJ of ‘EU-27 Electricity Grid Mix’	30

Figure 29- Validation Data of Output Flows to Make 3.6MJ of ‘EU-27 Electricity Grid Mix’	30

Figure 30- Example of Academic Paper Using GaBi	31

Figure 31- Reynolds Number - Variable Table	39

Figure 32- Moody's Friction Factor - Variable Table	40

Figure 33- Booklet Image 2	42

Figure 34- Booklet Image 1	42

Figure 35- Mapping Example	43

Figure 36- Elevation Finder - Mapping Tools	44

Figure 37- Google Maps Image of Valero Energy	44

Figure 38- Section Of Pipeline -Pipeline Owner- Start and End Point	45

Figure 39- Table of Initial Information -Length, Diameter, Operating Pressure	46

Figure 40- Table of Latitude, Longitude and Elevation for Each Section of Pipeline Investigated	46

Figure 41- Table of Average Velocities, Length of Pipe, Diameter of Pipe and Dynamic Viscosity (Gasoline)	47

Figure 42- Table of Reynolds Number, Darcy Friction Factor and Specific Gravity (Gasoline)	47

Figure 43- Table of Friction Losses (Gasoline)	47

Figure 44- Table of Average Velocities, Length of Pipe, Diameter of Pipe and Dynamic Viscosity (Diesel)	48

Figure 45-Table of Reynolds Number, Darcy Friction Factor and Specific Gravity (Diesel)	48

Figure 46- Final Pressure and Pressure Change Values (Gasoline)	49

Figure 47-Final Pressure and Pressure Change Values (Diesel)	49

Figure 48- Initial Pumping Power Required, Pumping Power to Overcome Losses and Total Pumping Power Required – (Gasoline)	54

Figure 49- Total Energy Required per Second, Volumetric Flow Rate and Mass Flow Rate (Gasoline)	54

Figure 50- Amount of Energy in 1Kg Of Fuel, Inefficiency and Efficiency	54

Figure 51- Initial Pumping Power Required, Pumping Power to Overcome Losses and Total Pumping Power Required – (Diesel)	55

Figure 52-Total Energy Required per Second, Volumetric Flow Rate and Mass Flow Rate (Diesel)	55

Figure 53-Amount of Energy in 1Kg Of Fuel, Inefficiency and Efficiency	55

Figure 54- Gabi Validation Result	56

Figure 55-Calculated Efficiency and Gabi Efficiency Comparison	57

Figure 56- Efficiency Vs Pump Efficiency Curve	58

Figure 57- Table of River freight ship – GaBi – Upstream (4000t capacity)	64

Figure 58-Table of River freight ship – GaBi – Downstream (4000t capacity)	64

Figure 59-Table of Oil Tanker - Ocean Going, average capacity	64

Figure 60- Table of Oil Tanker – Coastal, average, average capacity	64

Figure 61- Table of Oil Tanker – Ocean going 10000t-300000t capacity	64

Figure 62- Efficiency Table of Various Vessels Considered	65

Figure 63- Converted Figures into Capacity Format	66

Figure 64- Comparison of Dealer, Independent and Supermarket Fuel Throughput	69

Figure 65- Validation Data from GaBi - 1Kg of General Cargo Considered	74

Figure 66- Calculated Efficiency of Ship Compared to Gabi Calculated Efficiency	74

Figure 67- Altered Comparison Table into 100km	75

Figure 68- Graphical Description of NEDC Urban Stage	78

Figure 69- Graphical Description of NEDC Extra Urban Phase	78

Figure 70- Stepped Inertia Classes of Vehicles	79

Figure 71- Descriptive Table Including Graphs of Different Classes of the WLTP	80

Figure 72- Excel Plot of WLTP	81

Figure 73- Academic Journal Plot of WLTP	81

Figure 74- Acceleration Vs Time of NEDC	82

Figure 75- Acceleration Vs Time of WLTP	82

Figure 76- Velocity Vs Time of NEDC	83

Figure 77-Velocity Vs Time of WLTP	83

Figure 78- WLTP and NEDC Comparison Table 1	84

Figure 79- WLTP and NEDC Comparison Table 2	84

Figure 80- Vehicle Efficiency Plots Comparing NEDC and WLTP	85

Figure 81- Results of Battery Electric Vehicle (FSV - A Class)	87

Figure 82- Results of Battery Electric Vehicle FSV - C Class	87

Figure 83- Results of Gasoline Vehicle (Midsize and Compact)	88

Figure 84- Results of Gasoline (SUV)	89

Figure 85- Results of Diesel (Compact and Mid-size)	90

Figure 86- Results of Diesel (SUV)	91

Figure 87- Compact NEDC BASELINE - Compact	92

Figure 88- Energy Consumption of Compact Vehicles (NEDC)	92

Figure 89- Co2 Production for Compact Vehicles	92

Figure 90- (WLTP)(BASELINE)- Compact	93

Figure 91- Energy Consumption Comparison (WLTP)	93

Figure 92- Co2 Production (WLTP) Compact Vehicle	93

Figure 93- Example of Ice Vehicle Losses	94

Figure 94- Midsize NEDC (BASELINE)	95

Figure 95- Energy Consumption (NEDC) Mid-size	95

Figure 96- Co2 Production (NEDC) Mid-size	95

Figure 97- WLTP (BASELINE) Mid-Size	96

Figure 98- Energy Consumption (WLTP) Midsize	96

Figure 99- Co2 Production (WLTP) Mid-Size	96

Figure 100- Default Energy Demands Within Model	98

Figure 101- NEDC (BASEINE) SUV	99

Figure 102- Energy Consumption NEDC - SUV	99

Figure 103- Co2 Production NEDC - SUV	99

Figure 104- SUV (WLTP) (BASELINE)	100

Figure 105- Energy Consumption (WLTP) SUV	100

Figure 106- Co2 Production (WLTP) SUV	100

Figure 107- Attempted Comparison of Compact Results	102

Figure 108- Attempted Comparison of Mid-Size Results	102

Figure 109-Attempted Comparison of SUV Results	102

Figure 110- Comparison of Ford and World Autosteel NEDC Results (Compact)	103

Figure 111-Comparison of Ford and World Autosteel NEDC Results (Mid-size)	104

Figure 112- Original Well to Wheel Process of ICE Fuels and Electricity	111

Figure 113- Compositable Version of ICE fuels and Electricity	111

Figure 114- Refinery Efficiencies of Gasoline and Diesel Combined	112

Figure 115- Ice Transportation Efficiencies	112

Figure 116- Efficiency Comparison of ICE VS Electricity Throughout Each Stage	114

Figure 117-Overall Combined Efficiency Value for Electricity and ICE	115

# Introduction

Since the first fully operational internal combustion engine was invented in 1859, by J. J. Étienne Lenoir, (Vasilash, 2011) the modern world has progressively become dependent on the use of ICE in the transport industry. (Department of Transport, 2016) Interestingly, the idea of the electric vehicle is older than the internal combustion engine itself, which then begs the question of; why are electric vehicles not as widely utilized as gasoline or diesel-powered vehicles? (Husain, 2011)

Electric vehicles in the past shared much of the same complications of modern day E.Vs. The first usable electrified vehicle had a short range, heavy batteries and a very limited top speed, coincidentally some of the same issues are still prominent today. (Mehrdad Ehsani, 2010)

To add to the growing concerns of the usability of the electric vehicles, there are now highlighted infrastructure issues, as well as the controversy of how efficient the E.V can be when they depend on electricity that isn’t always generated in the most sustainable way. (International Energy Agency, 2011). It should be noted that there have been on-going considerate efforts globally to change the dynamic of producing electricity from mainly fossil fuels into renewable sources, however there is still a major reliance on the burning of fossil fuels. (World Enegry Council, 2016)

This paper will look to examine Electric and ICE powered vehicles using a method termed ‘the well to wheel process’, which involves considering electricity, gasoline and diesel from a life cycle perspective. Considering the energy used and emissions created from each stage of the WTW process will provide an expanded viewpoint on the possible effectiveness or ineffectiveness in the use of electricity to power a vehicle, in comparison to using gasoline or diesel.

The Well to Wheel process will cover the following aspects of Electricity, Gasoline and Diesel:

- The Refinery Stage of Gasoline and Diesel.

- Electricity Generation Considering the UK’s Electricity Grid Mix

- Transportation from Refinery to Storage/Distribution Points via Pipeline and Oil tanker

- Electricity Transportation via the National Grid

- Distribution Points to Local Fuel Stations via Lorry

- Use Phase of Gasoline, Diesel and Electric Vehicles of Various Sizes

To effectively carry out this expansive process in an organised fashion, two main aims will be considered whilst carrying out this investigation. 

Aim 1 – To effectively analyse the amount of energy lost within use of different energy sources, using the well to wheel process.

Aim 2 – To effectively analyse the amount of emissions produced within use of different energy sources, using the well to wheel process.

## Layout of Report:

Please note that because of the varied information contained within this report, each of the well to wheel processes are broken down into sections, with each section displayed in the usual technical report format. This unconventional format was necessary for readability purposes.

# Refinery Stage

## 1.0 Research

The first section of the well to wheel method covers a fundamental step in the utilisation of automotive fuels, the refinement process. This involves taking various types of energies whether it be from renewable or non-renewable sources and refining it into a state which can be easily used as a fuel source for passenger vehicles.

Topics discussed in this refinery research section will include the UK’s current and projected electricity mix, the distillation process of crude oil and electricity generation from renewable and non-renewable sources. Gasoline and Diesel are created predominantly from crude oil and for this reason details of crude oil extraction and refinement are detailed below.

### 1.1. Extraction of Crude Oil

Oil is created from carbon producing micro-organisms in the sea that similarly to coal experience extreme high pressures over extensive time periods eventually decaying into the substance we know as crude oil today. (John Andrews, 2007) Oil is extracted by drilling into ground that can be based on dry land or submerged offshore. Extraction of oil is often carried out deep beneath the sea bed where deep drilling and hydraulic/mechanical pumping is required. There is extensive research proving the harmful environmental effects that these actions cause, especially when incidents such as oil spills and mass contamination take place. (Maria Isabel Ramirez, 2016)

### 1.2 Transportation of Crude Oil

Oil is then transported most commonly via large cargo ships or pipelines to a refinery where a separation process will occur that allows several products to be obtained by fractional distillation. (Ömer Öztürkoğlu, 2016)

### 1.3 Refineries in Britain

Britain has seen a dramatic decrease it the number of operational refineries since the 60’s and 70’s of which there was a high dependency of fuel oil for power generation. Since then, more than half of Britain’s refineries have closed. (Times, 2010) The dependency on items such as fuel oil has not only decreased, but an increase in stringency in emission and efficiency regulations has caused many refineries to close because of the additional cost involved in keeping up with regulatory demands.

There are now only six major oil refineries throughout Britain which are detailed below:

- Essar Stanlow Refinery

- ExxonMobil Refinery Fawley

- Petroineos Grangemouth Refinery

- Phillips 66 Humber Refinery

- Total Lindsey Oil Refinery

- Valero Pembroke Refinery

(UK Petroleum Industry Assosiation, 2017)

### 1.4 Distillation Process of Crude Oil:

Crude oil is heated within a fractional distillation tower whereby the less dense products form vapours that travel up the tower through different levels that are separated by sieve like stages of various sizes that allow lighter (less dense) products to move up the levels and heavier (more dense) products to rest upon their suitable platform.

Figure 1- Fractional Distillation Tower

(Energy Education, 2015)

The image above shows the temperatures of which different crude oil product separate from its original state into vapours. Once vapours have travelled to their appropriate levels they are syphoned off and condensed back into liquid form. This process can be very energy intensive as vast amounts of heat energy are required. The image above also gives an indication of the energy required to harness various fuels, gasoline and kerosene rest near the top of the tower whereas diesel and fuel oil are situated toward the bottom demonstrating that more heat energy is required to separate diesel from crude than gasoline. This theory can be confirmed via the plot below:

Figure 2- Temperature of Distillation

(O. R. Abdurakhmanov, 2010)

The fractional distillation process can become more efficient if performed within a vacuum distillation unit, which allows the separation process to occur at lower temperatures. ( UK Petroleum Industry Assosiation, 2017)

### 1.5 Removal of Sulphur

The existence of sulphur in fuels has proven consequences in terms of increased sulphur dioxide emissions, tighter regulations are now in place to ensure that refineries must carry out a Desulphurisation process for both gasoline and diesel.

### 1.6 Crude Oil Products

Despite being a very energy intensive process, vast amounts of energy can be harnessed from the products separated from crude oil. Energy products include “liquid petroleum gas (LPG), gasoline, diesel, jet fuel, gas oil, heating oil and residues such as bitumen.” (UK Petroleum Industry Assosiation, 2017)

An explanation as to why the dependency on crude oil is still so apparent is the amount of energy that can be produced from crude oil related products such as gasoline; gasoline can provide approximately 45.7GJ of energy per metric ton.  (Golnik, 2003) Unfortunately, this dependency is not only based on a non-renewable and depleting source, it is a major source of greenhouse production, for example, burning a gallon of gasoline (4.546 Litres) produces approximately 8.89 Kg of Co2 (US Energy Information Administration, 2016)

### 1.7 Previous and Current Electricity Mix

Electric cars registered in the United Kingdom rely on the electricity mix specifically generated for consumption for the UK. This electricity mix consists of different amounts of varying energy sources, which are demonstrated below.

Figure 3- Previous Electricity Grid Mix - United Kingdom

(International Energy Agency, 2011)

Although the amount is decreasing, the UK is still greatly dependent on energy from coal and oil, suggesting that electric vehicles are also dependent on these finite and excessive emission producing energy sources.

### 1.7.1 Future Electricity Mix

Figure 4- Projection of Electricity Mix - United Kingdom 
(GaBi, Database, 2013)

The Projected electricity mix for the United Kingdom involves a continuation of large natural gas use, an increase in biofuels and nuclear, but lacking a substantial surge in renewables, however the use of coal falls progressively.

### 1.7.2 Assessing the United Kingdom’s General Energy Usage and transport Specific Usage

Recent Data for the United Kingdom shows that the transport industry is responsible forty percent of all fuel consumption, giving them a title of the main consuming sector in the UK. Within the transport sector, the amount of fuel used by vehicles operating on the road is seventy-four percent.

Figure 5- Energy Usage by Various Sectors

(Department for Business, Energy and Industrial Sector , 2017)

### 1.8 Renewable Energy Sources

Harnessing energy from natural resources that are renewable is beneficial as this process is more sustainable that utilizing finite sources. A brief overview of each renewable energy source available to the United Kingdom are detailed in the appendices. (Page 126)

### 1.9 Generating Electricity from Non-renewable sources

### 1.9.1 Producing Electricity from Coal

Coal is created as organic matter from thousands of years ago decays and experiences high pressures and temperatures. It is a highly concentrated energy source that is graded by its carbon content.

Coal fired power plants use coal to generate electricity by changing the phase state of water into steam by using heat generated from burning coal, steam is then used to move a turbine, which is connected to a generator, which produces an electric current. (John Andrews, 2007) (Godfrey Boyle, 2003)

The general statistic for the amount of energy released when one metric tonne of coal is burned is 28GJ. (Godfrey Boyle, 2003). Despite being an attractive energy source in terms of the amount of energy that can be utilised, coal produces large amounts of emissions, such as carbon dioxide and sulphur, contributing to issues such as global warming and acid rain. (John Andrews, 2007)

#### 1.9.2 Power Plant Processes

Steam power plants utilize different cycles to generate electricity, such as the Rankine cycle, Brayton Cycle, and combined cycles. Predominantly the cycles involve changing water into steam via heat generation from burning coal, but there are major differences in these cycles involving varying temperatures, and differences in the ‘steam quality’ produced. (John Andrews, 2007), to see brief overviews of each cycle, please refer to the appendices. (Page 132)

## 2.0 Methodology

### 2.1 Life cycle Assessments

Life cycle assessments are highly technical reports considering all inputs, outputs and processes involved in creating a product (Edelen, 2017). LCA’s are created by Life cycle assessment practitioners who are highly skilled professionals who are trained specifically to generate LCA’s that are a “factual analysis of a product’s entire life cycle in terms of sustainability” (PRé Consultants, 2017). Consequently, LCA’s are a highly credible and useful source when it comes to investigating the life cycle of automotive fuels.

They can also be used as an effective tool in decision-making processes for institutions such as governments and corporations, as it allows the assessor to gain a technical and expansive view of the situation being considered with relative ease. (LCA Consultants, 2015)

They allow organisations to consider their products from an environmental perspective, highlighting possible improvements that could be made at any stage in its production process. Clear numerical conclusions can be taken from LCA’s and used for persuasive marketing statements, for example, an LCA could highlight that a product produces lower co2 than its marketed counterparts. (ISO:14040, 1997-2006)

Life cycle assessments used in this report have been found at open source locations such as the European life cycle database, LCA database programs have also been be utilized for validation of data such as the industry recognised program Gabi. (Thinkstep-Gabi , 2018)

Unfortunately, specific datasets regarding the United Kingdom were unavailable at the time of this report, therefore datasets coving the European Union will be used instead as this is the closest most relevant data available.

## 3.0 Results

### 3.1 LCA Data Gathered from European Life Cycle Database

The inputs/outputs associated with the creation of gasoline at refinery stage are noted below, in reality the datasets include hundreds and sometimes thousands of inputs and outputs therefore only the relevant values that are related to emissions and energy consumption have been gathered and correlated into the tables below:

### 3.1.1 “Gasoline mix (regular) at refinery from crude oil and bio components”

Fuel supply – Production at Refinery

10 ppm Sulphur

5.75 wt % Bio Components

EU – 27”

| Input Flows to Make 1Kg of ‘Gasoline mix regular’ | MJ/KG | Amount |
| --- | --- | --- |
| Crude Oil | 43.3 MJ/Kg | 46.1339 MJ |
| Hard Coal | 26.3 MJ/Kg | 0.174546 MJ |
| Brown Coal | 11.9 MJ/Kg | 0.0916926 MJ |
| Natural Gas | 44.1 MJ/Kg | 5.01477 MJ |
| Uranium – Nuclear |  | 0.215316 MJ |
| Primary Energy – Geothermic |  | 0.00244361 MJ |
| Primary Energy – Hydro Power |  | 0.0534605 MJ |
| Primary Energy – Solar Energy |  | 1.43858 MJ |
| Primary Energy – Wind Power |  | 0.0234792 MJ |
| Secondary Fuel |  | 0.00349892 MJ |
| Secondary Fuel – Renewable |  | 3.3248 x 10^-4 MJ |

Figure 6- Input Flows to Make 1Kg of 'Gasoline Mix Regular'

(PE International , 2008-2015)

| Output Flows to Make 1Kg of ‘Gasoline mix regular’ |  | Amount |
| --- | --- | --- |
| Carbon Dioxide |  | 0.685973 Kg |
| Biogenic Carbon Dioxide |  | 0.00319409 Kg |
| Gasoline Regular |  | 1Kg |

Figure 7- Output Flows to Make 1Kg of 'Gasoline Mix Regular'

(PE International , 2008-2015)

| Total Energy Required | 50.152016 MJ |
| --- | --- |
| Total Gasoline Produced | 1KG |
| Total Energy Within 1kg of Gasoline | 47.14 MJ |
| Total Carbon Dioxide Produced | 0.6892Kg |

Figure 8- Summary Table of Flows for 'Gasoline Mix Regular'

### 3.1.2 “Diesel at refinery from crude oil and bio components”

Fuel supply – Production at Refinery

10 ppm Sulphur

5.75 wt % Bio Components

EU – 27”

| Input Flows to Make 1Kg of ‘Diesel mix regular’ | MJ/KG | Amount |
| --- | --- | --- |
| Crude Oil | 43.3 MJ/Kg | 45.0809 MJ |
| Hard Coal | 26.3 MJ/Kg | 0.145187 MJ |
| Brown Coal | 11.9 MJ/Kg | 0.0727566 MJ |
| Natural Gas | 44.1 MJ/Kg | 3.56146 MJ |
| Uranium - Nuclear |  | 0.163719 MJ |
| Primary Energy – Geothermic |  | 0.0018843 MJ |
| Primary Energy – Hydro Power |  | 0.0439534 MJ |
| Primary Energy – Solar Energy |  | 1.50183 MJ |
| Primary Energy – Wind Power |  | 0.018503 MJ |
| Secondary Fuel |  | 3.31968 x10^-4 MJ |
| Secondary Fuel – Renewable |  | 0.00348128 MJ |

Figure 9- Input Flows to Make '1Kg of Diesel Mix Regular’'

(PE International, 2008-2015)

| Output Flows to Make 1Kg of ‘Diesel mix regular’ |  | Amount |
| --- | --- | --- |
| Carbon Dioxide |  | 0.355244 Kg |
| Biogenic Carbon Dioxide |  | 0.0030279Kg |
| Gasoline Regular |  | 1Kg |

Figure 10-Output Flows to Make 1Kg of 'Diesel Mix Regular'

(PE International, 2008-2015)

| Total Energy Required | 50.593 MJ |
| --- | --- |
| Total Diesel Produced | 1KG |
| Total Energy Within 1kg of Diesel | 45.85 MJ |
| Total Carbon Dioxide Produced | 0.385523 kg |

Figure 11- Summary Table of Flows for 'Diesel Mix Regular'

### 3.1.3 Electricity – LCA Data

Electricity Grid Mix – EU-27

| Input Flows to Make 3.6MJ of ‘EU-27 Electricity Grid Mix’ | MJ/KG | Amount |
| --- | --- | --- |
| Crude Oil | 43.3 MJ/Kg | 0.682571MJ |
| Hard Coal | 26.3 MJ/Kg | 2.02965 MJ |
| Brown Coal | 11.9 MJ/Kg | 1.30847 MJ |
| Natural Gas | 44.1 MJ/Kg | 1.73318 MJ |
| Uranium – Nuclear |  | 2.80019 MJ |
| Primary Energy – Geothermic |  | 0.233813 MJ |
| Primary Energy – Hydro Power |  | 0.578962 MJ |
| Primary Energy – Solar Energy |  | 0.0976294 MJ |
| Primary Energy – Wind Power |  | 0.11346 MJ |
| Secondary Fuel |  | NA |
| Secondary Fuel – Renewable |  | NA |

Figure 12- Input Flows to Make 3.6MJ of 'EU 27 Electricity Grid Mix'

(PE International, 2002-2010)

| Output Flows to Make 1Kg of ‘EU-27 Electricity Grid Mix’ |  | Amount |
| --- | --- | --- |
| Carbon Dioxide |  | 0.52807 Kg |
| Electricity |  | 3.6 MJ |

Figure 13-Output Flows to Make 3.6MJ of 'EU 27 Electricity Grid Mix'

(PE International, 2002-2010)

| Total Energy Required | 9.57799 MJ |
| --- | --- |
| Total Electricity Produced | 3.6 MJ |
| Total Carbon Dioxide Produced | 0.52807 Kg |

Figure 14- Summary Table of Flows to Make 3.6MJ of 'EU 27 Electricity Grid Mix'

## 4.0 Analysis

### 4.1 Comparison of Gasoline, Diesel and Electricity

Equation used to interpret data:

Energy required to produce 1 Mj of fuel = Addition of relevant energy input flows  Number of MJ produced

| ELECTRICITY |  |
| --- | --- |
| Total Energy Required | 9.57799 MJ |
| Total Electricity Produced | 3.6 MJ |
| Total Carbon Dioxide Produced | 0.52807 Kg |

Figure 15-Electiricy Summary Table of Energy Required and Produced

(PE International, 2002-2010)

| GASOLINE |  |
| --- | --- |
| Total Energy Required | 50.15 MJ |
| Total Gasoline Produced | 1KG |
| Total Energy Within 1kg of Gasoline | 47.14 MJ |
| Total Carbon Dioxide Produced | 0.6892Kg |

Figure 16-Gasoline Summary Table of Energy Required and Produced

(PE International , 2008-2015)

| DIESEL |  |
| --- | --- |
| Total Energy Required | 50.593 MJ |
| Total Diesel Produced | 1KG |
| Total Energy Within 1kg of Diesel | 45.85 MJ |
| Total Carbon Dioxide Produced | 0.385523 kg |

Figure 17- Diesel Summary Table of Energy Required and Produced

(PE International, 2008-2015)

| Energy needed to create 1 MJ of Electricity | 3.6/3.6 = 1 9.57799/3.6 = 2.661 | 2.661MJ |
| --- | --- | --- |
| Carbon Dioxide produced to create 1MJ of Electricity | 0.52807/3.6 = 0.146686 | 0.146686 Kg |

Figure 18- Electricity Table - Per MJ

| Energy needed to create 1 MJ of Gasoline | 47.14/47.14 = 1 50.15/47.14 =1.06 | 1.06 MJ |
| --- | --- | --- |
| Carbon Dioxide produced to create 1MJ of Gasoline | 0.6892/47.14 = 0.014620 | 0.014620 Kg |

Figure 19- Gasoline Table - Per MJ

| Energy needed to create 1 MJ of Diesel | 50.43 MJ 50.593/45.85 =1.10 | 1.1 MJ |
| --- | --- | --- |
| Carbon Dioxide produced to create 1MJ of Diesel | 0.0385523/45.85 = 8.4×10-4 | 8.4×10-4 Kg |

Figure 20- Diesel Table - Per MJ

### 4.2 Efficiency Calculations:

Efficiency values in terms of energy are stated below, in the simplest form:

Energy OutEnergy In  ×100=Efficiency

(BBC, 2011)

EnergyUseful= EnergyPrimary -EnergyLosses

(Zoran Morvaj, 2010)

These equations can be modified to suite calculating Refinery Efficiency:

Overall Efficiency=1 MJ of usefuel energyAmount of energy required   ×100

Electricity

Overall EfficiencyElectricity=1 MJ 2.661 MJ  ×100=37.597 %

Gasoline

Overall EfficiencyGasoline=1 MJ 1.06 MJ  ×100=94.339 %

Diesel

Overall EfficiencyDiesel=1 MJ 1.1 MJ  ×100=90.90 %

Efficiency Summary Table:

| Electricity | Gasoline | Diesel |
| --- | --- | --- |
| 37.957 % | 94.339 % | 90.90 % |

Figure 21- Efficiency Summary Table at Refinery Stage

4.3 Graphical Analysis of Energy Consumption (Energy Required)

Figure 22-Graph of Energy Required at Refinery Stage to Create 1 MJ of Fuel

It can be seen from the graph above that generating electricity is far more energy intensive than the refinement of gasoline or gasoline. This is because the value above is affected greatly by the use of non-renewable fuels and low efficiency processes to create electricity such as coal. For example, a coal powered station has significant thermal losses of up to 70%. (Process Heating, 2014)

If electricity was to become the main source of automotive fuel in the future, the electricity generation detailed above must progress significantly by improving electricity generation processes and utilising greater renewable energy sources. (Simmons, 2014)

Interestingly there is a variation in the energy required to refine diesel and gasoline, this is due to the fractional distillation process, it takes significantly more heat to separate diesel from crude oil than gasoline, therefore technically more energy is required to refine diesel.  Validation of this can be seen below:

(O. R. Abdurakhmanov, 2010)

### 4.4 Graphical Analysis of Energy Consumption (Carbon Dioxide Produced)

Figure 23- Graph of Carbon Dioxide Produced to Create 1MJ of Fuel

The graph above compares the Co2 emissions produced as 1 MJ of fuel is refined/produced, as expected, electricity has produced a significant amount of Co2 as Co2 production is directly related to the energy consumption required.

There is a distinct difference between the co2 of electricity compared to gasoline and diesel at this stage as gasoline and diesel are being refined and not generated.

The large amount of co2 produced from electricity originates from the burning of large quantities of coal. The results above suggest that this produces greater amounts of emissions than the heating of the fractional distillation tower. (R.G.Gareev, 2002)

Unexpectedly the co2 produced from the refinement of gasoline is slightly larger than expected, it would be anticipated to be of a similar value to diesel or slightly less, as the energy required to refine both are relatively similar. It is well known that when gasoline is burnt it produces more co2 than diesel even when disregarding the efficiency of a diesel engine (US Energy Administration, 2017), however, within the fractional distillation tower, the fuels are only heated into a vapour as opposed to being burnt, therefore unfortunately the discrepancy is this result cannot be explained.

## 5.0 Validation data (Gabi)

The data below has been correlated in an attempt to validate the original data gathered from the European Life Cycle Database with secondary results from a different data source, GaBi.

### 5.1 Gasoline

| Input Flows to Make 1Kg of ‘Gasoline mix regular’ | MJ/KG |
| --- | --- |
| Primary Energy from Geothermics | 0.00289 |
| Primary Energy from Hydro Power | 0.0628 |
| Primary Energy from Solar Power | 2.45 |
| Primary Energy from Wave | 7.4x10^14 |
| Primary energy from Wind | 0.0554 |
| Natural Gas | 5.46 |
| Hard Coal | 4.29x10^13 |
| Crude Oil | 44.7 |

Figure 24- Validation Data of Input Flows to Make 1MJ of 'Gasoline Mix Regular'

| Output Flows to Make 1Kg of ‘Gasoline Mix Regular’ | Amount (MJ unless specified otherwise) |
| --- | --- |
| Gasoline Regular | 1 Kg |
| Carbon Dioxide | 0.646 |
| Carbon Dioxide (Biotic) | 0.0198 |

Figure 25-Validation Data of Output Flows to Make 1MJ of 'Gasoline Mix Regular'

(Thinkstep-Gabi , 2018)

### 5.2 Diesel

| Input Flows to Make 1Kg of ‘Diesel mix’ | MJ/KG |
| --- | --- |
| Primary Energy from Geothermics | 0.00197 |
| Primary Energy from Hydro Power | 0.0481 |
| Primary Energy from Solar Power | 2.35 |
| Primary Energy from Wave | 5.4x10^14 |
| Primary energy from Wind | 0.0359 |
| Hard Coal | 0.136 |
| Crude Oil | 44.1 |

Figure 26-Validation Data of Input Flows to Make 1MJ of 'Diesel Mix Regular'

| Output Flows to Make 1Kg of ‘Diesel mix’ | Amount (MJ unless specified otherwise) |
| --- | --- |
| Diesel Regular | 1Kg |
| Carbon Dioxide | 0.363 |
| Carbon Dioxide (Biotic) | 0.0188 |

Figure 27- Validation Data of Output Flows to Make 1MJ of 'Diesel Mix Regular'

(Thinkstep-Gabi , 2018)

### 5.3 Electricity

| Input Flows to Make 3.6MJ of ‘EU-27 Electricity Grid Mix’ | (MJ) (Unless stated otherwise) | Comparison Value from Original data – European LCA Database |
| --- | --- | --- |
| Primary Energy from Geothermics | 0.004 |  |
| Primary Energy from Hydro Power | 2.46 |  |
| Primary Energy from Solar Power | 1.32 |  |
| Primary Energy from Wave | 2.85x10-13 |  |
| Primary energy from Wind | 0.472 |  |
| Natural Gas | 1.33 |  |
| Hard Coal | 1.14 |  |
| Crude Oil | 0.161 |  |

Figure 28- Validation Data of Input Flows to Make 3.6MJ of ‘EU-27 Electricity Grid Mix’

| Output Flows to Make 3.6MJ of ‘EU-27 Electricity Grid Mix’ | Amount (MJ) (unless specified otherwise) |  |
| --- | --- | --- |
| Electricity | 3.6 |  |
| Carbon Dioxide | 0.295 |  |
| Carbon Dioxide (Biotic) | 0.1 |  |

Figure 29- Validation Data of Output Flows to Make 3.6MJ of ‘EU-27 Electricity Grid Mix’

(Thinkstep-Gabi , 2018)

### 5.4 Gabi Credibility

Gabi is a proven credible source of energy data and is used widely by academics and in industry, it is especially useful for studying the well to wheel processes. In order to check the credibility of GaBi, academic journal databases where utilized to find papers that have effectively used GaBi as an effective tool to aid their research and to carry out projects, positive results of the use of GaBi where found, such as this paper below;

Figure 30- Example of Academic Paper Using GaBi

(C.M Silva, 2006)

Unfortunately the above data does not correlate exactly to the original data set analysed, this is because the program Gabi will only provide the most recent information of the UE’s electricity grid mix which is  ‘EU 28’, whereas the European Life Cycle Database only provides the ‘EU 27’ electricity grid mix. The difference between ‘EU 27’ and ‘EU 28’ is the inclusion of Croatia, which are a country with a large natural gas output and usage, which will affect the overall average electricity mix. This would explain the differences between the datasets, as natural gas has a much stronger presence within the EU 28 dataset.

### 5.5 European Life Cycle Database Credibility

The European Life cycle database is also used regularly by industry and academics. (Daniel Garraín, 2015)

The opening description of the European Life Cycle database homepage perfectly describes why this data source is credible:

“The ELCD comprises Life Cycle Inventory (LCI) data from front-running EU-level business associations and other sources for key materials, energy carriers, transport, and waste management. The respective data sets are officially provided and approved by the named industry association” (European Life Cycle Database Joint Reserach Centre , n.d.)

## 6.0 Conclusion

This section of the well to wheel process has highlighted that producing electricity is far more energy intensive than the refinement of Gasoline and Diesel, due to the general lack of non-renewable sources that are used in the production of electricity, an example of this being the reliance of burning coal to generate steam. Therefore, at this stage it is the least environmentally friendly and least efficient of the fuels discussed. This result was expected as it correlates with academic journals assessing the subject. If electricity is to become a major contender as an automotive fuel, the energy consumption required and greenhouse gas emissions produced would have to reduce significantly in order for this to be a viable replacement from the dependence on non-renewable fuels such as gasoline and diesel.

At this point is the well to wheel process, gasoline has been proved to be the fuel that requires the least energy consumption, however, greater co2 emissions are produced within its refinement process than diesel but lesser than electricity.

It is worth noting that if this study were to be carried out using datasets from a single country perspective with a higher utilisation of renewable energies (such as Norway or Sweden), it would be probable that the energy consumption and greenhouse gas emission result for electricity would be improved.

# Refinery to Storage Distribution Points

# (Via Pipeline)

## 1.0 Research

The next phase in the well to wheel process is the transportation of fuel products from refineries to distribution points/storage facilities via pipeline or cargo ship.

In order for fuel to be transported from refineries to distribution storage points successfully, there must be a system in place that is capable of moving large amounts of product safely and efficiently whilst also maintaining reliability and cost effectiveness. (Petroleum Industry Association, 2017) Pipelines and cargo ships are the preferred choice at this stage due to the sheer mass of fuel having to be regularly distributed to storage terminals. (Corbett, 2008)

Underground pipelines are an effective solution to this enormous logistical task, (Pienaar, 2010) they serve as an efficient and cost-effective way of transporting fuel products whilst requiring less employees for operation (in comparison to vehicle-based transportation) and are a less complex operation in terms of logistics considering the infrastructure is already well established and maintained throughout Britain. (British Pipeline Industry, 2014) Cargo ships will be discussed in greater detail in the next section (page 62) of this report.

The use of pipeline allows a change in tactic in terms of refinery-based storage sites, instead of stockpiling masses of refined product at the refinery location, fuels can be piped directly into the underground pipeline system where they can begin their extensive journey (Acosta-Amado, et al., 2014) to the various storage facilities that are dispersed throughout Britain. (UK Petroleum Industry, 2017) Pipelines ensure safe movement of fuel without excessive energy consumption or emission production. (Canadian Energy Pipeline Assosiation, 2017) Despite this, the energy consumption and emission production still need to be taken into consideration in order to determine the overall effect on the well to wheel process being followed/investigated/used.

## 2.0 Methodology

To assess the change in energy that can occur along a pipeline, it is first important to contemplate where energy ‘losses’ may arise and at what point significant energy is needed in order for the pipeline system to operate.

At the beginning of a pipeline system there will be a pumping station situated that will pressurize the fuel to a specified amount and will allow fuel products to travel safely along the pipe at a particular velocity.  As the fuel travels along the pipeline, energy is ‘lost’ due to different forces acting within the system, such as the frictional force of the inner pipe surface, a change in inclination of the pipe routing and minor losses that occur due to pipe bends and elbows. This creates a requirement for a secondary pumping system along larger pipeline routes that are required to apply work to the fuel, increasing the pressure back to its original amount. (Acosta-Amado, et al., 2014)

Once the above is appreciated, they can be considered mathematically using a principle based on the conservation of energy within a system. This principle states that energy cannot be created or destroyed, it can merely change form. This is a useful concept when considering fuel travelling through a pipeline. Energy will change form into states that are not easily recoverable, such as heat. This ‘lost’ energy will need to be added back into the system via the additional pumping station along the pipeline route. (Vivien Mweene Chabalengula, 2012)

At a set distance along the pipeline route there will be additional pumping stations that will pressurise the fuel back to its original operating pressure. (Enbridge , 2018)

### 2.1 Forming a Case Study:

As it would be too large a task to investigate every underground pipeline running throughout Britain, a case study is proposed that is based on one of the major pipeline routes owned by the British pipeline agency. This case study will look to examine the energy losses that occur when gasoline and diesel are transported through this chosen route. This particular pipeline route running from Stanlow, Cheshire to Gatwick, West Sussex was chosen for the availability of information on the pipeline locations and dimensions. The information for this pipeline is taken from a booklet produced by a collaborative organisation of underground high-pressure pipeline companies, this information will be used to form data that allows the use of mathematical energy-based principals, such as Bernoulli’s theorem.

### 2.2 Description of Case Study

The case study begins to highlight the basic principle of energy conservation, specifically Bernoulli’s theorem. The use of this equation allows the author to calculate energy losses that occur when transporting liquid through a pipe. However, before Bernoulli’s theorem can be utilised, there are several variables that lie within Bernoulli’s theorem that must be calculated first. Each calculation for each variable is listed in subsections so that the methodology is clear throughout. Once each variable is discussed, a spreadsheet is utilised that includes all variables, allowing the author to evaluate the energy lost per section of pipeline.

The total energy needed to move a quantity of fuel through the pipeline equates to the energy required to initially pressurise the fuel and the energy required to maintain that pressure throughout each section of the pipeline.

Once values for energy loses are displayed and explained, calculations of initial pumping power required as well as the pumping power to overcome losses are carried out. This will aid the author in stating a value of efficiency for the operation of underground pipelines.

### 2.3 Case Study Starting Point

The case study begins by explaining the mathematical method of calculating energy losses through a pipe using Bernoulli’s equation:

P1+ 12 ρ v12+ ρgh1= P2+ 12ρv22+ ρgh2

Bernoulli’s equation is based on Newton’s first law, it states that “in a pipe of duct in which energy flows, the sum of the pressure, potential energy and kinetic energies are equal at any point” (Carvill, 1994). Therefore, this energy conservation rule can be applied to calculate energy losses that occur throughout a pipe when liquid fuel is being transported.

Fuel is considered to be moving at a constant velocity (for safety reasons) - therefore the terms involving velocity can be cancelled from the equation leaving:

P1+ 12 ρ v12+ ρgh1= P2+ 12ρv22+ ρgh2

P1+ ρgh1= P2+ ρgh2

P2=P1+ ρgh1-ρgh2

Equation modified to include frictional and minor loses to suite calculating the total loss of pressure:

P2=P1+ ρgh1-ρgh2- ∆hf+ ∆hm

Change in pressures=Elevation losses+ Fictional Losses+Minor Losses

By carrying out this method, the equation clearly states that any pressure change that occurs along a horizontal pipe at a constant velocity is due to frictional and minor losses.

h0=Initial height

h1=Final height

h1- h0=Change in height

∆hf=Frictional Losses

∆hm=Minor Losses

f=Darcy Fricton Factor

L=Length of pipe

D=Inside Diameter of Pipe

g=Specific Gravity

∆hf=f LD V22g

∆hm=Minor Losses

P0=Initial Pressure

P1=Final Presure

P0- P1=Change In Pressure

ρ=Density

V0=Initial Velocity

V1=Final Velocity

(Pipeflow, 2018)

The following variables must be known to calculate a value for frictional losses:

Darcy-Welsbach equation:

∆hf=f LD V22g

V=Velocity

f= Blasius’ Friction factor

L=Length of pipe

D=Inside Diameter of Pipe

g=Specific Gravity

(Pipeflow, 2018)

### 2.4 Velocity

The velocity of a fuel flowing through a high pressurised operational product pipeline can have a large effect on the operational safety of that pipeline. (Society of Petrolium Engineers International, 2015). To avoid static electricity build up, fuel normally travels approximately 3m/s when flowing in a high pressure pipeline. (Anup Sera, 2009).

Velocity=3m/s

### 2.5 Reynolds Number (required for Darcy’s Friction Factor)

Re= ρVDμ

(Aarsland, 2013)

| Variable | Symbol | Actual Value | Reference |
| --- | --- | --- | --- |
| Density of fuel being considered (Petrol) | ρp | 750 kg/m3 | (Martinez, 1992) |
| Density of fuel being considered (Diesel) | ρD | 830 kg/m3 | (Martinez, 1992) |
| Velocity | V | 3m/s | (Society of Petrolium Engineers International, 2015) |
| Length of Pipe | L | Example: 110690.68 m (Stanlow to Kingsbury) | Dependant on section of pipeline |
| Dynamic Viscosity of fluid petrol | μ | 0.005 poise = 0.0005 Kg/m3 | (Geotech, 2018) |
| Dynamic Viscosity of fluid Deisel | μ | 0.024 poise = 0.0024 Kg/m3 | (Geotech, 2018) |

Figure 31- Reynolds Number - Variable Table

Re= Density ×Velocity ×DiameterDynamic Viscoity

Re= ρVDμ

RePetrol= 747.5(3)(0.25)0.0005=1121250

ReDiesel= 830(3)(0.25)0.0024=259375

Reynolds number will change depending on the fuel chosen and the diameter of the pipe, (velocity is constant) therefore a different Reynolds number is calculated for each section of the pipeline. A table showing calculations for each section of pipeline can be found on page 47.

### 2.6 Two Methods for Calculating a Friction Factor (Turbulent Flow)

### 2.6.1 Blasius’ Law

The Blasius’ Law   fundamental part of the Darcy-Welsbach equation, it is a unit-less value that accounts for the frictional losses of a fluid due to the turbulence of the flow being consideredReynolds number Re. When working with turbulent flow, the Darcy friction factor can be calculated as follows:

f= 0.079Re1/4

F=0.079Re-0.25

(Trinh, 2009)

The above equation does not account for the roughness of the pipes surface, therefore a secondary method will have to be considered.

### 2.6.2 Moody’s friction factor:

f= 1.325ln e3.7D+ 5.74Re0.9 2

for 5000≤Re≤108 turbulent flow and 10-6≤eD≤ 10-2

(Tefaruk Haktanır, 2004)

| Variable | Symbol | Actual Value | Reference |
| --- | --- | --- | --- |
| Diameter | D | Example: 250 mm (Stanlow to Kingsbury) | Dependant on section of pipeline |
| Pipe Roughness | e | Commercial Steel: (0.045) | (J.M.McDonough, 2004) |
| Reynolds Number | Re | Example: 1121250 | Dependant on Pipeline Section |

Figure 32- Moody's Friction Factor - Variable Table

= 1.325ln 0.0453.7(0.25)+ 5.741121250 0.9 2=0.1449799

The friction factor will also change for each section of the pipeline since it is dependent on Reynolds number and pipe diameter.

*The pipeline is assumed to be constructed from commercial steel with a roughness of 0.045 (J.M.McDonough, 2004)

### 2.7 Specific Gravity/ Relative Density:

Specific gravity is the density of an object compared to the density of water. Specific gravity is a unit-less value as it a comparative ratio of two densities. The density of the chosen substance is usually compared to water:

Petrol=between 720kg/m3 and 775kg/m3  ∴average = 747.5 kg/m3 (Bosch, 2007)

Diesel=between 810kg/m3 and 850kg/m3  ∴average = 830 kg/m3  (Bosch, 2007)

Water=1000kg/m3 (Bosch, 2007)

Equation for specific gravity:

Specific Gravity=Density of fuelDensity of water

(Carvill, 1994) (D.Bacon, 1990)

Specific Gravity of Petrol =747.5 1000 =0.7475

Specific Gravity of Diesel =830 1000 =0.83

### 2.8 Length of Pipe, Diameter and Losses Due to Elevation

The methodology used to find the remaining variables that are needed to calculate the total losses are noted in individual sections below:

### 2.8.1 Elevation

To calculate losses due to elevation change, there must first be a method of calculating the height variation between different stages in a pipeline system.

It is a well-known fact that the exact location of pipelines is a difficult topic to research, it is crucial to pipeline owners that the locations are kept undisclosed to the public, if this were to become common knowledge there imposes greater risk of security threats, theft and safety concerns.

A safety booklet created by Linewatch, who are a joint initiative of major fuel companies and distributors including the British Pipeline Agency and the CLH Pipeline group, describes very generic locations of underground fuel pipes that run across the United Kingdom. (Linewatch, 2017) This booklet was produced to alert companies to consider the possibility of high pressure underground pipeline systems being present before undertaking ground work in certain areas. This booklet also provides broad start and endpoints of the subsections of pipeline that make up major fuel pipeline routes:

Figure 33- Booklet Image 2

(Linewatch, 2017)

*The Booklet also states the diameter and starting operating pressures of each subsection of pipeline, which will later be used in energy conservation calculations.

Figure 34- Booklet Image 1

### 2.8.2 Start and End Latitudinal/Longitudinal Coordinates:

Start and end point sections can then be investigated via an online satellite mapping system, to visually investigate the exact coordinates of the pumping stations situated on the generic locations provided by the safety booklet. Pumping stations are very obvious to locate through satellite mapping (google maps), as they are placed on fuel storage sites owned by well-known fuel manufacturers, fuel storage sites have very large white storage sites onsite of which a pumping station will be situated nearby, which are again very distinctive; abnormally shaped buildings with obvious heavy and complex pipework arising from the floor into very large fuel storage containers and into the building itself.

Figure 35- Mapping Example

This example is of the Kingsbury control centre, of which the pumping station and storage terminal can be been clearly seen in the satellite imagery. The generic location provided from the safety booklet coupled with the term ‘fuel storage facility’ is adequate information to input into a search engine that a location address will arise, this location is then inputted into a satellite mapping system and the exact location can be found by eye.

### 2.8.3 Mapping Tools

The exact latitudinal and longitudinal coordinates of the locations are recorded. Mapping tools are then utilised to find the elevation of the start and end point of each location.

Figure 36- Elevation Finder - Mapping Tools

Figure 37- Google Maps Image of Valero Energy

### 2.8.4 Length of Pipeline:

The approximate length of the pipeline could be calculated by plotting the start and end point of each subsection of pipeline and measuring the linear distance between each point. The pipeline length is assumed to run as linear as possible as it is routed underground, more complex modelling of the pipeline would be impossible due to the limited information available.

## 3.0 Results

### 3.1 Excel Workbook of Collected Data

From here, enough information can be found to map a major section of pipeline in excel. The subsection pipelines addresses are described below:

| Section of Pipeline | Address Starting Point | End Point | Pipeline Company |
| --- | --- | --- | --- |
| Stanlow- Kingsbury Storage Terminal | Mersey Pump Station Mill Lane, Little Stanney Nr Chester Cheshire CH2 4HY | Kingsbury Control Centre Trinity Road Kingsbury Nr Tamworth Staffordshire B78 2EE UK | BPA |
| Kingsbury - Bruncefield 1 | Kingsbury Control Centre Trinity Road Kingsbury Nr Tamworth Staffordshire B78 2EE UK | Bruncefield 15-17 St James' Mill Rd, Northampton NN5 5JW | BPA |
| Kingsbury - Bruncefield 2 | Kingsbury Control Centre Trinity Road Kingsbury Nr Tamworth Staffordshire B78 2EE UK | Bruncefield 15-17 St James' Mill Rd, Northampton NN5 5JW | BPA |
| Bruncefield - Thames | Bruncefield 15-17 St James' Mill Rd, Northampton NN5 5JW | The Manorway, Corringham, Stanford-le-Hope SS17 9LQ | BPA |
| Bruncefield - Heathrow 1 | Bruncefield 15-17 St James' Mill Rd, Northampton NN5 5JW | West London Terminal, Bedfont Rd, Staines-upon-Thames, Stanwell, Staines TW19 7LZ | BPA |
| Bruncefield - Heathrow 2 | Bruncefield 15-17 St James' Mill Rd, Northampton NN5 5JW | West London Terminal, Bedfont Rd, Staines-upon-Thames, Stanwell, Staines TW19 7LZ | BPA |
| Heathrow - Walton | West London Terminal, Bedfont Rd, Staines-upon-Thames, Stanwell, Staines TW19 7LZ | The Towpath Waterside Drive, Walton-On-Thames, Surrey, KT12 2JB | BPA |
| Walton - Gatwick | The Towpath Waterside Drive, Walton-On-Thames, Surrey, KT12 2JB | London Gatwick Airport, Perimeter Rd N, Horley, Gatwick RH6 0JE | BPA |

Figure 38- Section Of Pipeline -Pipeline Owner- Start and End Point

### 3.2 Results Applicable to Diesel and Gasoline

| Section of Pipeline | Length of Pipeline (miles) (approximately) | Diameter of Pipe (mm) | Operating pressure (pa) |
| --- | --- | --- | --- |
| Stanlow- Kingsbury Storage Terminal | 68.78 | 250 | 8273712 |
| Kingsbury - Bruncefield 1 | 39.83 | 300 | 8273712 |
| Kingsbury - Buncefield 2 | 39.83 | 250 | 8273712 |
| Bruncefield - Thames | 78.27 | 350 | 8273712 |
| Buncefield - Heathrow 1 | 57.33 | 200 | 8273712 |
| Bruncefield - Heathrow 2 | 57.33 | 150 | 8273712 |
| Heathrow - Walton | 61.64 | 250 | 8963188 |
| Walton - Gatwick | 19.3 | 250 | 8963188 |

Figure 39- Table of Initial Information -Length, Diameter, Operating Pressure

| Latitude A | Longitude A | Latitude B | Longitude B | Elevation A | Elevation B | Change in Elevation | Negative/ Positive Gradient |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 53.270481 | -2.843918 | 52.574346 | -1.672892 | 4.3 | 80.1 | -75.8 | uphill |
| 52.574346 | -1.672892 | 52.23383 | -0.910655 | 80.1 | 60.5 | 19.6 | Downhill |
| 52.574346 | -1.672892 | 52.23383 | -0.910655 | 80.1 | 60.5 | 19.6 | Downhill |
| 52.23383 | -0.910655 | 51.519136 | 0.511546 | 60.5 | 4 | 56.5 | Downhill |
| 51.519136 | 0.511546 | 51.450853 | -0.459456 | 4 | 17.1 | -13.1 | uphill |
| 51.519136 | 0.511546 | 51.450853 | -0.459456 | 4 | 17.1 | -13.1 | uphill |
| 51.450853 | -0.459456 | 51.401885 | -0.409433 | 17.1 | 10.1 | 7 | Downhill |
| 51.401885 | -0.409433 | 51.160541 | -0.184476 | 17.1 | 56.3 | -39.2 | uphill |
| 51.401902 | -0.409387 | 51.450794 | -0.459491 | 10.1 | 17.3 | -7.2 | uphill |
| 51.401902 | -0.409387 | 51.450794 | 0.540509 | 10.1 | 17.6 | -7.5 | uphill |
| 50.831633 | -1.363941 | 51.151389 | -0.958098 | 30.7 | 102.2 | -71.5 | uphill |
| 51.15146 | -0.958075 | 51.477177 | 0.247984 | 99.2 | 7.3 | 91.9 | Downhill |
| 50.830286 | -1.358888 | 51.526552 | -2.689138 | 30 | 2.1 | 27.9 | Downhill |
| 50.830286 | -1.358888 | 52.558812 | -2.231922 | 30 | 105.5 | -75.5 | uphill |

Figure 40- Table of Latitude, Longitude and Elevation for Each Section of Pipeline Investigated

*elevation change – it should be noted that elevation changes can only be considered from known locations. It is more than likely each section of pipe will see variation in gradient however this would be impossible to model without specific routing locations.

### 3.3 Gasoline Specific Result

| Average Velocity (m/s) | Density (Gasoline) (kg/m3) | Length of Pipe (m) | Diameter of Pipe (m) | Dynamic Viscosity (kg/m3) |
| --- | --- | --- | --- | --- |
| 3 | 747.5 | 110690.4052 | 0.25 | 0.0005 |
| 3 | 747.5 | 64100.0122 | 0.3 | 0.0005 |
| 3 | 747.5 | 64100.0122 | 0.25 | 0.0005 |
| 3 | 747.5 | 125963.0418 | 0.35 | 0.0005 |
| 3 | 747.5 | 92263.4622 | 0.2 | 0.0005 |
| 3 | 747.5 | 92263.4622 | 0.15 | 0.0005 |
| 3 | 747.5 | 99199.7176 | 0.25 | 0.0005 |
| 3 | 747.5 | 31060.262 | 0.25 | 0.0005 |

Figure 41- Table of Average Velocities, Length of Pipe, Diameter of Pipe and Dynamic Viscosity (Gasoline)

| Reynolds Number (Re) | Darcy Friction Factor | Specific Gravity kg/m3 |
| --- | --- | --- |
| 1121250 | 0.1449799 | 0.7454 |
| 1345500 | 0.128956087 | 0.7454 |
| 1121250 | 0.1449799 | 0.7454 |
| 1569750 | 0.117393424 | 0.7454 |
| 897000 | 0.169009629 | 0.7454 |
| 672750 | 0.209933731 | 0.7454 |
| 1121250 | 0.1449799 | 0.7454 |
| 1121250 | 0.1449799 | 0.7454 |

Figure 42- Table of Reynolds Number, Darcy Friction Factor and Specific Gravity (Gasoline)

Frictional Losses

387526.0402

166341.9652

224413.524

255059.7855

470689.3012

779549.753

347297.2538

108741.6775

Figure 43- Table of Friction Losses (Gasoline)

### 3.4 Diesel Specific Result

| Average Velocity (m/s) | Density (Diesel) (kg/m3) | Length of Pipe (m) | Diameter of Pipe (m) | Dynamic Viscosity (kg/m3) |
| --- | --- | --- | --- | --- |
| 3 | 830 | 110690.4052 | 0.25 | 0.0024 |
| 3 | 830 | 64100.0122 | 0.3 | 0.0024 |
| 3 | 830 | 64100.0122 | 0.25 | 0.0024 |
| 3 | 830 | 125963.0418 | 0.35 | 0.0024 |
| 3 | 830 | 92263.4622 | 0.2 | 0.0024 |
| 3 | 830 | 92263.4622 | 0.15 | 0.0024 |
| 3 | 830 | 99199.7176 | 0.25 | 0.0024 |
| 3 | 830 | 31060.262 | 0.25 | 0.0024 |

Figure 44- Table of Average Velocities, Length of Pipe, Diameter of Pipe and Dynamic Viscosity (Diesel)

| Reynolds Number (Re) | Darcy Friction Factor | Specific Gravity kg/m3 | Frictional Losses |
| --- | --- | --- | --- |
| 259375 | 0.144985307 | 0.83 | 348039.376 |
| 311250 | 0.128959935 | 0.83 | 149391.5682 |
| 259375 | 0.144985307 | 0.83 | 201547.0827 |
| 363125 | 0.117396334 | 0.83 | 229067.8035 |
| 207500 | 0.169017948 | 0.83 | 422733.8232 |
| 155625 | 0.20994865 | 0.83 | 700141.7836 |
| 259375 | 0.144985307 | 0.83 | 311909.6705 |
| 259375 | 0.144985307 | 0.83 | 97661.5289 |

Figure 45-Table of Reynolds Number, Darcy Friction Factor and Specific Gravity (Diesel)

### 3.5 Total Pressure Loss

Total Pressure loss can be calculated as the losses due to friction can been accounted for:

P2=P1+ ρgh1-ρgh2- ∆hf

P2=8273712+747.5×0.7454×4.3-747.50×0.7454×80.1-387526.0402

=7843951.223 Pa

### 3.6 Final Pressure and Pressure Change Values

Gasoline, Diesel

| Operating Pressure (Pa) | Section of Pipeline | Final Pressure (Pa) | Change in Pressure (end of pipe section) |
| --- | --- | --- | --- |
| 8273712 | Stanlow to Kingsbury | 7843951.223 | 429760.7769 |
| 8273712 | Kingsbury – Bruncefield 1 | 8118290.89 | 155421.1098 |
| 8273712 | Kingsbury – Bruncefield 2 | 8060219.331 | 213492.6686 |
| 8273712 | Bruncefield – Thames | 8050133.252 | 223578.7482 |
| 8273712 | Bruncefield – Heathrow 1 | 7795723.556 | 477988.4443 |
| 8273712 | Bruncefield – Heathrow 2 | 7486863.104 | 786848.8962 |
| 8963188 | Heathrow – Walton | 8619791.052 | 343396.9483 |
| 8963188 | Walton - Gatwick | 8832604.612 | 130583.3883 |

Figure 46- Final Pressure and Pressure Change Values (Gasoline)

| Operating Pressure (pa) | Section of Pipeline | Final Pressure | Change in Pressure |
| --- | --- | --- | --- |
| 8273712 | Stanlow- Kingsbury | 7873454.004 | 400257.996 |
| 8273712 | Kingsbury - Buncefield 1 | 8137822.872 | 135889.1282 |
| 8273712 | Kingsbury - Buncefield 2 | 8085667.357 | 188044.6427 |
| 8273712 | Buncefield - Thames | 8083567.047 | 190144.9535 |
| 8273712 | Buncefield - Heathrow 1 | 7841953.587 | 431758.4132 |
| 8273712 | Buncefield - Heathrow 2 | 7564545.626 | 709166.3736 |
| 8963188 | Heathrow - Walton | 8656100.629 | 307087.3705 |
| 8963188 | Walton - Gatwick | 8838521.591 | 124666.4089 |

Figure 47-Final Pressure and Pressure Change Values (Diesel)

### 3.7 Pipeline Efficiency

Pipeline efficiency is usually discussed in two ways; Economic Efficiency and Transportation Efficiency. For the purpose of this paper, the Transport Efficiency is discussed.

### 3.7.1 Hydraulic and Thermal Efficiency

Transport efficiency consists of two major factors:

Hydraulic Efficiency – The efficiency is the measure of the energy lost (change in pressure) which is a result of the frictional loses and elevation changes.

Thermal Efficiency - The efficiency of the motor or engine that is doing work to the fuel in order to pressurise it to the required operating level.

(Interstate Natural Gas Assosiation of America , 2010)

### 3.7.1.1 Hydraulic Efficiency:

The previous calculations have provided enough information to determine the hydraulic efficiency of the pipeline.

Efficiency values in terms of energy are stated below, in the simplest form:

Energy OutEnergy In  ×100=Efficiency

(BBC, 2011)

EnergyUseful= EnergyPrimary -EnergyLosses

(Zoran Morvaj, 2010)

These equations can be modified to suite calculating pipeline efficiency:

Overall Effiecnecy=Energy of fuel being transported-Pipline loses Energy of total mass of fuel transported  ×100

The above equation is a comparison of how much energy is required to transport the fuel compared to the energy of the fuel being transported.

So far, the energy lost is calculated in terms of pressure loss, which now needs to be considered in terms of energy, in order to do this, pressure must be converted into units that are more relatable to energy loss.

Pressure can be converted into power, by considering the pumping power required to pressurise the fuel to the required value of pressure.

The pressure loses can also be converted into pumping power by considering the value of pressure change from the start of the pipeline to the end of the pipeline. One kilowatt of power is equivalent to one kilojoule per second, (US Energy Information Administration, 2017) therefore, if the pumping power is known, the amount of energy that is required per second can also be calculated.

### 3.8 Pumping Power Required

An example of the pumping power required is demonstrated below using the data relevant to the first section of pipeline.

Equations Used:

Power=Force ×Velocity

Pumping Power=Pressure ×Area ×Mean Velocity

Area= π ×(radius of pipe)2

(D.Bacon, 1990)

P1= 8273712   @ Start of pipe (calculated from Bernoulli’s equation)

P2= 7843951 @ End of pipe (calculated from Bernoulli’s equation)

P∆ =  8273712-7843951= 429761 Pa

Pumping Power=ρAu

@Start of pipe: 	Pumping Power=8273712 0.0493=1218400 w=1218.4Kw

(Carvill, 1993)

@ Extra Pumping Power Required to overcome height change and friction

Pumping Power=429761 0.0493=63287w=63.29 Kw

Therefore:

Overall pumping power needed=Pumping power at start+Additional Pumping Power required

Overall PP Needed=1216.24+63.17Kw=1281.69Kw

1281.69Kw=1281.69 Kj/s

The above states the energy required to move an amount of fuel in one second, the amount of fuel moved is one second is dependent on the area of the pipe and the velocity that the fuel is travelling at. A volumetric flow rate equation is used to calculate the amount of fuel being moved in one second.

### 3.9 Volumetric Flow Rate:

An equation for volumetric flow rate is utilised as a step to working out the mass flow rate of the fuel being transported per second:

Volume of fuel moved per second = Volumetric flow rate

Volumetric flow rate = Area ×Velocity

Volumetric Flow rate=0.049 ×3=0.147m3/s

∴0.147m3s of fuel is flowing

(Carvill, 1993)

### 3.10 Mass Flow Rate:

Converting the volumetric flow rate into a mass flow rate will determine the amount of fuel in Kg that is flowing per second:

Mass flow rate=Density ×Volume flow rate

m=747.5 kgm3×0.147=109.88 Kg of fuel is flowing per second

(Carvill, 1993)

The amount of fuel in Kg is then converted into joules by considering how many joules of energy are present in 1 Kg of fuel.

According to recent Life cycle analysis data, there are 47.14 MJ of energy present in 1Kg of gasoline fuel.

Therefore:

109.88 ×47140=5179861.05 KJ

∴5179743.2 KJ fuel energy energy in the amount of fuel being moved in one second

Now that the amount of energy used to transport a known value in one second is known, the following conclusion can be made:

1281.69Kj of energy is needed to move 5179861.05 Kj of fuel in one second.

### 3.11 Efficiency and Inefficiency

The above statement can now be used to form an inefficiency value:

Overall Ineffiecnecy=Energy required to transport fuel Energy of total mass of fuel transported  ×100

Gasoline:

Energy Required to Transport Fuel = 1281.69Kj

Total Energy Transported = 5179861 Kj

Overall Inefficiency=1281.695179861 ×100= 0.0247 %

100%-0.0247%=99.975%

Diesel:

Energy Required to Transport Fuel = 1277.35 Kj

Total Energy Transported = 5037998 Kj

Overall Inefficiency=1277.355037998 ×100= 0.0253 %

100%-0.0253%=99.974%

This value could be applied to larger amounts of fuel, for example, 10kg of fuel could be considered over a ten second time period, or 100kg over 100 seconds, and so on.

*The above example was based on one section of the pipeline in question, each pipeline section efficiency will differ very slightly as they are dependent on the pipeline area and varying pressure loses. To see the values for all sections of pipeline, please refer to the tables below:

### 3.12 Gasoline Specific Result:

| Section of Pipeline | Initial Pumping Power Required (Kw) | Pumping Power to Overcome Losses (Kw) | Total Pumping Power Required (Kw) |
| --- | --- | --- | --- |
| Stanlow to Kingsbury | 1218.404664 | 63.28749841 | 1281.692163 |
| Kingsbury – Bruncefield 1 | 1754.502717 | 32.95821262 | 1787.460929 |
| Kingsbury – Bruncefield 2 | 1218.404664 | 31.43939059 | 1249.844055 |
| Bruncefield – Thames | 2388.073142 | 64.5323893 | 2452.605531 |
| Bruncefield – Heathrow 1 | 779.7789851 | 45.04934956 | 824.8283347 |
| Bruncefield – Heathrow 2 | 438.6256791 | 41.71430326 | 480.3399824 |
| Heathrow – Walton | 1319.938386 | 50.56937484 | 1370.507761 |
| Walton - Gatwick | 1319.938386 | 19.22999124 | 1339.168378 |

Figure 48- Initial Pumping Power Required, Pumping Power to Overcome Losses and Total Pumping Power Required – (Gasoline)

| Section of Pipeline | Total Energy Required Per Second (Mj/s) | Volumetric Flow Rate m3/s | Mass Flow Rate (Kg/s) |
| --- | --- | --- | --- |
| Stanlow to Kingsbury | 1281.692163 | 0.147 | 109.8825000 |
| Kingsbury – Bruncefield 1 | 1787.460929 | 0.212 | 158.4700000 |
| Kingsbury – Bruncefield 2 | 1249.844055 | 0.147 | 109.8825000 |
| Bruncefield – Thames | 2452.605531 | 0.288 | 215.2800000 |
| Bruncefield – Heathrow 1 | 824.8283347 | 0.094 | 70.2650000 |
| Bruncefield – Heathrow 2 | 480.3399824 | 0.053 | 39.6175000 |
| Heathrow – Walton | 1370.507761 | 0.147 | 109.8825000 |
| Walton - Gatwick | 1339.168378 | 0.147 | 109.8825000 |

Figure 49- Total Energy Required per Second, Volumetric Flow Rate and Mass Flow Rate (Gasoline)

| Section of Pipeline | Amount of energy in 1 Kg of fuel (Kj) | Inefficiency % | Efficiency % |
| --- | --- | --- | --- |
| Stanlow to Kingsbury | 5179861.05 | 0.024743756 | 99.9752562 |
| Kingsbury – Bruncefield 1 | 7470275.8 | 0.023927643 | 99.9760724 |
| Kingsbury – Bruncefield 2 | 5179861.05 | 0.024128911 | 99.9758711 |
| Bruncefield – Thames | 10148299.2 | 0.024167651 | 99.9758323 |
| Bruncefield – Heathrow 1 | 3312292.1 | 0.024902041 | 99.9750980 |
| Bruncefield – Heathrow 2 | 1867568.95 | 0.025720067 | 99.9742799 |
| Heathrow – Walton | 5179861.05 | 0.026458389 | 99.9735416 |
| Walton - Gatwick | 5179861.05 | 0.025853365 | 99.9741466 |

Figure 50- Amount of Energy in 1Kg Of Fuel, Inefficiency and Efficiency

### 3.13 Diesel Specific Result:

| Stanlow- Kingsbury Storage Terminal | Initial Pumping Power Required (Kw) | Pumping Power to Overcome Losses (Kw) | Total Pumping Power Required (Kw) |
| --- | --- | --- | --- |
| Kingsbury - Bruncefield 1 | 1218.404664 | 58.94285531 | 1277.34752 |
| Kingsbury - Bruncefield 2 | 1754.502717 | 28.81630936 | 1783.319026 |
| Bruncefield - Thames | 1218.404664 | 27.69185944 | 1246.096524 |
| Bruncefield - Heathrow 1 | 2388.073142 | 54.88226523 | 2442.955407 |
| Bruncefield - Heathrow 2 | 779.7789851 | 40.69227177 | 820.4712569 |
| Heathrow - Walton | 438.6256791 | 37.5960128 | 476.2216919 |
| Walton - Gatwick | 1319.938386 | 45.22234815 | 1365.160734 |
| Walton - Heathrow | 1319.938386 | 18.35864411 | 1338.29703 |

Figure 51- Initial Pumping Power Required, Pumping Power to Overcome Losses and Total Pumping Power Required – (Diesel)

| Stanlow- Kingsbury Storage Terminal | Total Energy Required Per Second (MJ/s) | Volumetric Flow Rate | Mass Flow Rate (Kg/s) |
| --- | --- | --- | --- |
| Kingsbury - Bruncefield 1 | 1277.34752 | 0.147 | 109.8825000 |
| Kingsbury - Bruncefield 2 | 1783.319026 | 0.212 | 158.4700000 |
| Bruncefield - Thames | 1246.096524 | 0.147 | 109.8825000 |
| Bruncefield - Heathrow 1 | 2442.955407 | 0.288 | 215.2800000 |
| Bruncefield - Heathrow 2 | 820.4712569 | 0.094 | 70.2650000 |
| Heathrow - Walton | 476.2216919 | 0.053 | 39.6175000 |
| Walton - Gatwick | 1365.160734 | 0.147 | 109.8825000 |
| Walton - Heathrow | 1338.29703 | 0.147 | 109.8825000 |

Figure 52-Total Energy Required per Second, Volumetric Flow Rate and Mass Flow Rate (Diesel)

| Stanlow- Kingsbury Storage Terminal | Amount of energy in 1 Kg of fuel (KJ) | Inefficiency % | Efficiency % |
| --- | --- | --- | --- |
| Kingsbury - Bruncefield 1 | 5179861.05 | 0.02465988 | 99.97534012 |
| Kingsbury - Bruncefield 2 | 7470275.8 | 0.023872198 | 99.9761278 |
| Bruncefield - Thames | 5179861.05 | 0.024056563 | 99.97594344 |
| Bruncefield - Heathrow 1 | 10148299.2 | 0.02407256 | 99.97592744 |
| Bruncefield - Heathrow 2 | 3312292.1 | 0.024770498 | 99.9752295 |
| Heathrow - Walton | 1867568.95 | 0.025499551 | 99.97450045 |
| Walton - Gatwick | 5179861.05 | 0.026355161 | 99.97364484 |
| Walton - Heathrow | 5179861.05 | 0.023512081 | 99.97648792 |

Figure 53-Amount of Energy in 1Kg Of Fuel, Inefficiency and Efficiency

### 3.14 Validation of result in comparison to Gabi data

Gabi is a proven credible source of energy data and is used widely by academics and in industry, it is especially useful for studying the well to wheel processes. In order to check the credibility of GaBi, academic journal databases where utilized to find papers that have effectively used GaBi as an effective tool to aid their research and to carry out projects, positive results of the use of GaBi where found, such as this paper; “A Tank-to-wheel-analysis tool for energy and emissions studies in road vehicles” (C.M Silva, 2006)

3.15 Gabi Results:

Figure 54- Gabi Validation Result

( Gabi - Thinkstep, 2016)

### 3.16 Summary of Pipeline Efficiencies

The above data gathered from Gabi states that for 1Kg of product moved, 0.001MJ of energy is used or lost.

In 1Kg of gasoline, contains 47.14 MJof energy, therefore the efficiency using data provided from Gabi shows:

Overall Effiecnecy=Energy of fuel being transported-Pipline loses Energy of total mass of fuel transported  ×100

Inefficiency= 0.0049547.14 ×100=%

Inefficiency=100-0.0105=99.989%

In 1Kg of gasoline, contains 45.85 MJof energy, therefore the efficiency using data provided from Gabi shows:

Overall Effiecnecy=Energy of fuel being transported-Pipline loses Energy of total mass of fuel transported  ×100

Inefficiency= 0.0049545.85 ×100=0.0108 %

Inefficiency=100-0.0108=99.989%

There is seemingly little to no difference between Gasoline and Diesel result, therefore the same value will be used for both:

| Calculated Efficiency | Gabi Efficiency (Gasoline + Diesel) |
| --- | --- |
| 99.975 % | 99.989 % |

Figure 55-Calculated Efficiency and Gabi Efficiency Comparison

### 3.17 Thermal efficiency:

The thermal efficiency must also be accounted for, once this has been calculated, this can be combined with the hydraulic efficiency to form an overall efficiency. Thermal efficiency is the efficiency of the pump, powered by motor or engine that is doing work to the fuel within the pipeline.

### 3.17.1 Efficiency Vs Pump Efficiency Curve

Figure 56- Efficiency Vs Pump Efficiency Curve

(DEFRA Envirement Agency , 2012)

The graph above demonstrates how pumping efficiencies vary depending on the specific operating pump speed. For the purpose of this report, the pump is assumed to be operating at a suitable speed for better values of efficiency, therefore the efficiency of the pump can be considered to be approximately 80%.

From the above graphs, efficiency of the pumps doing work on the fuel can be considered to be approximately 80%, when this efficiency is combined with the hydraulic efficiency calculated previously:

Hydraulic effieciency ×Thermal Efficiency=Overall efficiency

0.9997 ×80=79.98

## 4.0 Analysis

### 4.1 Pipeline Loses – General Analysis

Overall, the amount of energy consumed via pipeline loses compared to the large amount of energy transported indicates that this system is efficient, frictional loses in terms of energy are minor in comparison to the energy content of the large amount of fuel being transported, however when also accounting for loses due to the thermal efficiency of a pump, the efficiency decreases significantly.

The main contributing factor to the total loses is the thermal efficiency of the pump of which has not been investigated as thoroughly as the losses due to the pipeline itself. The methodology for calculating the pipeline loses were time consuming as many variables had to be considered, which left little time to properly evaluate the thermal efficiency. However, thermal efficiencies of pumps are extremely varied and case specific efficiencies are difficult to come by as they would have to match the specific factors of this case study (such as specific pump speed and pump sizing), therefore the only suitable method is to form an average figure that can be generally applied.

It can be assumed that the pumps within the power stations would be specifically matched to the throughput and speed that is required of them, therefore providing good efficiency.

### 4.2 Gasoline and Diesel Results Compared

Any difference between the gasoline and diesel result is due to the variation in densities between the two fuels as this value is a variable that effects the Reynolds number and specific gravity, when specific gravity is changed this has an effect on the frictional losses of the pipe, when Reynolds number is changed it will affect the friction factor which will also change the value of frictional losses. Interestingly, diesel has a greater value of density than gasoline, but the frictional losses have decreased, this is because the specific gravity of each fuel which also has a direct effect on the frictional losses equation.

### 4.3 Elevation

The results coincide with the idea that losses due to elevation increase as the difference in elevation when positive is larger, this has a direct effect on the pumping power required. The results also show logically that the pumping power required to overcome losses is greater when the pipeline is gaining elevation (travelling uphill) and lesser when elevation is decreasing (travelling downhill).

### 4.4 Volumetric and Mass Flow Rate

Flow rate results show that increasing the diameter of the pipe allows a greater volumetric flow rate to be achieved. Gasoline and diesel achieve the same volumetric and mass flow as this value is measured in Kg/s, however one kilogram of diesel contains a slightly lesser amount of energy than a one kilogram of gasoline, therefore technically, gasoline allows greater amounts of energy moved per second even though the flow rates are the same.

### 4.5 Differences in Efficiency between Gasoline and Diesel

The differences in efficiency between gasoline and diesel are very small, this is because the differences in frictional losses are minute, which is the only effective variable that changes when the type of fuel considered is changed.

### 4.6 Roughness of the Pipe

The surface roughness of the inside of the pipe does affect the frictional force experienced by the fluid, this has been assumed to be constant for the sake of maintaining simplicity for this model, however in reality the smoothness of the pipe would change over time and the value would depend on the age of the pipe, for example, if corrosion build up took place, this would have an effect on the surface roughness and also the inside diameter measurement of the pipe. (Christensen, 2009) (NACE International , 2018)

### 4.7 Pressure Losses

Pressure losses in this model are affected by friction and elevation with minor loses assumed to be negligible, it can be seen by the results that increasing both friction and elevation does increase pressure loses significantly, which would mean greater pumping power would be required to re-pressurise the fluid back to its operating pressure.

### 4.8 Length of Pipe

As expected, pipes of greater length experience greater frictional losses, which is due to length being a dependant variable in the frictional loses equation:

∆hf=f LD V22g

∴ The greater distance that the fluid has to travel means more energy is lost due to friction

### 4.9 Gabi Result Analysis

The Discrepancy between GaBi results and the authors calculated results is due to two main factors:

1) The differences between the author’s method of calculating the pipeline loses as GaBi makes use of an average utilisation ratio which accounts for the time that the pipeline is operational, and how much product is being distributed compared to the maximum product that is being distributed, whereas the author assumed that the pipeline is distributing at maximum product capacity and that the pipeline is constantly operational. It was also assumed that minor loses were negligible, whereas it is not known if GaBi accounts for the minor losses of the pipeline, however if Gabi did account for minor losses it would decrease the efficiency level, bringing it further away from the author calculated value.

2) GaBi’s results are also based on the averages of many different product pipelines, not just pipelines dedicated to fuel product, the product carried is defined in terms of ‘cargo’ of 1Kg.

### 4.9.1 Efficiency Value

The efficiency value created from both the hand calculated and GaBi results describe the system to be of very high efficiency before the thermal loses are taken into account, however it should be understood what this efficiency value represents. The value is comparing the amount of energy consumed in order to transport a designated mass of fuel energy, which because of the high energy content of both fuels being transported will always show losses to be minimal in comparison, therefore if this case study were to be repeated, a more effective way of comparing efficiencies could be utilised.

## 5.0 Conclusion

This section of the well to wheel method has proved that the use of pipelines to transport fuel product is effective as long as the pumping stations are operating efficiently, as pumping efficiency has a far greater effect on the overall efficiency than the overall losses from the pipeline.

The calculated model has provided results that are close to the results provided by GaBi, which validates the case study methodology.

# Refinery to Storage Distribution Points

# (Via Ship)

## 1.0 Research

Ships can carry vast amounts of product at a time and can be distributed with relative ease to river or seaside ports without the concern of road based logistical issues. (Helmy, 2016) Although sea-based travel has an advantage over a method such as road fuel tankers, there are still a few major drawbacks to consider. With vehicle regulations increasing in stringency year by year, there is still growing concern that the marine industry is falling behind in terms of emissions and fuel consumption. (Kristensen, 2002)

It should be noted that fuel pipelines are the preferred mode of transport, however because some product is still moved via ship, it is necessary to investigate this stage in the well to wheel process. (UK Petroleum Industry, 2017)

### 1.1 Fuel consumption of Oil Tankers, Ocean and Coastal

The fuel consumption and emissions production of large ships has been under discussion for some time, shipping has been named responsible for approximately 2.5% of the world’s overall greenhouse gas emissions. It has also been said that there is the potential to reduce emissions and energy consumption by 75%. (European Commission, 2017)

Ships can be powered by various fuels such as “Heavy Fuel Oil, Marine Diesel Oil, Marine Gas Oil and biodiesel.” The type of fuel used will vary depending on the specification of the ship in question. (Gusti, 2016)

There are several different sizes of tankers that are capable of carrying fuel product, some of which are detailed below:

(United Nations Conference on Trade and Developement , 2016)

### 2.0 Methodology

The well to wheel process used in this report is based upon the transportation from refineries to oil storage being predominantly carried out by pipeline, however there are some instances where this can be carried out by ship, therefore ship transportation will be discussed but not to the extent of the pipeline section.

Gabi results have been collected to provide the fuel consumption of various ship sizes travelling by river or sea, upstream and downstream:

## 3.0 Results

River freight ship – GaBi – Upstream (4000t capacity)

| Cargo Transported | Mass Of Diesel Used to Transport Cargo (Per 100km) | CO2 Produced per 1Kg transported (Per 100km) |
| --- | --- | --- |
| 1 Kg | 0.000425Kg | 0.00129 Kg |

Figure 57- Table of River freight ship – GaBi – Upstream (4000t capacity)

River freight ship – GaBi – Downstream (4000t capacity)

| Cargo Transported | Mass Of Diesel Used to Transport Cargo (Per 100km) | CO2 Produced per 1Kg transported (Per 100km) |
| --- | --- | --- |
| 1 Kg | 0.000137 Kg | 0.000416 Kg |

Figure 58-Table of River freight ship – GaBi – Downstream (4000t capacity)

Oil Tanker - Ocean Going, average capacity

| Cargo Transported | Mass Of Heavy fuel Oil Used to Transport Cargo (Per 100km) | CO2 Produced per 1Kg transported (Per 100km) |
| --- | --- | --- |
| 1 Kg | 0.000139 Kg | 0.00049 Kg |

Figure 59-Table of Oil Tanker - Ocean Going, average capacity

Oil Tanker – Coastal, average, average capacity

| Cargo Transported | Mass Of Light fuel Oil Used to Transport Cargo (Per 100km) | CO2 Produced per 1Kg transported (Per 100km) |
| --- | --- | --- |
| 1 Kg | 0.000335Kg | 0.00116 Kg |

Figure 60- Table of Oil Tanker – Coastal, average, average capacity

Oil Tanker – Ocean going 10000t-300000t capacity

| Cargo Transported | Mass Of Heavy fuel Oil Used to Transport Cargo (Per 100km) | CO2 Produced per 1Kg transported (Per 100km) |
| --- | --- | --- |
| 1 Kg | 0.000139 Kg | 0.000439 Kg |

Figure 61- Table of Oil Tanker – Ocean going 10000t-300000t capacity

### 3.1 Efficiency calculations:

Efficiency is calculated using the same method as the pipeline and lorry sections:

Efficiency =100- amount of fuel consumed (Kg)amount of fuel transported (Kg)

| Vessel Considered | Equation | Efficiency |
| --- | --- | --- |
| River freight ship –Upstream (4000t capacity) | 100-0.000425 Kg1Kg | 99.999 |
| River freight ship –Downstream (4000t capacity) | 100-0.000137 Kg1Kg | 99.999 |
| Oil Tanker - Ocean Going, (average capacity) | 100-0.000139 Kg1Kg | 99.999 |
| Oil Tanker – Coastal, average, (average capacity) | 100-0.000335 Kg1Kg | 99.999 |
| Tanker – Ocean going (10000t-300000t capacity) | 100-0.000139 Kg1Kg | 99.999 |

Figure 62- Efficiency Table of Various Vessels Considered

### 3.2 Values converted to capacity

CO2 produced and energy consumed is stated in terms of per Kg of product transported, which is why this value seems so small, in reality each ship considered carries much larger quantities of fuel than a single Kilogram, this disguises the large fuel consumption that ships tend to have. Therefore, each value will be adjusted in the table below to give an idea of the total fuel consumption per 100Km of each of the vessels considered:

| Vessel Considered | Original value of fuel consumption (Kg) | Original value of CO2 Production (Kg) | Total Kg of cargo | New Fuel Consumption Value (Kg Per 100km) | New CO2 Production value (Kg Per 100Km) |
| --- | --- | --- | --- | --- | --- |
| River freight ship –Upstream (4000t capacity) | 0.000425 | 0.00129 | 4×106 | 1700 | 5160 |
| River freight ship –Downstream (4000t capacity) | 0.000137 | 0.000416 | 4×106 | 548 | 1664 |
| Oil Tanker – Ocean going 10000t-300000t capacity | 0.000139 | 0.000439 | 1×107 And 3×108 | 1390 And 41700 | 4390 And 131700 |

Figure 63- Converted Figures into Capacity Format

## 4.0 Analysis

4.1 Co2  Results 
It can be seen from the last section of this table that a greater amount of Co2 is produced when compared to the original value of fuel, this is because of the chemical reaction between the carbon within the fuel and the oxygen in the air, carbon atoms combine with oxygen atoms to create a heavier molecule after the reaction. (US Energy Administration , 2018)

CO2 Emissions are still prominent despite the high efficiency values of fuel consumption. However, when compared to transportation via lorry, less emissions are produced when 1 kilogram of fuel is transported by ship rather than by lorry.

4.2 Energy Consumption
River Freight ships travelling upstream consumed the greatest amount of fuel out of the data considered, whereas the same ship travelling downstream consumed the least.

Contradictory to expectations, the largest sea going oil tanker (Oil Tanker – Ocean going 10000t-300000t) has a better fuel capacity than the river freight ship (Upstream -4000t capacity), which suggests that the resistance by the upstream river flow has a larger effect than the weight of the cargo being carried.

### 4.3 Efficiency

As mentioned previously within the pipeline section, efficiency values are exaggerated by the fact that a small number (fuel consumed) is being compared to a very large number (fuel transported), however because each efficiency is being calculated in the same way, this result is still comparable.

This efficiency value for energy consumed compared to energy transported has been the highest so far, because of the vast capacity of oil tanker ships outweighing the fuel consumption results. However, in terms of transporting sheer mass of product, the efficiency values do conclude that this is the most efficient option.

### 4.4 Additional Transport

It is also important to consider than once fuel has been transported by ship, it will need to undergo another form of transportation if the fuel demand is coming from in land.

## 5.0 Conclusion

A variety of results covering the varying types of fuel carrying sea vessels have been collected and analysed, determining ship transportation more efficient than expected.

# Distribution from Storage Points to Local Garage

# (Via Lorry)

### 1.0 Research

Once fuel has been transported in a refined state via pipelines to various distribution points, it then becomes the job of large road tankers to distribute the refined product to numerous fuel stations.

*It is important to note that an assumption has made in this part of the model regarding the way that fuel is transported from distribution points to filling stations. Road tankers are the most common method of transportation at this stage. (Energy Institute, 2018)Therefore, the data being worked with will be concentrated to Road tanker usage, as other variations of transport have minimal input within this model.

### 1.1 Amount of fuel sold in the UK 2017:

The amount of fuel needed to successfully supply filling stations varies massively depending on the type of fuel station in question. The two main types of fuel station considered are independent fuel stations and supermarket filling stations.

Figure 64- Comparison of Dealer, Independent and Supermarket Fuel Throughput

The above shows the vast difference in the average amount of throughput experienced by independent and supermarket fuel stations. In 2017, supermarkets have been seen to distribute approximately 10.5 million litres of fuel, compared to a much lower figure of 2.4 million litres for independent stations. Collectively, 12.9 million litres of fuel were sold in 2017.

(UKpia, 2017)

## 2.0 Methodology

### 2.1 Amount of fuel a single large road tanker can carry at full capacity

Large fuel tankers will generally carry fuel at maximum capacity (98% for liquids) predominantly for safety reasons. (Europe Inland Transport Committee , 2017) A half laden fuel lorry can be very dangerous due to the instability caused by the movement of the fuel within the tanker. (J. L. Bautista-Jacobo, 2015).

A typical maximum capacity of a large fuel tanker can vary, the average figure for a large tanker is assumed to be approximately 43,000 litres, this is based on data collected from one of the larger UK logistical companies ‘Suttons’. (Suttons, 2018)

### 2.2 Maximum Road Tanker Journeys

The stated maximum working hours for a driver of a heavy goods vehicle is 11 hours, with a mandatory hour’s break. (GOV, 2017) This predetermines the maximum distance a road tanker can travel within a driver’s working hours. The above figures regulate the maximum journey to 4.5 hours each way (including the mandatory hours brake that is taken at midway through the journey).

The Highway Code dictates that the maximum speed for a vehicle weighing more than 7.5 tonnes travelling on the motorway is 60 mph and 55 mph on dual carriageways (Departement for Transport, 2012), 55 mph is used in this model as a middle ground of the average speed that a road tanker would normally achieve.

Combining the above figures:

Therefore, the maximum round trip that a lorry driver can make is 4.5 hours in each direction, coupled with the average speed of a large lorry (55mph), the maximum mileage a lorry could cover in a single shift would be:

4.5 ×2 ×55=495 Miles maximum, one way

4952=247.5 Miles each way

This coincides with data collected from the interview mentioned above, the maximum road journey stated by Sutton’s group was approximately 250 miles each way. (Suttons, 2018)

### 2.3 Average Road Tanker Journeys

The average Journey stated by Sutton’s group was believed to be 90 miles each way, 180 miles collectively. (Suttons, 2018)

A separate report carried out by the Transport Studies Group, states that the ‘average length of liquid haul by articulated tanker is 132km (81.97 miles) each way. (Transport Studies Group, 2004)

*This report was compiled in 2004, where there was a larger number of refineries and distribution point, which would have an impact on average journey lengths.

This statement coincides with an article taken from a ‘commercial motor’ periodical, which states that the average length of journeys for haulage is increasing. (Harris, 2007)

With all of the above considered, the average journey length used in this model will be kept to the data gathered from the large logistical transport firm ‘Sutton’s Group’ declaring an average of a 180-mile round trip.

### 2.4 Lorry Fuel Usage per Journey

A typical large cargo vehicle expects to achieve approximately 8 mpg (1.76 miles per litre). (Suttons, 2018) (Department for Transport, 2017)

Consequently, a tanker on the assumed average journey would use 1.76 miles per litre of travel:

1.76 ×180 =316.8 (Litres of fuel used per average Journey )

### 2.5 Energy Usage per Journey

An American gallon of gasoline contains 1.3×108 Joules of Energy.

1 American Gallon= 3.78541 Litres= 1.3×1083.78541= 34342382.99=3.434×107J

316.8 ×  3.434×107=1.1×1010J (energy used in fuel per average journey)

### 2.6 Co2 Produced per average Journey

When a Litre of Diesel is burned, approximately 2.3035Kg of carbon dioxide is produced. (Carmen Data Ltd , 2018)

2.3035 ×316.8=729.7488 Kg of Co2 produced per Journey

### 2.7 Total energy within the fuel being transported

43,000 litres of petrol in terms of energy = 43,000 × 3.434×107

=1.477×1012Joules

### 2.8 Efficiency

Efficiency is calculated by comparing the amount of fuel consumed over a typical lorry tanker journey in comparison to the fuel energy transported in the tanker.

Total energy consumed in one average journey = 1.1×1010J

Total Emissions produced per average journey = 729.7488 Kg

Efficiency value for journey=Total energy carried-energy consumedTotal energy carried

EfficiencyGasoline value for journey =  1.477×1012- 1.1×10101.477×1012=0.9925 × 100

=99.25%

EfficiencyGasoline value for journey =  1.477×1012- 1.1×10101.477×1012=0.9925 × 100

### 2.9 Amount of Co2 produced per 1Mj of fuel energy carried within average journey

729.7488 Kg of Co2= 1.477×1012 joules of fuel energy carried

729.7488 1.477×1012=4.94×10-10 of Co2 produced per 1MJ of fuel carried

### 2.10 Energy consumed per 1Mj of energy carried

1 MJ × 0.9925=0.9925 J

1-0.9925=0.0075 J

## 3.0 Validation

Gabi is used once again as a secondary tool to calculate the efficiency of fuel lorry transportation. The data collected from Gabi is noted below:

### 3.1 Euro 5 Regulated Diesel Tanker Truck, Gross weight above 32t

| Portion of Cargo Considered | 1Kg |
| --- | --- |
| Amount of energy in 1 Kg of Gasoline | 47.14 MJ |
| Amount of energy in 1Kg of Diesel | 50.593 MJ |
| Amount of energy consumed in diesel to transport 1 Kg (proportional) | 0.00142kg = 50.593×0.00142=0.07184J |
| Amount of CO2 Produced when 1Kg of fuel is transported | 0.00428 Kg |

Figure 65- Validation Data from GaBi - 1Kg of General Cargo Considered

( Gabi Thinkstep, 2018)

Efficiency value for journey=Total energy carried-energy consumedTotal energy carried

Efficiency value for journeyDiesel= 50.593-0.0718450.593=99.858%

Efficiency value for journeyPetrol =47.14-0.0718447.14=99.847%

### 3.2 Comparison to case study calculated data:

| Calculated Efficiency | Gabi Efficiency |
| --- | --- |
| 99.25% (Gasoline) | 99.858% (Gasoline) |

Figure 66- Calculated Efficiency of Ship Compared to Gabi Calculated Efficiency

## 4.0 Analysis

Although efficiency levels calculated by the author and by GaBi show lorry tanker transportation to be very efficient, it must be recognised what the stated efficiency values represent. The efficiency of the lorry is stated as the remaining energy left when loses are accounted for, when loses represent the energy used by the lorry. When an efficiency of 99.858% is stated, this means the lorry will consume the equivalent of 0.142% of its fuel load within the tanker per 100km. An average tanker carries approximately 43,000 litres of fuel; therefore, the lorry consumes 0.142 % of this value which represents a fuel consumption of 61.06 litres an hour.

If one assumes this model to be accurate, it states that every time a large fuel tanker travels 100km, approximately 61 litres of fuel is burnt, considering that there are thousands of fuel tanker journeys carried out each day, some of which travel well over 100km, massive amounts of fuel are used despite what the efficiency rating may appear to represent. Which brings a secondary point made evident by the analysis of this section of the well to wheel process, deceptive efficiency ratings.

The reliability of efficiency ratings are restricted in their usefulness by the values that are being compared, for example, the efficiency value above is comparing the amount of fuel used by the lorry as a ratio of the total fuel carried by the tanker and because the tanker is carry vast amounts of fuel, the fuel used by the lorry is always going to be a significantly smaller number in comparison, this in turn will provide an efficiency value that seems remarkably high if it is not properly understood what this value represents. In hindsight, this was not the best way to represent energy consumption because of the confusion that could arise from the percentages calculated.

Additionally, the author has used a different value for the average length a typical journey consists of (approximately 180 miles return trip), Gabi values represent the efficiency over 100km trip, therefore the efficiency values are not actually comparable as they are based upon very different values.

To make these values comparable, 180 miles should be converted into equivalent Km, which is approximately 290km, which means the authors calculated values are based on a journey that is approximately 1.6 times larger than the journey references in GaBi, which consequently means that the losses would be approximately 1.6 times larger also. Therefore, the author’s in-efficiency value can be corrected by reducing it by 1.6 times.

Orginal inefficeny=0.75%

Recalculated inefficiency=0.75 ×1.6= 0.46%

Which now changes the comparison table:

| Calculated Efficiency | Gabi Efficiency |
| --- | --- |
| 99.531% (Gasoline) | 99.858% (Gasoline) |

Figure 67- Altered Comparison Table into 100km

The author’s value is now closer to the value calculated by Gabi, however there is still a 0.327 discrepancy which may seem small, but as explained in the previous paragraph, this small percentage would represent a large difference in energy consumption.

The energy consumption of this section of the well to wheel process is reliant on the average fuel consumption achieved by the type of lorry tanker being considered, ways of improving the efficiency of this process would be increased mpg and greater logistical awareness toward lorry routes.

## 5.0 Conclusion

An effective study of the energy consumed by lorry tankers when distributing fuel from storage to local fuel station sites has been carried out. However, if this process were to be repeated, a different method would be chosen to calculate the efficiency at this stage. The method chosen is accurate and informative, although could be easily misinterpreted if the understanding of what the efficiency value represents was not fully understood.

The main point demonstrated by this report is that Lorries do consume large amounts of energy throughout average journeys, however the amount of fuel consumed compared to the amount of product transported makes the fuel consumption of the lorry seem relatively small.

It is also important to highlight that the results calculated by both the author and GaBi are based on average journey length and average lorry tanker fuel consumption, any variation to these average figures would directly affect results.

# Usage Phase

## 1.0 Research

1.1 Drive cycle testing
Drive cycle testing is an essential tool in determining the fuel consumption and emission production of passenger vehicles.  The results taken from test procedures allow manufacturers to determine if their vehicles meet current emission legislation standards. (T J Barlow, 2009) Results are also useful from a consumer perspective, Emission and fuel consumption ratings can affect consumer decision making when purchasing a new vehicle. (Vehicle Certification Agency , 2017)

1.2 NEDC
The New European Drive Cycle test procedure was initially created more than three decades ago and is under constant negative scrutiny for being outdated and in-effective for representing real world driving conditions. This is mainly due to the structure and set conditions of the test procedure. The NEDC consists of two main stages which are described as the ‘Urban phase’ and ‘Extra Urban Phase’.

1.2.1 Urban Phase
The Urban Phase consists of four repetitive stages of the ECE 15 cycle which is described via the graph below, the drive cycle is supposed to represent a pattern of driving in a busy European city.

Figure 68- Graphical Description of NEDC Urban Stage

1.2.2 Extra Urban Phase 
The Extra Urban phase aims to characterise a driving pattern that would be associated with roads external to the city.

Figure 69- Graphical Description of NEDC Extra Urban Phase

### 1.3 The Effect of Modern Dynamometers on NEDC Vehicle Inertia Categories

Before the existence of modern dynamometers that can simulate any vehicle weight, mechanical dynamometers were used which simulated resistive forces by the addition of physical incremented weights, which is an inaccurate method as vehicle weights are normally varying rather than incremented.  (The International Council on Clean Transportaton, 2011) (D.Tsokolis, 2016) Vehicles were assessed via various inertia categories, which meant that the resistance provided to the dyno rollers for the vehicle to overcome varied depending on the inertia category of the vehicle. Inertia categories were calculated via a step system which is described graphically below:

Figure 70- Stepped Inertia Classes of Vehicles

(The International Council on Clean Transportaton, 2011)

This step system allows a leeway within each step that a vehicle can be classified in, which means the inertia class would be more of an approximation of vehicle weight rather than actual vehicle weight. “This gave manufacturers the incentive to squeeze their vehicles into a lower inertia class. Vehicles at the upper boundary of an inertia class are tested on a dyno simulating less inertia weight than the vehicle’s engine must overcome in real-world operation on the road. The engine therefore can work less hard, burn less fuel and emit fewer pollutants” (D.Tsokolis, 2016) (The International Council on Clean Transportaton, 2011).

1.4 WLTP
The New European driving cycle is used regularly to validate the ‘well to wheel’ efficiency of a vehicle. But as previously mentioned, over the recent years it has come under major criticism for not being an effective representative of ‘real world’ driving. (Joachim Demuynck, 2012) As of September 2017, vehicles now must adhere to new legislative testing called the worldwide harmonized light vehicle test procedure, which is said to be much more demonstrative of actual driving. Vehicles will also soon be made to demonstrate WLTP information such as the fuel consumption and CO2 output on legislative stickers on the forecourt so that buyers can consider these aspects before purchasing.  (WLTPfacts, 2017) The WLTP is seen to be a more extensive and thorough procedure of which will produce higher results that the NEDC as it is a more demanding and realistic duty cycle. (G.O.Duarte, 2016) There are three classes of WLTP test procedure. Each class covers vehicles with various power to mass ratios. (International Council on Clean Transportation, 2018)

| Class | Description | Duty Cycle |
| --- | --- | --- |
| Class One | Vehicles with very low PMR, PMR≤22 covering low and medium speed ranges. (International Council on Clean Transportation, 2018) | (Nicolas, 2013) |
| Class Two | Vehicles with medium values of PMR, 22≤PMR≤34 covering low medium and high speeds. (International Council on Clean Transportation, 2018) | (Nicolas, 2013) |
| Class 3 | Class 3 – represents vehicles with higher values of PMR, PMR>34 covering low medium and high speeds. (International Council on Clean Transportation, 2018) | (Nicolas, 2013) |

Figure 71- Descriptive Table Including Graphs of Different Classes of the WLTP

### 1.5 Graphically described differences between NEDC and WLTP

To effectively describe differences in results between the NEDC and WLTP drive cycles, Excel spreadsheets that model the specific cycle speed and time data are utilised and graphs are created to visually highlight major variations between both cycles. The excel data was obtained from an open web source and can be validated by comparing the speed time plot to images of the WLTP speed time plots that are found throughout the internet in journals and websites:

Excel Plot of WLTP Speed Vs Time: 	              WLTP Plot of Speed Vs Time from academic journal:

Figure 72- Excel Plot of WLTP

Figure 73- Academic Journal Plot of WLTP

(NEDC Excel Workbook,2017)

(D.Tsokolis, 2016)

### 1.5.1 Acceleration Variation

The plots below show the differences in rates of acceleration between both duty cycles, the NEDC’s accelerations/decelerations are smooth and stylised, whereas the WLTP is far less gradual and more varied shown by the more erratic plot of data, this will increase the energy demand as more fuel is consumed when frequent and uneven accelerations take place in comparison to smooth and gradual accelerations.  (Hanna Larsson, 2009) (David J. Chang, 2005)

Figure 74- Acceleration Vs Time of NEDC

Figure 75- Acceleration Vs Time of WLTP

### 1.5.2 Velocity differences

The velocities differ greatly between both duty cycles, the speed increases and decreases in a much more gradual fashion within the NEDC as opposed to the WLTP where changes in speed are irregular, which more closely represents a busy urban area as traffic is unpredictable.

There are less instances in the WLTP where the vehicle comes to a complete stop, the WLTP models more a lifting off and on situation to account for slow moving traffic rather than completely stopping and starting, this could be better representative of the greater occurrence of roundabouts as opposed to traffic lights within Europe, whereby traffic often slows down on the approach to a roundabout but doesn’t always come to a complete stop.

Figure 76- Velocity Vs Time of NEDC

Figure 77-Velocity Vs Time of WLTP

### 1.5.4 General Differences

The WLTP and NEDC are compared regularly as the WLTP is referred to as the improved replacement. There are a few major differences of how the NEDC and WTLP differ, the tables below highlight some of the changes that make the WTLP an ‘improved’ test procedure:

Figure 78- WLTP and NEDC Comparison Table 1

(WLTPfacts, 2017)

Figure 79- WLTP and NEDC Comparison Table 2

(S. Tsiakmakis, 2016)

Comparative tests have shown that NEDC and WLTP have seen differences in CO2 readings of up to 11% and discrepancies of energy usage of up to 44% (Jelica Pavlovic, 2016).

To effectively account for the differences between using the NEDC and the more accurate WTLP, the percentage differences of inaccuracies of the NEDC compared to the WTLP will be researched further and considered when collaborating data.

### 1.6 Differences in the vehicle efficiency predicted by the WLTP and NEDC

There are several academic papers stating that there is a general discrepancy of the vehicle efficiency values between the NEDC and WLTP. (Georgios Triantafyllopoulos, 2017) (D.Tsokolis, 2016) The graph below originates from a paper investigating Fuel consumption and CO2 emissions of passenger cars over the New Worldwide Harmonized Test Protocol, this paper suggests that there is a variance between the drive cycles method of calculating engine vehicle efficiency:

Figure 80- Vehicle Efficiency Plots Comparing NEDC and WLTP

(D.Tsokolis, 2016)

## 2.0 Methodology

The UCSB Automotive Energy & GHG Model was created by the World Auto Steel Association as a tool to be used by vehicle makers generally to assess the overall impact that their vehicles will have on the environment in terms of energy usage and Greenhouse gas emissions. (WorldAutoSteel, 2017)

Using the program UCSB Automotive Energy & GHG Model, the author was able to access in depth information regarding the energy consumed over the NEDC and WLTP Class three duty cycles by various vehicles powered by Electricity, Gasoline and Diesel. The spreadsheet consists of an ‘input sheet’ of which different settings of powertrain, vehicle size and drive cycle can be selected, from these inputs the spreadsheet will utilise numerous LCA data sets to calculate the energy usage used and greenhouse gas emitted from the selected settings.

## 3.0 Results Gathered from the UCSB Automotive Energy & GHG Model:

Battery electric Vehicle results using different model size settings, FSV A-Class (compact) and FSV C-Class (midsize). The Vehicle Sizes were run through both the NEDC and WLTP duty cycle and the results can be seen below:

Figure 81- Results of Battery Electric Vehicle (FSV - A Class)

| Battery Electric Vehicle | Vehicle Size | Vehicle Mass (Kg) | Electricity Source | Electricity Demand (Mj/100km) | Upstream Greenhouse gas emissions (per Km) gCO2 eq/km |
| --- | --- | --- | --- | --- | --- |
| NEDC Baseline | FSV A-Class | 1097 | Europe | 37.74 | 55 |
| NEDC Contender 1 | FSV A-Class | 981 | Europe | 35.12 | 51 |
| NEDC Contender2 | FSV A-Class | 943 | Europe | 34.28 | 50 |
| WLTP Baseline | FSV A-Class | 1097 | Europe | 34.02 | 49 |
| WLTP Contender 1 | FSV A-Class | 981 | Europe | 31.60 | 46 |
| WLTP Contender 2 | FSV A-Class | 943 | Europe | 30.82 | 45 |

| Battery Electric Vehicle | Vehicle Size | Vehicle Mass (Kg) | Electricity Source | Total Fuel Consumption (Electricity) (MJ) | Upstream Greenhouse gas emissions (per Km) gCO2 eq/km |
| --- | --- | --- | --- | --- | --- |
| NEDC Baseline | FSV C-Class | 1334 | Europe | 45.92 | 66 |
| NEDC Contender 1 | FSV C-Class | 1186 | Europe | 42.53 | 61 |
| NEDC Contender2 | FSV C-Class | 1138 | Europe | 41.45 | 60 |
| WLTP Baseline | FSV C-Class | 1334 | Europe | 42.16 | 61 |
| WLTP Contender 1 | FSV C-Class | 1186 | Europe | 38.88 | 56 |
| WLTP Contender 2 | FSV C-Class | 1138 | Europe | 37.83 | 55 |

Figure 82- Results of Battery Electric Vehicle FSV - C Class

### 3.1 Gasoline Results:

There are three available size setting for gasoline powered vehicles within the UCSB Automotive Energy & GHG Model, Compact, Mid-size and SUV. All three sizes were run through the NEDC and WLTP drive cycle and the results obtained can be seen below:

Figure 83- Results of Gasoline Vehicle (Midsize and Compact)

| ICE - Gasoline | Vehicle Size | Vehicle Mass (Kg) | Fuel Type | Liquid Fuel Demand MJ/100km | Upstream Greenhouse gas emissions (per Km) gCO2 eq/km |
| --- | --- | --- | --- | --- | --- |
| NEDC Baseline | Compact | 1260 | Gasoline | 163.53 | 143 |
| NEDC Contender 1 | Compact | 1143 | Gasoline | 154.04 | 135 |
| NEDC Contender2 | Compact | 1106 | Gasoline | 151 | 132 |
| WLTP Baseline | Compact | 1260 | Gasoline | 160.74 | 141 |
| WLTP Contender 1 | Compact | 1143 | Gasoline | 150.22 | 132 |
| WLTP Contender 2 | Compact | 1106 | Gasoline | 146.85 | 129 |

| ICE - Gasoline | Vehicle Size | Vehicle Mass (Kg) | Fuel Type | Liquid Fuel Demand MJ/100km | Upstream Greenhouse gas emissions (per Km) gCO2 eq/km |
| --- | --- | --- | --- | --- | --- |
| NEDC Baseline | Mid-Size | 1640 | Gasoline | 233.97 | 205 |
| NEDC Contender 1 | Mid-Size | 1510 | Gasoline | 219.87 | 193 |
| NEDC Contender2 | Mid-Size | 1468 | Gasoline | 215.35 | 189 |
| WLTP Baseline | Mid-Size | 1640 | Gasoline | 226.42 | 199 |
| WLTP Contender 1 | Mid-Size | 1640 | Gasoline | 212.57 | 186 |
| WLTP Contender 2 | Mid-Size | 1640 | Gasoline | 208.14 | 183 |

### 3.1.2 Gasoline Continued:

| ICE - Gasoline | Vehicle Size | Vehicle Mass (Kg) | Fuel Type | Liquid Fuel Demand MJ/100km | Upstream Greenhouse gas emissions (per Km) gCO2 eq/km |
| --- | --- | --- | --- | --- | --- |
| NEDC Baseline | SUV | 2195 | Gasoline | 324.99 | 285 |
| NEDC Contender 1 | SUV | 2020 | Gasoline | 304.01 | 267 |
| NEDC Contender2 | SUV | 1963 | Gasoline | 297.30 | 261 |
| WLTP Baseline | SUV | 2195 | Gasoline | 330.10 | 289 |
| WLTP Contender 1 | SUV | 2020 | Gasoline | 309.20 | 271 |
| WLTP Contender 2 | SUV | 1963 | Gasoline | 302.51 | 265 |

Figure 84- Results of Gasoline (SUV)

### 3.2 Diesel Results:

| ICE – Diesel | Vehicle Size | Vehicle Mass (Kg) | Fuel Type | Liquid Fuel Demand MJ/100km | Upstream Greenhouse gas emissions (per Km) gCO2 eq/km |
| --- | --- | --- | --- | --- | --- |
| NEDC Baseline | Compact | 1350 | Diesel | 161.56 | 124 |
| NEDC Contender 1 | Compact | 1233 | Diesel | 151.85 | 117 |
| NEDC Contender2 | Compact | 1196 | Diesel | 148.75 | 115 |
| WLTP Baseline | Compact | 1350 | Diesel | 160.83 | 124 |
| WLTP Contender 1 | Compact | 1233 | Diesel | 150.47 | 116 |
| WLTP Contender 2 | Compact | 1196 | Diesel | 147.16 | 113 |

The same size settings were available for the diesel powertrain setting, Compact, Mid-size and SUV, once again all three size settings were run through the NEDC and WLTP Drive cycles.

Figure 85- Results of Diesel (Compact and Mid-size)

| ICE – Diesel | Vehicle Size | Vehicle Mass (Kg) | Fuel Type | Liquid Fuel Demand MJ/100km | Upstream Greenhouse gas emissions (per Km) gCO2 eq/km |
| --- | --- | --- | --- | --- | --- |
| NEDC Baseline | Mid-size | 1740 | Diesel | 194.76 | 150 |
| NEDC Contender 1 | Mid-size | 1610 | Diesel | 183.90 | 142 |
| NEDC Contender2 | Mid-size | 1568 | Diesel | 180.42 | 139 |
| WLTP Baseline | Mid-size | 1740 | Diesel | 197.35 | 152 |
| WLTP Contender 1 | Mid-size | 1610 | Diesel | 185.37 | 143 |
| WLTP Contender 2 | Mid-size | 1568 | Diesel | 181.54 | 140 |

### 3.2.1 Diesel Results Continued:

| ICE – Diesel | Vehicle Size | Vehicle Mass (Kg) | Fuel Type | Liquid Fuel Demand MJ/100km | Upstream Greenhouse gas emissions (per Km) gCO2 eq/km |
| --- | --- | --- | --- | --- | --- |
| NEDC Baseline | SUV | 2320 | Diesel | 263.53 | 203 |
| NEDC Contender 1 | SUV | 2145 | Diesel | 248.99 | 192 |
| NEDC Contender2 | SUV | 2088 | Diesel | 244.34 | 188 |
| WLTP Baseline | SUV | 2320 | Diesel | 263.10 | 203 |
| WLTP Contender 1 | SUV | 2145 | Diesel | 247.41 | 191 |
| WLTP Contender 2 | SUV | 2088 | Diesel | 242.38 | 187 |

Figure 86- Results of Diesel (SUV)

## 4.0 Analysis

The analysis is organised in a vehicle size and duty cycle layout so that graphs can be easily comparable.

### 4.1 Compact – NEDC

| COMPACT (NEDC) (BASELINE) |  |  |
| --- | --- | --- |
| Battery Electric Vehicle Electricity Demand(MJ) | ICE Gasoline Liquid Energy Demand (MJ) | ICE Diesel Liquid Energy Demand (MJ) |
| 37.74 | 163.53 | 161.56 |
| Battery Electric Vehicle CO2 gCO2 eq/km | ICE Gasoline CO2 Emission gCO2 eq/km | ICE Diesel CO2 Emission CO2 gCO2 eq/km |
| 55 | 143 | 124 |

Figure 87- Compact NEDC BASELINE - Compact

Figure 88- Energy Consumption of Compact Vehicles (NEDC)

Figure 89- Co2 Production for Compact Vehicles

### 4.1.1 Compact - WLTP

| COMPACT (WLTP) (BASLINE) |  |  |
| --- | --- | --- |
| Battery Electric Vehicle Electricity Demand(MJ) | ICE Gasoline Liquid Energy Demand (MJ) | ICE Diesel Liquid Energy Demand (MJ) |
| 34.02 | 160.74 | 160.83 |
| Battery Electric Vehicle CO2 gCO2 eq/km | ICE Gasoline CO2 Emission gCO2 eq/km | ICE Diesel CO2 Emission CO2 gCO2 eq/km |
| 49 | 141 | 124 |

Figure 90- (WLTP)(BASELINE)- Compact

Figure 91- Energy Consumption Comparison (WLTP)

Figure 92- Co2 Production (WLTP) Compact Vehicle

### 4.1.2 ICE and BEV differences in energy consumption:

The graph of energy consumed by different powertrain set ups clearly states that the energy consumed by an electric vehicle is significantly less in both the NEDC and WLTP drive cycles, this is can be explained by the differences in efficiency between the electric vehicle set ups and the ICE set ups. The ICE set ups loose significantly more energy throughout the system due to the frictional, drivetrain, pumping and incomplete combustion loses, an example of ICE losses are described graphically below:

Figure 93- Example of Ice Vehicle Losses

(National Renewable Energy Laboratory, 1994)

Due to the significant efficiency differences between the battery operated and ICE driven vehicles, ICE consumed far more energy than the BEVs and as a result of this more fuel energy is required, which when burnt produces additional greenhouse gases. (Mark A. Delucchi, 2000)

### 4.1.3 Diesel and Gasoline Energy Consumption Differences:

When a compact vehicle is run through the NEDC drive cycle, the diesel set ups have used remarkably similar amounts of energy in comparison to the gasoline, this is an unexpected result as diesel engines are usually referred to as more efficient than gasoline.

This result could be due to the differences in weight between the diesel and gasoline compact vehicles, the diesel compact vehicle weighs an additional 100kg (approximately 7.45% increase in overall weight) which is a significant increase in weight for a small vehicle, and this can therefore lead to an increased fuel consumption.

### 4.1.4 Decreased Greenhouse Gas Values for the Diesel Set Up

Despite diesel and gasoline having a similar energy consumption from this particular set up, the Co2 emissions produced by diesel appears to be far less than gasoline, this coincides with a study carried out by the Centre for Environmental Policy, Imperial College London which carried out Practical portable emissions testing on a number of gasoline, diesel and hybrid vehicles, the tests described diesel to produced substantially more NOx than gasoline, however the diesels also produced 13-66% less Co2 emissions than gasoline. (Rosalind O'Driscoll, 2017)

### 4.2 Mid-Size NEDC

| MID SIZE (NEDC) (BASELINE) |  |  |
| --- | --- | --- |
| Battery Electric Vehicle Electricity Demand(MJ) | ICE Gasoline Liquid Energy Demand (MJ) | ICE Diesel Liquid Energy Demand (MJ) |
| 45.92 | 233.97 | 194.76 |
| Battery Electric Vehicle CO2 gCO2 eq/km | ICE Gasoline CO2 Emission gCO2 eq/km | ICE Diesel CO2 Emission CO2 gCO2 eq/km |
| 66 | 205 | 150 |

Figure 94- Midsize NEDC (BASELINE)

Figure 95- Energy Consumption (NEDC) Mid-size

Figure 96- Co2 Production (NEDC) Mid-size

### 4.2.1 Mid- Size WLTP

| MID SIZE (WLTP) (BASELINE) |  |  |
| --- | --- | --- |
| Battery Electric Vehicle Energy Consumption (MJ) | ICE Gasoline Energy Consumption (MJ) | ICE Diesel Energy Consumption (MJ) |
| 42.16 | 215.35 | 197.35 |
| Battery Electric Vehicle CO2 gCO2 eq/km | ICE Gasoline CO2 Emission gCO2 eq/km | ICE Diesel CO2 Emission CO2 gCO2 eq/km |
| 61 | 189 | 152 |

Figure 97- WLTP (BASELINE) Mid-Size

Figure 98- Energy Consumption (WLTP) Midsize

Figure 99- Co2 Production (WLTP) Mid-Size

### 4.2.3 Mid-Size Vehicle Analysis

### Comparison of Energy Consumption for Mid-Size Vehicles (NEDC + WLTP)

Once again, the battery-operated vehicles in both drive cycles have used significantly less energy and have produced far lesser amounts of greenhouse gases.

### 4.2.4 Differences between Gasoline and Diesel

Interestingly, when assessing the mid-sized vehicles through both drive cycles the diesel has consumed a lesser amount of energy than the gasoline, which is contradictory to the previous compact vehicle results, yet this coincides more with known efficiency characteristics of a diesel engine.

it is important to note there is still a difference in weight between diesel and gasoline models, but the overall weights of both midsize vehicles are larger (and the weight difference is still 100kg) therefore the 100Kg weight discrepancy has a lesser effect overall than it would for the compact vehicles.

### 4.2.5 WLTP Showing Lower Energy Consumption for Both Gasoline and Diesel

The value of gasoline consumption according to the WLTP is 233.97 MJ/100km, whereas the NEDC states a value of 215.35MJ/100Km, this is a conflicting result as the WLTP is usually expected to predict greater energy consumption than the NEDC of up to 20% (Inland Transport Committee, 2014) which sheds doubt upon the gasoline result. Consumption of the diesel vehicle between the WLTP and NEDC demonstrates the reverse with the WLTP cycle shown to be more slightly more energy intensive, which would be more expected.

Once this contradictory result was found, the author repeated the data collection process from the UCSB Automotive Energy & GHG Model and found the result to be consistent, once again showing the NEDC to use more energy than the WLTP for a mid-size gasoline vehicle, this indicated that there may be an issue with the model in question.

After looking deeper into the UCSB Automotive Energy & GHG Model, it could be seen that there is a questionable section of data that would affect the overall usage result.

The model contains some base data that is predetermined by the creators, the data in question is called the “Default fuel energy demand (EDf in MJf/100km)”. This selection of values determines the energy demand from the vehicle as it is running through the chosen drive cycles, this data is shown below:

### 4.2.6 Default Fuel Energy Demand

Figure 100- Default Energy Demands Within Model

(WorldAutoSteel, 2017)

The table above shows the NEDC to be a more energy intensive drive cycle for the NEDC ICEV-G compact, NEDC ICEV-G midsize, NEDC ICEV-D compact, NEDC ICEV-D SUV. However, for the set ups NEDC ICEV-G SUV and NEDC ICEV-D Midsize, the default energy demand is greater when run through the WLTP Cycle, which would be far more expected.

This is a complicated and mostly contradictory result for the author, the next few paragraphs are dedicated to trying to find the reasoning for such seemingly irregular base data.

### 4.2.7 Reasoning for Unexpected WLTP Result:

After numerous time was taken to try and understand the reasoning behind the unexpected result, the above default fuel energy demand seemed to be the main discrepancy, but this discrepancy could not be logically explained, therefore the author contacted World Autosteel directly for assistance. The full email correspondence can be found within the appendix on page …

World Auto steel replied with a very helpful comprehensive email explaining the results and the default energy demand values of which the main reasoning is detailed below:

“The powertrain model was developed before the WLTP test protocol was finalized.  In the addendum to the 2014 Global technical regulation on the WLTC, it was stated in 3.3.2.6., “At the option of the Contracting Party, the Extra High3 phase may be excluded.”, so it was left out of the first version of the powertrain model (the thinking being that no automaker would include it voluntarily).  This is the version of the powertrain model on which the UCSB Model values are currently based. In the final version of the driving cycle used for type approval, the Extra High3 phase is included, so the powertrain model was recently updated. Dr. Geyer, who is also the author of the UCSB Model, is updating the Energy Demand and Energy Savings values, and an updated version of the UCSB Model will be released in the next few weeks as well.” (Balzer, 2018)

This gives reasoning to the results obtained from the model, unfortunately the author would not be able to provide revised results until the next release of this model. The author would like to mention the effectiveness and usability of the UCSB Automotive Energy & GHG Model and the quick response provided by the technical director when a query was made, the next version of the model is highly anticipated.

### 4.3 SUV – NEDC

| SUV (NEDC) (BASELINE) |  |  |
| --- | --- | --- |
| Battery Electric Vehicle Energy Consumption (MJ) | ICE Gasoline Energy Consumption (MJ) | ICE Diesel Energy Consumption (MJ) |
| NA | 324.99 | 263.53 |
| Battery Electric Vehicle CO2 gCO2 eq/km | ICE Gasoline CO2 Emission gCO2 eq/km | ICE Diesel CO2 Emission CO2 gCO2 eq/km |
| NA | 285 | 203 |

Figure 101- NEDC (BASEINE) SUV

Figure 102- Energy Consumption NEDC - SUV

Figure 103- Co2 Production NEDC - SUV

### 4.3.1 SUV - WLTP

| SUV (WLTP) (BASELINE) |  |  |
| --- | --- | --- |
| Battery Electric Vehicle Energy Consumption (MJ) | ICE Gasoline Energy Consumption (MJ) | ICE Diesel Energy Consumption (MJ) |
| NA | 330.10 | 263.10 |
| Battery Electric Vehicle CO2 | ICE Gasoline Energy Consumption CO2 | ICE Diesel Energy Consumption CO2 |
| NA | 289 | 203 |

Figure 104- SUV (WLTP) (BASELINE)

Figure 105- Energy Consumption (WLTP) SUV

Figure 106- Co2 Production (WLTP) SUV

### 4.3.2 SUV Vehicle Analysis

The SUV Vehicle results show a gasoline powered SUV analysed through the NEDC drive cycle consumes less energy and produces lesser co2 emissions than when analysed through the WLTP cycle, this is consistent with what would be the expected result. Yet, when the diesel results are investigated in the same manor they show the NEDC and WLTP result to be remarkably similar.

Unfortunately, because of the very recent change from NEDC to WLTP, it is difficult to find another open access usage model that includes the WLTP drive cycle, which makes validation of the above results and analysis difficult. However, there is still an abundant of manufacturers stated NEDC values for an abundance of compact, midsize and SUV vehicles, which makes validating the NEDC usage model predictions from the UCSB Automotive Energy & GHG Model possible. Please see page 103 for a comparison of the UCSB Automotive Energy & GHG Model and Manufacturers stated NEDC values.

### 4.4 Percentage Difference in Readings Between the NEDC and WLTP

| COMPACT | COMPACT | COMPACT |
| --- | --- | --- |
| Battery Electric Vehicle Energy Consumption (MJ) | ICE Gasoline Energy Consumption (MJ) | ICE Diesel Energy Consumption (MJ) |
| Percentage difference of WLTP from the NEDC (%) | Percentage difference of WLTP from the NEDC (%) | Percentage difference of WLTP from the NEDC (%) |
| -10.93474427 | -1.735722284 | -0.453895418 |

Figure 107- Attempted Comparison of Compact Results

| MID SIZE | MID SIZE | MID SIZE |
| --- | --- | --- |
| Battery Electric Vehicle Energy Consumption (MJ) | ICE Gasoline Energy Consumption (MJ) | ICE Diesel Energy Consumption CO2 |
| Percentage difference of WLTP from NEDC | Percentage difference of WLTP from NEDC | Percentage difference of WLTP from NEDC |
| -8.918406072 | -8.646389598 | 1.312389156 |

Figure 108- Attempted Comparison of Mid-Size Results

| SUV | SUV | SUV |
| --- | --- | --- |
| Battery Electric Vehicle Energy Consumption (MJ) | ICE Gasoline Energy Consumption (MJ) | ICE Diesel Energy Consumption CO2 |
| Percentage difference of WLTP from NEDC | Percentage difference of WLTP from NEDC | Percentage difference of WLTP from NEDC |
| NA | 1.548015753 | -0.163435956 |

Figure 109-Attempted Comparison of SUV Results

This section of the report was created in a hope to demonstrate the clear differences between the NEDC and WLTP energy consumption and Co2 production, however because of the discrepancies in results obtained, this could not be suitably demonstrated.

### 4.5 Validation of Results from Ford compared to UCSB Automotive Energy & GHG Model:

Values of energy consumption and GHG emissions from ford (according to the NEDC) have been compared to the values of the compact and midsize energy consumption and GHG emissions taken from the UCSB Automotive Energy & GHG Model.

| Vehicle Type | NEDC (Urban and Extra Urban Combined) Fuel Consumption in Mpg (L/100km) | Fuel Consumption In MJ/100km Gasoline = 31.88Mj/litre | C02 Emissions (g/km) |
| --- | --- | --- | --- |
| - Fiesta EcoBoost (125 PS) Gasoline | 5.2 | 5.2 ×31.88=165.776 | 118 |
|  | (Ford, 2018) | (Ford, 2018) | (Ford, 2018) |

### 4.5.1 Compact

| Fuel Consumption In MJ/100km Comparison Value by World Autosteel Model (Compact Vehicle) | C02 Emissions (g/km) Comparison Value by World Autosteel Model (Compact Vehicle) |
| --- | --- |
| 163.53 | 143 |
| (WorldAutoSteel, 2017) | (WorldAutoSteel, 2017) |

Figure 110- Comparison of Ford and World Autosteel NEDC Results (Compact)

Differences, vehicle weight from Autosteel model = 1260Kg (WorldAutoSteel, 2017)
Vehicle weight ford fiesta = 1550Kg (Ford, 2018)

| Vehicle Type | NEDC (Urban and Extra Urban Combined) Fuel Consumption in Mpg (L/100km) | Fuel Consumption In MJ/100km Gasoline = 31.88Mj/litre | C02 Emissions (g/km) |
| --- | --- | --- | --- |
| 2.0 Focus EcoBoost (125 PS) Gasoline | 6.8 (Ford, 2018) | 6.8 ×31.88=216.784 (Ford, 2018) | 159 (Ford, 2018) |

| Fuel Consumption In MJ/100km Comparison Value provided by World Autosteel Model (Compact Vehicle) | C02 Emissions (g/km) Comparison Value provided by World Autosteel Model (Compact Vehicle) |
| --- | --- |
| 233.97 | 205 |
| (WorldAutoSteel, 2017) | (WorldAutoSteel, 2017) |

### 4.5.2 Midsize

Figure 111-Comparison of Ford and World Autosteel NEDC Results (Mid-size)

Differences, vehicle weight from Autosteel model = 1640Kg (WorldAutoSteel, 2017)
Vehicle weight ford focus = 1900Kg (Ford, 2018)

### 4.6 Efficiency Calculations:

#### 4.6.1 Ice and Electric Vehicle Efficiencies

According the official US Government source for fuel economy information (which includes the US department of energy and the united states environmental protection agency), ICE vehicle efficiencies are approximately 17% - 21%, whereas electric vehicles are approximately 59%–62%. For the purpose of this section the averages of both of these estimates will be used to represent each vehicles efficiency.  (Fuel Economy.GOV, 2016)

ICE Efficiency Average = 19 %

Electric EfficiencyAverage = 60.5 %

Incorporating this into the data gathered for this section allows a calculation of how much of the fuel energy is effectively used:

Fuel Energy DemandElectricy (NEDC-compact)= 37.78MJ/100Km × 0.605=22.857MJ/100Km

Fuel Energy Demand Gasoline (NEDC-compact)= 163.53MJ/100Km ×0.19=31.0707MJ/100Km

Fuel Energy DemandDiesel (NEDC-compact)= 161.56MJ/100Km ×0.19=30.696MJ/100Km

## 5.0 Conclusions

The amount of fuel consumption recorded by the UCSB Automotive Energy & GHG Model compared to a Ford Fiesta (a compact vehicle produced by ford) are reasonably similar, which suggests that the NEDC predictions by the UCSB Automotive Energy & GHG Model are adequate. Due to the newness of the WLTP model, this process could not be repeated to validate the WLTP results.

Despite the difficulty in obtaining vehicle usage results for the WLTP, the most important point that has still been demonstrated by the NEDC results is that electric vehicles use significantly less energy within the usage phase. This corresponds to general understanding and academic papers related to this subject. (Mark A. Delucchi, 2000) (G.O.Duarte, 2016)

# Electricity Grid Losses (Electricity Transportation)

## 1.0 Electricity Grid Losses

### 1.1 Research

Electricity grid loses are defined as the difference in energy entering the system compared to energy leaving the system.

Losses= Energy In-Energy out

(Sohn Assosiates, 2009)

### 1.2 Types of Loses

There are two types of loses to consider, technical and non-technical:

Technical – Physical loses for example losses due to resistance within power lines.

Non-Technical – Measurement inaccuracies, for example worn electricity meters or miscalculations through data collection.

(UK Power Networks, 2017)

*For the purpose of this paper, only technical loses will be discussed

### 1.3 Technical Loses – Causes

Technical Loses are Predominantly Caused By:

Long distance /High voltage power lines - energy in lost as heat due to resistance in the wires

Distribution line losses – Once again energy is lost due to resistance but additionally transformers are needed to step the energy down into household suitable voltages which also creates loses.

Loses can vary depending on surrounding temperatures and fluctuating demands, for example, transmission wires involving copper increase in resistance significantly as the surrounding temperature is increased.

(Sohn Assosiates, 2009)

### 1.4 Electric Vehicles and the National grid

There are substantial concerns regarding the feasibility of charging the rapidly increasing number of electric vehicles from the current electricity grid within the UK. (The Guardian , 2017)

A frequently discussed solution to this issue is the introduction of ‘Smart Charging’, which hopes to alleviate additional electricity demand from peak times. Smart charging allows a control over the timing of which electric vehicles are being charged to avoid peak demand times, the general idea being that a vehicle is plugged in and the grid’s smart charging system will determine at what point is suitable to start charging the vehicle. Smart charging is also hoped to allow a more effective use of inconsistent renewable electricity supply.  (Nicolò Daina, 2016)

2.0 Results

A report created by the National Grid Electricity has been used to demonstrate reliable data describing an example of the amount of losses due to the transmission system.

(National Grid Electricity Plc, 2016)

## 3.0 Analysis and Conclusion

The total Losses throughout the system are remarkably low considering the extent of the grid Infrastructure. Unfortunately, due to the scope of this project and the time constraint given, this section of the well to wheel process has not been investigated as thoroughly as other sections, however, results have been found from a technical report by the National Grid, which will provide enough information to complete the well to wheel investigation.

# Overall Analysis

## 1.0 Efficiency Comparison of Each Section

Throughout each section of the well to wheel process gasoline, diesel and electricity have been analysed in terms of energy consumption and carbon dioxide emissions, with indications toward the efficiency level of each section depending on the fuel in question.

In order for an approximation to be made on the feasibility of the electric vehicle compared to the conventional internal combustion engine, efficiency values will be used to highlight the electric vehicle well to wheel process and the internal combustion well to wheel process.

These values will have to be converted into equivalent sections in order for a comparison to be made. At this point each well to wheel process has been described like so:

Electric Vehicle: Refinery Stage – Transportation Via National Grid to Consumer – Use Phase ICE: Refinery Stage – Pipeline/Ship Transportation to Storage Areas – Distribution to Local Garages – Use Phase

Figure 112- Original Well to Wheel Process of ICE Fuels and Electricity

As it can be seen from above there are variations between the electric vehicle process and ICE process, therefore the author takes the time to summarise the ICE phase into a three-stage process to make it more comparable to the electric vehicle process, like so:

Electric Vehicle: Refinery Stage - Electricity Transportation Phase – Use Phase ICE: Refinery Stage – ICE Overall Transportation Phase – Use Phase

Figure 113- Comparative Version of ICE fuels and Electricity

In order to achieve this, the various stages of ICE fuel transportation must be combined:

ICE Refinery Stage = Average efficiency of combined gasoline and diesel

Ice Transportation stage = Average of pipeline and ship(gasoline and diesel combined ×Averge of lorry (gasoline and diesel combined)

### 1.1 ICE Refinery Stage Calculations:

| Efficiency of Gasoline % | Efficiency of Diesel % | Combined Efficiency % |
| --- | --- | --- |
| 94.34 | 90.90 | 94.34+90.902=92.62 |

Figure 114- Refinery Efficiencies of Gasoline and Diesel Combined

### 1.2 ICE Transportation Stage Calculations:

| Pipeline Combined % | Ship Combined % | Lorry Combined % | Overall Combined: (Pipeline x Ship x Lorry) % |
| --- | --- | --- | --- |
| 79.98 | 99.999 | 99.858 | 79.9 |

Figure 115- Ice Transportation Efficiencies

### 1.3 Electricity Refinery Value

Electricity efficiency refinery value = 37.957 %

### 1.4 Electricity Transportation Value (Overall Grid Losses):

Overall Grid Loses = 1.77%

Therefore Efficiency = 100-1.77 = 98.23 %

### 1.5 Ice Usage Efficiency

Average = 19 %

### 1.6 Electricity Usage Efficiency

Average =60.5 %

### 1.7 Overall Efficiency Calculation:

Overall EfficiencyElectricity =RefineryElectricity×TransportationElectricity×UsageElectricity

Overall EfficiencyElectricity =0.37957 ×0.9823 ×0.6050= 0.2255 ×100

=22.56 %

Overall EfficiencyIce= Refinery ICE×TransportationICE ×UsageICE

Overall EfficiencyIce=92.62 ×79.9 ×19=0.14 ×100

=14.60  %

## 2.0 Graphical Comparison

#### 2.1 Sectional Comparison

Now that the data has been formatted into sections that are more easily comparable, each section can be compared graphically.

Figure 116- Efficiency Comparison of ICE VS Electricity Throughout Each Stage

An electric vehicles biggest downfall is the amount of energy that is required to generate electricity, which is accurately depicted in the graph above. However, in terms of transportation the total losses in the electricity grid are lesser than the loses that occur over the overall transportation process of Gasoline and Diesel. As expected, the stage that has affected the overall efficiency of the electric vehicle the most is the use phase, as electric vehicle loses are far less of those experienced by the Internal Combustion Engine.

### 2.2 Overall Efficiency

From the individual Sections, an overall efficiency can be calculated:

Figure 117-Overall Combined Efficiency Value for Electricity and ICE

Undoubtedly, according to the process followed throughout this report, the electric vehicle has proved to be overall the most efficient choice in terms of energy consumption which is predominantly due to the major loses the internal combustion experiences within the use phase.

In general, the uncertainties associated with the electric vehicle come from the idea that the burning of fuel to create electricity is occurring in the same way as an ICE fuelled vehicle burns Gasoline or Diesel, except for electricity this occurs at the refinery stage. However, the data collaborated for this report suggests that despite this concern being justified by the inefficient creation of electricity, the dramatic difference in efficiency of both vehicles at the use stage deems the electric vehicle to be more effective overall.

# Overall Conclusion and Evaluation

## Evaluation

The content of this report is vast and varied but was necessary in order to evaluate each section of the well to wheel process effectively.

The use of Life Cycle Analysis data from Gabi and The European Life Cycle Database as well as the results provided by the UCSB Automotive Energy & GHG Model, has been fundamental in providing accurate data for each section so that each stage of the well to wheel process could be analysed and compared.

The overall analysis is based on efficiency values that were calculated on a case study bases for each section, each case study was made to be as realistic and generically applicable as possible, but it is understood that a case study outlines a particular situation rather than multiple, which could be seen as an idealistic approach. However, with such as vast and complicated topic, this was the only feasible way of carrying out this investigation within the time frame provided.

## Conclusion

The author has maintained an unbiased view throughout this research paper coupled with a fair and comparative methodology in order to effectively compare ICE and electrically powered vehicles.

The general suggestion that electric vehicles are not a feasible solution in reducing energy consumption due to the inefficiencies associated with electricity generation has been addressed and it has been concluded that this factor does not make electric vehicles any less efficient overall than vehicles powered by internal combustion.

To conclude the findings of this report, an overall analysis of all data collected suggests that electric vehicles are more efficient in terms of energy consumption when considered from a well to wheel perspective.

# Bibliography

Gabi - Thinkstep, 2016. Pipeline Average Energy Consumption. s.l.:Gabi.

Gabi Thinkstep, 2018. Euro 5 Regulated Diesel Tanker, 32t Plus, s.l.: s.n.

UK Petroleum Industry Assosiation, 2017. Refining & UK Refineries - More Details. [Online] 
Available at: http://www.ukpia.com/docs/default-source/default-document-library/refinery-process.pdf?sfvrsn=0
[Accessed 17 January 2018].

A. Abou El-Azm Aly, H. A. A. F. S. A. H., 2016. Numerical Study of the Semi-Open Centrifugal Pump Impeller Side Clearance, Elkobbah: Research Gate .

Aarsland, S. E., 2013. An Experimental and Numerical Study of, Trondheim: Norweigan University of Science and Technology .

Acosta-Amado, R. J., Garcia-Diaz, A., Atuahene, I. & Ojeda, A. F. A., 2014. The Refined Products Distribution Problem via Pipeline Systems: A Survey, Tennessee: Department of Industrial and Systems Engineering.

Alternative Fuels Data Centre , 2017. Alternative Fuels Data Centre. [Online] 
Available at: https://www.afdc.energy.gov/fuels/emerging_biobutanol.html
[Accessed 15 November 2017].

Anon., n.d. s.l.:s.n.

Anup Sera, 2009. Pipelines And Storage. Pipeline and Gas Journal, 236(12).

Balzer, R., 2018. UCSB Automotive Energy & GHG Model Query [Interview] (16 April 2018).

BBC, 2011. Heat transfer and efficiency. [Online] 
Available at: http://www.bbc.co.uk/schools/gcsebitesize/science/aqa_pre_2011/energy/heatrev6.shtml
[Accessed 9 March 2018].

BBC, n.d. Global Energy Guide, s.l.: BBC mmx.

Bosch, 2007. Automotive Handbook. 7th ed. Plochingen: Robert Bosch.

British Geological Survey, 2011. Geothermal heat, UK: NERC.

British Hydropower Assosiation, n.d. Hydro Facts. [Online] 
Available at: http://www.british-hydro.org/hydro_facts.html
[Accessed 16 Novemeber 2017].

British Pipeline Industry, 2014. Bpa. [Online] 
Available at: https://www.bpa.co.uk/experience/what-we-operate/
[Accessed 4 April 2018].

C.M Silva, G. G. T. F. J. M. -. L., 2006. A tank-to-wheel analysis tool for energy and emissions studies in road vehicles. Science of the Total Envirement, March, 1049(001), p. 445.

Canadian Energy Pipeline Assosiation, 2017. Pipline Industry Performance Report, Canada: CEPA.

Carmen Data Ltd , 2018. Kg CO2 per litre of petrol. [Online] 
Available at: https://comcar.co.uk/newcar/companycar/poolresults/co2litre.cfm
[Accessed 12 Febuary 2018].

Carvill, J., 1993. Mechanical Engineer's Data Handbook. 1st ed. Oxforc: Reed Elsevier .

Carvill, J., 1994. Mechanical Engineers Data Handbook. 1st ed. Oxford: Reed Elsevier.

Christensen, R., 2009. Age Effects on Iron Based Pipes , Utah: Utah State University.

CNN, 2017. [Online] 
Available at: http://edition.cnn.com/2017/09/14/africa/nigeria-china-hydropower/index.html
[Accessed 14 November 2017].

Corbett, J., 2008. The Impacts of Globalisation on International Maritime Transport , Mexico: OECD.

D.Bacon, R., 1990. Mechanical Technology , Oxford: Reed Elsevier.

D.Tsokolis, S. A. G. P. B. ,., 2016. Fuel consumption and CO2 emissions of passenger cars over the New Worldwide Harmonized Test Protocol. Applied Energy, Volume 172, p. 1161.

Daniel Garraín, S. F. C. d. l. R. M. R. Y. L. F. M., 2015. Background qualitative analysis of the European Reference Life Cycle Database (ELCD) , s.l.: Springer Plus.

darvill.clara, 2015. Tidal Power. [Online] 
Available at: http://www.darvill.clara.net/altenerg/tidal.htm
[Accessed 2017 November 2017].

David J. Chang, E. K. M., 2005. Vehicle Speed Profiles to Minimize Work and Fuel Consumption. Journal of Transportation Engineering, p. 173.

DEFRA Envirement Agency , 2012. Pumping Station Efficiency: Guidance, Bristol: Envirment Agency .

Departement for Transport, 2012. Vehicle Speeds Statistics , London: UK Government.

Department for Business, Energy and Industrial Sector , 2017. Energy Consumption in the UK, United Kingdom : s.n.

Department for Transport, 2017. Average Heavy Goods Vehicle Fuel Consumption, Great Britian, 2013-2016, London: DfT.

Department of Transport, 2016. Vehicle Licensing Statistics, London : Department of Transport .

dr inż. Artur Wyrwa, d. M. B., 2016. LCA of lignin derived cyclic oxygenates as biofuels obtained via fast pyrolysis in the US, Krackow: s.n.

Edelen, A., 2017. The creation, management, and use of data quality information, Berlin: Springer-Verlag.

Enbridge , 2018. projects-and-infrastructure. [Online] 
Available at: https://www.enbridge.com/projects-and-infrastructure/projects/~/media/2509CBF391B74FB9B33935264971D1C1.ashx
[Accessed 1 March 2018].

Energy Education, 2015. Fractional Distillation. [Online] 
Available at: http://energyeducation.ca/encyclopedia/Fractional_distillation
[Accessed 6 November 2017].

Energy Institute, 2018. Petroleum road tankers. [Online] 
Available at: https://www.energyinst.org/technical/distribution-and-marketing/petroleum-road-tankers
[Accessed 6 Febuary 2018].

EPA , 2017. Student Guide to Global Climate Change , s.l.: EPA.

Europe Inland Transport Committee , 2017. ADR - European Agreement concerning the international carraige of dangerous goods by road , UN: United Nations.

European Commission, 2017. Reducing emissions from the shipping sector. [Online] 
Available at: https://ec.europa.eu/clima/policies/transport/shipping_en

European Life Cycle Database Joint Reserach Centre , n.d. s.l.:European Commision.

Evans, S., 2017. The UK generated more electricity from wind than from coal in the full calendar year of 2016. [Online] 
Available at: https://www.carbonbrief.org/analysis-uk-wind-generated-more-electricity-coal-2016
[Accessed 13 November 2017].

Ford, 2018. All New Fiesta. [Online] 
Available at: https://www.ford.co.uk/content/dam/guxeu/uk/documents/feature-pdfs/FT-All_New_Fiesta.pdf
[Accessed 2 April 2018].

Ford, 2018. Specification. [Online] 
Available at: https://www.ford.co.uk/content/dam/guxeu/uk/documents/brochures/cars/BRO-Focus.pdf
[Accessed 2 April 2018].

Fuel Economy.GOV, 2016. All Electric Vehicles. [Online] 
Available at: https://www.fueleconomy.gov/feg/evtech.shtml
[Accessed 10 April 2018].

G.O.Duarte, G., 2016. Analysis of fuel consumption and pollutant emissions of regulated and alternative driving cycles based on real-world measurements, Lisboa: Elsevier.

GaBi, Database, 2013. Projection of Elecricity Mix - United Kingdom , United Kingdom: GaBi.

Georgios Triantafyllopoulos, A. K. D. T. L. N., 2017. Potential of energy efficiency technologies in reducing vehicle. Energy, Volume 140, p. 372.

Geotech, 2018. Average Viscosities of Miscellaneous Liquids. [Online] 
Available at: http://www.geotechenv.com/Reference_Pages/average_viscosities_liquids.pdf
[Accessed 5 April 2018].

Godfrey Boyle, B. E. J. R., 2003. Energy Systems and Sustainability. Oxford: Oxford University Press .

Godfrey Boyle, B. E. J. R., 2003. Energy Systems and Sustainability. Milton Keynes: Oxford University Press .

Golnik, A., 2003. Energy Density Of Gasoline. [Online] 
Available at: https://hypertextbook.com/facts/2003/ArthurGolnik.shtml
[Accessed 10 November 2017].

GOV, 2017. Drivers' hours. [Online] 
Available at: https://www.gov.uk/drivers-hours/driving-under-both-eu-and-gb-domestic-rules

GreenMatch , 2017. Types of Solar Panels. [Online] 
Available at: https://www.greenmatch.co.uk/blog/2015/09/types-of-solar-panels
[Accessed 13 Novemebr 2017].

Gusti, A. P., 2016. The Effect of Vessel Speed on Fuel Consumption and Exhaust. American Journal of Engineering and Applied Sciences, p. 1047.

Hanna Larsson, E. E., 2009. The effects of an acceleration advisory tool in vehicles for reduced fuel consumption and emissions. Transportation Research, Volume 14, p. 143.

Harris, D., 2007. Bigger trucks taking more freight further. Commercial Motor, 4 October, 206(5249), p. 14.

Helmy, C. S., 2016. Mega Container Ships, Pros, Cons. Journal of Shipping and Ocean Engineering, Volume 6, p. 285.

Husain, I., 2011. Design Fundalmentals of Electric and Hyrbrid Vehicles. 2nd ed. Florida: Taylor and Francis Group LLC.

Inland Transport Committee, 2014. Proposal for a new global technical regulation on the, Geneva: Economic and Social Council.

International Council on Clean Transportation, 2018. transportpolicy. [Online] 
Available at: https://www.transportpolicy.net/standard/international-light-duty-worldwide-harmonized-light-vehicles-test-procedure-wltp/
[Accessed 11 April 2018].

International Energy Agency, 2011. Total Primary Energy Supply,1973 to 2020, Paris: IEA Publications.

Interstate Natural Gas Assosiation of America , 2010. Interstate Natural Gas Pipeline Efficiency , Waahington: INGAA.

ISO:14040, 1997-2006. Enviremental Managment - Life Cycle Assesment - Principles and Frameowrk, Europe: ISO.

J. L. Bautista-Jacobo, E. R.-M. J. J. M.-R. H. G.-C., 2015. Effect of Baffles on the Sloshing in Road Tankers Carrying LPG: A Comparative Numerical Study. Mathematical Problems in Engineering, 2015(359470), p. 1.

J.M.McDonough, 2004. Elementary Fluid Dynamics, Lexington: University of Kentucky .

Jelica Pavlovic, A. M. B. C., 2016. CO2 emissions and energy demands of vehicles tested under the NEDC. Applied Energy, Volume 177, p. 1.

Joachim Demuynck, D. B. D. P. c. F. M. V., 2012. Recommendations for the new WLTP cycle based on an analysis of vehicle. Energy Policy 49, 17 July, 49(0301- 4215), p. 1.

John Andrews, N. J., 2007. Energy Science. Oxford: Oxford University Press .

Kristensen, H. O., 2002. Cargo transport by sea and road--technical and economic environmental factors, New York: Marine Technology and SNAME News.

LCA Consultants, 2015. Consequential LCA. [Online] 
Available at: https://consequential-lca.org/clca/why-and-when/
[Accessed 1 April 2018].

Linewatch, 2017. Special Requirements for safe working in close proximity to high pressure pipelines, United Kingdom: Linewatch.

Maria Isabel Ramirez, A. P. A. S. S. N. B.-M., 2016. Contamination by oil crude extraction - Refinement and their effects on human health. Environmental Pollution, p. 416.

Mark A. Delucchi, A. B. T. L. M. M., 2000. Electric and Gasoline Vehicle Lifecycle Cost and Energy-Use Model, California: Institute of Transportation Studies, University of California.

Martinez, I., 1992. Thermodynamica Basica Aplicada. Madrid: Dossat.

Mehrdad Ehsani, Y. G. E., 2010. Global Warming. In: T. a. Francis, ed. Modern Elecrtic, Hybrid Electric and Fuel Cell Vehicles. Florida: Taylor and Francis Group,LLC, pp. 3-4.

NACE International , 2018. Pipelines & Underground Systems. [Online] 
Available at: https://www.nace.org/Pipelines-Tanks-Underground-Systems/
[Accessed 10 March 2018].

National Geographic , 2015. Wind Power. [Online] 
Available at: https://www.nationalgeographic.com/environment/global-warming/wind-power/
[Accessed 13 November 2017].

National Grid Electricity Plc, 2016. Special Condition 2K.4 – Transmission Losses Report, Warwick: National Grid Electricity Plc.

National Renewable Energy Laboratory, 1994. ADVISOR Software Power Mode Losses Result , s.l.: s.n.

Nicolas, R., 2013. Car-Engineer. [Online] 
Available at: http://www.car-engineer.com/the-different-driving-cycles/
[Accessed 11 April 2018].

Nicolò Daina, A. S. J. W. P., 2016. Electric vehicle charging choices: Modelling and implications. Transportation Research Part C, p. 37.

O. R. Abdurakhmanov, Z. S. S. S. M. S. K. N. B. a. B. S. U., 2010. Intensification of the Process of Primary Distillation, Bukhara: Russian Journal of Applied Chemistry.

Ömer Öztürkoğlu, O. L., 2016. The integrated network model of pipeline, sea and road distribution of. An International Journal of Optimization and Control: Theories & Applications, p. 151.

PE International , 2008-2015. Gasoline mix (regular) at refinery. [Online] 
Available at: http://eplca.jrc.ec.europa.eu/ELCD3/datasetdetail/process.xhtml?uuid=1ea201ab-eec9-4356-93d8-3a4905ea35cd&version=00.00.000&stock=default
[Accessed 17 January 2018].

PE International, 2002-2010. Electricity Grid Mix. [Online] 
Available at: http://eplca.jrc.ec.europa.eu/ELCD3/showProcess.xhtml?uuid=0a1b40db-5645-4db8-a887-eb09300b7b74&stock=default
[Accessed 17 January 2018].

PE International, 2008-2015. Diesel mix at refinery. [Online] 
Available at: http://eplca.jrc.ec.europa.eu/ELCD3/datasetdetail/process.xhtml?uuid=1ea201ab-eec9-4356-93d8-3a4905ea35cd&version=00.00.000&stock=default
[Accessed 17 January 2018].

Petroleum Industry Association, 2017. Distribution. [Online] 
Available at: http://www.ukpia.com/industry_information/distribution.aspx
[Accessed 4 April 2018].

Pienaar, W., 2010. Logistical Aspects of Petroleum Based Pipelines, s.l.: Department of Logistics - Stellenbosch University.

Pipeflow, 2018. Flow of fluid through a pipe. [Online] 
Available at: http://www.pipeflow.co.uk/public/articles/Darcy_Weisbach_Formula.pdf
[Accessed 5 January 2018].

PRé Consultants, 2017. The basics of the LCA methodology. [Online] 
Available at: https://www.pre-sustainability.com/disclaimer
[Accessed 2 April 2018].

Process Heating, 2014. Capturing Waste Heat Lowers Energy Cost, Reduces Pollution. Process Heating, October, p. 6.

R.G.Gareev, 2002. Power Supply and Energy Conservation in Fractional Distillation of Crude Oil. Chemistry and Technology of Fuels and Oils , 38(2).

Racetechnology, 2009. Tomorrow's technology passes its first test. Race Technology , December, p. 47.

Renewable Energy UK , n.d. Wave & Tidal Energy. [Online] 
Available at: http://www.renewableuk.com/page/WaveTidalEnergy
[Accessed 15 November 2017].

Rosalind O'Driscoll, M. E. S. N. M. T. O. H. M. A., 2017. Real world CO2 and NOx emissions from 149 Euro 5 and 6 diesel, gasoline and hybrid passenger cars. Science Of the Total Environment , Volume 621, p. 282.

S. Tsiakmakis, D. T. A. D. G. F. P. P. B. C. Z. S., 2016. Fuel consumption and CO2 emissions of passenger cars over the New Worldwide Harmonized Test Protocol. Applied Energy, Volume 179, p. 1161.

Simmons, E. D. C. a. R. A., 2014. Efficiency and Vehicle Technology. In: Understanding the Global Energy Crises. Indiana: Global Policy Research Institute by Purdue University Press, p. 218.

Society of Petrolium Engineers International, 2015. Pipeline design consideration and standards. [Online] 
Available at: http://petrowiki.org/Pipeline_design_consideration_and_standards#Velocity_considerations
[Accessed 4 April 2018].

Sohn Assosiates, 2009. Electricity Distribution Systems Losses, s.l.: Sohn Assosiates.

Suttons, M. E. o., 2018. Technical Questions of Fuel Road Tanker Operations [Interview] (8 Febuary 2018).

T J Barlow, S. L. I. S. M. P. B., 2009. A Reference Book of Driving Cycles, Wokingham: Transport Research Laboratory.

Tefaruk Haktanır, M. A., 2004. Numerical modeling of Darcy–Weisbach friction factor. Advances in Engineering Software , Issue 35, p. 774.

The European Wind Energy Assosiation, 2016. Wind Energy. [Online] 
Available at: http://www.ewea.org/wind-energy-basics
[Accessed 13 November 2017].

The Guardian , 2017. Electric cars will fuel huge demand for power, says National Grid. The Guardian , 13th July.

The Independant , 2017. Solar power sets new electricity record supplying a quarter of UK demand. [Online] 
Available at: http://www.independent.co.uk/environment/solar-power-electricity-record-uk-energy-quarter-demand-supply-renewables-green-a7757836.html
[Accessed 12 November 2017].

The International Council on Clean Transportaton, 2011. Inertia classes, vehicle emissions tests. [Online] 
Available at: https://www.theicct.org/blogs/inertia-classes-vehicle-emissions-tests-and-dead-hand-past
[Accessed April 2018].

The Telegraph , 2010. Noise complaints about one in six wind farms. [Online] 
Available at: http://www.telegraph.co.uk/news/earth/earthnews/7377641/Noise-complaints-about-one-in-six-wind-farms.html
[Accessed 13 November 2017].

Thinglink, n.d. Central mareomotriz. [Art] (Thinglink).

Thinkstep-Gabi , 2018. Gabi Software. [Online] 
Available at: http://www.gabi-software.com/uk-ireland/index/
[Accessed 2 April 2018].

Times, F., 2010. UK refineries up for sale at worst of times. [Online] 
Available at: https://www.ft.com/content/25ff2ea8-53c5-11df-aba0-00144feab49a
[Accessed 20 November 2017].

Transport Studies Group, 2004. Bulk Tanker Operations In Great Britain, UK: Institute of Logistics and Transport.

Trinh, K. T., 2009. On the Blasius correlation for friction factors, New York : Cornell University .

UK Petroleum Industry Assosiation, 2017. Industry Information. [Online] 
Available at: http://www.ukpia.com/industry_information/refining-and-uk-refineries.aspx
[Accessed 15 January 2018].

UK Petroleum Industry, 2017. UK Petroleum Industry - Distribution. [Online] 
Available at: http://www.ukpia.com/industry_information/distribution.aspx
[Accessed 4 April 2018].

UK Power Networks, 2017. What are electrical losses?. [Online] 
Available at: https://www.ukpowernetworks.co.uk/losses/index.html

UKpia, 2017. Statistical Review 2017, London: United Kingdom Petroleum Industry Association.

United Nations Conference on Trade and Developement , 2016. Review of Maritime Transport , New York and Geneva: United Nations .

US Energy Administration , 2018. Frequently Asked Questions. [Online] 
Available at: https://www.eia.gov/tools/faqs/faq.php?id=82&t=11

US Energy Administration, 2017. How much carbon dioxide is produced from burning gasoline and diesel fuel?. [Online] 
Available at: https://www.eia.gov/tools/faqs/faq.php?id=307&t=11
[Accessed 16 April 2018].

US Energy Information Administration, 2016. Oil:Crude and Petroleum Products Explained. [Online] 
Available at: https://www.eia.gov/energyexplained/index.cfm?page=oil_home

US Energy Information Administration, 2017. Energy Conversion. [Online] 
Available at: https://www.eia.gov/energyexplained/index.cfm?page=about_energy_conversion_calculator
[Accessed 9 March 2018].

Vasilash, G. S., 2011. Federal -Mogul Advances Ignition. Automotive Design and Production, Dec, p. 60.

Vehicle Certification Agency , 2017. CSV Euro Standard 6, s.l.: Vehicle Certification Agency .

Vivien Mweene Chabalengula, F. M., 2012. Energy Conservation. 1st ed. s.l.:INTECH.

Walker, K., 2013. AZO CleanTech. [Online] 
Available at: https://www.azocleantech.com/article.aspx?ArticleID=350
[Accessed 15 November 2017].

WLTPfacts, 2017. wltpfacts. [Online] 
Available at: http://wltpfacts.eu/from-nedc-to-wltp-change/
[Accessed 1 11 2017].

World Enegry Council, 2016. World Energy, London: WECouncil.

WorldAutoSteel, 2017. Steel Industry Provides Improved Tools to Contribute to Sustainable Automotive Industry. [Online] 
Available at: http://www.worldautosteel.org/life-cycle-thinking/ucsb-energy-ghg-model/
[Accessed January 2018].

Zoran Morvaj, V. B., 2010. Energy Efficiency Policy. 1st ed. Croatia: United Nations Development Programme.

# Appendices

### 1.0 Renewable Energy Sources

Harnessing energy from natural resources that are renewable is beneficial as this process is more sustainable that utilizing finite sources. A brief overview of each renewable energy source available to the United Kingdom are detailed blow:

### 1.1 Solar Power

Within the UK, energy produced by solar power is gaining in popularity, whilst it is hard to estimate how much of the electricity mix is provided by solar power, there are often articles describing the potential of solar power when weather conditions are adequate.

(The Independant , 2017)

Solar power is generated via different techniques of converting the energy produced by sunlight, depending on the particular type of panel used. The table below describes the most widely used panels, outlining advantages, disadvantages and efficiency rates.

(GreenMatch , 2017)

### 1.2 Wind Power

Energy produced from wind power in the UK is becoming more and more prominent, with certain instances where wind energy production has over taken coal energy production. (Evans, 2017) According to the European wind energy association, one on shore wind turbine with an average capacity is capable of producing enough electricity to power approximately 1,500 houses. (The European Wind Energy Assosiation, 2016)

(BBC, n.d.)

Large turbine blades are spun by the wind whilst connected to a generator, producing electricity. There are three main types of wind turbine set ups, on shore, off shore, and small wind turbines. On shore wind turbines are less expensive to establish and operate, but are not subjected to the high winds seen at sea. (National Geographic , 2015)

Although wind energy is an effective renewable source, there are still issues regarding public opinions toward their operation. There are also implications regarding the storage of energy created by wind power.

(The Telegraph , 2010)

The initial creation of a wind turbine, for example, the process of creating, moulding and assembling will produce emissions, and will also consume considerable energy. (There will be emissions and energy consumption incurred from maintenance) (GaBi, Database, 2013)

### 1.3 Geothermal

Geothermal energy harnesses the heat produced from the internals of the earth, where radioactive substances have decayed for a prolonged amount of time. (EPA , 2017) Geographically, the heat that can be harnessed varies widely, some countries have far more geothermal potential than others, depending on the internals of the earth in that particular region, see map below for an example on the varying geothermal potential in the UK;

The diagram to the right describes the process of converting the geothermal energy into electricity throught the use of turbines and steam, after the heat is harnessed, the geothermal water is injected back into the ground to be heated again. (Godfrey Boyle, 2003)

(EPA , 2017)

(British Geological Survey, 2011)

It is important that the hot water taken from the ground is replenished with water that has a change to heat up again, otherwise, the internal temperature starts to decrease, depleting the natural heat resource which essentially makes this process non-renewable and unsustainable.  (EPA , 2017)

### 1.4 Hydroelectricity

Hydroelectricity is produced from utilizing the energy from water flow. A hydroelectric plant is generally established by building large dams and can sometimes involve flooding large areas. Once again, this renewable involves the spinning of turbines that operate a generator, but this time flowing water is used instead of heating water into steam.

(CNN, 2017)

(BBC, n.d.)

It has been stated by the British Hydropower Association that hydropower has previously produced 17% of the world’s electricity mix. (British Hydropower Assosiation, n.d.)

Hydropower is a very clean way of producing energy, but does not come without its negatives. Often building large dams effects the surrounding community greatly, by interrupting fishing and increasing the risk of water related disease. (Godfrey Boyle, 2003). If an area has to be flooded, then communities often have to leave their homes, of which they may have inhabited for a long time.

### 1.5 Biofuels

Biofuels are generated from crops and organic sources and are used as an alternative fuel to diesel and gasoline. Alternative fuels are often criticized for their energy density in comparison to gasoline, for example, ethanol has an energy density of 63%, which means more fuel is needed to produce the same amount of energy, resulting in more refuelling. However, Bio-butanol has an energy density of 85-90% (Racetechnology, 2009) , and produces far less emissions, it is worth noting biofuels generally need to be blended with gasoline to be useable. (Alternative Fuels Data Centre , 2017)

There are many different types of biofuels produced from different sources, the table below demonstrates additional information on biofuels:

(dr inż. Artur Wyrwa, 2016)

### 1.6 Tidal

Tidal Energy exploits the energy of the tides to spin underwater turbines connected to generators. Establishing a tidal energy system can be difficult initially, but once set up, is a sustainable energy source, it is also noticeable more efficient than wind turbines and solar power (Walker, 2013)

There are different designs of tidal energy technology, including tidal barrages, offshore turbines and tidal reefs, some are more effective than others, whilst some are very effective but costly to create. (darvill.clara, 2015)

(Thinglink, n.d.)

Renewable energy UK states that tidal power has the capability of supplying 20% of the UKs electricity mix.

(Renewable Energy UK , n.d.) Unfortunately, because the energy is dependent on the tides, the energy produced is intermittent

#### 2.0 Power Plant Processes

Steam power plants utilize different cycles to generate electricity, such as the Rankine cycle, Brayton Cycle, and combined cycles. Predominantly the cycles involve changing water into steam via heat generation from burning coal, but there are major differences in these cycles involving varying temperatures, and differences in the ‘steam quality’ produced. (John Andrews, 2007), to see brief overviews of each cycle, please see below:

#### 2.1 The Carnot Cycle

The Carnot cycle a theoretical process, despite being termed the cycle that is ‘the most efficient for a closed cycle thermal power plant’ (John Andrews, 2007), it has been proven that in practice it is not to be an ideal process to use in a steam power plant, predominantly because of the high amounts of energy needed to produce the volume of steam that this cycle requires. (Rosa, 2005)

#### 2.2 The Rankine Cycle

The Rankin Cycle is better suited to steam power plants, as the volume of steam needed is reduced, therefore less energy is required. The Rankine cycle is often implemented with a reheat process that involves rewarming the steam multiple times. This cycle involving the reheat process is proven more effective. This process requires high temperatures to aid efficiency values, but these high temperatures cause stress and increased pressures that the equipment has to withstand, often resulting in regular replacement of parts. (Rosa, 2005)

#### 2.3 The Brayton Cycle

The Brayton cycle varies from the Rankine and Carnot, as it does not use water as its working fluid. Instead it utilises the constant pressurisation of gas to produce a combustion of which the ‘waste energy’ from the exhaust is put through a power turbine. (NASA, 2015)