ACBA5017 –

Building Services 2 - Integration Assignment 2: Case study analysis - Low energy design

David Deeth (2223671)

## Contents

Contents	1

List of Figures:	1

List of Abbreviations:	1

1.	Introduction	2

1.1.	EPC vs PHPP	2

2.	Critical Appraisal of Scenario Building	3

2.1.	Description of Building:	3

2.2.	Understanding of Current AD: L Design:	4

2.3.	LETI Variation	5

3.	Comparison	6

3.1.	HT 641 Operational outputs	10

3.2.	Justifications:	10

3.4.	HT 341 Operational Energy Losses & Gains	12

4.	Conclusion	13

5.	References	14

6.	Appendix	15

6.1.	Figures:	15

## List of Figures:

Figure 1: Spring Designs_HT 641_ Floor Plan	4

Figure 2: aroTHERM plus (Vaillant.co.uk, 2025)	5

Figure 3: envirovent energiSava 200 (Envirovent.com, n.d.)	5

Figure 4: Mixergy cylinder (BEE - the smart heating merchants, 2026)	6

Figure 5: Total Annual Costs (www.passivhaustrust.org.uk, n.d.)	12

Figure 6: Spring Designs HT 641_Site Plan & Elevations	15

Figure 7: Spring Designs_HT 641_Floor Plans	16

Figure 8: Spring Designs_HT 641_Elevations	16

Figure 9: aroTHERM Specifications (Vaillant.co.uk, 2025)	18

## List of Abbreviations:

| ASHP | Air Source Heat Pump |
| --- | --- |
| AD: L | Approved Document L (Wales) 2025 |
| EPC | Energy Performance Certificate |
| EUI | Energy Use Intensity |
| HWC | Hot Water Cylinder |
| LETI | Low Energy Transformation Initiative |
| M&E | Mechanical, Electrical and Ventilation Services |
| MVHR | Mechanical Ventilation Heat Recovery |
| SAP | Standard Assessment Procedure |

## Introduction

Residential buildings contribute towards a large percent of the UK’s greenhouse gas emissions and energy consumption and therefore is a major factor in achieving National and Welsh Government Net Zero carbon targets.

Approved Document L (Wales) 2025 [AD: L], which covers conservation of fuel and power, establishes the minimum required energy efficiency rates for new buildings.

The Low Energy Transformation Initiative [LETI] provides alternative, more ambitious guidance towards reducing energy demands, minimising dwelling occupational carbon emissions, and whole-life environmental impact considerations.

Achieving LETI standards requires making improvements to the building fabric through careful design, specification and commissioning of the mechanical, electrical and ventilation [M&E] services to maximise their efficiency.

This report aims to critically appraise the existing M&E design of the scenario building which has been designed to meet AD:L standards and, whilst keeping the building fabric as designed, utilise the Passive House Planning Package [PHPP] to identify design differences which could be made to achieve LETI standards.

### EPC vs PHPP

Energy Performance Certificates [EPC] indicate the running costs of a building by providing a score between 1 and 100, divided into 7 bands, based on the Standard Assessment Procedure [SAP] set out by the UK Government.

However, SAPs are designed to be a compliance tool for AD: L, and the EPC methodology uses the overall cost-based ratings to drive energy efficiency.

As carbon emissions related to expensive electricity reduce, the EPC rating system could potentially show inaccurate results, with high EPC rated dwellings also producing a high rate of emissions.

PHPP is more rigorous and detailed assessment tool, providing in depth analysis of the building’s energy balance and specific component performance.

To improve building energy efficiency and lower emissions, energy efficiency should be measured using space heating as the primary metric, derived from PHPP or SAP calculations, assisted by EPC’s cost-based index.

## Critical Appraisal of Scenario Building

| Scenario building: Designer: Spring Designs Location: Vale of Glamorgan Domestic dwelling Gross Internal Area [GIA]: 110.0m² Stories: 2 6 Person 4 Bed House Bathrooms: 2 Plant: MVHR, ASHP, HWC | Spring Designs_HT 64 (Davis, 2025) |
| --- | --- |

Property Location:

- Vale of Glamorgan (The Vale as it’s colloquially known): area of approximately 130 square miles between Bridgend, Rhondda Cynon Taf, Cardiff, and the Bristol Channel.

- Main towns: Barry, Penarth, Llantwit Major, and Cowbridge

- Population: approximately 130k

- Attractions: Barry Pleasure Park resort, Barry Tourist Railway.

- Proximity to: Cardiff, Newport, Swansea, and the A4.

- Transport: Cardiff Airport – domestic and international flights, Transport for Wales & Great Western Railway – services between Swansea, London Paddington, and Manchester Picadilly.

### Description of Building:

- New build dwelling in wealthy rural area of Wales

- Two-story, Four-bedroom detached house with 110m2 internal floor area: presumed to allow occupancy of 4-5 people – typical family size home.

- Designed to comply with Approved Document L, Welsh Building Regulations

- Electricity: supplied from National Grid. No photovoltaic panels in current design

- Water: supplied from the mains water

- Standard UK climate data and building orientation

- House type: detached

- Floor area: 110m2 internal floor area

- External wall thickness: 550mm

- Internal wall thickness: 100mm

- Floors: 400mm

- Existing services: ASHP, MVHR, DHW

### Understanding of Current AD: L Design:

Figure 1: Spring Designs_HT 641_ Floor Plan

| Building Fabric |  |
| --- | --- |
| Air permeability Thermal bridges | 5.00 m³/m²/hr 0.200 W/mK |
| Doors & Windows |  |
| Frames Uf Installation thermal bridging Glazing Ug Glazing g-value Glazing edge | 1.400 W/m²K uPVC 0.040 W/mK 1.120 W/m²K double low-e glazing 16mm Ar 0.64 0.040 W/mK |
| Ventilation Strategy |  |
| Ventilation rate Ventilation unit Heat recovery efficiency Ducting Duct insulation (inlet/ exhaust) | 30m³ per person per hour via MEV MEV N/A (extract only) |
| Heating |  |
| ASHP SCOP Controls | Generic 3.30 On/off |

AD: L Variation (www.passivhaustrust.org.uk, n.d.)

This design utilises the fabric u-value parameters from AD: L with the ventilation strategy changed to MEV to align with Future Homes and Buildings Standard: 2023 consultation for England.

### LETI Variation

| Building Fabric |  |
| --- | --- |
| Air permeability Thermal bridges | 0.50 m³/m²/hr -0.050 W/mK |
| Doors & Windows |  |
| Frames Uf Installation thermal bridging Glazing Ug Glazing g-value Glazing edge | 0.810-0.088 W/m²K Rehau Artevo uPVC 0.009-0.023 0.55 W/m²K triple glazing (4-18Ar-4-18Ar-4) 0.63 0.025 W/mK |
| Ventilation Strategy |  |
| Ventilation rate Ventilation unit Heat recovery efficiency Ducting Duct insulation (inlet/ exhaust) | 30m³ per person per hour via MVHR Zehnder ComfoAir 225 92% Semi-rigid radial 50mm closed cell Armaflex |
| Heating |  |
| ASHP SCOP Controls | Vaillant aroTHERM plus 4.01 Acc. to ambient temperature |

LETI Variation (www.passivhaustrust.org.uk, n.d.)

These values were developed to ensure that even with the least favourable building topology, the HT 641 dwelling design could still comply with LETI standards.

| ASHP: Unit: Vaillant aroTHERM plus 5 kW Location: external, to the rear of the property (see appendix for full specifications) | Figure 2: aroTHERM plus (Vaillant.co.uk, 2025) |
| --- | --- |

| MVHR: Unit: envirovent energiSava 200 Location: ground floor utility room. Volume flow: 40-245m³/h Heat recovery: 89.0% Specific fan power: 0.79W/l/s Maximum Airflow: 241m3/h (67l/s) | Figure 3: envirovent energiSava 200 (Envirovent.com, n.d.) |
| --- | --- |

Provides fresh air and extracts stale air utilising a heat recovery system to reduce energy costs.

Baffle filters and designed ducting reduce operation noise and noise transfer between rooms.

| HWC: Unit: Mixergy cylinder 150 / 210L Location: first floor boiler cupboard Mixergy 210L for HT 421 | Figure 4: Mixergy cylinder (BEE - the smart heating merchants, 2026) |
| --- | --- |

## Comparison

As AD: L already requires high-quality building fabric, the LETI variation will use the same fabric with identical u-values to the AD: L variant.

This allows the LETI variant to demonstrate potential upfront carbon reductions without changing the operational performance of the dwelling.

Both variants use readily available materials which comply with building regulations.

Internal finishes, fixtures and fittings are not included in the analysis due to the high variance in carbon density.

The main differences come from: improved airtightness, mitigating thermal bridging, external doors and window quality, and the efficiency and optimisation of installed technologies.

| Element | AD: L | LETI | Difference |
| --- | --- | --- | --- |
| Building Fabric |  |  |  |
| Air permeability | 5.00 m³/m²/hr | 0.50 m³/m²/hr | 4.50 m3/m2/hr |
| Thermal bridges | 0.200 W/mK | -0.050 W/mK | 0.250 W/mK |
| Doors & Windows |  |  |  |
| Frames Uf | 1.400 W/m²K uPVC | 0.810-0.088 W/m²K Rehau Artevo uPVC | 0.590-1.312 W/m²K |
| Installation thermal bridging | 0.040 W/mK | 0.009-0.023 W/mK | 0.031-0.017 W/mK |
| Glazing Ug | 1.120 W/m²K double low-e glazing 16mm Ar | 0.55 W/m²K triple glazing (4-18Ar-4-18Ar-4) | 0.57 W/m²K |
| Glazing g-value | 0.64 | 0.63 | 0.01 |
| Glazing edge | 0.040 W/mK | 0.025 W/mK | 0.015 W/mK |
| Ventilation Strategy | Extract Air Unit | Balanced PH ventilation with HR | High heat recovery, Continuous supply of filtered air, Better controls |
| Ventilation rate | 30m³ per person per hour via MEV | 30m³ per person per hour via MVHR |  |
| Ventilation unit | MEV | Zehnder ComfoAir 225 | Higher indoor air quality, comfort, and long-term performance |
| Heat recovery efficiency | N/A (extract only) | 92% | 92% |
| Ducting | N/A | Semi-rigid radial | Dedicated ducting to each room, reduced noise transfer |
| Duct insulation (inlet/ exhaust) | N/A | 50mm closed cell Armaflex | Prevents condensation, Minimises heat loss, Improved acoustic dampening |
| Heating |  |  |  |
| ASHP | Standard air/water heat pump | Vaillant aroTHERM plus |  |
| SCOP | 3.30 | 4.01 | 0.71 |
| Controls | On/off | Acc. to ambient temperature | Higher seasonal efficiency with lower electricity costs |

| Element | AD: L | LETI |
| --- | --- | --- |
| Foundations | Trench Fill | Trench Fill |
| Identical foundation Pros | - Strong, load-bearing foundation | Quick construction time |
| Cons | - High embodied carbon in concrete | - High material cost |
| Ground Floor | Screed, PIR, slab on grade | Steico joist w/ Warmcel |
| Pros | - Large thermal mass, stores solar gains - Long lifespan - Straightforward construction process | - Very low embodied carbon - Easy to install / run services through floor - Timber floor provides a more natural feel |
| Cons | - High embodied carbon - PIR insulation is petrochemical-based - Slow heating and cooling times - Difficult to alter after construction | - Low thermal mass / heat retention - High level of detailing required to achieve airtightness - Higher material cost |
| External Walls Brickwork finish externally | Plasterboard on dabs, masonry cavity w/ PIR | Plasterboard w/ service zone, SMARTPLY OSB, 292mm twin stud w/ Warmcel |
| Pros | - Traditional & familiar construction method - High durability - Good thermal mass - Thin wall width due to PIR’s high insulation value | - Low embodied carbon - Excellent thermal efficiency - Twin-stud design reduces thermal bridging - Easier to achieve Passive House level of fabric performance - Good airtightness levels |
| Cons | - Thermal bridging issues - Low embodied carbon - Variable airtightness – requires skilled workmanship | - Thicker walls - Less familiar construction process - Careful design of moisture details to avoid interstitial condensation - High upfront cost |
| Party Walls All to Robust Details | Wet plaster, blockwork, ROCKWOOL filled cavity, blockwork, wet plaster | Plasterboard, SMARTPLY OSB, timber w/ Warmcel, cavity w/ Warmcel, timber w/ Warmcel, SMARTPLY OSB, plasterboard |
| Pros | - Traditional, well-established construction - High fire resistance - Good acoustic insulation - Adaptable to occupancy changes | - Low embodied carbon - Excellent acoustic performance - Lightweight construction - Good thermal continuity between INT and EXT wall elements - Easier adaptability for services / routing |
| Cons | - Requires high weight bearing foundations - Highly labour intensive - Higher embodied carbon - Difficult to alter services / routing once constructed | - High levels of detail design - High-quality control and workmanship levels required - Less familiar construction process- - Lower fire resistance |
| Internal Walls | Wet plaster, blockwork, wet plaster | Plasterboard, stud w/ Warmcel, plasterboard |
| Pros | - High durability - Long lifespan - Good thermal mass - Good fire resistance | - Fast and simple construction - Lower embodied carbon - Lightweight construction - Services can be routed through the stud cavity |
| Cons | - High embodied carbon - Longer construction and drying times - Skilled labour required - Difficult to alter services / routing once constructed | - Lower durability - Lower thermal mass - Harder to mount wall furnishings - Prone to moisture and impact damage |
| Intermediate Floor | Chipboard, Posi-joist w/ ROCKWOOL, plasterboard | Chipboard, Posi-joist w/ Warmcel, plasterboard |
| Pros | - Excellent acoustic insulation between floors - Durable insulation with high fire and moisture resistance - Well established construction method | - Low embodied carbon in recycled cellulose insulation - Excellent acoustic insulation between floors - Blown insulation efficiently fills gaps to increase performance |
| Cons | - Higher embodied carbon vs cellulose-based insulation - High energy usage manufacturing - Insulation fibres can cause skin irritation | - Specialist equipment required for blown insulation - Insulation density and settlement can affect performance - Susceptible to moisture damage - Requires fire-resistant treatment |
| Separating Floor All to Robust Details | Screed, resilient layer, beam & block, suspended ceiling w/ ROCKWOOL & plasterboard | Screedboard, OSB deck, resilient layer, Posi-joist w/ ROCKWOOL, resilient bar w/ plasterboard, suspended ceiling w/ plasterboard |
| Pros | - Good thermal and acoustic insulation - Fire and moisture resistant insulation - Posi-joist construction allows for easy routing of services | - Low embodied carbon in recycled cellulose insulation - Good thermal and acoustic insulation - Blown insulation efficiently fills the cavity providing better performance - Posi-joist construction allows for easy routing of services |
| Cons | - Higher embodied carbon compared to cellulose - Energy intensive insulation production - Insulation fibres can cause skin irritation | - Specialist equipment required for blown insulation - Insulation density and settlement can affect performance - Susceptible to moisture damage - Requires fire-resistant treatment |
| Roof | Plasterboard, polythene VCL, ROCKWOOL, concrete tiles | Plasterboard, SMARTPLY OSB, Warmcel, Welsh slate |
| Pros | - High durability and longevity - Good thermal and acoustic insulation - Lower cost compared to slate | - Lower embodied carbon - Higher rigidity and airtightness - Very long lifespan with lower maintenance - Good thermal and acoustic insulation - Natural aesthetics |
| Cons | - High embodied carbon - Heavy materials, requires a strong roof structure - Polythene prone to damage | - Higher upfront material costs - Heavy materials, requires a strong roof structure - Higher skill requirement and labour time - Warmcel requires specialist installation equiptment |

(Davis, 2025, p.20)

### HT 641 Operational outputs

| Operational scenarios | Space heating demand | Energy use intensity (EUI) | Annual energy use per dwelling | CO2e emissions ex. PV (60 yrs) | kWp of PV to Net Zero per dwelling |
| --- | --- | --- | --- | --- | --- |
| AD: L | 79.5 kWh/m²/yr | 65.6 kWh/m²/yr | 7,242 kWh | 27.20 tonnes | 10.0 kWp |
| LETI | 15.1 kWh/m²/yr | 44.5 kWh/m²/yr | 4,913 kWh | 18.50 tonnes | 6.8 kWp |
| Difference | 64.4 kWh/m²/yr | 21.1 kWh/m²/yr | 2,329 kWh | 8.7 tonnes | 3.2 kWp |

(Davis, 2025, p.27)

### Justifications:

Performance comparison between buildings with identical building fabric and u-values shows significant improvements can be made through detailed airtightness, thermal bridging, and ventilation strategies.

Reduced space heating demand from higher quality development allows the ASHP requirements to be reduced from 7kW to 3.5 kW.

Efficient operation and installation of MVHR system reduce the EUI and CO2e emissions to 32%, allowing for fewer PV panels required to reach Net Zero targets and reduce capital costs.

  - Costs

| AD L (Wales) 2025 upgrade to LETI |  |  |  |
| --- | --- | --- | --- |
| Building Element |  |  |  |
| Fabric | Double glazing to triple glazing | additional cost | + £623.63 |
|  | ASHP reducing in size | cost saving | - £6,000.00 |
|  | MVHR addition | additional cost | + £4,000.00 |
|  | MEV omission | cost saving | - £900.00 |
| Generation | PV array decreasing in size | cost saving | - £1,200.00 |
| TOTAL | Cost per unit |  | - £3,476.37 |
|  | Cost per m² of GIA |  | - £31.49 |

(Davis, 2025, p.29)

| Cost Analysis |  |  |  |
| --- | --- | --- | --- |
| Element | AD: L | LETI | Difference |
| Foundations | £8,750.00 | £8,750.00 | £0.00 |
| Ground Floor | £19,345.00 | £19,345.00 | £0.00 |
| External Walls | £54,230.00 | £54,230.00 | £0.00 |
| Party Walls | N/A | N/A | N/A |
| Internal Walls | £9,435.00 | £9,435.00 | £0.00 |
| Internal Floor | £5,225.00 | £5,225.00 | £0.00 |
| Separating Floor | N/A | N/A | £0.00 |
| Roof | £15,958.00 | £15,958.00 | £0.00 |
| Doors / Windows | £7,018.76 | £7,642.39 | + £623.63 |
| M&E | £31,200.00 | £26,800.00 | - £4,100.00 |
| TOTAL | £150,861.76 | £147,385.39 | - £3,476.37 |
| Cost per m2 | £1,366.50 | £1,335.01 | - £31.49 |

Projected Annual Electricity Bills:

AD: L	£1,774.29

LETI	£1,203.69

| Topic | AD:L | LETI |
| --- | --- | --- |
| Operational Energy | meets minimum standards | minimises demand, uses high efficiency systems |
| Embodied Carbon | Limited considerations | Considered from the design stage |
| Controls | Basic compliance | Intelligent optimisation and monitoring |
| Renewable energy | Optional | Strongly encouraged where appropriate |
| Whole-life cost | Lower upfront cost, higher lifetime cost | High upfront cost, lower lifetime cost |

### HT 341 Operational Energy Losses & Gains

Figure 5: Total Annual Costs (www.passivhaustrust.org.uk, n.d.)

## Conclusion

This report has critically appraised the M&E design for the scenario building and evaluated improvements which would bring the design closer to reaching LETI’s low-energy performance targets.

The current AD: L design complies with Building Regulations and incorporates low carbon heating and heat recovery ventilation which support the transition towards renewable energy.

If they are correctly designed and installed, these systems can deliver good levels of occupant comfort and indoor air quality.

However, this design only achieves the minimum AD: L standards which could lead to higher operational energy usage, higher heat loss through thermal bridging, and lower airtightness thresholds affecting the efficiency of the installed services, reducing the dwelling’s overall lifetime operational performance.

This appraisal hopes to demonstrate that adopting a ‘fabric-first’ approach through careful selection of the building materials, improvements to airtightness, continuous insulation, and reducing thermal bridging would greatly reduce the building’s heating and energy demands and allow the MVHR and ASHP units to operate at a higher efficiency level.

Integrating the building fabric, services, and construction methods together in the design stage and focusing on higher quality installation, commissioning, and maintenance of the services greatly improves the energy performance levels and life-time emissions.

For further improvements, installation of PV panels and battery storage would allow for future electrical infrastructure and reduce reliance on Grid electricity.

The suggested improvements reduce operational carbon emissions, lower annual energy consumption, and decrease operational energy costs whilst keeping high occupant comfort and indoor air quality.

In summary, this report aims to highlight the benefits of adopting LETI performance principles whilst still complying with AD: L and support the UK’s Net Zero objectives.

## References

Approved Document F (2021). Approved Document F: Volume 1 applies to dwellings. [online] Available at: https://assets.publishing.service.gov.uk/media/61deba42d3bf7f054fcc243d/ADF1.pdf.

BEE - the smart heating merchants. (2026). Mixergy x slimline indirect unvented cylinder 120l - 210l. [online] Available at: https://beetbg.com/products/mixergy-210l-indirect-unvented-cylinder-485mm-dia.

Davis, J. (2025). Net Zero Carbon Buildings​ Feasibility Study & Cost Assessment.

Envirovent.com. (n.d.). energiSava® 200 | envirovent. [online] Available at: https://www.envirovent.com/product-details/energisava-200/.

EPCs as Efficiency Targets. (n.d.). [online] Available at: https://www.passivhaustrust.org.uk/UserFiles/File/research%20papers/EPCs/2020.04.17-

EPCs%20as%20efficiency%20targets-v9.pdf.

HM Government (2023). L Conservation of Fuel and Power the Building Regulations 2010 Volume 1: Dwellings Requirement L1: Conservation of Fuel and Power Requirement L2: On-site Generation of Electricity Regulations. [online] Available at: https://assets.publishing.service.gov.uk/media/662a2e3e55e1582b6ca7e592/Approved_Document_L__Conservation_of_fuel_and_power__Volume_1_Dwellings__2021_edition_incorporating_2023_amendments.pdf.

Mixergy. (2025a). Mixergy X. [online] Available at: https://www.mixergy.co.uk/solutions/hot-water/mixergy-x/.

Spring-Consultancy. (2025b). Spring-Consultancy. [online] Available at: https://www.spring-consultancy.co.uk/.

Vaillant.co.uk. (2025c). aroTHERM plus air source heat pump 5kW. [online] Available at: https://professional.vaillant.co.uk/for-installers/products/arotherm-plus-air-source-heat-pump-5kw-146560.html.

www.passivhaustrust.org.uk. (n.d.). Design Support. [online] Available at: https://www.passivhaustrust.org.uk/design_support.php.

‌

## Appendix

Heating demand = Amount of energy required to heat a space annually

Energy	 / 	area (TFA) 	/ 	annum

kWh 	/ 	m²TFA 		/	 yr

Energy use intensity (EUI) = All energy consumed by a building annually*

Energy	 / 	area (GIA)	 /	 annum

kWh 	/ 	m²GIA		 / 	yr

*excludes EV charging

Net Zero (operational) = Balancing annual consumption with on-site generation

### Figures:

Figure 6: Spring Designs HT 641_Site Plan & Elevations

Figure 7: Spring Designs_HT 641_Floor Plans

Figure 8: Spring Designs_HT 641_Elevations

aroTHERM plus air source heat pump 5kW Specifications Unit; 5kW VWL 55 / 6. General information. Width; mm; 1,100. Height; mm; 765. Depth; mm; 450. Weight, ready for operation; kg; 114. Connection, heating circuit; G 1 1/4". Rated voltage; V; 230 V (+10%/- 15%), 50 Hz, 1~/N/PE. Rated current, maximum; A; 14.3. Fuse size; A; 16. Fuse Type; Characteristic C, slow-blow, singlepole switching. RCD type; Type B rated to 20kHz or greater and with a minimum trigger point of 150mA above 1kHz.. eBUS (2-core communication cable); mm2; 0.75. Maximum length eBUS cable (polarity sensitive); m; 50. IP rating; m; IP 15 B. Fan, power consumption; W; 40. Fan quantity; 1. Fan, air flow, maximum; m3/h; 2,300. Heating pump, power consumption; W; 2 - 50. Heating circuit. Heating water temperature, minimum/maximum; °C; 20 - 75. Operating pressure, minimum; bar; 0.5. Operating pressure, maximum; bar; 3.0. Volume flow, minimum; l/h; 400. Volume flow, maximum; l/h; 860. Water volume, in the outdoor unit; l; 1.5. Minimum open water volume; l; 15. Refrigerant circuit. Fluid type; R290. Fluid fill quantity; kg; 0.6. Refrigerant, Global Warming Potential (GWP); 3. Noise emissions, heating mode. Sound power, EN 12102, EN 14511 LWA, A7/W35; dB(A); 48. Sound power, EN 12102, EN 14511 LWA, A7/W55; dB(A); 48. Efficiency. Energy efficiency class 35°C; (A+++ to F); A++. Energy efficiency class 55°C; (A+++ to F); A. Combination with uniTOWER. Energy efficiency class; (A+++ to F); A+++. Energy efficiency class for hot water supply; (A+ to F); A++.

Figure 9: aroTHERM Specifications (Vaillant.co.uk, 2025)