Lab – Understanding Ethernet Cable Standards

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC-A | NIC | 192.168.10.1 | 255.255.255.0 | N/A |
| PC-B | NIC | 192.168.10.2 | 255.255.255.0 | N/A |

Objectives

Part 1: Analyze Ethernet Cabling Standards and Pinouts

Part 2: Build an Ethernet Crossover Cable

Part 3: Test an Ethernet Crossover Cable

Background / Scenario

In this lab, you will build and terminate an Ethernet crossover cable and test it by connecting two PCs together and pinging between them. You will first analyze the Telecommunications Industry Association/Electronic Industries Association (TIA/EIA) 568-A and 568-B standards and how they apply to Ethernet cables. You will then construct an Ethernet crossover cable and test it. Finally, you will use the cable you just constructed to connect two PCs together and test it by pinging between them.

Note: With autosensing capabilities available on many devices, such as the Cisco 1941 Integrated Services Router (ISR) switch, you may see straight-through cables connecting like devices.

Required Resources

One length of cable, either Category 5 or 5e. Cable length should be 0.6 to 0.9m (2 to 3 ft.)

2 RJ-45 connectors

RJ-45 crimping tool

Wire cutter

Wire stripper

Ethernet cable tester (optional)

2 PCs (Windows 7 or 8)

Analyze Ethernet Cabling Standards and Pinouts

The TIA/EIA has specified unshielded twisted pair (UTP) cabling standards for use in LAN cabling environments. TIA/EIA 568-A and 568-B stipulates the commercial cabling standards for LAN installations; these are the standards most commonly used in LAN cabling for organizations and they determine which color wire is used on each pin.

With a crossover cable, the second and third pairs on the RJ-45 connector at one end of the cable are reversed at the other end, which reverses the send and receive pairs. The cable pinouts are the 568-A standard on one end and the 568-B standard on the other end. Crossover cables are normally used to connect hubs to hubs or switches to switches, but they can also be used to directly connect two hosts to create a simple network.

Reflection:

Write a research report (Max word count 600) on Cat 5e and 6 Ethernet cable standards.

Analyze diagrams and tables for the TIA/EIA 568-A standard Ethernet cable.

The following table and diagrams display the color scheme and pinouts, as well as the function of the four pairs of wires used for the 568-A standard.

Note: In LAN installations using 100Base-T (100 Mb/s), only two pairs out of the four are used.

568-A 10/100/1000Base-TX Ethernet

| Pin Number | Pair Number | Wire Color | 10Base-T Signal 100Base-TX Signal | 1000Base-T Signal |
| --- | --- | --- | --- | --- |
| 1 | 2 | White/Green | Transmit | BI_DA+ |
| 2 | 2 | Green | Transmit | BI_DA- |
| 3 | 3 | White/Orange | Receive | BI_DB+ |
| 4 | 1 | Blue | Not Used | BI_DC+ |
| 5 | 1 | White/Blue | Not Used | BI_DC- |
| 6 | 3 | Orange | Receive | BI_DB- |
| 7 | 4 | White/Brown | Not Used | BI_DD+ |
| 8 | 4 | Brown | Not Used | BI_DD- |

The following diagrams display how the wire color and pinouts align with an RJ-45 jack for the 568-A standard.

Analyze diagrams and tables for the TIA/EIA 568-B standard Ethernet cable.

The following table and diagram display the color scheme and pinouts for the 568-B standard.

568-B 10/100/1000-BaseTX Ethernet

| Pin Number | Pair Number | Wire Color | 10Base-T Signal 100Base-TX Signal | 1000Base-T Signal |
| --- | --- | --- | --- | --- |
| 1 | 2 | White/Orange | Transmit | BI_DA+ |
| 2 | 2 | Orange | Transmit | BI_DA- |
| 3 | 3 | White/Green | Receive | BI_DB+ |
| 4 | 1 | Blue | Not Used | BI_DC+ |
| 5 | 1 | White/Blue | Not Used | BI_DC- |
| 6 | 3 | Green | Receive | BI_DB- |
| 7 | 4 | White/Brown | Not Used | BI_DD+ |
| 8 | 4 | Brown | Not Used | BI_DD- |

Build an Ethernet Crossover Cable

A crossover cable has the second and third pairs on the RJ-45 connector at one end, reversed at the other end (refer to the table in Part 1, Step 2). The cable pinouts are the 568-A standard on one end and the 568-B standard on the other end. The two following diagrams illustrate this concept.

Build and terminate a TIA/EIA 568-A cable end.

Determine the cable length required. (Your instructor will let you know the cable length you should make.)

Note: If you were making a cable in a production environment, the general guideline is to add another 12 in. (30.48 cm) to the length.

Cut off a piece of cable to the desired length and using your wire stripper, remove 5.08 cm (2 in.) of the cable jacket from both ends.

Hold the four pairs of twisted cables tightly where the jacket was cut away. Reorganize the cable pairs into the order of the 568-A wiring standard. Refer to the diagrams, if necessary. Take as much care as possible to maintain the twists in the cable; this provides noise cancellation.

Flatten, straighten, and line up the wires using your thumb and forefinger.

Ensure that the cable wires are still in the correct order for the 568-A standard. Using your wire cutters, trim the four pairs in a straight line to within 1.25 to 1.9 cm (1/2 to 3/4 in.).

Place an RJ-45 connector on the end of your cable, with the prong on the underside pointing downward. Firmly insert the wires into the RJ-45 connector. All wires should be seen at the end of the connector in their proper positions. If the wires are not extending to the end of the connector, take the cable out, rearrange the wires as necessary, and reinsert the wires back into the RJ-45 connector.

If everything is correct, insert the RJ-45 connector with cable into the crimper. Crimp down hard enough to force the contacts on the RJ-45 connector through the insulation on the wires, thus completing the conducting path. See the following diagram for an example.

Build and terminate a TIA/EIA 568-B cable end.

Repeat steps 1a to 1g using the 568-B color wiring scheme for the other end.

Test an Ethernet Crossover Cable

Test the cable.

Many cable testers will test for length and mapping of wires. If the cable tester has a wire map feature, it verifies which pins on one end of the cable are connected to which pins on the other end.

If your instructor has a cable tester, test the crossover cable for functionality. If it fails, check with your instructor first as to whether you should re-cable the ends and re-test.

Connect two PCs together via NICs using your Ethernet crossover cable.

Working with a lab partner, set your PC to one of the IP addresses shown in the Addressing Table (see page 1). For example, if your PC is PC-A, your IP address should be set to 192.168.10.1 with a 24-bit subnet mask. You partner’s IP address should be 192.168.10.2. The default gateway address can be left empty.

Using the crossover cable you made, connect the two PCs together via their NICs.

On the PC-A command prompt, ping the PC-B IP address.

Note: The Windows firewall may have to be temporarily disabled for pings to be successful. If the firewall is disabled, make sure you re-enable it at the conclusion of this lab.

Repeat the process and ping from PC-B to PC-A.

Assuming IP addressing and firewall are not issues, your pings should be successful if the cables were properly made.

Reflection

Which part of making cables did you find the most difficult?

- The physical dexterity required to keep all the cables in the same pin slots while crimping.

Why do you have to learn how to make a cable if you can easily buy pre-made cables?

You can make cables yourself to fit custom lengths, reuse spare cable to save costs, to be more employable.

Report: 

Write a research report (Max word count 600) on Cat 5e and 6 Ethernet cable standards.

Both Cat5e and Cat 6 Ethernet cabling are standards in the telecommunications cabling systems. They are both unshielded twisted pair cables (UTP). 

Cat5e provides performance of up to 100Mhz and is suited for 1000BASE-T, which is a standard for Gigabit Ethernet over copper wiring (1) most Cat 5 cables including the e variant are unshielded, so care has to be taken when installing the cables so as not to be interfered with by unwanted frequencies. Cat 5 cables are available in both stranded forms and solid conductor varieties. 

The stranded variant can withstand bending without breaking more so than the solid core variant. Solid core is used when installation would be more permanent due to the fragile nature of the solid core. The Cat5e variety took over from the original Cat 5 standard in 2001 as it was an improvement overall, including extending frequency response which reduced the amount of interference via crosstalk. 

The Cat6 cable is similar to the Cat5e, but unlike the former which can operate up to 100Mhz, Cat6 cables operate anywhere from 0-250Mhz.

Both cables have an absolute maximum length of 100 meters, to ensure optimum performance, however for Cat 6, it can operate up to on Gigabit Ethernet (e.g. 10GBASE-T) at speeds theoretically possible of up to 10Gbps. For this mode of operation, the length can be no more than 55 meters in length, lowered to 33 meters in high cross talk conditions.  Cat 5e is the theoretically slower cable, in being able to provide 1000Mbps, but typically it works in modes of operation at 10, 100 and 1000 Mbps. Because of the connectors used, both Cat5e and 6 cables are backwards compatible with the 5 variant and Cat 3 also.

Because of the higher performance of Cat 6, it is typically priced higher than Cat5e. One source shows that a 50m Coil of Cat5e is £8.63 ex Vat, whereas a 10m Coil of Cat 6 cable would be £4.66, so to multiply that figure by 5 to match, would be £23.30, almost triple the cost.