| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| July 2022 | Gorffennaf 2022 |

MODULE CODE: ACDA4006

Electrical & Electronic Principles

TIME:  2 HOURS

answer all questions

Equipment or materials required – Calculator, Formulae Sheet

Adnoddau neu ddefnyddiau sydd eu hangen – Cyfrifiannell, Taflen Fformiwla

6 Questions

Total marks available = 60

ANSWER ALL QUESTIONS

- A 12 V battery has an internal resistance of 3Ω,

  - Calculate the power dissipated by various load resistances and complete the table below.

| Load resistance (Ω) | Load power (W) |
| --- | --- |
| 1 | 9 |
| 2 | 11.52 |
| 3 | 12 |
| 4 | 11.755 |
| 5 | 11.25 |

[10 Marks]

  - Plot a graph of the load power against load resistance

[6 Marks]

Graph showing data from table above. Marks for: Axis labels, Units, Scale, Headings, Axis right way round, Data plotted as per table

- For the circuit shown in Fig Q2, given R1=1kΩ, R2=470Ω, R3=1.5kΩ, R4=330Ω, R5=560Ω, R6=2.7kΩ, and the battery has a voltage of 9V. Calculate:

  - Total circuit resistance

[4 Marks]

RT=1000+800×1500800+1500+560+2700=4781.74 Ω

  - Circuit current

[2 Marks]

I=VR=94781.74=1.882 mA

  - Total power supplied by the battery

[2 Marks]

P=IV=1.882×10-3×9=16.938 mW

  - Current flowing through R4

[2 Marks]

I=1.882×10-3×1500800+1500=1.227 mA

  - Power dissipated by R3

[4 Marks]

IR3=1.882×10-3-1.227×10-3=0.655 mA

P=I2R=(0.655×10-3)2×1500=643.538 µW

  - R3 is replaced by a 10Ω resistor. What is the new circuit current?

[4 Marks]

RT=1000+800×10800+10+560+2700=4269.877 Ω

I=VR=94269.877=2.108 mA

Fig Q2

- For the symmetrical sinusoid shown in Fig Q3 below:

  - Determine its period τ

[1 Mark]

τ=1f=110000=100 µs

  - Determine its peak-peak voltage.

[1 Mark]

Vpk-pk=Vpk×2=50×2=100 V

  - Determine the RMS value of the waveform.

[2 marks]

VRMS=Vpk×0.707=50×0.707=35.35 V

  - Sketch the diagram immediately below and indicate on it any two points between which the PERIOD (T) could be measured

[1 Mark]

Accept any two identical points on consecutive cycles

10kHz

50V

Fig Q3

- For the circuit shown in fig Q4, given the resistor is 50Ω, the inductor is 10mH, and the capacitor is 1µF, and the supply is 15V RMS at a frequency of 1500Hz.

Determine the following:

  - Total Impedance

[2 Marks]

XC=12πfC

XC=12×π×1500×1µ=106.103 Ω

XL=2πfL

XL=2×π×1500×0.01=94.248 Ω

Z=R2+XL-XC2

Z=502+94.248-106.1032=51.386 Ω

  - Total current supplied to the circuit

[2 Marks]

I=VZ=1551.386=291.908 mA

  - Phase difference between the supply voltage and current

[2 Marks]

φ=tan-1 XTR

φ=tan-1 94.248-106.10350=-13.339 °

  - Voltage across the resistor.

[2 Marks]

V=I×R=0.291908×50=14.595 V

  - Voltage across the capacitor

[2 Marks]

V=I×XC=0.291908×106.103=30.972 V

  - Voltage across the inductor

[2 Marks]

V=I×XL=0.291908×94.248=27.516 V

  - The resonant frequency

[2 Marks]

fres=12πLC=12π0.01×1µ=1591.549 Hz

Fig Q4

  - State Kirchhoff’s voltage law

[2 Marks]

The sum of voltages around a closed loop are equal to zero.

  - State Kirchhoff’s current law

[2 Marks]

The sum of currents around a circuit node is equal to zero.

  - Show how 4 diodes can be connected in a circuit to create a DC voltage source from an AC voltage source.

[2 Marks]

  - What component can be added to this circuit to smooth the DC output voltage?

[1 Mark]

A capacitor can be added to the output of the circuit to smooth the ripple from the DC voltage.

Diwedd/End

# Formulae Sheet

|  |  | RT=R1+R2+…+Rn 1RT=1R1+1R2+…+1Rn 1CT=1C1+1C2+…+1Cn CT=C1+C2+…+Cn LT=L1+L2+…+Ln 1LT=1L1+1L2+…+1Ln | VOut=VinRBRA+RB I=IinRBRA+RB |
| --- | --- | --- | --- |
|  |  |  | XC=12πfC XC=-j1ωC XL=2πfL XL=jωL |
| Z=R+jXL-XC Z=R2+XL-XC2 | ω=2πf XT=XL-XC | φ=tan-1XTR pf=cosφ | fres=12πLC |
| VC=Vs1-e-tτ IC=VRe-tRC VC=V0e-tτ IC=-VRe-tRC τ=RC | τ=LR VL=Ve-RtL VL=-Ve-RtL IL=VR1-e-RtL IL=VRe-RtL | f=12πRC AV=20logVoutVin AP=10logPoutPin | Q=XLR Q=XCR Q=fRfU-fL Q=1RLC |
|  |  | E=12CV2 E=12LI2 |  |
| VB =RB2(RB1 + RB2)× VCC VE=VB-0.7 IC=IE VC=VCC-IC×RC Av=vcvb VE=10%VCC RIN=11RB1+1RB2+1hFE×RE ROUT=RC AV≈-RCRE |  | Vout=-R2R1×Vin Vout=1+R2R1×Vin Vout=R2R1×V2-V1 Vout=-R2R1×V1+V2 Vout=-RR1×V1+RR2×V2+…+RRn×Vn |  |

Detach this page and place in answer book

Student Number ………………………….	Question Number ……………………….