ACDA6020 Lab Session 4: Methyl 3 Nitrobenzoate Melting point

By Daniel Garton 2219350

ACDA6020

2219350@student.uwtsd.ac.uk

Contents

Introduction	2

Variables	2

Theory	2

Results	3

Discussion	3

Errors	4

OME applications	4

Alternatives	4

Conclusion	5

Bibliography	5

# Introduction

The aim of this investigation is to obtain the melting point of the student synthesised Methyl 3 nitrobenzoate samples to evaluate the purity of the samples. These samples were synthesised in lab report 3. The samples which are purer will be within a closer range to the stated melting point. The stated melting point of Methyl 3 Nitrobenzoate is 78-80 degrees [1]. 4 samples are being used within this report from 4 pairs of students.

## Variables

  - Independent: The different samples.

  - Dependent: The observed melting point temperature.

  - Controlled: Ramp rate (2°C/min), plateau temperature setting, and the specific apparatus (Cole-Parmer M200).

## Theory

Key theory used in this investigation is the melting point of a material. The melting point is the temperature where the material changes state from a solid to a liquid [2]. The melting point is used to determine the purity of the sample. This is due to the Structure of the substance, a pure chemical like MNB the molecules pack densely and uniformed lattice, this means that all the bonds break at the same temperature resulting in a tight melting point usually. When a substance is contaminated, the molecules aren’t able to pack densely or uniformed, due to this the forces holding attracting the molecules together is weaker resulting in a lower melting point.

Purity of samples was dictated in the synthesis process during the recrystallization step. The samples were recrystallized using hot ethanol. Ethanol is used because it can dissolve the methyl 3 Nitrobenzoate when it is hot and the contaminants, however when the ethanol cools the methyl 3 nitrobenzoate is not soluble by ethanol leaving the product behind and removing contaminants.

# Results

The melting point of methyl 3 nitrobenzoate is 78-80 degree Celsius as previously stated [1].

| Group | Inspection | Trial 1 MP | Trial 2 MP | Average MP |
| --- | --- | --- | --- | --- |
| DG | Clumpy | 77 | 77 | 77 |
| OM | White | 75 | 75 | 75 |
| LM | Odour, yellowing | 74 | 73 | 73.5 |
| B J | Clear | 77 | 77 | 77 |

DG and B + J samples were the closest to the stated melting point of MNB being just outside by 1 degree. With LM being the furthest form the melting point 4.5 degree away. DG was stated to be clumpy which suggests a texture difference but not necessarily an impurity. LM was observed to have an odour and a yellowing which deviates from the stated description. OM sample is the closest to the stated sample.

# Discussion

DG and B + J had a MP of 77 degrees is the closest to the stated MP of MNB suggesting they could be the highest percent purity samples. There is evidence to suggest impurity within the LM sample due to the low melting point of the sample at just 73.5 degrees average. This is due to the impurities affecting structure of the sample. Adding to this evidence the odour and yellow stain suggests that the sample is impure. This impurity may be due to the sample being cooled too rapidly trapping contaminants in the crystals. Another potential reason for contamination could be inadequate drying which could suggest the presence of ethanol contamination.

## Errors

The Cole-Parmer M200 used as the method of heating the samples and observing the melting points. The Cole Parmer M200 has a stated accuracy of -/+1 at 20 c and -/+ 2.5 at 300c [3]. Therefore, at 80c could have discrepancy of 1.5c. This range could exacerbate or minimise our results compared to the stated MP of MNB.

The capillary tubes require being packed correctly to ensure the temperature is transferred to the chemical effectively. The tube should be filled with material about 2-3mm wide [4] and packed tightly. The sample should be completely dry for accurate results.

This experiment is completed by an operator recognising a state change within the capillary tube. This allows for human error to creep into the results. If the sample is contaminated by solvent this will create droplets of liquid in the tube first before the MNB melts. This can create the illusion that the sample is melting this is called sweating. Sintering is another factor which is where the solid molecules contract and can be mistaken for “sagging” creating the illusion of melting.

## OME applications

Purity of samples in the OME worlds is critical to product safety and quality. Contamination of an Energetic material decreases the insensitivity of the munition. This reduces the safety of the product and will increase manufacturing, transportation, disposal and firing accidents [5].

## Alternatives

Another option of assessing the purity of the sample to confirm the results would be to use FTIR. FTIR works by measuring the absorption of infrared light by chemical bonds. This can be used to check the purity of the sample [6].

# Conclusion

The samples of DG and B + J are determined to be the purest by this experiment. The theory of contaminated samples would melt before the stated melting point of MNB. For more confirmation of this data it would be recommended to complete other methods stated above like DSC.

# Bibliography

| [1] | “Thermo Fischer Scientific SDS Methyl 3 Nitrobenzoate,” 23 02 2011. [Online]. Available: https://www.fishersci.com/store/msds?partNumber=AC168445000&countryCode=US&language=en. [Accessed 02 01 2026]. |
| --- | --- |
| [2] | “MELTING POINT ANALYSIS- IDENTITY AND PURITY,” LibreTexts chemistry, [Online]. Available: https://chem.libretexts.org/Bookshelves/Organic_Chemistry/Book%3A_How_to_be_a_Successful_Organic_Chemist_(Sandtorv)/05%3A_HOW_TO_INTERPRET_YOUR_RESULTS/5.03%3A_MELTING_POINT_ANALYSIS-_IDENTITY_AND_PURITY. [Accessed 03 01 2026]. |
| [3] | “Colepalmer MP-200D-120 Digital melting point apparatus,” Cole Palmer, [Online]. Available: https://www.coleparmer.co.uk/i/cole-parmer-mp-200d-120-digital-melting-point-apparatus-120-vac/0301140. [Accessed 02 01 2026]. |
| [4] | “A Fast and Reliable Method for Packing Capillaries for X-ray Powder Diffraction Analysis,” ACS Publications, 1999. |
| [5] | Y. Wang, “Preparation and Characterization of RDX with reduced senseitivity,” MDPI, 2017. |
| [6] | “How an FTIR Spectrometer Operates,” Libretexts Chemistry, [Online]. Available: https://chem.libretexts.org/Bookshelves/Physical_and_Theoretical_Chemistry_Textbook_Maps/Supplemental_Modules_(Physical_and_Theoretical_Chemistry)/Spectroscopy/Vibrational_Spectroscopy/Infrared_Spectroscopy/How_an_FTIR_Spectrometer_Operates. [Accessed 03 01 2026]. |
| [7] | “Melting,” Libretexts chemistry, [Online]. Available: https://chem.libretexts.org/Bookshelves/Introductory_Chemistry/Introductory_Chemistry_(CK-12)/13%3A_States_of_Matter/13.11%3A_Melting. [Accessed 04 01 2026]. |

7. Appendix

Materials:

Cole-Parmer M200

glass capillaries

The methyl-3-nitrobenzoate samples from each group.