# Computer Architecture Tutorial – Solutions

## Part A – Basic Concepts

1.

Control Unit: Directs operations of the CPU and controls execution of instructions.

Arithmetic Logic Unit: Performs arithmetic and logical operations.

Register: Small high-speed storage location inside the CPU.

System Clock: Generates timing signals that synchronise CPU operations.

2.

PC: Holds address of next instruction.

MAR: Holds memory address to be accessed.

MDR: Temporarily holds data transferred between CPU and memory.

IR: Holds the current instruction being executed.

ACC: Stores intermediate results of arithmetic operations.

3. Machine instruction: A command stored in memory that tells the CPU what operation to perform.

Example: ADD A,B

Opcode = ADD

Operands = A and B

## Part B – Instruction Cycle

4. Fetch, Decode, Execute

5. PC → MAR → Memory → MDR → IR

6. Fetch sequence:

PC contains address 200

Address copied to MAR

Control unit sends READ signal

Memory[200] instruction transferred to MDR

Instruction moved to IR

PC increments to next instruction

7. Opcode and Operands

8. Arithmetic Logic Unit (ALU)

## Part C – System Clock

9. Synchronises and coordinates CPU operations by generating timing signals.

10. 3 billion clock cycles per second

## Part D – System Bus

11. Data Bus, Address Bus, Control Bus

12.

Data Bus – transfers data and instructions

Address Bus – carries memory addresses

Control Bus – carries control signals (READ, WRITE)

13.

a) False

b) True

c) True

## Part E – Applying the Concepts

14.

Address Bus → memory address sent from CPU

Control Bus → READ signal sent by control unit

Data Bus → instruction returned from memory to CPU

15.

CPU retrieves value of A

CPU retrieves value of B

ALU performs A + B

Result stored in register or memory