Model a bullets position over time

Position (x, y): Initial position in 2D space, usually defined as (x0, y0).

Velocity (vx, vy): Initial velocity components in the horizontal (vx) and vertical (vy) directions.

Acceleration:

- Gravity (g): Acts downward, usually defined as g = −9.81

- Air Resistance: Can be modeled as either linear drag (c⋅v) or quadratic drag (c⋅v2), where c is a drag coefficient.

Mass (m): Mass of the bullet, which affects acceleration under air resistance.

Drag Coefficient (c): Determines how much air resistance slows down the bullet.

Time Step (deltaTime): Small time interval for each update, e.g., 0.01 seconds.

// Constants and initial conditions

const float g = -9.81;  // Gravity in m/s^2 (downward)

const float c = 0.05; // Drag coefficient

const float m = 0.01; // Mass of the bullet in kg

const float deltaTime = 0.01; // Time step in seconds

// Initial position and velocity

float x = 0.0; // Initial x position

float y = 0.0; // Initial y position (ground level)

float vx = 300.0; // Initial velocity in x direction (m/s)

float vy = 300.0; // Initial velocity in y direction (m/s)

float time = 0.0; // Start time