#include <main.h>

#define LCD_ENABLE_PIN PIN_D2

#define LCD_RS_PIN PIN_D0

#define LCD_RW_PIN PIN_D1

#define LCD_DATA4 PIN_D4

#define LCD_DATA5 PIN_D5

#define LCD_DATA6 PIN_D6

#define LCD_DATA7 PIN_D7

#include <lcd.c>

void analog(void);						//function for reading the adc																				// define function/subroutine prototypes

void motorSA(void);						// function for RV1  controlling motor and printing

void motorSC(void);						// Motor A control via button press

void PosB(void);						// function for printing RV1 value, no motor control

void kitt(void);						// Kitt mode lights

unsigned int adc;						// global var to store adc value in

void main()							// Start of main sequence

{

setup_adc_ports(sAN0);					// setting up adc ports	and operation														// initialise adc channel

setup_adc(ADC_CLOCK_INTERNAL|ADC_TAD_MUL_0);		// adc clock

lcd_init();							// initalise the lcd

while(TRUE)						// always loop

{

analog();				// calls the analog function																								// read the analog input

if (input(pin_a2))			// Calls motorSA function if on switch position 1

{

motorSA();

}

if (input(pin_a3))			// Calls PosB function if on switch position 2

{

PosB();

}

if (input(pin_a4))			// Calls motorSC function if on switch position 3

{

motorSC();

output_b(0);				// Clear LED's status

}

if (input(pin_a5))			// Calls Kitt function if on switch position 4

{

kitt();

}

}								// End of the while loop

}								// End of main function

void analog(void)						// start of the analogs function

{

set_adc_channel(0);			// set adc channel to 0

delay_us(30);				// wait 30us

adc = read_adc();			// assign the value of the adc pin to adc

}							// end of analog function

void motorSA(void)						// Start of motorSA function

{

unsigned int state;					// create local State variable

if (adc < 75)						// adc pin < 75

{

lcd_gotoxy(4,2);					// go to middle of lcd screen line 1

printf(lcd_putc, "Anti Clockwise") ;		//print to lcd

output_low(pin_c0); 				//turn motor anti clockwise

output_high(pin_c1);

}

if (adc > 175)						// adc pin > 175

{

lcd_gotoxy(4,2);					// middle of lcd screen

printf(lcd_putc, "  Clockwise  ") ;			//print to lcd

output_low(pin_c1); 				// turn motor clockwise

output_high(pin_c0);

}

if ((adc > 76) && (adc < 174))				// adc value between 76 and 174

{

lcd_gotoxy(4,2);					// lcd middle of line 2

printf(lcd_putc, "   Stopped    ") ;		// print to lcd

output_low(pin_c0); 				// motor stop

output_low(pin_c1);

}

lcd_gotoxy(6,1);					// lcd middle of line 1

printf(lcd_putc, "RV1= %3u",adc) ;			// print adc to lcd

output_B(adc);						// display adc on leds as binary

if (adc !=state)					// check buttons previous state

printf("ADC= %u\n\r", adc);				// print once to terminal

state = adc;						// set state var to adc to store previous state

}							// end of motorSA function

void PosB(void)							// start of PosB function

{

output_low(pin_c0);					// stop motor if on

output_low(pin_c1);

unsigned int state;					// create local state var

lcd_gotoxy(1,2);

printf(lcd_putc,"                    ");		// clear unused lines

lcd_gotoxy(21,1);

printf(lcd_putc,"                    ");		// clear unused lines

lcd_gotoxy(21,2);

printf(lcd_putc,"                    ");		// clear unused lines

lcd_gotoxy(6,1);

printf(lcd_putc, "RV1= %3u",adc) ;			// print adc to lcd

output_B(adc);						// display adc on leds as binary

if (adc !=state)					// check buttons previous state

printf("ADC= %u\n\r", adc);				// print once to terminal

state = adc;						// set state var to adc to store previous state

}							// end of PosB function

void motorSC(void)						// start of motorSC function

{

unsigned int button =0;				// set the state of the button to 0

unsigned int state;					// create local state var

lcd_gotoxy(1,1);

printf(lcd_putc,"                    ");		// clear unused lines on lcd

lcd_gotoxy(21,2);

printf(lcd_putc,"                    ");		// clear unused lines on lcd

if(input(pin_a1))					// check if button is pressed

{

output_low(pin_c1); 				// spin motorA clockwise

output_high(pin_c0);

button = 1;						// set the state of the button to pressed

lcd_gotoxy(21,1);

printf(lcd_putc, "   Button pressed    " );		// print button status on lcd

if (state != 1)					// check button status

printf("Pressed \n\r");				// print status of button to terminal

state = 1;						// set state var to 1

}

else							// if the button isnt pressed

{

output_low(pin_c1); 				// motorA stopped

output_low(pin_c0);

button = 0;						// state of button to off

lcd_gotoxy(21,1);

printf(lcd_putc, " Button not pressed   " );	// print status of button to lcd

if (state != 0)					// print to terminal if the button was previously pressed

printf("Not pressed \n\r");			// print to terminal

state = 0;						// reset state to 0

}  							// end else

}  							// end motorSC function

void kitt(void)										// start of kitt function

{

unsigned int state;								// local var state

lcd_gotoxy(21,1);

printf(lcd_putc,"                    ");					// clear unused lines on lcd

lcd_gotoxy(21,2);

printf(lcd_putc,"     Kitt mode   ") ;					// print to lcd

if (state !=1)								// print "Kitt mode" once to the terminal

printf("Kitt mode \n\r");

state=1;									// set state to 1 to only print once to terminal

unsigned int LEDs[16] = {128,64,32,16,8,4,2,1,2,4,8,16,32,64,128};		// define Leds ports and positions

unsigned int index = 0;							// define var for position in leds array

for(unsigned state = 0; state < 14; state++, index++)			// loop itterating state and index

{

output_b(0);								// turns off the leds on port b

output_b(leds[index]);							// ouput to b ports on position index

delay_ms(200);

}										// end of for loop

}											// end of kitt function

Comments

2.. 20   3.. 30    4.. 40   5.. 10

****************************************

2.. Your code has a number of structures and all work. Your statements indicate what each line does 75%

3.. Your program flows nicely 75%

4.. Your program achieves the desired results. 75%

5.. Your program is nicely laid out and well indented. 75%

Total 75%