Christopher B (2322626)

[Email address]

Abstract

The following document contains the code produced for the embedded c assignment

Embedded c assignment

ACDA4013

#include <main.h>

#define LCD_ENABLE_PIN PIN_D2

#define LCD_RS_PIN PIN_D0

#define LCD_RW_PIN PIN_D1

#define LCD_DATA4 PIN_D4

#define LCD_DATA5 PIN_D5

#define LCD_DATA6 PIN_D6

#define LCD_DATA7 PIN_D7

#include <lcd.c>

void analog(void);                      //function for reading the adc                                                                              // define function/subroutine prototypes

void motorSA(void);                     // function for RV1  controlling motor and printing

void motorSC(void);                     // Motor A control via button press

void PosB(void);                        // function for printing RV1 value, no motor control

void kitt(void);                        // Kitt mode lights

unsigned int adc;                       // global var to store adc value in

void main()                         // Start of main sequence

{

setup_adc_ports(sAN0);                  // setting up adc ports and operation                                                       // initialise adc channel

setup_adc(ADC_CLOCK_INTERNAL|ADC_TAD_MUL_0);        // adc clock

lcd_init();                          // initalise the lcd

while(TRUE)                     // always loop

{

analog();               // calls the analog function                                                                                                // read the analog input

if (input(pin_a2))          // Calls motorSA function if on switch position 1

{

motorSA();

}

if (input(pin_a3))          // Calls PosB function if on switch position 2

{

PosB();

}

if (input(pin_a4))          // Calls motorSC function if on switch position 3

{

motorSC();

output_b(0);             // Clear LED's status

}

if (input(pin_a5))          // Calls Kitt function if on switch position 4

{

kitt();

}

}                               // End of the while loop

}                               // End of main function

void analog(void)                       // start of the analogs function

{

set_adc_channel(0);         // set adc channel to 0

delay_us(30);               // wait 30us

adc = read_adc();           // assign the value of the adc pin to adc

}                           // end of analog function

void motorSA(void)                      // Start of motorSA function

{

unsigned int state;                 // create local State variable

if (adc < 75)                      // adc pin < 75

{

lcd_gotoxy(4,2);                    // go to middle of lcd screen line 1

printf(lcd_putc, "Anti Clockwise") ;        //print to lcd

output_low(pin_c0);                 //turn motor anti clockwise

output_high(pin_c1);

}

if (adc > 175)                     // adc pin > 175

{

lcd_gotoxy(4,2);                    // middle of lcd screen

printf(lcd_putc, "  Clockwise  ") ;         //print to lcd

output_low(pin_c1);                 // turn motor clockwise

output_high(pin_c0);

}

if ((adc > 76) && (adc < 174))             // adc value between 76 and 174

{

lcd_gotoxy(4,2);                    // lcd middle of line 2

printf(lcd_putc, "   Stopped    ") ;        // print to lcd

output_low(pin_c0);                 // motor stop

output_low(pin_c1);

}

lcd_gotoxy(6,1);                   // lcd middle of line 1

printf(lcd_putc, "RV1= %3u",adc) ;         // print adc to lcd

output_B(adc);                     // display adc on leds as binary

if (adc !=state)                   // check buttons previous state

printf("ADC= %u\n\r", adc);             // print once to terminal

state = adc;                       // set state var to adc to store previous state

}                           // end of motorSA function

void PosB(void)                         // start of PosB function

{

output_low(pin_c0);                    // stop motor if on

output_low(pin_c1);

unsigned int state;                    // create local state var

lcd_gotoxy(1,2);

printf(lcd_putc,"                    ");       // clear unused lines

lcd_gotoxy(21,1);

printf(lcd_putc,"                    ");       // clear unused lines

lcd_gotoxy(21,2);

printf(lcd_putc,"                    ");       // clear unused lines

lcd_gotoxy(6,1);

printf(lcd_putc, "RV1= %3u",adc) ;         // print adc to lcd

output_B(adc);                     // display adc on leds as binary

if (adc !=state)                   // check buttons previous state

printf("ADC= %u\n\r", adc);             // print once to terminal

state = adc;                       // set state var to adc to store previous state

}                           // end of PosB function

void motorSC(void)                      // start of motorSC function

{

unsigned int button =0;                // set the state of the button to 0

unsigned int state;                    // create local state var

lcd_gotoxy(1,1);

printf(lcd_putc,"                    ");       // clear unused lines on lcd

lcd_gotoxy(21,2);

printf(lcd_putc,"                    ");       // clear unused lines on lcd

if(input(pin_a1))                  // check if button is pressed

{

output_low(pin_c1);                 // spin motorA clockwise

output_high(pin_c0);

button = 1;                     // set the state of the button to pressed

lcd_gotoxy(21,1);

printf(lcd_putc, "   Button pressed    " );     // print button status on lcd

if (state != 1)                 // check button status

printf("Pressed \n\r");              // print status of button to terminal

state = 1;                      // set state var to 1

}

else                           // if the button isnt pressed

{

output_low(pin_c1);                 // motorA stopped

output_low(pin_c0);

button = 0;                     // state of button to off

lcd_gotoxy(21,1);

printf(lcd_putc, " Button not pressed   " );    // print status of button to lcd

if (state != 0)                 // print to terminal if the button was previously pressed

printf("Not pressed \n\r");          // print to terminal

state = 0;                      // reset state to 0

}                              // end else

}                              // end motorSC function

void kitt(void)                                     // start of kitt function

{

unsigned int state;                              // local var state

lcd_gotoxy(21,1);

printf(lcd_putc,"                    ");                 // clear unused lines on lcd

lcd_gotoxy(21,2);

printf(lcd_putc,"     Kitt mode   ") ;                   // print to lcd

if (state !=1)                               // print "Kitt mode" once to the terminal

printf("Kitt mode \n\r");

state=1;                                 // set state to 1 to only print once to terminal

unsigned int LEDs[16] = {128,64,32,16,8,4,2,1,2,4,8,16,32,64,128};       // define Leds ports and positions

unsigned int index = 0;                          // define var for position in leds array

for(unsigned state = 0; state < 14; state++, index++)            // loop itterating state and index

{

output_b(0);                             // turns off the leds on port b

output_b(leds[index]);                           // ouput to b ports on position index

delay_ms(200);

}                                       // end of for loop

}                                          // end of kitt function