| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| July 2018 | Gorffennaf 2018 |

MODULE CODE: ACDA4002

Electrical and Electronic Principles

TIME:  2 HOURS

answer all questions

Equipment or materials required – Calculator

Adnoddau neu ddefnyddiau sydd eu hangen - Dim

ANSWER ALL QUESTIONS

  - For the circuit shown in Fig Q1a, calculate the following assuming Vs = 20V.

    - Total resistance

[2 Marks]

RT=10k×24k10k+24k+1122k+115k+112k

RT=7058.8235+5116.2791=12175.1026 Ω

    - Circuit current

[1 Mark]

I=VR=2012175.1026=1.643×10-3 A

    - Total power dissipated

[2 Marks]

P=V×I=20×1.643×10-3=32.853×10-3 W

Fig Q1a

  - A 110 V step-up transformer has a 200W lamp connected to its secondary winding. The turns-ratio of the transformer is 2:1.  Calculate:

    - The secondary current supplied to the load

[2 Marks]

VS=n×VP

VS=2×110=220 V

I=PV=200220=0.909 A

    - The primary current taken from the supply

[3 Marks]

IP=n×IS

IP=2×0.909=1.818 A

Oscilloscope Settings:

20 mV/div

5 ms/div

DC Coupled

Centre = 0V

Fig Q1b

For the waveform shown in Fig Q1b:

    - Estimate the period of the waveform.

[1 mark]

5 ms ×4 div=20 ms

    - From the estimated period, determine its frequency.

[1 mark]

f=1T=120ms=50 Hz

    - Determine its peak-peak amplitude

[1 mark]

Vpk-pk=2 div ×20 mV=40 mV

    - Determine the RMS value of the waveform.

[2 marks]

Mark space ratio is 50% therefore RMS value of waveform will be:

50%×40 mV=20 mV

  - Explain the following terms:

    - Resistance

Opposition to flow of current – DC circuit property

    - Reactance

Opposition to CHANGE of flow of current – AC circuit property

    - Impedance

Vector sum of resistance and reactance – Describes total opposition to current – AC and DC circuit property

[3 Marks]

  - Convert the following into polar form, using any method.

    - 6 – j6

6-j6=8.485 ∠-45°

    - 4 + j3

4+j3=5 ∠ 30.96°

[2 Marks]

  - Convert the following into rectangular form, using any method.

    - 5∠53.13°

5 ∠ 53.13°=3+j4

    - 25∠-25°

25 ∠-25°=22.658-j10.565

[2 Marks]

  - Determine the total impedance of the following components connected in series:

    - A 10Ω resistor, +j6Ω inductor and a -j10Ω capacitor.

[1 Mark]

10+j6-j10=10-j4 Ω

    - A 15Ω resistor, +j10Ω inductor and a -j10Ω capacitor.

[1 Mark]

15+j10-j10=15 Ω

  - Calculate the resonant frequency of a series connected RLC circuit consisting of a 120Ω resistor, a 100mH inductor and a 1µF capacitor.

[2 Marks]

fr=12πLC=12π100×10-3×1×10-6=503.292 Hz

  - Calculate the magnitude of the current flowing in the series connected RLC circuit described in part e, when it is connected to a 15V 440Hz power supply.

XC=12πfC

XC=12×π×440×1×10-6=397.887 Ω

XL=2πfL

XL=2×π×440×100×10-3=276.460 Ω

Z=R2+XL-XC2

Z=1202+276.460-397.8872=170.718 Ω

I=VZ=15170.718=87.864×10-3 A

[4 Marks]

Diwedd/End