Lab – Understanding Ethernet Cable Standards

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC-A | NIC | 192.168.10.1 | 255.255.255.0 | N/A |
| PC-B | NIC | 192.168.10.2 | 255.255.255.0 | N/A |

Objectives

Part 1: Analyze Ethernet Cabling Standards and Pinouts

Part 2: Build an Ethernet Crossover Cable

Part 3: Test an Ethernet Crossover Cable

Background / Scenario

In this lab, you will build and terminate an Ethernet crossover cable and test it by connecting two PCs together and pinging between them. You will first analyze the Telecommunications Industry Association/Electronic Industries Association (TIA/EIA) 568-A and 568-B standards and how they apply to Ethernet cables. You will then construct an Ethernet crossover cable and test it. Finally, you will use the cable you just constructed to connect two PCs together and test it by pinging between them.

Note: With autosensing capabilities available on many devices, such as the Cisco 1941 Integrated Services Router (ISR) switch, you may see straight-through cables connecting like devices.

Required Resources

One length of cable, either Category 5 or 5e. Cable length should be 0.6 to 0.9m (2 to 3 ft.)

2 RJ-45 connectors

RJ-45 crimping tool

Wire cutter

Wire stripper

Ethernet cable tester (optional)

2 PCs (Windows 7 or 8)

Analyze Ethernet Cabling Standards and Pinouts

The TIA/EIA has specified unshielded twisted pair (UTP) cabling standards for use in LAN cabling environments. TIA/EIA 568-A and 568-B stipulates the commercial cabling standards for LAN installations; these are the standards most commonly used in LAN cabling for organizations and they determine which color wire is used on each pin.

With a crossover cable, the second and third pairs on the RJ-45 connector at one end of the cable are reversed at the other end, which reverses the send and receive pairs. The cable pinouts are the 568-A standard on one end and the 568-B standard on the other end. Crossover cables are normally used to connect hubs to hubs or switches to switches, but they can also be used to directly connect two hosts to create a simple network.

Reflection:

Write a research report (Max word count 600) on Cat 5e and 6 Ethernet cable standards.

A category 5e Ethernet standard can deliver speeds from 10, 100 to 1000Mbps depending on the capability of the device’s network standard. The category 5e has a cable limitation of 100 meters. Although, if the cable was running at the max speed then the cable has a restriction of 50 meters.

The category 5e Ethernet standard improves network transfer speeds and signal strength compared to the other Ethernet standards by using a feature called crosstalk. Crosstalk is simply done by crossing the negative and positive channels to reduce interference within the Ethernet cable. Interference causes poor quality when data is sent, so therefore reducing the interference, it improves the service and speeds of the cable.

Cat 5e - https://www.comms-express.com/infozone/article/cat5e-max-speed-cat5e-cable-speed/

A category 6 Ethernet cable standard is compatible with category 5e cables. The standard is capable of speeds up to 10Gbps at a bandwidth capacity of 250MHz. Compared to the category 5e, which has a max speed of 1000Mbps. If a device was only capable of 1050Mbps then a category 5e cable would still be suitable for the task at hand. The category 6 Ethernet standard is designed for networks that run past 1Gbps. Although, by using a category 6 Ethernet cable standard for slower speeds than 1Gbps wouldn’t make the network slower, and would offer more length in cable size. The network quality will be much higher. However, the category 6 Ethernet standard is more costly compared to the category 5e Ethernet standard.

Cat 6 - https://www.xxpert.com/cat5-vs-cat6-ethernet-cables-and-why-you-should-care/

Analyze diagrams and tables for the TIA/EIA 568-A standard Ethernet cable.

The following table and diagrams display the color scheme and pinouts, as well as the function of the four pairs of wires used for the 568-A standard.

Note: In LAN installations using 100Base-T (100 Mb/s), only two pairs out of the four are used.

568-A 10/100/1000Base-TX Ethernet

| Pin Number | Pair Number | Wire Color | 10Base-T Signal 100Base-TX Signal | 1000Base-T Signal |
| --- | --- | --- | --- | --- |
| 1 | 2 | White/Green | Transmit | BI_DA+ |
| 2 | 2 | Green | Transmit | BI_DA- |
| 3 | 3 | White/Orange | Receive | BI_DB+ |
| 4 | 1 | Blue | Not Used | BI_DC+ |
| 5 | 1 | White/Blue | Not Used | BI_DC- |
| 6 | 3 | Orange | Receive | BI_DB- |
| 7 | 4 | White/Brown | Not Used | BI_DD+ |
| 8 | 4 | Brown | Not Used | BI_DD- |

The following diagrams display how the wire color and pinouts align with an RJ-45 jack for the 568-A standard.

Analyze diagrams and tables for the TIA/EIA 568-B standard Ethernet cable.

The following table and diagram display the color scheme and pinouts for the 568-B standard.

568-B 10/100/1000-BaseTX Ethernet

| Pin Number | Pair Number | Wire Color | 10Base-T Signal 100Base-TX Signal | 1000Base-T Signal |
| --- | --- | --- | --- | --- |
| 1 | 2 | White/Orange | Transmit | BI_DA+ |
| 2 | 2 | Orange | Transmit | BI_DA- |
| 3 | 3 | White/Green | Receive | BI_DB+ |
| 4 | 1 | Blue | Not Used | BI_DC+ |
| 5 | 1 | White/Blue | Not Used | BI_DC- |
| 6 | 3 | Green | Receive | BI_DB- |
| 7 | 4 | White/Brown | Not Used | BI_DD+ |
| 8 | 4 | Brown | Not Used | BI_DD- |

Build an Ethernet Crossover Cable

A crossover cable has the second and third pairs on the RJ-45 connector at one end, reversed at the other end (refer to the table in Part 1, Step 2). The cable pinouts are the 568-A standard on one end and the 568-B standard on the other end. The two following diagrams illustrate this concept.

Build and terminate a TIA/EIA 568-A cable end.

Determine the cable length required. (Your instructor will let you know the cable length you should make.)

Note: If you were making a cable in a production environment, the general guideline is to add another 12 in. (30.48 cm) to the length.

Cut off a piece of cable to the desired length and using your wire stripper, remove 5.08 cm (2 in.) of the cable jacket from both ends.

Hold the four pairs of twisted cables tightly where the jacket was cut away. Reorganize the cable pairs into the order of the 568-A wiring standard. Refer to the diagrams, if necessary. Take as much care as possible to maintain the twists in the cable; this provides noise cancellation.

Flatten, straighten, and line up the wires using your thumb and forefinger.

Ensure that the cable wires are still in the correct order for the 568-A standard. Using your wire cutters, trim the four pairs in a straight line to within 1.25 to 1.9 cm (1/2 to 3/4 in.).

Place an RJ-45 connector on the end of your cable, with the prong on the underside pointing downward. Firmly insert the wires into the RJ-45 connector. All wires should be seen at the end of the connector in their proper positions. If the wires are not extending to the end of the connector, take the cable out, rearrange the wires as necessary, and reinsert the wires back into the RJ-45 connector.

If everything is correct, insert the RJ-45 connector with cable into the crimper. Crimp down hard enough to force the contacts on the RJ-45 connector through the insulation on the wires, thus completing the conducting path. See the following diagram for an example.

Build and terminate a TIA/EIA 568-B cable end.

Repeat steps 1a to 1g using the 568-B color wiring scheme for the other end.

Test an Ethernet Crossover Cable

Test the cable.

Many cable testers will test for length and mapping of wires. If the cable tester has a wire map feature, it verifies which pins on one end of the cable are connected to which pins on the other end.

If your instructor has a cable tester, test the crossover cable for functionality. If it fails, check with your instructor first as to whether you should re-cable the ends and re-test.

Connect two PCs together via NICs using your Ethernet crossover cable.

Working with a lab partner, set your PC to one of the IP addresses shown in the Addressing Table (see page 1). For example, if your PC is PC-A, your IP address should be set to 192.168.10.1 with a 24-bit subnet mask. You partner’s IP address should be 192.168.10.2. The default gateway address can be left empty.

Using the crossover cable you made, connect the two PCs together via their NICs.

On the PC-A command prompt, ping the PC-B IP address.

Note: The Windows firewall may have to be temporarily disabled for pings to be successful. If the firewall is disabled, make sure you re-enable it at the conclusion of this lab.

Repeat the process and ping from PC-B to PC-A.

Assuming IP addressing and firewall are not issues, your pings should be successful if the cables were properly made.

Reflection

Which part of making cables did you find the most difficult?

The first time I aligned the wires correctly and pushed them in easily with no problems. I crimped it but not hard enough. This resulted all my hard work down the drain as the ends pulled out of the connector. After doing this process I found it hard to put the ends inside the new connector. I completed the task by shortening the cable again and managing to get it in the new connector, which proved as a chore.

Why do you have to learn how to make a cable if you can easily buy pre-made cables?

It reduces cost and gives size customization. Companies would possibly ask networking engineers to design cables to fit to their specifications.