| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 5 | LEFEL 5 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| JANuary 2021 | IONAWR 2021 |

MODULE CODE: ACCA5024

COD Y MODWL: ACCA5024

ADVANCED NETWORKING

TIME:  2 HOURS

Name			:

Student ID		:

Date & Time		:

This is an individual practical test, held under examination conditions. You will be assigned three Switches and three Routers; you should configure these devices based on the given specifications.

This paper is marked out of 100.

Topology

Addressing Table: IPv4 (5 marks)

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | Gi0/0 |  |  | N/A |
|  | S0/0/0 |  |  | N/A |
|  | S0/0/1 |  |  | N/A |
| R2 | S0/0/0 |  |  | N/A |
|  | Gi0/0 |  |  | N/A |
|  | Lo1 |  |  | N/A |
|  | Lo2 |  |  | N/A |
| R3 | S0/0/1 |  |  | N/A |
|  | Lo209 |  |  | N/A |
| PC1 |  |  |  |  |
| PC2 |  |  |  |  |

- Addressing Table: IPv6 (5 marks)

| Device | Interface | IPv6 Address | Link-local | Gateway |
| --- | --- | --- | --- | --- |
| R1 | Gi0/0 |  |  | N/A |
|  | S0/0/0 |  |  | N/A |
|  | S0/0/1 |  |  | N/A |
| R2 | S0/0/0 |  |  | N/A |
|  | Gi0/0 |  |  | N/A |
|  | Lo60 |  |  | N/A |
|  | Lo61 |  |  | N/A |
| R3 | S0/0/1 |  |  | N/A |
|  | Lo209 |  |  | N/A |
| PC1 |  |  |  |  |
| PC2 |  |  |  |  |

Task 1: - Topology Creation (5 marks)

Using Packet Tracer software create the topology shown above and complete all tasks below.

Task 2: - Device Configuration (75 marks)

- On all devices,

  - Configure hostname on all devices

  - Set privilege EXEC password

  - Configure line console password

  - Set logging synchronous

  - Configure line VTY password

  - Disable DNS lookup

  - Set password encryption

  - Set banner “Authorized Users Only!”

- On all devices, configure interfaces with IPv4 and v6 addresses as shown in the topology

- On R1, configure IPv4 static route to R2 and R3 networks, excluding Lo209 network.

- On R2, configure IPv4 static route to R1 and R3 networks, excluding Lo209 network.

- On R3, configure IPv4 static router to R1 and R2 networks.

- Configure default static route to reach Lo209 network using next-hop ip address.

- Set the bandwidth on serial links to 512kbps.

- Set the clock rate on DCE to be 64000

- On R1, configure IPv6 static route to R2 and R3 networks, excluding Lo209 network.

- On R2, configure IPv6 static route to R1 and R3 networks, excluding Lo209 network.

- On R3, configure IPv6 static router to R1 and R2 networks.

- Configure IPv6 default static route to reach Lo209 network using next-hop ip address.

- Using 1024 bit RSA keys configure SSH access on R1. Use domain soac.com, username: admin and password: Sw@ns3a.

Task3: Network connectivity (10 marks)

Using ping command, verify end-to-end connectivity for both IPv4 and v6 networks:

From PC1, ping PC2. Is it successful? ___yes/no_____

From PC1, ping Lo209. Is it successful? ___yes/no_____

From PC2, ping Lo209. Is it successful? ___yes/no_____

Verify SSH connectivity:

From PC1, establish telnet connection to R1. Is it successful? __yes/no____(this should fail)

From PC1, establish SSH connection to R1. Is it successful? ___yes/no____

Submission Requirement

The packet tracer file should be saved as “studentname_id” and uploaded onto Moodle. In addition to packet tracer file, you must also submit

- the output of show running-configuration command in a text file for all devices configured.

- This document with Task 3 completed

All documentation for your solution should be submitted electronically on Moodle.

It is your responsibility that you submit all the outputs and files correctly.