Lonyé Sarah Browne (2114209)

UWTSD

ACBA4001

ASSessment 1 L4

Table of Contents

Site details                                                                                                                  2

Site History                                                                                                                3

Environmental Appraisal                                                                                           6

Archaeological Appraisal                                                                                          7

Geological Appraisal                                                                                                 8

Recommendations for Specialist Geotechnical Contractor                                       8

Conceptual Ground Model                                                                                        9

Foundation Sketch                                                                                                   10

Reference List                                                                                                          11

Site Details

The site is located in Swansea very close to the shoreline, the coordinates are “51.613749, -3.9451024”. It is located between Swansea Mariott Hotel and Swansea City & Council & Library.

The site is an open, flat, paved car park of approximately 1600m2 in area and is very easily accessible from the main road.

The site is currently utilized as a car park for the hotel and persons going to the beach and as an accessway/footpath to the beach.

The proposed use of the site is a three-storey building that would consist of a restaurant, café bar, ice-cream parlour, kitchen, and food stores. On the second floor, there would be a self-contained manager’s apartment and staff welfare facilities. The building is to have a Gross Internal Area of 300m2 at the ground floor.

Aerial view of the site and the area(m2) of the site.

Google Maps

Site History

1870 - The site itself was an open pasture near an active railway. The surrounding area had a school and a hospital for contagious diseases, a hotel and a sawmill.

The site in 1870, highlighted in the purple rectangle.

Digimaps Historic Roam

1890- Half of the site was constructed into a timber yard. In the surrounding areas, the hospital for contagious diseases had been constructed into an engine shed, and a timber yard was built between it and the school. Everything else remained.

The site in 1890, highlighted in the purple rectangle.

Digimaps Historic Roam

1910- The part of the site that was a timber yard was changed to a canister works. Everything in the surrounding areas remained the same.

The site in 1910, highlighted in the purple rectangle.

Digimaps Historic Roam

1950 - A paint factory was built near the site/canister works to the top left and a timber yard was built near the bottom right. Repairs were being done on the railway. The school was no longer, but the engine shed and sawmill remained.

The site in 1950, highlighted in the purple rectangle.

Digimaps Historic Roam

1970- The site was now a warehouse with the paint factory remaining nearby, but not the timber yard. The engine shad was demolished and the railway dismantled.

The site in 1970, highlighted in purple.

Digimaps Historic Roam

1990- The site was constructed into the parking lot with the Mariott hotel to the right. The City Council and library were constructed in the space the engine shed used to be.

The site in 1990, highlighted in purple.

Digimaps Historic Roam

Environmental Appraisal

Land and soil become contaminated by past or current activities, such as industrial activity, poor waste management, spills, heavy metals and chemicals. It is important to investigate soil contamination when constructing as there is a possibility of harming the environment through groundwater flow and reintroducing buried substances into the environment due to excavations.

The Mumbles Railway- Steam trains were used in the 1900s, they were fueled using coal. The coal was burned as a means of creating heat for the water to produce steam. Coal causes pollution or contamination of land/soil through acid rain. The surrounding area of a railway also has a high concentration of heavy metals. Creosote is used to treat the railroad ties and it takes 30-100 years to fully decompose, it is incredibly harmful to the environment. The surrounding area of a railway can be potentially contaminated with hydrocarbons(coal), coolants, solvents and asbestos from ash.

The Engine Shed- This is where the trains were refuelled (with coal), greased/lubricated and generally maintained. The sheds were used to store the coal and were also the disposal site for the ash. Improper disposal of the waste (ash) would lead to soil contamination as coal ash has a high content of arsenal and other metals. Air pollution is also very likely if the ash was not properly contained to prevent spreading my wind.

The Paint Factory- Paints contain numerous chemicals that are toxic to the environment such as solvents, monomers, softening agents and biocides. It is more than likely that lead paint was the type of paint being manufactured and used in the 1950s. Lead is a heavy metal that is very toxic to the environment and can exist as dust in the environment for as long as forever, from its initial introduction. Chipping of paint from walls contribute to land and/or soil contamination and the lead piles up in dangerous concentrations. Poor waste management of a paint factory is very capable of contaminating the soil in much more dangerous concentrations than paint chipping. A paint factory also greatly contributes to air pollution, which can cause land pollution through precipitation.

Hydrology and Hydrogeology- The site is just 100m from the sea/course and approximately 1000m from River Tawe. Swansea has a moderately productive aquifer which means that there is a reasonable amount of groundwater. There is a possibility of flooding as construction can stop the normal flow of groundwater.

Map describing the Swansea aquifer and key.

BGS Maps

Potential contamination/pollution from the work to be done on the site itself- Like all construction sites, pollution is inevitable. This site will cause noise pollution to the nearby businesses such as City Council & Library and the Mariott Hotel. Air pollution will arise as a result of all the dust and sands. Water pollution because of dust being carried to the nearby water bodies as well as contaminating the groundwater, which then flows out into the sea which can affect marine life. The construction at the site may also cause traffic or parking issues in the area as the space is currently utilized by the hotel and it would not be available for use once construction has begun.

Archaeological Appraisal

I think the likelihood of locating archaeology below the site is very low as none of the past uses; a timber yard and a canister works would be able to suggest anything about the past life. The fact that the site is currently paved also reduces the risk of finding anything now- meaning, if there was archaeology it would have already been discovered. Another contributing factor to the low likelihood of finding archaeology is that everywhere else around the site has already been built up and foundations were dug and there are no reports that support the findings of archaeology.

Geological Appraisal

The site consists of made ground (ashes, sand, bricks and some clays) up to 5m deep. The layer up to 7m is very dense sand gravel with traces of sandy clay and it is 2m thick. At almost 8m there is dense sand with some gravel and it is 0.35m thick. The next layer ends at 11m and is a gravelly, boulder clay that is 3m thick. The subsequent layer ends at 18.6m deep and is a very stiff boulder clay (grey) and it is 7.60m thick. The final rock is grey mudstone that is 3.40m thick and ends the borehole at 22m deep.

There is no mining in the area and there are no voids in the borehole. The stratification of the rock is very low and materials vary most where there is made ground.

The risk of subsidence is low though the very last soil is weak, there is a very thick layer of stiff boulder clay. The stiff boulder clay is a very tough type of clay that completely lacks stratification and is said to present a few geotechnical problems.

Map showing the make-up of the site and key.

Digimaps Geology Roam

Recommendations for Specialist Geotechnical Contractor

For the detailed site investigation, I suggest that significant testing be done on the soil before excavations. This testing would be done for traces of heavy metals and creosote. This is a precautionary measure to reduce the possibility of re-polluting the land by digging up the heavy metals and creosote. Digging up these elements and substances can also have a negative effect on humans they are toxic to the body. It is also to prevent polluting the ground water, or just generally knowing how to move forward if the substances are still present in the soil.

There is 1-3% of ground gas present at the site. I suggest monitoring and testing what these gases are to see if it is necessary to take gas protection measures. Carbon dioxide and methane are ground gases, they move from the soil up and cause cracks and fissures in a structure. These gases are very harmful to human beings.

Map showing the radon potential and key.

BGS Maps

Before the excavation starts, I recommend properly investigating the groundwater flow and the water table level. This is to ensure that when the construction begins, it would not have a negative effect on the flow of the water such as stopping the flow and causing the water to accumulate which can lead to water seeping into the building and can also cause subsidence. Monitoring the water table level is important as well, to ensure that the level is not depleted which can cause irregular settlement of the soil which will affect the foundation.

These recommendations are by the codes of practice BS10175 for site investigations and BS8575 for ground gases.

Conceptual ground model

I have decided to go with friction piling foundation. It is a type of deep foundation and it is a vertical structure that is made of reinforced concrete or steel. Though the building will not be as big as the Mariott hotel or the City Council I think that it is more suitable for this site as the stronger rocker is very deep. I chose friction piles rather than end-bearing piles because of how deep the stiff boulder clay is- too deep for the end-bearing to transmit the load directly to it. The friction pile uses friction between the pile itself and the soil to transmit the load to the soil. Another reason I chose friction piling is because it lowers the effect of the bulb pressure. This is important as there are surrounding structures that are huge and it lowers the risks of the bulbs overlapping. When pressure bulbs overlap, it causes one of the bulbs to grow larger and extend further into the earth. When all of this pressure is exerted, it causes the buildings to lean.

Foundation Sketch

Reference List

https://www.google.com/maps/place/Swansea+Marriott+Hotel/@51.6136697,-3.9453683,192m/data=!3m1!1e3!4m16!1m7!3m6!1s0x0:0x8521ab875b731b23!2zNTHCsDM2JzQ5Lj

UiTiAzwrA1Nic0Mi40Ilc!3b1!8m2!3d51.6137491!4d-3.9451025!3m7!1s0x486e8ab49338a0b1:0x927675fd7a0f0c1b!5m2!4m1!1i2!8m2!3d51.613715!4d-3.944272

Digimaps- Historic Roam

Digimaps- Geology Roam

BGS http://mapapps.bgs.ac.uk/geologyofbritain/home.html

https://environmentlive.unep.org/landsoilpollution

https://www.socotec.co.uk/media/blog/contaminated-land-on-railway-sites

https://link.springer.com/chapter/10.1007/978-3-319-57496-7_6

https://www.michigan.gov/egle/0,9429,7-135-3307_29693_30031-90418--,00.html

https://www.architectureanddesign.com.au/suppliers/greenpainters/paint-industry-impacts-environment-greenpainters#

https://www.middletonrailway.org.uk/index.php/development-projects/build-a-running-shed

http://mapapps2.bgs.ac.uk/geoindex/home.html?layer=BGSHydroMap&_ga=2.157711420.709622637.1641418865-1115552953.1641186113

https://www.intertek.com/blog/2020-11-10-groundwater/

https://home.howstuffworks.com/real-estate/buying-home/how-does-ground-water-level-affect-development.htm

https://home.howstuffworks.com/real-estate/buying-home/how-does-ground-water-level-affect-development.htm#:~:text=Groundwater%20can%20be%20blocked%20from,the%20slab%20and%20cause%20dampness.

https://www.breckland.gov.uk/media/1258/Ground-Gas-Protection-Measures/pdf/ground_gas.pdf?m=636682028601300000