University of Wales Trinity Saint David

Wales Institute of Science & Art

Engine Performance

Characteristics

Motorcycle/Motorsport/vehicle Technology

(ACHA4010)

Kowsar Kashim

2304744

March 24th, 2024

Introduction

In this performance test we will be using the dyno to test the fuel consumption of the Honda engine when increasing different variables such as the under-load pressure and the revolutions per minute to test the engine to the desired parameters then after collecting that raw data we will put it into a spreadsheet and by using calculations we will find out other measurements that are not already being measured by the dyno such as the horsepower and brake horsepower, specific fuel consumption and even torque from the under-load pressure. After these results, they will be plotted on to appropriate graphs to see if there are any relationships between the specific data points noted down. An evaluation will then be done to see if their process could have been done more accurately and efficiently by us to see if any improvements are warranted or needed to be made to improve the overall quality of the performance test.

Equipment Introduction

In the background you can see a yellow box. Within that box is the supply box that has the main fuel pump which gives the engine the fuel needed to run the engine. The fuel comes from a fuel tank at the back of the room. The fuel is filtered through the supply box that is monitored by the fuel pump then sent straight to our engine

The engine is connected to a computer. Mainly the throttle so that we can increase rpm without needing to leave the computer.  The performance test was a Honda 1.6L v10 from roughly 2002 from an imported model car which are normally weaker. The sensors are a coolant meter to motor temperature, also a flow fuel meter and lastly a lambda sensor. Then finally the engine is connected to the dynameter.

In the background you can see a dynameter that's connected to the engine by the crank shaft. The dynameter is a shcienk ws230 with a 10,000 rpm and maximum load of 230KW. The only thing a dynameter gives us is a voltage rating between 0v to 10v but with the various sensors we can find out different measurements and speeds that we may need.

This system can create a lot of heat so as you can see in the bottom left corner there are two pipes which are linked to the coolant tank on the roof which maintains the dynameters temperatures with fans and radiators.

These are the screen sensors connected to the engine and dynameter they have a magnitude of numbers that are being calculated from the sensors and dynameter itself these numbers can the then be used to find out different conclusions and questions that could be asked such as how different fuels run within an engine or how different engines deal with loads at different speeds.

Graphs and Tables

At the beginning the engine uses a lot of fuel to start which is what occurs as the start of an engine cycle the atomizing the fuel with the air as the fuel needs to become vapor so that the engine can starts so a lot of fuel is added but as time goes on less fuel is needed to power the engine until it starts to stagnate towards the higher rpm’s.

The Torque in engine creates more power than if there was less torque this is very useful as when the engine needs to reach higher speeds more torque is needed and as the torque becomes too much the engine starts to stall as there isn't enough RPMs to keep the balance where if there's too much on either side the engine starts to stall as shown above .

In this graph it shows a more positive correlation in the positive direction as there is more RPMs there is a need for more brake horsepower is needed to slow down the engine an appropriate amount otherwise if there was a small amount of bhp the RPMs will decrease at a much slower rate than what would most likely be needed to. Also, towards the end of the graph the points deviate from the line of best fit and this is most likely down to the maximum brake power that engine can give. This is due to braking power could damage the car more than the brake power could give out.

Evaluation

All in all, the report offers a thorough summary of the dynamometer performance test performed on a Honda engine. It describes the procedures, tools, and methods for gathering and analyzing data. This is a review of the report:

Advantages:

Simple Methodology: The report provides a clear explanation of the procedures used to conduct the performance test, including the factors examined, the techniques used to collect data, and the analysis that came after.

Specific Tools Introduction: The reader is given a comprehensive grasp of the setup, including the engine, dynamometer, sensors, and cooling system, by the description of the equipment utilized. This improves transparency and makes it possible to repeat the experiment.

Graphs and Tables: The report's conclusions are made clearer by using graphs and tables to display the data. Understanding trends and the connections between variables is made easier with the aid of these graphic aids.

Safety Measures: Bringing up the cooling system for controlling heat produced during testing shows safety measures are considered, which is important in machinery-based experimental setups.

Opportunities for Development:

Data Interpretation: Although the report offers some interpretations of the data, more in-depth research and conversation are still necessary. It might examine the effects of the trends that have been noticed on engine efficiency, performance optimization, or comparison with theoretical models or industry standards, for instance.

Technical Details: Although the report gives readers a general understanding of the tools and configuration used, more technical information could help them understand the dynamometer's and sensors' operations as well as the calculations needed to determine metrics like horsepower and torque.

The report might benefit from some portions being organized or presented more clearly. To help the reader navigate the analysis, the explanation of the graphs and tables may use some additional organization and targeted headings or labels.

Suggestions for Improvement: Although the report notes that the procedure was evaluated to uncover possible enhancements, it might offer suggestions considering the results. This could involve recommendations for refining the equipment setup for further studies, improving data accuracy, or streamlining testing methods.

A succinct conclusion that highlights the major conclusions, their ramifications, and any suggestions for more study or testing would be beneficial to the paper.

In summary, although the report offers a strong basis for comprehending the performance test carried out on the Honda engine, applying the recommended modifications could increase its lucidity, comprehensiveness, and general caliber even more.